3GPP TSG-RAN5 Meeting #93-e	R5-217273r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2605 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SOR test case 6.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the premable there is no PDU Session established and UE is powered on in the test proceure so if 5GS registration type is set as Mobility Registration, there is no handling of PDU session establishment in that branch of test procedure. - Some conformance requirements are redundant |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified steps 26b3-26b5a1 to call the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with 'connected without release' and added new step 26b6 to handle UE requested PDU session establishments. - Removed the redundant conformance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-217273->R5-217273r1, removed the introduction of new steps 21Aa1 to 21Aa7, restored the conformance requirements, removed the redundant requirements |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.3	Steering of UE in roaming during registration/security check unsuccessful/Automatic mode

6.3.1.3.1	Test Purpose (TP)

(1)

with {UE being in automatic PLMN selection mode and current VPLMN not part of “PLMNs where registration was aborted due to SOR” List and not part of "User Controlled PLMN Selector with Access Technology" list and UE's USIM configured with indication that the UE is to receive Steering of Roaming information due to initial registration in a VPLMN}

ensure that {

when {Security check on the steering of roaming information SOR Transparent container is unsuccessful}

then {UE sends REGISTRATION COMPLETE message to the serving AMF without including an SOR transparent container and releases the current N1 NAS signalling connection locally and attempts to obtain service on a higher priority PLMN}

}

6.3.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause C.2. Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause C.2]

8)	If:

a)	the UE's USIM is configured with indication that the UE is to receive the steering of roaming information due to initial registration in a VPLMN, but neither the list of preferred PLMN/access technology combinations nor the secured packet nor the HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided' is received in the REGISTRATION ACCEPT message, when the UE performs initial registration in a VPLMN or if the steering of roaming information is received but the security check is not successful; and

b)	the current chosen VPLMN is not contained in the list of "PLMNs where registration was aborted due to SOR", not part of "User Controlled PLMN Selector with Access Technology" list, the UE is not in manual mode of operation and the PDU session for emergency services is not pending to be activated;

then the UE shall send the REGISTRATION COMPLETE message to the serving AMF without including an SOR transparent container, release the current N1 NAS signalling connection locally, store the PLMN identity in the list of "PLMNs where registration was aborted due to SOR" and attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired, with an exception that the current PLMN is considered as lowest priority, and skip steps 9 to 12;

NOTE 2:	When the UE is in the manual mode of operation or the current chosen VPLMN is part of the "User Controlled PLMN Selector with Access Technology" list, the UE stays on the VPLMN.

6.3.1.3.3	Test Description

6.3.1.3.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.3.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.3.3.1-1. The MCC and MNC values corresponding to the PLMN identifier shall understood as those specified in TS 36.523-1 [42], Table 6.0.1-1.

Table 6.3.1.3.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 11 | PLMN2 |
| NR Cell 12 | PLMN13 |
| NR Cell 13 | PLMN14 |

NR Cell 11 is set to ''Serving Cell'';

NR Cell 12 is set to ''Serving Cell'';

NR Cell 13 is set to “Serving Cell'';

System Information Combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 of TS 38.508-1 [4] will be used.

Preamble:

-	The UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.3.1.3.4	Test procedure sequence

Table 6.3.1.3.4-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE | - | - | - | - |
| 2-14 | Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 | - | - | - | - |
| 15 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 16 | SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) | - | - | - | - |
| 17 | The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container | --> | NR RRC: ULInformationTransfer 5G MM: REGISTRATION COMPLETE | 1 | P |
| 18-21a1 | Void | - | - | - | - |
| 22 | The SS locally releases the RRC connection. | - | - | - | - |
| 23 | Check: Does the UE transmits an RRCSetupRequest message on NR Cell 12 before tmax expires? (Note 1, 2 and 3) | --> | NR RRC: RRCSetupRequest | 1 | P |
| 24 | The SS transmits an RRCSetup message on NR Cell 12. | <-- | NR RRC: RRCSetup | - | - |
| 25 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 12. (Note 4) | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| - | EXCEPTION: Steps 26a1 to 26b5a1 describe behaviours which depend on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 26a1-26a15a1 | IF 5GS registration type is set as Initial Registration in step 25, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 12. | - | - | - | - |
| 26b1-26b2 | Void | - | - | - | - |
| 26b3-26b5a1 | ELSE IF 5GS registration type is set as Mobility Registration in step 25, THEN steps 4 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with 'connected without release' are performed on NR Cell 12. | - | - | - | - |
| 26b6 | The UE performs the generic procedure for UE-requested PDU session(s) establishment, specified in TS 38.508-1 [4] subclause 4.5A.2, performing establishment of UE-requested PDU session(s). | - | - | - | - |
| 27-28a1 | Void. | - | - | - | - |
| Note 1: Timer tmax in step 16 and 23 are derived from the high priority PLMN search timer T defined by EFHPPLMN Note 2: Following attempts to access the HPLMN/EHPLMN/higher priority PLMN in VPLMN is operator specific setting (Refer to TS 23.122 Rel-12). Hence, window between 120s to T+Tolerance is being used , where the high priority PLMN search timer T defined by EFHPPLMN Note 3: Tolerance of 5min is added to allow time for the UE to find the proper PLMN Note 4: The 5GS registration type shall be only set as Mobility Registration for Rel-16 UEs according to TS 24.501 subclause 5.2.3.2.5 specified in Release 16. The EXCEPTION description applies only to Rel-15 UEs. |  |  |  |  |  |

6.3.1.3.5	Specific message contents

Table 6.3.1.3.5-1: REGISTRATION ACCEPT for NR Cell 13 (step 15, Table 6.3.1.3.4-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |
| SoR-MAC-IAUS | Set to mismatch the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 1 | The list type is a PLMN ID and access technology list |  |
| Acknowledgement (ACK) value | 1 | Acknowledgement requested |  |
| PLMN ID 1 | PLMN2 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |

Table 6.3.1.3.5-2: Void

Table 6.3.1.3.5-3: REGISTRATION COMPLETE for NR Cell 13 (step 17, Table 6.3.1.3.4-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Not Present | The SOR transparent container is not included in REGISTRATION COMPLETE message |  |

Table 6.3.1.3.5-4: REGISTRATION ACCEPT for NR Cell 13 (step 26a10, Table 6.3.1.3.4-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |
| SoR-MAC-IAUS | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 1 | The list type is a PLMN ID and access technology list |  |
| Acknowledgement (ACK) value | 1 | Acknowledgement requested |  |
| PLMN ID 1 | PLMN13 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |

Table 6.3.1.3.5-5: REGISTRATION COMPLETE for NR Cell 13 (step 26a11, Table 6.3.1.3.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |
| SOR data type | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |