3GPP TSG-RAN5 Meeting 93-e	R5-217566r2

# Electronic Meeting, 8th Nov – 19th  Nov 2021

Title:		RRM known issue #7 - Absolute thresholds / large UE gain range

Source:			Ericsson

Agenda Item:			5.3.1.17

Document for:			Endorsement

# 1.	Introduction

In this document we discuss the issue of large UE antenna gain range for test cases where UE reports absolute SS-RSRP and propose a UE preamble method to make the test more stringent.

# 2.	Discussion

As listed on the RRM issue list in [1] and outlined in [2], for certain test cases UE makes comparison based on reported SS-RSRP which is subjected to wide gain variations. What contributes to the total MU can be seen in below excerpt from 38.903 [6].

The biggest contributors to the total uncertainty is the reporting accuracy (up to ±8dB) and UE gain variation ±15dB for PC3 UE.

Moreover, in spherical coverage further variations needs to be considered (±5.45dB for n257).

As an example, for a UE power class 3 in band n257, with applied Io > -70dBm, the variation of SS-RSRP would be (±5.45dB ±15dB ±8dB) = ±28.45dB. This together with the Test system downlink power uncertainty of ±5.65dB, can add up to over ±34dB. It is possible that some test cases with very wide range of reported values may be considered irrelevant.

Example from TT analysis to TC 7.7.1.1 [7] where UE can report SS-RSRP between -106.03 dBm to -48.71 dBm:

Observation 1: Some test cases with very wide range of reported values may be considered irrelevant.

Observation 2: For test cases where UE needs to compare the signal level to absolute value UE gain variation of +/- 15 dB is largest contributor to the total measurement uncertainty.

Originally this issue was discussed in terms of testability of test cases with absolute thresholds comparison such as a4-threshold. As it was proven in TT analysis [3] and [4], no such issue exists. However, in our understanding it is still beneficial to introduce preamble method for RRM to obtain the UE antenna gain to lower the very wide range of reported values as for example in FR2 SSB-based L1-RSRP reporting cases where, as showed in [5] the reported value of L1-RSRP spans almost 60 dB.

Thus, it is proposed to introduce such preamble method for RRM into 38.508-1 clause 7.1.3

For simplicity the proposed method called preamble will be limited to SS-RSRP and L1-RSRP absolute measurements in single AoA from Rx Beam peak direction (AoA Setup 1). The method can later be extended to cover other AoA setups. In other words, measurement performance test cases 5.7.1.x, 7.7.1.x, 8.5.2.1.2 are excluded from the discussion as of now.

Proposal 1: Introduce preamble method for RRM AoA Setup 1 into 38.508-1 clause 7.1.3 applicable for test cases other than measurement performance test cases 5.7.1.x, 7.7.1.x, 8.5.2.1.2.

As there already is calibration method used for signalling test cases in 38.508-1 clause 6.1.3, we propose to base the method for RRM on the method used for signalling. The specific implementation of the procedure needs to be verified separately for each test case and needs to be done in TS 38.533 and/or test tolerance analysis in 38.903.

Proposal 2: Base the method for RRM on the method used for signalling in 38.508-1 clause 6.1.3.

Proposal 3: Procedure to be implemented on a case-by-case basis in 38.533 and/or TT analysis in 38.903

For relative SS-RSRP measurements where signals are coming from the same directions, UE gain is considered to cancel out, so no preamble is necessary. However, for relative SS-RSRP measurements where signals are coming different directions, UE gain is not considered to be canceling out. The preamble method for that scenario is not the part of this discussion paper and is FFS.

Observation 3: The preamble method for relative SS-RSRP measurements where signals are coming different directions is FFS.

The list of absolute RSRP measurement test cases with AoA setup 1 consists of 25 items.

| Test Case | Title | AoA |
| --- | --- | --- |
| EN-DC Test Cases |  |  |
|  | Measurement procedures |  |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | Setup 1 |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | Setup 1 |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Setup 1 |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Setup 1 |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Setup 1 |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Setup 1 |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Setup 1 |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Setup 1 |
| SA Test Cases |  |  |
|  | Handover |  |
| 7.3.1.1 | NR SA FR1-FR2 handover with unknown target cell | Setup 1 |
| 7.3.1.2 | NR SA FR2 handover with unknown target cell | Setup 1 |
| 7.3.1.3 | NR SA FR2-FR2 handover with unknown target cell | Setup 1 |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | Setup 1 |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | Setup 1 |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | Setup 1 |
|  | Measurement procedures |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | Setup 1 |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | Setup 1 |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Setup 1 |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Setup 1 |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Setup 1 |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | Setup 1 |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Setup 1 |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Setup 1 |
|  | Inter-RAT measurements |  |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | Setup 1 |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | Setup 1 |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | Setup 1 |

Corresponding 38.508-1 CR in [8]. New clause proposed in 38.508-1:

# 3.	Conclusion

Observation 1: Some cases with very wide range of reported values may be considered irrelevant.

Observation 2: For test cases where UE needs to compare the signal level to absolute value UE gain variation of +/- 15 dB is largest contributor to the total measurement uncertainty.

Observation 3: The preamble method for relative SS-RSRP measurements where signals are coming different directions is FFS.

Proposal 1: Introduce preamble method for RRM AoA Setup 1 into 38.508-1 clause 7.1.3.

Proposal 2: Base the method for RRM on the method used for signalling in 38.508-1 clause 6.1.3.

Proposal 3: Procedure to be implemented on a case-by-case basis in 38.533 and/or TT analysis in 38.903

# 4.	References

[1] R5-215410, “FR2 RRM test cases: Known Issue List - after RAN5_92e”, Ericsson, RAN5#92e

[2] R5-212634, “FR2 RRM test cases: Known Issues ”, Anritsu, RAN5#89e

[3] R5-213943, Test Tolerance analysis for FR2 event triggered reporting test cases, Ericsson, RAN5#91e

[4] R5-213944, Test Tolerance analysis for FR2 event triggered reporting test cases, Ericsson, RAN5#91e

[5] R5-216363, Test Tolerance analysis for FR2 SSB-based L1-RSRP measurement for beam reporting test cases, Ericsson, RAN5#92e

[6] 38.903, NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases

[7] R5-213940, Update TT analysis for 5.7.1.1, ROHDE & SCHWARZ, RAN5#91

[8] R5-217567, Procedure for UE preamble for RRM, Ericsson, RAN5#93e