3GPP TSG-RAN5 Meeting #93-e                      	R5-217571r1

# Electronic Meeting, 8th Nov – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1562 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RRM HST E-UTRA NR FR1 Cell reselection test case 8.2.1.2 including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM test case 8.2.1.2 is incomplete in 38.533 |  |  |  |  |  |  |  |  |
| R4- |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM test case 8.2.1.2 has been corrected, following has been added/corrected: Test tolerance has been applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 8.2.1.2 will remain incomplete in 38.533 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependant R1: Corrected signal levels in Table 8.2.1.2.5-1 and Table 8.2.1.2.5-2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

#### 8.2.1.2	E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

8.2.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in TS 36.133 [23] clause 4.2.2.5.6.

8.2.1.2.2	Test applicability

This test applies to all types of NR UE configured with highSpeedInterRAT-NR-r16 from Release 16 onwards.

8.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.1.2.

8.2.1.2.4	Test description

8.2.1.2.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.1.2.4.1-1.

Table 8.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.2.1.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.1.2.4.1-2.

Table 8.2.1.2.4.1-2: Initial conditions for E-UTRA – NR FR1 cell re-selection to lower priority NR target cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.2.1.2.4.1-3.

2.	Message contents are defined in clause 8.2.1.2.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1 and Cell 2 is configured according to Annex C.1.1 and C.1.2.

4. In SIB of the E-UTRA cell, highSpeedInterRAT-NR-r16 is configured and the carrier of NR cell is configured with highSpeedCarrierNR-r16.

Table 8.2.1.2.4.1-3: General test parameters for E-UTRA – NR FR1 cell re-selection to lower priority NR target cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 87 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

8.2.1.2.4.2	Test procedure

Two cells are deployed in the test, which are one E-UTRA Cell 1 and NR Cell 2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cells are already identified by the UE prior to the start of the test. NR Cell 2 is of lower priority than E-UTRA Cell 1. UE is camped on E-UTRA Cell 1 during the initial phase of the test.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure.

- Ensure the UE is in State RRC_IDLE with generic procedure parameters Connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5.

- Set the parameters according to T1 in Table 8.2.1.2.5-1 and 8.2.1.2.5-2, T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority NR Cell 2.

4. The SS shall switch the power setting from T1 to T2 as specified in Table Table 8.2.1.2.5-1 and 8.2.1.2.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority E-UTRA Cell 1.

6. If the UE responds on lower priority cell, NR Cell 2 during time duration T2 within 3 seconds from the beginning of time period T2, then count a success for the event “Re-select lowe priority Cell 2”. Otherwise count a fail for the event “Re-select lower priority Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

8. 	Switch off and on the UE and ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 2.

9. Repeat step 2-9 until a test verdict has been achieved.
Each of the events “Re-select higher priority Cell 1” and “Re-select lower priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

8.2.1.2.4.3	Message contents

Message contents are same as in 8.2.1.1.4.3 with the following exceptions:

Table 8.2.1.2.4.3-1: highSpeedCarrierNR-r16 in SystemInformationBlockType24

| Derivation Path: TS 36.508, Table 4.4.3.3-20 with Condition FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| CarrierFreqNR-v1610 ::= SEQUENCE {(SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| highSpeedCarrierNR-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.1.2.4.3-2: highSpeedInterRAT-NR-r16 configuration

| Derivation Path: 36.508 Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB::= SEQUENCE { |  |  |  |
| highSpeedInterRAT-NR-r16 | TRUE |  |  |
| } |  |  |  |

Table 8.2.1.2.4.3-3: SIB2

| Derivation Path: TS 38.508-1, Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not sent |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.1.2.5	Test requirement

Tables 8.2.1.2.4.1-3, 8.2.1.2.5-1 and 8.2.1.2.5-2 define the primary level settings including test tolerances for higher priority E-UTRA cell re-selection test case.

Table 8.2.1.2.5-1: NR Cell specific test parameters for E-UTRA – NR FR1 cell re-selection to lower priority NR target cell for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |
|  | dB | 1, 4 | 17.45 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Note2 | dBm/SCS | 1, 4 | -99.9 |  |
|  |  | 2, 5 | -99.9 |  |
|  |  | 3, 6 | -96.9 |  |
| Note2 | dBm/15 kHz | 1, 4 | -99.9 |  |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | 17.45 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -82.45 | -85.90 |
|  |  | 2, 5 | -82.45 | -85.90 |
|  |  | 3, 6 | -79.45 | -82.90 |
| Io | dBm/9.36 MHz | 1, 4 | -70.40 | -56.38 |
|  | dBm/9.36 MHz | 2, 5 | -70.40 | -56.38 |
|  | dBm/38.16 MHz | 3, 6 | -64.30 | -50.30 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6 | Not sent |  |
| Threshx, high | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, low | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, low | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN 3334 Note 4 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |

Table 8.2.1.2.5-2: E-UTRA Cell specific test parameters for for E-UTRA – NR FR1 cell re-selection to lower priority NR target cell for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
| Note 2 | dBm/15 kHz | -99.9 |  |
| RSRP Note 3 | dBm/15 KHz | -103.55 | -84.45 |
|  | dB | -3.65 | 15.45 |
|  | dB | -3.65 | 15.45 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | 50 |  |
| Threshx, high | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN 1944 Hz Note4 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |

The cell reselection delay to a lower priority NR cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Tevaluate, NR	See Table 4.2.2.5.6-2 in clause 4.2.2.5.6 in [15]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 2.24 s, allow 3 s for the cell re-selection delay to a lower priority NR cell.

///End of changes