3GPP TSG-RAN5 Meeting #93-e	R5-217596r2

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5068 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NB-IoT / Cell reselection / MFBI for NS_04 NB-IoT in North America |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA Inc., Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correction to test points for NB-IoT. RAN4 has updated 36.101 Table 5.5F-1 E-UTRA operating bands for NB-IoT in North America based on feedback from the FCC in R4-2107330. R4-2111486 (RAN4#99e, May ’21) introduced NS_04 to discriminate between frequency band limits in Table 5.5-1 and Table 5.5F-1 E-UTRA operating bands for NB-IoT in North America. Test points for NB-IoT were updated in R5-215823, using Network Signaling to identify when specific test points are used to meet regulatory rquirements in North America. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NB-IoT / Cell reselection / MFBI updated to allow Network Signaling NS_04. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test points are used for NB-IoT in North America. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.10.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: updated coversheet to include source to TSG r2: added Table 22.2.10.3.3-0 and the defined the condition when NS_04 applies |  |  |  |  |  |  |  |  |

<<< skip unchanged sections >>>

### 22.2.10	NB-IoT / Cell reselection / MFBI

22.2.10.1	Test Purpose (TP)

(1)

with { UE in  RRC_IDLE state }

ensure that {

when { an equal priority Intra-frequency neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { the UE reselects the new cell }

}

(2)

with { UE in  RRC_IDLE state }

ensure that {

when { an equal priority Inter-frequency neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { the UE reselects the new cell }

}

22.2.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.2a and 5.2.4.6, TS 36.331, clause 5.2.2.7, 5.2.2.12 and 6.7.3.1.

[TS 36.304, clause 5.2.4.2a]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP, the UE may choose not to perform intra-frequency measurements

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NB-IoT inter-frequencies which are indicated in system information:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP, the UE may choose not to perform inter-frequency measurements.

-	Otherwise, the UE shall perform inter-frequency measurements.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Except for NB-IoT, equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. For NB-IoT equals to QoffsetDedicatedfrequency for any frequency other than the frequency of the dedicated frequency offset, if QoffsetDedicatedfrequency is valid, otherwise this equals to Qoffsetfrequency (if QoffsetDedicatedfrequency is valid Qoffsetfrequency is not used). |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] |
| QoffsetSCPTM | Offset temporarily applied to an SC-PTM frequency as specified below. The offset is applied to all cells on the SC-PTM frequency. If QoffsetSCPTM is valid, Qoffset for inter-frequency neighbour cells is not used. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2 (5.2.3.2a for NB-IoT), but may exclude all CSG cells that are known by the UE not to be CSG member cells.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

1>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

1>	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	if attachWithoutPDN-Connectivity is received for the selected PLMN:

3>	forward the attachWithoutPDN-Connectivity to upper layers;

2>	else

3>	indicate to upper layers that attachWithoutPDN-Connectivity is not present;

2>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

2>	else:

3>	apply the additionalSpectrumEmission in SystemInformationBlockType2-NB and the p-Max;

1>	else:

2>	consider the cell as barred in accordance with TS 36.304 [4]; and

2>	perform barring as if intraFreqReselection is set to notAllowed.

[TS 36.331, clause 5.2.2.12]

Upon receiving SystemInformationBlockType5-NB, the UE shall:

1>	if in RRC_IDLE, or in RRC_CONNECTED while T311 is running:

2>	if, for the frequency band selected by the UE (from multiBandInfoList) to represent a non-serving NB-IoT carrier frequency, the freqBandInfo is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

2>	else:

3>	apply the p-Max;

[TS 36.331, clause 6.7.3.1]

SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

…

SystemInformationBlockType2-NB field descriptions

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1-NB, listed in the same order.

…

SystemInformationBlockType5-NB

The IE SystemInformationBlockType5-NB contains information relevant only for inter-frequency cell re-selection i.e. information about other NB-IoT frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

…

SystemInformationBlockType5-NB field descriptions

multiBandInfoList Indicates the list of frequency bands, with the associated additionalPmax and additionalSpectrumEmission values as defined in TS 36.101 [42, 6.2.4], in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common.

22.2.10.3	Test description

22.2.10.3.1	Pre-test conditions

System Simulator:

-	Ncell 1, Ncell 3, and Ncell 11 have different tracking areas according to TS 36.508 [18] Table 8.1.4.2-1A and are MFBI capable cells.

- 	Ncell 1 and Ncell 11 belong to the absolute centre frequency which overlaps between bands controlled by IXITs
px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	Ncell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs
px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	System information combination 3 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used in NB-IoT inter-frequency multi cell scenario.

UE:

-	UE does not support the px_OverlappingNotSupportedFrequencyBand_MFBI band.

Preamble:

-	The UE is in state Registered, Idle mode (State 3-NB) on Ncell 1 (serving cell) according to TS 36.508 [18] clause 8.1.5.1

22.2.10.3.2	Test procedure sequence

Table 22.2.10.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at
various time instants of the test execution. The configurations marked "T1" and "T2" is applied at the point indicated in the Main
behaviour description in Table 22.2.10.3.2-2.

Table 22.2.10.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 11 | Ncell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | NRS EPRE | dBm/15kHz | -85 | -79 | “Off” | The power level values are set so that Ncell 1 < Ncell 11. (NOTE 1) |
| T2 | NRS EPRE | dBm/15kHz | “Off” | -85 | -73 | The power level values are set so that Ncell 1 < Ncell 11. (NOTE 1) |
| Note 1: Power level “Off” is defined in TS 36.508 Table 8.3.2.2.1-1. |  |  |  |  |  |  |

Table 22.2.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second. (to ensure that 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 22.2.10.3.2- 1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on E-UTRAN Ncell 11? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 3 | The SS changes the cells power level setting according to the row "T2" in table 22.2.10.3.2- 1. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on E-UTRAN Ncell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 |  |

22.2.10.3.3	Specific message contents

Table 22.2.10.3.3-0: SystemInformationBlockType1-NB for Ncell 1, Ncell 3 and Ncell 11 (preamble and all steps,
Table 22.2.10.3.2-2)

| Condition | Explanation |
| --- | --- |
| NS_04 | When using the NS_04 frequency table variants in TS 36.508 subclause 8.1.3.1 |

Table 22.2.10.3.3-1: SystemInformationBlockType1-NB for Ncell 1, Ncell 3 and Ncell 11 (preamble and all steps,
Table 22.2.10.3.2-2)

| Derivation Path: 36.508 table 8.1.4.3.2.-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| freqBandIndicator-r13 | FreqBandIndicator-NB-r13 An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupp ortedFrequencyBandMF BI). | Ncell 1 Ncell 3 Ncell 11 |  |
| multiBandInfoList-r13 SEQUENCE { |  |  |  |
| freqBandIndicator –r13 | An overlapping Band under test (px_MFBI_FrequencyBan d). | Ncell 1 Ncell 3 Ncell 11 |  |
| freqBandInfo –r13 SEQUENCE{ |  |  |  |
| NS-PmaxList-NB-r13 SEQUENCE{ |  |  |  |
| additionalPmax-r13 | Not Present | Ncell 1 Ncell 3 Ncell 11 |  |
| additionalSpectrumEmission-r13 | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |  |
|  | 4 (NS_04) |  | NS_04 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.10.3.3-2: SystemInformationBlockType5-NB for Ncell 1, Ncell 3 and Ncell 11 (preamble)

| Derivation Path: 36.508 table 8.1.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5-NB-r13 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-r13 SEQUENCE { | The same number of entries as the configured inter-freq carriers. For Signalling test cases, see clause 8.3.2.3. | n denotes the index of the entry |  |
| dl-CarrierFreq-r13[n] | Downlink EARFCN under test |  |  |
| multiBandInfoList-r13 SEQUENCE { |  |  |  |
| freqBandIndicator –r13 | An overlapping Band under test (px_MFBI_FrequencyBand). | Ncell 1 Ncell 3 Ncell 11 |  |
| freqBandInfo –r13 SEQUENCE{ |  |  |  |
| NS-PmaxList-NB-r13 SEQUENCE{ |  |  |  |
| additionalPmax-r13 | Not Present | Ncell 1 Ncell 3 Ncell 11 |  |
| additionalSpectrumEmission-r13 | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |  |
|  | 4 (NS_04) |  | NS_04 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 22.2.11	Void

<<< skip unchanged sections >>>