Title: 	Discussion on test point selection for MPR, SEM and ACLR for E-UTRA-NR V2X inter operation

Source: 	Huawei

Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for MPR, SEM and ACLR for E-UTRA/NR Uu and E-UTRA/NR SL inter operation in 38.521-3.

Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

UE maximum output power for Intra-band V2X

For MPR, SEM and ACLR of intra-band V2X, no exception requirements apply to NR sidelink operation or E-UTRA V2X operation. Given only single switched SL is supported for all intra-band configurations, the requirements can be well covered in E-UTRA SL only test casee of TS 36.521-1 and NR SL only test case of TS 38.521-1 and don’t need to be tested again

Proposal 1: No test point is needed for intra-band V2X.

UE maximum output power for Inter-band concurrent V2X

For inter-band concurrent V2X configurations, concurrent operation is allowed. The analysis in this clause applies to both inter-band E-UTRA Uu + NR sidelink and NR Uu + E-UTRA V2X configurations.

    - Test Environment

The test environment of MPR/SEM/ACLR for Uu only or SL only test cases are selected as below table. The same environment selection can be adopted for V2X test cases.

Table 2.2.1-1 Test environment configuration summary

| Test Case | Test Environment |
| --- | --- |
| MPR | Normal, TL/VL, TL/VH, TH/VL and TH/VH |
| ACLR | Normal,TL/VL, TL/VH, TH/VL and TH/VH |
| SEM | NC |

Proposal 2: For inter-band concurrent V2X testing, define test environment as Normal, TL/VL, TL/VH, TH/VL and TH/VH for MPR and ACLR, and Normal for SEM.

    - Test Subcarrier Spacing

Test subcarrier spacing for NR Uu or NR SL is selected as 15kHz so that the time slot could be aligned with E-UTRA subframe.

Proposal 3: For inter-band concurrent V2X testing, select 15kHz SCS for NR Uu or NR SL carrier.

    - Test Frequencies

Test frequencies are selected as Low, Mid and High range for Uu only or SL only test cases. For the sake of saving test time, it could be reduced to Low and High for inter-band concurrent V2X requirements.

Proposal 4: For inter-band concurrent V2X testing, test Low range for each carrier and High range for each carrier.

    - Test Channel Bandwidth

Test channel bandwidth is selected as Lowest, Mid and Highest for Uu only or SL only test cases. For the sake of saving test time, it could be reduced to Lowest and Highest for each carrier for inter-band concurrent V2X requirements.

Proposal 5: For inter-band concurrent V2X testing, test Lowest channel bandwidth for each carrier and Highest channel bandwidth for each carrier.

    - Modulations and RB allocation

For each carrier, the combination of modulation and RB allocation shall be selected such that all MPR value could be covered. If multiple combinations require the same MPR value, only the highest modulation and largest RB allocation is selected.

For E-UTRA Uu and NR sidelink, the modulation and RB allocation are selected as below table.

Table 2.2.5-1 Test modulations and RB allocation configuration summary

| E-UTRA Uu carrier |  |  | NR sidelink Carrier |  |  |
| --- | --- | --- | --- | --- | --- |
| Modulation | PUSCH RB allocation (Note 1) | MPR | Modulation | PSCCH and PSSCH RB allocation (Note 2) | MPR |
| QPSK | Partial_Allocation | 0dB | CP-OFDM 16QAM | Inner_Full | 2.5dB |
| QPSK | Full_Allocation | 1dB | CP-OFDM 16QAM | Outer_Full | 4.5dB |
| 16QAM | Partial_Allocation | 1dB | CP-OFDM 64QAM | Outer_Full | 4.5dB |
| 16QAM | Full_Allocation | 2dB | CP-OFDM 256QAM | Outer_Full | 7.0dB |

For NR Uu and E-UTRA sidelink, the modulation and RB allocation are selected as below table.

Table 2.2.5-2 Test modulations and RB allocation configuration summary

| NR Uu carrier |  |  | E-UTRA V2X Carrier |  |  |
| --- | --- | --- | --- | --- | --- |
| Modulation | PUSCH RB allocation (Note 1) | MPR | Modulation | PSCCH and PSSCH RB allocation (Note 2) | MPR |
| DFT-s-OFDM QPSK | Inner_Full | 0dB | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW | 1.5dB |
| DFT-s-OFDM QPSK | Outer_Full | 1dB | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW | 1.5dB |
| DFT-s-OFDM 16QAM | Inner_Full | 1dB | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW | 1.5dB |
| DFT-s-OFDM 16QAM | Outer_Full | 2dB | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW | 1.5dB |
| DFT-s-OFDM 64QAM | Outer_Full | 2.5dB | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW | 1.5dB |
| DFT-s-OFDM 256QAM | Outer_Full | 4.5dB | QPSK | 48@2 for 10MHz BW 96@2 for 20MHz BW | 1.5dB |
| CP-OFDM QPSK | Inner_Full | 1.5dB | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW | 2dB |
| CP-OFDM 16PSK | Inner_Full | 2dB | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW | 2dB |
| CP-OFDM 16PSK | Outer_Full | 3dB | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW | 2dB |
| CP-OFDM 64PSK | Outer_Full | 3.5dB | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW | 2dB |
| CP-OFDM 256PSK | Outer_Full | 6.5dB | 16QAM | 48@2 for 10MHz BW 96@2 for 20MHz BW | 2dB |

Proposal 6: For inter-band concurrent V2X testing, modulation and RB allocation shall be selected as table 2.2.5-1 and 2.2.5-2 for MPR and ACLR. Only subset of Full allocation is selected for SEM.

    - Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.3.6-1 Total test points of Inter-band concurrent V2X

| Test case | Environment conditions | Test SCS | Test Frequencies | Test ChBW | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- | --- |
| MPR | 5 | 1 | 2 | 2 | 4 for E-UTRA Uu + NR SL 11 for NR Uu + E-UTRA SL | 80 220 |
| SEM | 1 | 1 | 2 | 2 | 2 for E-UTRA Uu + NR SL 7 for NR Uu + E-UTRA SL | 8 28 |
| ACLR | 5 | 1 | 2 | 2 | 4 for E-UTRA Uu + NR SL 11 for NR Uu + E-UTRA SL | 80 220 |

Conclusion

It is proposed that RAN5 agrees below proposals on test points of MPR, SEM and ACLR for NR V2X:

Proposal 1: No test point is needed for intra-band V2X.

Proposal 2: For inter-band concurrent V2X testing, define test environment as Normal, TL/VL, TL/VH, TH/VL and TH/VH for MPR and ACLR, and Normal for SEM.

Proposal 3: For inter-band concurrent V2X testing, select 15kHz SCS for NR Uu or NR SL carrier.

Proposal 4: For inter-band concurrent V2X testing, test Low range for each carrier and High range for each carrier.

Proposal 5: For inter-band concurrent V2X testing, test Lowest channel bandwidth for each carrier and Highest channel bandwidth for each carrier.

Proposal 6: For inter-band concurrent V2X testing, modulation and RB allocation shall be selected as table 2.2.5-1 and 2.2.5-2 for MPR and ACLR. Only subset of Full allocation is selected for SEM.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-3 v16.9.0 User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

[3] 3GPP TS 38.521-3 v17.2.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone