Title:	Discussion on test point selection for NS_202 A-MPR in FR2

Document for:	Discussion and approval

# Introduction

The purpose of this paper is to provide test point selection proposal for 38.101-2 A-MPR requirements when NS_202 is signalled (band n257 and n258). Additional RF emission requirements apply in this scenario in the form of Spurious emissions, meaning this proposal applies for test cases 6.2.3 (A-MPR) and 6.5.3.3 (Additional Spurious emission).

Test point selection is considered for PC1, PC2, PC3 and PC4.

Both single Tx and UL-MIMO are covered.

# Discussion

Since A-MPR for NS_202 in RAN4 is defined together with A-Spurious requirements, a combined analysis is required. In general, the following non-compliant UE behaviors need to be checked:

- UE apply too much A-MPR (more than RAN4 allow)

- UE apply to little A-MPR (causing too much RF emissions)

Case A can be verified in A-MPR test case.

Case B can be verified in A-Spurious test case and does not require check of UE output power if it is ensured that the same test point is tested inside A-MPR test.

A-MPR for NS_202 is defined in 38.101-2 [1] as:

For power class 1, A-MPR for NS_202 shall be 11.0 dB

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

For power class 3, A-MPR for NS_202 shall be 1.0 dB

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

A-Spurious for NS_202 is defined in 38.101-2 [2] as:

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

## Test Environment

RF emission requirements for NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Normal conditions, and the same selection is appropriate also for Additional Spurious emission.

## Test Frequencies

Protected range for NS_202 is both below and above band n257 and n258. It is judged that the worst case is when the carrier frequency is as close as possible to the protected range, which implies Low and High frequency range.

## Test Channel Bandwidth

The A-MPR requirements are the same for a particular power class for any channel bandwidth. Therefore, test channel bandwidth can be decided based on what is most demanding for Spurious emission. Selection can therefore be reused from normal spurious test case which means Highest channel bandwidth.

## Test Subcarrier Spacing

RF emission requirements for NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Lowest SCS, and the same selection is appropriate also for Additional Spurious emission.

However, the MPR test is using 120 kHz SCS as a simplification since it is the only supported SCS at 400 MHz channel Bandwidth. Therefore, the proposal is changed to test only 120 kHz SCS.

## UL Modulation

RF emission requirements for NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with QPSK DFT-s-OFDM modulation to maximize UE output power (lowest power backoff). Testing at a high output power is considered more important than testing a more demanding modulation scheme for Spurious testing.

For PC1 where A-MPR is always higher than MPR regardless of modulation also 64QAM is tested. The reason is that in this case 64QAM does not give the UE more allowed MPR and should therefore be added as a worst case.

## Uplink RB allocation

RF emission requirements for NS_202 are defined as Spurious emissions.

For PC2, PC3 and PC4:

For outer allocation A-MPR is smaller than MPR and hence does not need to be considered in test point selection.

For inner allocation A-MPR is larger than MPR which indicates that this is a worst case for testing.

Regular spurious emissions for FR2 is tested with Inner_1RB and Inner_Full allocation, and the same selection is appropriate also for Additional Spurious emission.

For PC1:

A-MPR is always 11 dB meaning the UE output power may be the same regardless of test parameters. A worst case to test will then be an outer allocation closest to the band edge with high order modulation.

Inner_Partial allocation is appropriate due to high MPRnarrow values applicable to Inner_1RB waveform. In order to avoid possible testability issue caused by large MPR value, it can be considered to use partial allocation of 0 MPR

## Check of alignment of test parameters between power measurement test case (A-MPR) and RF emissions test case (A-Spur)

Now that test point proposal is completed, it is required to check that all test points selected in RF emissions test case (Spurious in this case) is covered in A-MPR test case.

Result:

The test points in this paper matches the test points in A-Spurious and A-MPR test cases in TS38.521-2 for all power classes.

## Exceptions for UL-MIMO

Extracted from Clause 6.2D.3

Observation 1: Same A-MPR and A-Spurious requirements as specified for single-antenna port apply to UE supporting UL-MIMO.

According to [2], CP-OFDM is mandatory to UL-MIMO transmission with two layers, therefore DFT-s-OFDM is precluded from the test points for A-MPR UL-MIMO measurement. Therefore CP-OFDM needs to be configured in UL MIMO test case.

# Proposal

Following test points are proposed for NS_202 A-MPR and A-Spur test cases (PC1)

- Test environment: Normal Conditions

- Test frequencies: Low and High range

- Test Channel Bandwidth: Highest

- Test SCS: 120 kHz

- RB allocation and modulation: Inner_Partial (DFT-s-OFDM QPSK ), Outer_Full (64QAM)

Following test points are proposed for NS_202 A-MPR and A-Spur test cases (PC2, PC3 and PC4)

- Test environment: Normal Conditions

- Test frequencies: Low and High range

- Test Channel Bandwidth: Highest

- Test SCS: 120 kHz

- Modulation: DFT-s-OFDM QPSK

- RB allocation: Inner_1RB, Inner_Full

The proposal applies also for UL-MIMO with the exception modulation is CP-OFDM QPSK

# Number of Test Points

The number of test points for NS_202 A-MPR is 4 (both single Tx and UL-MIMO)

The number of test points for NS_202 A-Spurious is 4, same as in Tx Spurious emissions test

# References

- TS 38.101-2 v16.0.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 16)

- R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019