3GPP TSG-RAN5 Meeting #93-e	R5-217674r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1473 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of CBW 70MHz into 6.5A of 38.521-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CBW 70M is currently missing in TC 6.5A. Test requirement for 70MHz CBW need to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated CBW 70M into TC 6.5A.1.1, Updated CBW 70M into TC 6.5A.2.2.1, Updated CBW 70M into TC 6.5A.2.4.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CBW 70MHz will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.1.1 6.5A.2.2.1 6.5A.2.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: Undo the revision of note in Table 6.5A.2.2.1.5-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5A.1.1	Occupied bandwidth for CA (2UL CA)

Editor’s Note:

- For UL non-contiguous intra-band operation when the signalling is absent for dualPA-Architecture IE, the number of test frequency points is under investigation in TS 38.508-1, e.g. “Low” and “High”, or “Mid”.

- MU needs to be reassessed.

6.5A.1.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits for 2 UL CA

6.5A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL and 2UL CA

6.5A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.1.0.

6.5A.1.1.4	Test description

6.5A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.1.1.4.1-1 for inter-band UL CA, table 6.5A.1.1.4.1-2 for intra-band contiguous CA and table 6.5A.1.1.4.1-3 for intra-band non-contiguous CA. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range for both PCC and SCC (NOTE 4) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest for both PCC and SCC Highest for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCC | SCC |
| 1 | N/A for this test | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.3A.4-1. NOTE 3: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.3-1. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

Table 6.5A.1.1.4.1-2: Intra band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | All aggregated channel bandwidth |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |
|  |  | (NOTE 2) |  |
| 1 | N/A for this test | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |

Table 6.5A.1.1.4.1-3: Intra band non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | For test frequencies refer to “Range” columns |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | All aggregated channel bandwidth |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Smallest supported SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |
| ID | CA config |  |  |  |  | DL config | UL config |  |  |
|  | PCC |  | SCC |  | Wgap |  | CC MOD | RB allocation (NOTE 1) |  |
|  | Band | Range | Band | Range |  |  |  | PCC | SCC |
| 1 | nX | CC1 | nX | CC2 | Max (NOTE 4) | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The RB allocation is defined in table 6.1-1 for each CC. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.2-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.1.1.4.1-1 to Table 6.5A.1.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.1.1.4.3.

6.5A.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.1.1.4.1-1 or Table 6.5A.1.1.4.1-2 as appropriate. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

6.	For inter-band CA: measure the power spectrum distribution of both PCC and SCC within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency.

For Intra-band contiguous CA: measure the power spectrum distribution over all component carriers within two times or more range over the aggregated channel bandwidth requirement for Occupied Bandwidth specification centring on the centre of aggregated channel bandwidth. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is at least 1ms over consecutive active uplink slots.

For intra-band non-contiguous CA: measure the power spectrum distribution of each sub-block over all component carriers within the requirement for Occupied Bandwidth for CA specification, centring on the carrier frequency of each sub-block in CA configuration. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

7.	Calculate the total power within the range of all frequencies measured in step 6 and save this value as “Total power”. “Total power” is calculated for each CC separately for inter-band carrier aggregation, and for all CCs together for intra-band contiguous and non-contiguous carrier aggregation.

8.	Identify the measurement window whose centre is aligned on the centre of the channel bandwidth on each carrier for inter-band carrier aggregation, or on the centre of the aggregated channel bandwidth for intra-band contiguous carrier aggregation or centring on each carrier frequency of each sub-block for which the sum of the power measured is 99% of the “Total power”.

9.	The “Occupied Bandwidth” is the width of the measurement window obtained in step 8.

6.5A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.1.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |

Table 6.5A.1.1.4.3-1: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  | Power class 3 and Bandwidth class B |
|  | 10 |  | Power class 3 and Bandwidth class C |

6.5A.1.1.5	Test requirements

For inter-band carrier aggregation, he measured Occupied Bandwidth for each component carrier shall not exceed values in Table 6.5A.1.1.5-1.

Table 6.5A.1.1.5-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Occupied channel bandwidth (MHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |

For intra-band contiguous carrier aggregation, the measured Occupied Bandwidth shall not exceed the aggregated channel bandwidth as defined in subclause 5.3A.3.

For intra-band non-contiguous carrier aggregation, the measured Occupied Bandwidth shall not exceed values of channel bandwidth as defined in section 5.5A.2.

<Skipped>

6.5A.2.2.1	Spectrum emission mask for CA (2UL CA)

6.5A.2.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth for 2UL CA.

6.5A.2.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL and 2UL CA.

6.5A.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause6.5A.2.2.0.

6.5A.2.2.1.4	Test description

6.5A.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.2.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest for both PCC and SCC Highest for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Low | N/A | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 3 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 4 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 5 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 6 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 7 | Default |  | DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| 8 | Default |  | DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| 9 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 10 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 11 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 12 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 13 | Default |  | CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| 14 | Default |  | CP-OFDM 256 QAM | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.2.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.2.2.1.4.3.

6.5A.2.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.2.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.2.2.1.4.1-1 on both PCC and SCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the mean power of the UE in the channel bandwidth of the radio access mode for each CC according to the test configuration, which shall meet the requirements described in clause 6.2A.2. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths for each CC according to Table 6.5A.2.2.1.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5A.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.2.2.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 3, 4, 7 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 5, 6, 13 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 1, 2, 8-12 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 14 |

6.5A.2.2.1.5	Test requirement

The measured UE mean power in the channel bandwidth for each CC, derived in step 6, shall fulfil requirements in clause 6.2A.2 as appropriate, and the power of any UE emission for each CC, derived in step 7, shall fulfil requirements in Table 6.5A.2.2.1.5-1.If for some frequency spectrum emission masks of component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a component carrier spectrum emission mask overlaps with the channel bandwidth of another component carrier, then the emission mask does not apply for that frequency.

Table 6.5A.2.2.1.5-1: NR General spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.2.2.1.5-2: Test Tolerance for Spectrum emission mask

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5dB | 1.8dB |
| 40MHz < BW ≤ 100MHz | 1.5dB | 1.8dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<Skipped>

6.5A.2.4.2.1	UTRA ACLR for CA (2UL CA)

6.5A.2.4.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent UTRA channels in terms of Adjacent Channel Leakage power Ratio (UTRA ACLR) for 2UL CA.

6.5A.2.4.2.1.2	Test applicability

This test case applies for network signalling values NS_3U, NS_5U, NS_43U, and NS_100 to all types of NR UE release 15 and forward that supports 2DL and 2UL inter-band CA.

6.5A.2.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.2.4.2.0.

6.5A.2.4.2.1.4	Test description

6.5A.2.4.2.1.4.1	Initial conditions

Same as in subclause 6.2A.3.1.4.1 with the following exepction;

-	Only network signalling values NS_3U, NS_5U, NS_43U, and NS_100 with the corresponding CA configuration defined in Table 6.2A.3.1.4.1-1 need to perform UTRA ACLR test.

-	Message contents in step 6 are defined in clause 6.5A.2.4.2.1.4.3.

6.5A.2.4.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.2.4.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable test configuration tables in clause 6.2A.3.1.4.1 on both PCC and SCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

7.	Measure the rectangular filtered mean power for the assigned NR channel on PCC.

8.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the assigned NR channel on PCC, respectively.

9.	Calculate the ratios of the power between the values measured in step 7 over step 8 for lower and upper UTRA ACLR, respectively.

10.	Measure the rectangular filtered mean power for the assigned NR channel on SCC.

11.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the assigned NR channel on SCC, respectively.

12.	Calculate the ratios of the power between the values measured in step 10 over step 11 for lower and upper UTRA ACLR, respectively.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5A.2.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

6.5A.2.4.2.1.4.3.1	Message contents exceptions (network signalling value "NS_100" on PCC)

Table 6.5A.2.4.2.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.5A.2.4.2.1.4.3.2	Message contents exceptions (network signalling value "NS_43U" on PCC)

Table 6.5A.2.4.2.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (NS_43U) |  |  |

6.5A.2.4.2.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 6, shall fulfil requirements in clause 6.2A.3.1 as appropriate, and if the measured adjacent channel power is greater than –50 dBm, then the measured UTRA ACLR for each CC, derived in step 9 and step 13, shall be higher than the limits in Table 6.5A.2.4.2.1.5-2.

Table 6.5A.2.4.2.1.5-1: Measurement bandwidth for NR carrier

|  | NR channel bandwidth / UTRA ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR channel measurement bandwidth (MHz) | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| UTRA channel Measurement bandwidth (MHz) | 3.84 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1st Adjacent channel centre frequency offset | ± 2.5 MHz from NR channel edge |  |  |  |  |  |  |  |  |  |  |  |  |
| 2nd Adjacent channel centre frequency offset | ± 7.5 MHz from NR channel edge |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.2.4.2.1.5-2: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB -TT |
| UTRAACLR2 | 36 dB - TT |

Table 6.5A.2.4.2.1.5-3: Test Tolerance for UTRA ACLR

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 100MHz | 0.8dB | 0.8dB |

<End of Changes>