3GPP TSG-WG5 Meeting #93-e	R5-217680r1

# Electronic Meeting, 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2646 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of testcase 7.1.3.5.7 Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Removal of editors note if test loop mode B modification for NR and EHC is approved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of editors note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EHC functionality testing method will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.7 Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.509 CR 0048 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updated pre-test conditions. Editorial corrections. Ethernet address provided to UE with test loop functionality introduced in R5-217697r2. Added steps 9 - 14 to test UE behaviour with changing ethernet address. |  |  |  |  |  |  |  |  |

------------------------------------------------- Start of changes -------------------------------------------------

------------------------------------------------- Skipped chapters -------------------------------------------------

##### 7.1.3.5.7	Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression

7.1.3.5.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Ethernet header compression functionality is taken into use on DRB }

then { UE performs correct ethernet header compression and decompression function on DRB }

}

7.1.3.5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.12. These are Rel-16 requirements.

[TS 38.323, clause 5.12.2]

PDCP entities associated with DRBs can be configured by upper layers TS 38.331 [3] to use EHC. Each PDCP entity carrying user plane data may be configured to use EHC. Every PDCP entity uses at most one EHC compressor instance and at most one EHC decompressor instance.

[TS 38.323, clause 5.12.3]

The usage and definition of the parameters shall be as specified below.

-	MAX_CID_EHC_UL: This is the maximum CID value that can be used for uplink. One CID value shall always be reserved for uncompressed flows. The parameter MAX_CID_EHC_UL is configured by upper layers (maxCID-EHC-UL in TS 38.331 [3]);

[TS 38.323, clause 5.12.4]

If EHC is configured, the EHC protocol generates two types of output packets:

-	EHC compressed packets (i.e. EHC full header packets and EHC compressed header packets), each associated with one PDCP SDU;

-	standalone packets not associated with a PDCP SDU, i.e. EHC feedback.

An EHC compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU. The header compression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP SDU.

EHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered.

[TS 38.323, clause 5.12.5]

If EHC is configured by upper layers for PDCP entities associated with user plane data, the PDCP Data PDUs are decompressed by the EHC protocol after performing deciphering as explained in clause 5.8. The header decompression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP Data PDU.

[TS 38.323, clause 5.12.6.1]

When an EHC feedback is generated by the EHC protocol, the transmitting PDCP entity shall:

-	submit to lower layers the corresponding PDCP Control PDU as specified in clause 6.2.3.3 i.e. without associating a PDCP SN, nor performing ciphering.

[TS 38.323, clause 5.12.6.2]

At reception of a PDCP Control PDU for EHC feedback from lower layers, the receiving PDCP entity shall:

-	deliver the corresponding EHC feedback to the EHC protocol without performing deciphering.

7.1.3.5.7.3	Test description

7.1.3.5.7.3.1	Pre-test conditions

Pre-test conditions as in clause 7.1.3.0 with condition pc_NG_RAN_NR, one PDN/PDU session, message with condition UM used for step 7 in 4.5.4.2 according to [4] and with exception of using UE TEST LOOP MODE B.

7.1.3.5.7.3.2	Test procedure sequence

Table 7.1.3.5.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure DRB to use ethernet header compression. SS transmits unique full ethernet header information to the UE to be used in looped back ethernet messages. | <-- | RRCReconfiguration TC: UPDATE UE ETHERNET ADDRESS | - | - |
| 2 | UE transmits RRCReconfigurationComplete message to confirm that reconfiguration with ethernet header compression configuration is completed. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS transmits a ciphered PDCP Data PDU with unique Context ID containing ethernet packet with full ethernet header information different to the one used in step 1. | <-- | PDCP Data PDU #0 | - | - |
| 4 | CHECK: Does UE transmit unciphered EHC feedback packet with Context ID set in step 3? | --> | PDCP Control PDU #0 | 1 | P |
| 5 | CHECK: Does UE transmit ciphered looped back PDCP Data PDU with unique Context ID containing ethernet packet with full ethernet header set in step 1? | --> | PDCP Data PDU #0 | 1 | P |
| 6 | SS transmits unciphered EHC feedback packet with Context ID set in step 5. | <-- | PDCP Control PDU #0 | - | - |
| 7 | SS transmits a ciphered PDCP Data PDU with unique Context ID set in step 3 containing compressed header ethernet packet. | <-- | PDCP Data PDU #1 | - | - |
| 8 | CHECK: Does UE transmit ciphered looped back PDCP Data PDU with Context ID set in step 5 containing compressed header ethernet packet? | --> | PDCP Data PDU #1 | 1 | P |
| 9 | SS transmits full ethernet header information to the UE that is different to the ones used in step 1 and step 3. | <-- | TC: UPDATE UE ETHERNET ADDRESS | - | - |
| 10 | SS transmits a ciphered PDCP Data PDU with unique Context ID set in step 3 containing compressed header ethernet packet. | <-- | PDCP Data PDU #2 | - | - |
| 11 | CHECK: Does UE transmit ciphered looped back PDCP Data PDU with Context ID different to step 5 containing ethernet packet set in step 10 with full ethernet header set in step 9? | --> | PDCP Data PDU #2 | 1 | P |
| 12 | SS transmits unciphered EHC feedback packet with context ID set in step 11. | <-- | PDCP Control PDU #1 | - | - |
| 13 | SS transmits a ciphered PDCP Data PDU with unique Context ID set in step 3 containing compressed header ethernet packet. | <-- | PDCP Data PDU #3 | - | - |
| 14 | CHECK: Does UE transmit ciphered looped back PDCP Data PDU with Context ID set in step 11 containing compressed header ethernet packet? | --> | PDCP Data PDU #3 | 1 | P |

7.1.3.5.7.3.3	Specific message contents

Table 7.1.3.5.7.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.5.7.3.2-1)

| Derivation Path: TS 38.508 [4], clause 4.6.1, table 4.6.1-13 RRCReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-EHC |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

------------------------------------------------- Skipped chapters -------------------------------------------------

------------------------------------------------- End of changes -------------------------------------------------