3GPP TSG RAN WG5 Meeting #93-e	                          R5-217701r2

Electronic Meeting, 8th - 19th November 2021

Agenda item:	5.3.1.17

Source:	Apple Hungary Kft., Keysight

Title:                         Discussion on handling ETC for FR2 RF CA test case scenarios

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The purpose of this discussion paper is to provide a view on the feedback received during the presentation of [1] done in the last meeting, as well as to propose a possible way forward on the ETC coverage for FR2 RF CA test scenarios.

The ETC applicability of Single Carrier (SC) EIRP, EIS and Frequency Error and additional test cases had been discussed and agreed ([3]-[6]), However, similar discussion related to these tests cases needs to be further understood and optimized for the CA implementation. An in-depth analysis of nuances related to CA implementation might confirm that definitions agreed for SC cannot be directly implemented on CA test case scenarios.

# 2	Discussion

In general, the optimization of test time for ETC CA for FR2 RF tests is a reasonable requirement and assumption based on the discussion from [2].

Considering the testing time optimization described above, the following statements describes our feedback to comments received for [1] in RAN5#92:

Feedback 1: The work on single CC should be concluded and those findings should be used as starting point for CA ETC

Findings related to single CC FR2 ETC is progressing significantly and already provides a good starting point for CA FR2 ETC investigation. Both single CC and CA could progress in parallel trying to minimize additional delays on getting FR2 ETC functionality fully defined.

Observation 1: Findings related to single CC can already provide a good starting point for CA ETC investigation. Both single CC and CA could progress in parallel trying to minimize additional delays on getting FR2 ETC functionality fully defined.

Feedback 2: Limiting tests to MOP and REFSENS might not be enough, frequency error and EVM should be considered as well.

Frequency error can be considered to be added alongside MOP and REFSENS, as it is a key parameter for proper UL signal demodulation. However, EVM core requirements are exempt from testing under ETC according to core requirements:

Observation 2: CA EVM cannot be extended to ETC as these testing conditions are exempt from core requirements.

Observation 3: CA Frequency error could be extended instead, adding a significative value to FR2 ETC CA coverage.

Feedback 3: More data driven analysis is needed to justify the trade-off and additional considerations when compared with single CC ETC definitions

Based on FR2 Single CC test cases (whose applicability to ETC has been already agreed) and based on FR1 CA test cases currently tested under ETC, the following test coverage vs testing time trade-off is proposed at this stage:

- Update CA REFSENS, frequency error initial conditions to include FR2 ETC testing for any number of CCs (as done in FR1).

  - To be noticed that:

    - MOP test case is tested without allocation in any SCC to achieve MPR=0. Extending this test to ETC does not provide much value.

    - EVM requirements is exempt from ETC according to core requirements.

- Defer discussions on ETC applicability of CA FR2 time mask test cases until testability issues are solved for single CC case.

- Defer discussions on ETC applicability of CA MPR, Minimum Output power and CA ACLR until SCC power prioritization and potential further relaxations compared to single CC due to CA analysis are concluded.

Proposal 1: Update CA REFSENS, frequency error initial conditions for any number of CCs to include FR2 ETC testing.

Proposal 2: Defer discussions on ETC applicability of CA FR2 time mask test cases until testability issues are solved for single CC test case.

Proposal 3: Defer discussions on ETC applicability of CA MPR, Minimum Output power and CA ACLR until SCC power prioritization and relaxations due to CA analysis are concluded.

# 3	Observations and Proposals

This document handles the feedback received on FR2 CA ETC discussions and makes the following observation and proposals:

Observation 1: Findings related to single CC can already provide a good starting point for CA ETC investigation: Both single CC and CA could progress in parallel trying to minimize additional delays on getting FR2 ETC functionality fully defined.

Observation 2: CA EVM cannot be extended to ETC as these testing conditions are exempt from core requirements.

Observation 3: CA Frequency error could be extended instead, adding a significative value to FR2 ETC CA coverage.

Proposal 1: Update CA REFSENS, frequency error initial conditions for any number of CCs to include FR2 ETC testing.

Proposal 2: Defer discussions on ETC applicability of CA FR2 time mask test cases until testability issues are solved for single CC test case.

Proposal 3: Defer discussions on ETC applicability of CA MPR, Minimum Output power and CA ACLR until SCC power prioritization and relaxations due to CA analysis are concluded.

# 4	References

R5-215560, Discussion on handling ETC for FR2 RF CA test case scenarios, Apple, RAN5 #92-e

R5-213811, FR2 Extreme testing conditions applicability, Keysight, RAN5 #91-e

R5-216092, 38.521-2 CR FR2 ETC MU & TT updates, Keysight Technologies UK Ltd, RAN5 #92-e

R5-214049, Removal of for further study notes about ETC Testing, CAICT, Keysight Technologies, RAN5 #91-e

R5-194265, Correction to FR2 EIS test configurations, Dish Network, RAN5#83

R5-194264, Correction to FR2 EIRP test configurations, Dish Network, RAN5#83