3GPP TSG-RAN5 Meeting #93-e                                                    R5-217703r2

Electronic Meeting, 8th – 19th November 2021

Title: 		        Beam correspondence Measurement Uncertainties derivation

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.1.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Measurement uncertainties for FR2 beam correspondence tolerance is still undefined ([1][2]).

This document makes a proposal on how to derive such measurement uncertainty.

# 2.	Core requirements overview

Analysing minimum conformance requirements and test procedure for beam correspondence tolerance, it can be seen that beam correspondence tolerance is defined as:

6.6.1.3.3.2 Beam correspondence tolerance for PC3 The beam correspondence tolerance requirement ∆EIRPBC for power class 3 UEs is defined based on a percentile of the distribution of ∆EIRPBC, defined as ∆EIRPBC = EIRP2 - EIRP1 over the link angles spanning a subset of the spherical coverage grid points, such that - EIRP1 is the total EIRP in dBm calculated based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping. - EIRP2 is the best total EIRP (beam yielding highest EIRP in a given direction) in dBm which is based on beam correspondence with relying on UL beam sweeping. - The link angles are the ones corresponding to the top Nth percentile of the EIRP2 measurement over the whole sphere, where the value of N is according to the test point of EIRP spherical coverage requirement for power class 3, i.e. N = 50. For power class 3 UEs, the requirement is fulfilled if the UE’s corresponding UL beams satisfy the maximum limit in Table 6.6.1.3.3.2-1. Table 6.6.1.3.3.2-1: UE beam correspondence tolerance for power class 3 Operating band; Max ∆EIRPBC at 85 %-tile ∆EIRPBC CDF (dB). n257; 3.0. n258; 3.0. n260; 3.2. n261; 3.0. NOTE: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1.

As shown, beam correspondence tolerance requirement is defined as a given power percentile around the sphere of a differential EIRP measurement.

The EIRP difference for a compliant UE will be no higher than 3.2 dB for at least 85% of the measurement grid points around the sphere. Hence, it seems to be reasonable to assume that test system configuration to make each of these measurements will be similar.

# 3.	Measurement uncertainties contributions

Similarly as in [3], for the beam correspondence tolerance measurement, it is thought that all the MU factors associated to calibration stage in EIRP absolute power measurement MU budget table are cancelled out as the 2 signals experience the same absolute level distortion.

In the same way, the following MU factors in the DUT measurement stage also cancel out in the FR2 beam correspondence tolerance measurement MU budget:

- Measure distance uncertainty

- Quality of the Quiet Zone

- Mismatch

- Standing wave between the DUT and measurement antenna

- Phase curvature

- Random uncertainty

- Influence of XPD

- Insertion Loss Variation

- RF leakage (from measurement antenna to the receiver/transmitter)

- Multiple measurement antenna uncertainty

- Positioning misalignment

Hence the MU factors remaining will be:

- Positioning misalignment

- Uncertainty of the RF Power measurement equipment

- Amplifier uncertainties

- Influence of noise

Proposal 1: Adopt N/A for the MU elements other than uncertainty of the RF Power measurement equipment, amplifier uncertainties and influence of noise in FR2 Beam Correspondence tolerance measurement MU.

Regarding the amplifier MU, compared to relative power tolerance measurement the situation in beam correspondence tolerance is different as for beam correspondence a high number of grid points have to be measured resulting in a longer runtime. Therefore, the amplifier temperature can change resulting in gain variations.

In addition, the measured power can range from Nth percentile to EIRPmax (43 dBm), so that compression effects need to be accounted for.

Therefore, it is proposed to use the same amplifier MU as in the EIRP spherical coverage test case.

Proposal 2: Adopt 2.1 dB for the uncertainty value of amplifier uncertainty in FR2 Beam Correspondence tolerance measurement MU.

Proposal 3: In the absence of a CDF reference cure and to enable progress on this priority 1 test case, select one of the 3 options below:

- Option 3.1: Adopt 1.0dB (systematic error) as influence of noise(SNR) for FR2 Beam Correspondence tolerance measurement MU as proposed in [3] for FR2 relative power measurement MU.

- Option 3.2: Open an action point for UE vendors to provide typical ∆EIRPBC CDFs knowing the complete ERP1 vs link angle and EIRP2 vs link angle data sets so further analysis can be carried out.

- Option 3.3: The influence of noise to be calculated by considering the ΔSNR at the level of “50%-tile at minimum EIRP – beam correspondence tolerance”, for example 8.5 dBm = 11.5 dBm – 3.0 dB for n257.

Proposal 4: Adopt +/- 0.4dB (as expanded uncertainty) for Uncertainty of the RF Power measurement equipment as proposed in [3].

# 4.	Conclusion

This document describes how to calculate FR2 beam correspondence tolerance measurement MU and makes the following proposals:

Proposal 1: Adopt N/A for the MU elements other than uncertainty of the RF Power measurement equipment, amplifier uncertainties and influence of noise in FR2 Beam Correspondence tolerance measurement MU.

Proposal 2: Adopt 2.1 dB for the uncertainty value of amplifier uncertainty in FR2 Beam Correspondence tolerance measurement MU.

Proposal 3: In the absence of a CDF reference cure and to enable progress on this priority 1 test case, select one of the 3 options below:

- Option 3.1: Adopt 1.0dB (systematic error) as influence of noise(SNR) for FR2 Beam Correspondence tolerance measurement MU as proposed in [3] for FR2 relative power measurement MU.

- Option 3.2: Open an action point for UE vendors to provide typical ∆EIRPBC CDFs knowing the complete ERP1 vs link angle and EIRP2 vs link angle data sets so further analysis can be carried out.

- Option 3.3: The influence of noise to be calculated by considering the ΔSNR at the level of “50%-tile at minimum EIRP – beam correspondence tolerance”, for example 8.5 dBm = 11.5 dBm – 3.0 dB for n257.

Proposal 4: Adopt +/- 0.4dB (as expanded uncertainty) for Uncertainty of the RF Power measurement equipment as proposed in [3].

# 5.	References

[1] 38.521-2 v16.9.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

[2] 38.903 v16.9.0 “Derivation of test tolerances and measurement uncertainty for User equipment (UE) conformance test cases”

[3] R5-210845, “On FR2 relative power measurement uncertainty”, Anritsu, RAN5#90-e

[4] R5-215319, “On MU of FR2 power control test cases”, Anritsu, RAN5#92-e