3GPP TSG-RAN5 Meeting #93e	R5-217733r1

# Electronic meeting, 8-19 November 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2654 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDU session authentication and authorization Test Case 10.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 38.331 section 5.3.1.1: “A configuration with SRB2 without DRB or DRB without SRB2 is not supported”. So: - When all DRBs are released and SRB2 is still active, the RRC connection must be released. At step 1 of this test case, the only DRB is released while SRB2 is still active |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 0, another PDU session is activated, so that a DRB is still active throughout this test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compliant UE will release RRC connection after step 1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updated coversheet |  |  |  |  |  |  |  |  |

[Start of changes]

10.1.2.2.3.2	Test procedure sequence

Table 10.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | Cause the UE to request establishment of PDU session Y to the DN. (Note 1) | - | - | - | - |
| 0B | The PDU session establishment procedure as specified in TS 38.508-1 [4] subclause 4.5A.2 takes place | - | - | - | - |
| 1 | The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed message with PDU session ID IE is set to the same value as the first PDU session ID for Internet in PDU SESSION ESTABLISHMENT REQUEST message in preamble. | - | - | - | - |
| 2 | Void | - |  | - | - |
| 3 | The SS transmits a PDU session modification command message with PDU session ID IE is set to the same value in PDU SESSION RELEASE COMMAND message. This message is included in a DLInformationTransfer message. | <-- | PDU SESSION MODIFICATION COMMAND | - | - |
| 4 | Check: Does the UE transmit a 5GSM STATUS with the 5GSM cause IE indicating #43 "invalid PDU session identity"? | --> | 5GSM STATUS | 1 | P |
| Note 1: The request of connectivity to an additional PDU session may be performed by MMI or AT command. |  |  |  |  |  |

### [End of changes]