| 3GPP TR 38.918 V0.3.0 (2021-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 5G NR User Equipment (UE) Full Stack Testing for Network Slicing Technical Report (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General	8

4.1	Background	8

4.2	Study Item Objective	9

5	Study on 5G NR UE Full Stack Testing for Network Slicing	9

5.1	Definition of 5G NR UE Full Stack Testing for Network Slicing	9

5.1.1	Definition of Full Stack	9

5.2	Test Configurations	9

5.2.1	UE Full Stack Network Slicing Test Equipment	9

5.2.2	UE Full Stack Network Slicing Test Equipment Connection Diagrams	10

5.2.3	Network Slicing Configurations	10

5.2.4	Message and Information Elements Contents	11

5.2.5	UE Specific Items	12

5.3	Application Simulation	12

5.3.1	Application Client Simulator	12

5.3.2	Test Models	12

5.4	Statistical Analysis	12

5.4.1	Overview of Mapping Application to Network Slicing	12

5.4.2	Overview of Application Layer Throughput With Network Slicing	13

5.4.3	Overview of Application Layer Latency With Network Slicing	13

5.5	Test System Uncertainty and Test Tolerance	13

5.5.1	Test System Uncertainty and Test Tolerance for FR1 testing	13

5.5.1.1	Recommended Uncertainty of Test System	13

5.5.1.2	Test Tolerances	13

5.5.1.3	Impact of Test System Uncertainty on Test Results	13

6	Conclusions	13

Annex A: Test Procedures	15

A.1	Purpose of annex	15

A.2	5G NR /URSP Configuration and Application Mapping	15

A.2.1	5G NR /URSP Configuration	15

A.2.1.1	5G NR / URSP Configuration / Signalling	15

A.2.2	5G NR / Mapping Application to Network Slicing	18

A.2.2.1	5G NR / Mapping Application to Network Slicing / DNN	18

A.2.2.2	5G NR / Mapping Application to Network Slicing / APP ID	21

A.2.2.3	5G NR / Mapping Application to Network Slicing / FQDN	21

A.2.2.4	5G NR / Mapping Application to Network Slicing / IP 3 Tuples	21

A.2.2.5	5G NR / Mapping Application to Network Slicing / Connection Capabilities	21

A.2.2.6	5G NR / Mapping Application to Network Slicing / URSP Update	21

A.3	5G NR / Service Performance Testing with Network Slicing	21

A.3.1	5G NR / Service Performance / Single Network Slicing	21

A.3.1.1	5G NR / Service Performance / Single Application with Single Network Slicing	21

A.3.1.2	5G NR / Service Performance / Multiple Applications with Single Network Slicing	23

A.3.2	5G NR / Service Performance / Multiple Network Slicing	23

A.3.2.1	5G NR / Service Performance / Multiple Applications with Multiple Network Slicing	23

A.3.3	5G NR / Service Performance / URSP Update	23

A.3.3.1	5G NR / Service Performance / Single Network Slicing / URSP update	23

A.3.3.2	5G NR / Service Performance / Multiple Network Slicing / URSP update	24

Annex B: Applicability	25

Annex C: Change history	26

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document contains the findings of the Study on 5G NR UE full stack testing of network slicing and the proposed test cases.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5] 	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[6] 	3GPP TS 38.508-1: "5GS;User Equipment (UE) conformance specification; Part 1: Common test environment".

[7] 	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	 3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[9] 	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); stage 3".

[10]	 3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[11] 	3GPP TS 23.003: "Numbering, addressing and identification"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

FFS

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	General

4.1	Background

5G Network Slicing is one of the most distinctive features provided by 5G NR and is key to meet diversified services requirements in 5G era. With the introduction of 5G network slicing technology, operators will be able to provide network capabilities with different functional characteristics, which will provide e.g. "exclusive" network for users with different KPI requirements to ensure a high-quality of service and meet differentiated scenario requirements. 5G network slicing can also help to achieve the goal of improving the efficiency of network resource utilization, optimizing the network construction investment of operators, and building a flexible and agile 5G network.

The user experience resulting from the use of 5G network slicing is critical for commercial success, and therefore testing the operation of 5G network slicing is necessary to ensure consistent predictable behaviour. To enable testing the 5G UEs supporting network slicing as they are used in the field the application layer at the device side needs to be included in the testing, as URSP based network slice selection and traffic routing procedures are based on traffic descriptor components (see TS 24.526) passed on by the application layer to the UE protocol stack. The traffic descriptor components that are passed onto the UE protocol stack to be used as a base for the network slice selection when a specific application requests a PDU session are configured within the device. There is a strong industry demand to define the test methods and test procedures that include the full related UE application layer in the testing of the network slice selection functionality for 5G UE supporting network slicing.

4.2	Study Item Objective

The technical objectives of this study item are:

-	To study how to define test procedures that will allow the full protocol stack testing of a 5G NR UE supporting network slicing. The test procedures for full protocol stack testing of the network slicing functionality, e.g. how the UE uses configurations matching UE application to traffic descriptor components to select S-NSSAI, will be provided.

-	To study how to define test procedures that will allow the service performance testing of a 5G NR UE supporting network slicing. The test procedures for performance testing of the network slicing service, e.g. application layer throughput and latency, will be provided.

NOTE 1: 	The Study Item should take into account industry recommendation from relevant organizations such as GSMA, NGMN and GTI pending on RAN5's decision on a case by case basis.

NOTE 2: 	The Study Item should take into account the User Privacy and/or implementation confidentiality.

NOTE 3:	 The Study Item will not define the specific traffic descriptor or related mapping mechanism.

5	Study on 5G NR UE Full Stack Testing for Network Slicing

5.1	Definition of 5G NR UE Full Stack Testing for Network Slicing

5.1.1	Definition of Full Stack

The test procedures defined will allow the full stack testing of a 5G NR UE supporting network slicing. The term "full stack" means multilayer combined performance of the following layers:

1	UE's Radio link protocol data processing performance (PHY, MAC, RLC, PDCP)

2	UE's Radio link protocol signalling processing capability (RRC, NAS)

3	UE's TCP/IP processing performance

4	UE's Internet Application processing performance

5.2	Test Configurations

5.2.1	UE Full Stack Network Slicing Test Equipment

Editor's note: This sub-clause should define the  architecture of the test equipment utilized for 5G NR UE full stack testing of network slicing

The test equipment utilized for 5G NR UE Full Stack Testing for Network Slicing should consist of the following items:

- System Simulator(s) suitable for the 5G NR radio technology used for network slicing testing. System simulator(s) could receive network slice configuration scheme from main controller and send network slicing related configuration to UE via signalling. System simulator(s) could receive application data from UE and send application data to application server simulator(s).

- Application Server Simulator(s) suitable for receiving application data from UE via system simulator(s) and handling the received application data. Application Server Simulator(s) could be either implemented in Main Controller or implemented as an independent unit.

- Main Controller suitable for generating network slice configuration scheme and configuring the network slice configuration scheme to System Simulator(s).

- Application Client Simulator(s) suitable for generating Traffic Descriptor of application(s) and corresponding application data on UE side.

5.2.2	UE Full Stack Network Slicing Test Equipment Connection Diagrams

Editor's note: This sub-clause should give the reference diagrams  of the test equipment defined in 5.2.1

The UE Full Stack Network Slicing Test Equipment connection diagram is shown in Figure 5.2.2-1.

Figure 5.2.2-1: UE Full Stack Network Slicing Test Equipment Connection Diagram

5.2.3	Network Slicing Configurations

Table 5.2.3-1: Network Slicing Configurations

| Application | OS App id | DNN | FQDN | IP 3 Tuples | S-NSSAI |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SST | SD |
| APP A | pc_OS_App_ID | pc_APN_ID_Specific | FFS | FFS | ‘00000001'B | 0x000001 |
| APP B | FFS | FFS | FFS | FFS | ‘00000001'B | 0x000002 |

5.2.4	Message and Information Elements Contents

The common message and information elements contents are defined in TS 38.508-1 [6] clause 4. In addition, the following message and information elements contents apply:

–	MANAGE UE POLICY COMMAND

Table 5.2.4-1: MANAGE UE POLICY COMMAND

| Derivation Path: TS 24.501 Table D.5.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| MANAGE UE POLICY COMMAND message identity |  | ’0000 0001’B |  |  |
| UE policy section management list |  | See Table 5.2.4-2 |  |  |

–	UE policy section management list

Table 5.2.4-2: UE policy section management list

| Derivation Path: TS 24.501 Figure D.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management list IEI |  |  |  |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  |  |  |  |
| UE policy section management sublist (PLMN-1) |  |  |  |  |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management sublist contents |  |  |  |  |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UE policy section contents |  |  |  |  |
| UE policy part |  | See Table 5.2.4-3 |  |  |

–	UE policy part

Table 5.2.4-3: UE policy part with UE policy part type = {URSP}

| Derivation Path: TS 24.501 Figure D.6.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B |  | URSP |
| UE policy part contents |  |  |  |  |
| URSP rule 1 |  | See Table 4.8.5.2-1 in TS 38.508-1 |  |  |
| URSP rule 2 |  | See Table 4.8.5.2-1 in TS 38.508-1 |  |  |

5.2.5	UE Specific Items

The following parameters are recommended to be provided by UE or application supplier to indicate the information of application using network slicing during test.

| Parameter Name | Parameter Type | Description |
| --- | --- | --- |
| pc_APN_ID_Specific | charstring | APN/DNN value of the simulation application client defined in clause 5.3.1 The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [11], subclause 9.1 |
| pc_OS_App_ID | charstring | OS App Id value of the simulation application client defined in clause 5.3.1 The coding of OS App Id is up to UE implementation |

5.3	Application Simulation

5.3.1	Application Client Simulator

To enable testing an Application Client Simulator on device is used to simulate the application which configured with network slicing. The following functionality is recommended for the Application Client Simulator to support.

-	The Application Client Simulator is installed on device

-	The Application Client Simulator may start automatically on device power-up or manually be started by the operator at the beginning of the test campaign.

-	The Application Client Simulator could receive the type of traffic descriptor to be tested and generate the corresponding traffic descriptor which conform to Network Slicing Configurations defined in clause 5.2.3

-	The Application Client Simulator could generate data packets matching the traffic descriptor in Network Slicing Configurations and transfer data packets to operating system and communication unit on device

-	The Application Client Simulator could directly send the generated data packets to application server simulator via cable connection

5.3.2	Test Models

Editor's note: This sub-clause should define the test models including app simulator and test equipment

5.4	Statistical Analysis

5.4.1	Overview of Mapping Application to Network Slicing

When the mapping of applications to network slicing is tested, the fundamental goal is to verify the procedure of associating applications to PDU sessions based on URSP rules. URSP rules contain Traffic Descriptors determining which URSP rule is applicable for the specific application and a list of Route Selection Descriptors determining the corresponding PDU session.

With this knowledge, the following aspects are recommended to be checked to verify the procedure of mapping application to Network Slicing

- The selection of applicable URSP rule according to the Traffic descriptors in UE Route Selection Policy Rule within URSP rules and information from the specific application

- The selection of existing PDU session or establishment of new PDU session for specific application based on the selected Route Selection Descriptors within the applicable URSP rule.

The way to trigger URSP rule selection is UE implementation specific and driven by application. To enable testing then the UE is recommended to be equipped with the application simulation client defined in clause 5.3.1 to trigger the required selection procedures

5.4.2	Overview of Application Layer Throughput With Network Slicing

Editor's note: This sub-clause should define the statistical analysis method of application layer throughput with network slicing configured

5.4.3	Overview of Application Layer Latency With Network Slicing

Editor's note: This sub-clause should define the statistical analysis method of application layer latency with network slicing configured

5.5	Test System Uncertainty and Test Tolerance

5.5.1	Test System Uncertainty and Test Tolerance for FR1 testing

5.5.1.1	Recommended Uncertainty of Test System

5.5.1.2	Test Tolerances

5.5.1.3	Impact of Test System Uncertainty on Test Results

6	Conclusions

The UE full stack testing for network slicing study item was initiated by RAN5 to include UE full stack network slicing capability. The following aspects were included as part of the present document:

Definition of UE Full Stack Testing for Network Slicing

Test configurations

Application Simulation

Statistical analysis

Test uncertainty and test tolerance

The present document includes a set of recommended test procedures for UE supporting network slicing that is in-line with the study item objectives identified in clause 4.

The test procedures are contained in Annex A.

Annex A:
Test Procedures

A.1	Purpose of annex

This annex specifies the test cases for 5G NR UE Full Stack Testing for Network Slicing. The test cases are the result of the study item and are the recommended test cases to be used when evaluating UE capability of supporting network slicing. The use of "shall" in the test cases listed herein is only used to indicate that the test case purpose, procedure, and/or result may not be as expected if the specified "shall" or "must" item is not used or followed. As the Technical Report is informative in nature, the use of "shall" or "must" in the test cases is not meant to imply a specific requirement. Their use is meant to indicate instances where the test objectives of the recommended test cases may not be achieved.

A.2	5G NR /URSP Configuration and Application Mapping

A.2.1	5G NR /URSP Configuration

A.2.1.1	5G NR / URSP Configuration / Signalling

A.2.1.1.1	Definition

The provisioning of URSP rules from network to UE is one of the fundamental functional requirements for network slicing.

A.2.1.1.2	Test Purpose

To verify that UE could support the provisioning of URSP rules from network to UE over NAS messages

A.2.1.1.3	Test Parameters

Void

A.2.1.1.4	Test Description

A.2.1.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.1.1.4.2	Test Procedure

1.	UE is switched on.

2.	Steps 2 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed.

3.	The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI and Configured NSSAI.

4.	The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message.

5.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to configure URSP rules

6.	The UE transmits an UL NAS TRANSPORT message including UE Policy Container and a MANAGE UE POLICY COMPLETE message is included

7.	The SS transmits an RRCRelease message

Table A.2.1.1.4.2-1: REGISTRATION ACCEPT (step 3)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000001 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table A.2.1.1.4.2-2: DL NAS TRANSPORT (step 5)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0101’B | UE policy container |  |
| Payload container |  | See Table A.2.1.1.4.2-3 |  |  |

Table A.2.1.1.4.2-3: MANAGE UE POLICY COMMAND (step 5)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10001000’B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000100’B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10100000’B |  | OS App Id type |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of 'OS App Id' in bytes |  |  |
| OS App Id |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000002 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000100’B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.2.2	5G NR / Mapping Application to Network Slicing

A.2.2.1	5G NR / Mapping Application to Network Slicing / DNN

Editor’s note:

-	The test parameters is FFS.

A.2.2.1.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.1.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of DNN.

A.2.2.1.3	Test Parameters

FFS

A.2.2.1.4	Test Description

A.2.2.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.1.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.1.4.2-1

2.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 1

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.1.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10001000’B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000100’B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.2.2.1.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0001’B | N1 SM information |  |
| PDU session ID |  | PSI-1 |  |  |
| Request type |  | ‘001’B | Initial request |  |
| S-NSSAI |  | Not Present |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.2.2.1.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.2.2.2	5G NR / Mapping Application to Network Slicing / APP ID

Editor’s note:

-	The test parameters is FFS.

A.2.2.2.1	Definition

The UE procedure for associating applications to PDU sessions based on URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.2.2	Test Purpose

To verify that UE could support the mapping of applications to URSP rules and the establishment of PDU sessions based on the Traffic Descriptor of OS App Id.

A.2.2.2.3	Test Parameters

FFS

A.2.2.2.4	Test Description

A.2.2.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.2.4.2-1

2.	Using the Application Client Simulator, generate traffic matching OS App Id value in Traffic descriptor in URSP rule provisioned to the UE in step 1

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10100000’B |  | OS App Id type |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |

Table A.2.2.2.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0001’B | N1 SM information |  |
| PDU session ID |  | PSI-1 |  |  |
| Request type |  | ‘001’B | Initial request |  |
| S-NSSAI |  | Not Present |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | 0x000001 |  |  |

Table A.2.2.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | 0x000001 |  |  |

A.2.2.3	5G NR / Mapping Application to Network Slicing / FQDN

A.2.2.4	5G NR / Mapping Application to Network Slicing / IP 3 Tuples

A.2.2.5	5G NR / Mapping Application to Network Slicing / Connection Capabilities

A.2.2.6	5G NR / Mapping Application to Network Slicing / URSP Update

A.3	5G NR / Service Performance Testing with Network Slicing

A.3.1	5G NR / Service Performance / Single Network Slicing

A.3.1.1	5G NR / Service Performance / Single Application with Single Network Slicing

Editor’s note:

-	The test parameters is FFS.

A.3.1.1.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.1.1.2	Test Purpose

To measure the performance of single service in application layer of the 5G NR UE with single network slicing.

A.3.1.1.3	Test Parameters

FFS

A.3.1.1.4	Test Description

A.3.1.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.1.1.4.2	Test Procedure

1.	UE is switched on.

2.	Steps 2 to 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed.

3.	Using the Application Client Simulator, begin uplink data transfer. Wait for 15 seconds and then start to measure the throughput result and latency in [transport layer]. (This is iteration 1) Continue data transfer for 1 minutes.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer

5.	Calculate and record the average application layer data throughput and latency across three iterations.

6.	Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed.

7.	UE is brought back to operation

8.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the MANAGE UE POLICY COMMAND message is defined in Table A.3.1.1.4.2-1

9.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 8

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator

13.	Wait for 15 seconds and then start to measure the throughput result and latency in [transport layer] in parallel (This is iteration 1). Continue data transfer for 1 minute.

14.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer

15. Calculate and record the average application layer data throughput and latency across three iterations.

16.	Observe that the throughput in step 15 is not lower than the throughput benchmark in step 5 and the latency in step 15 is not higher than the latency benchmark in step 5

Table A.3.1.1.4.2-1: MANAGE UE POLICY COMMAND (step 8)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10001000’B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000100’B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.3.1.2	5G NR / Service Performance / Multiple Applications with Single Network Slicing

A.3.2	5G NR / Service Performance / Multiple Network Slicing

A.3.2.1	5G NR / Service Performance / Multiple Applications with Multiple Network Slicing

A.3.3	5G NR / Service Performance / URSP Update

A.3.3.1	5G NR / Service Performance / Single Network Slicing / URSP update

A.3.3.2	5G NR / Service Performance / Multiple Network Slicing / URSP update

Annex B:
Applicability

FFS

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Cat | Subject/Comment | New |
| 2021-05 | RAN5#91e | R5-213433 | - |  | - | Draft skeleton | 0.1.0 |
| 2021-05 | RAN5#91e | R5-213433 | - |  | - | Text Proposal for TR 38.918 on Definition of Full Stack Testing for Network Slicing | 0.1.0 |
| 2021-05 | RAN5#91e | R5-213434 | - |  | - | Text Proposal for TR 38.918 on Test Equipment | 0.1.0 |
| 2021-05 | RAN5#91e | R5-213435 | - |  | - | Text Proposal for TR 38.918 on Test Equipment Connection Diagrams | 0.1.0 |
| 2021-08 | RAN5#92e | R5-216352 | - |  | - | Text Proposal on Test Configuration | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215247 | - |  | - | Text Proposal on Application Simulation | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215248 | - |  | - | Text Proposal on Analysis of mapping application to network slicing | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215249 | - |  | - | Text Proposal on Test Procedure A.2.1.1 | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215250 | - |  | - | Text Proposal on Test Procedure A.2.2.1 | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215251 | - |  | - | Text Proposal to Update References | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215252 | - |  | - | Editorial changes on wording | 0.2.0 |
| 2021-08 | RAN5#92e | R5-215283 | - |  | - | Text Proposal on Test Procedure A.3.1.1 | 0.2.0 |
| 2021-11 | RAN5#93e | R5-217264 | - |  | - | Text Proposal on Test Procedure A.2.2.2 | 0.3.0 |
| 2021-11 | RAN5#93e | R5-217265 | - |  | - | Text Proposal to Update Message Contents | 0.3.0 |
| 2021-11 | RAN5#93e | R5-217266 | - |  | - | Text Proposal to Update Network Slicing Configurations | 0.3.0 |
| 2021-11 | RAN5#93e | R5-217267 | - |  | - | Editorial changes on wording | 0.3.0 |