3GPP TSG-RAN5 Meeting #94-e	R5-220103

# Electronic Meeting, 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2173 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update chapter 4.5.1 General |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The connectivity NR is not fully aligned with TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The connectivity has been clarified to NR/5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN5 used RRC terminology might be slightly ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 4.5	Generic procedures

### 4.5.1	General

The generic procedures are used by test cases to get UE under test into SWITCHED_OFF, RRC_IDLE/Ipsec SA not established, RRC_INACTIVE, RRC_CONNECTED/Ipsec SA or Out of Coverage established state.

A test case controls the SS by specifying the required RRC state and a set of generic procedure parameters applicable for the intended testing.

The connectivity EN-DC and E-UTRA/EPC are both a UE connected to the EPC. The connectivity E-UTRA/5GC, NR/5GC, NGEN-DC, NE-DC and NR-DC are all a UE connected to the 5GC.

Table 4.5.1-1: Generic procedure parameters

| Parameter | Values | Description | Parameter condition |
| --- | --- | --- | --- |
| Connectivity | E-UTRA/5GC | E-UTRA connected to 5GC | Mandatory |
|  | NR/5GC | NR connected to 5GC |  |
|  | EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |  |
|  | NGEN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to 5GC |  |
|  | NE-DC | NR E-UTRA Dual Connectivity |  |
|  | NR-DC | NR-NR Dual Connectivity |  |
|  | WLAN | Un trusted non 3GPP access over WLAN |  |
|  | E-UTRA/EPC | E-UTRA connected to EPC |  |
| Bearers | MCG(s) and SCG | MCG and SCG | Mandatory when Connectivity is set to EN-DC, NGEN-DC, NE-DC and NR-DC and when the generic procedures are used by test cases to get UE under test into RRC_CONNECTED state. s=1 if MULTI_PDN= FALSE and s=2 if MULTI_PDN=TRUE. N/A otherwise. |
|  | MCG(s) and split | MCG and split |  |
| Test Mode | On | UE test mode active as specified in TS 38.509 [11], clause 5.2.2. | Optional |
| Test Loop Function | On | UE test mode active with one of the UE test loop modes activated as specified in TS 38.509 [11], clauses 5.2.2 and 5.3.2. | Optional |
| Connected without release | On | Enter RRC_Connected with Ipsec_SA_Established and without any release. | Optional N/A for NR-DC. |
| Interworking without N26 interface supported | On | The NWK sets the REGISTRATION ACCEPT message, IE 5GS network feature support, IWK N26 (octet 3, bit 7) = 1 | Optional, Depends on test case scenario. Default message content for REGISTRATION ACCEPT is set to Interworking without N26 interface NOT supported |
| Unrestricted nr PDN | On | Allow unrestricted numbers of PDNs. | Optional for Connectivity E-UTRA/EPC. N/A otherwise. |
| Sidelink | On | NR sidelink | Optional |
| Cast type | Unicast | To establish unicast sidelink and PC5-RRC connection. | Optional |
| GNSS Sync | On | To use GNSS as the synchronization reference source. | Optional |

Editor’s Note: The following values are not available to use in the current version of this specification because details are still FFS: Connectivity (E-UTRA/5GC and NGEN-DC).

<End of modified section>