3GPP TSG-RAN5 Meeting #94-e	R5-220110

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1581 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of SA FR1 TC 6.5.3.1 and 6.5.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current wording in steps 5 & 7 may cause misleading interpretation of starting time of T2 & T3, which are defined as below in the cases. The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reword steps 5 and 7 in 6.5.3.1 and 6.5.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The starting time of T2 and T3 in test steps will be inconsistent with the definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1.4.2 and 6.5.3.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle

6.5.3.1.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements in TS 38.133 [6] clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

6.5.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and supporting 2DL CA.

6.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.3.1.

6.5.3.1.4	Test description

6.5.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.3.1.4.1-1.

Table 6.5.3.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.5.3.1-1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 6.5.3.1-2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6.5.3.1-3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.3.1.4.1-2 and Table 6.5.3.1.4.1-3.

Table 6.5.3.1.4.1-2: Initial conditions for known FR1 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.3.1.5-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

Table 6.5.3.1.4.1-3: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. |

1.	Message contents are defined in clause 6.5.3.1.4.3.

2.	There are two NR carriers, each with one cell. Cell 1 is NR FR1 PCell and Cell 2 is the deactivated SCell. Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

6.5.3.1.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in TS 38.133 [6] clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3, where Ninterruption is the interruption length given in TS 38.133 [6] clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 6.5.3.1.4.1-3 and 6.5.3.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2. T1 starts.

3.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1 and provide measurement configurations.

3a.	The UE sends a MeasurementReport message.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n and T2 starts in slot n. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

6.	The UE shall start sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell during SCell activation.

-	If the first CSI report for SCell is received by the SS in a slot n+1+THARQ+3msNR slot length,

-	or slot n+1+THARQ+3ms+TXNR slot length+Ninterruption+1 if the slot n+1+THARQ+3msNR slot length was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length,

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length

-	and DTX is not observed by the SS outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 9.

7.	When T2 expires, the SS deactivate SCC by sending the deactivation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m and T3 starts in slot m. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 8, otherwise go to step 9.

8.	The UE shall stop sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell during SCell deactivation.

-	If the last CSI report is received by the SS earlier than or equal to slot m+THARQ+3msNR slot length

-	and DTX is not observed by the SS outside the slot m+1+THARQNR slot length to m+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

9.	When T3 expires, or Activation in step 5 was not acknowledged, or a fail was counted for the event “Activation” in step 6, or Deactivation in step 7 was not acknowledged, the SS shall transmit a RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

10.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

11.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

12.	Repeat steps 2-11 until a test verdict has been achieved.

Each of the events “Activation” and “Deactivation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

6.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-7 with Condition Deactivated SCell; Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.3.1.4.3-2: RRCReconfiguration: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 6.5.3.1.4.3-2A |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCell | Table 6.5.3.1.4.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.3.1.4.3-2A: MeasConfig (Table 6.5.3.1.4.3-2)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR for SCell | entry 2 Table 6.5.3.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 6.5.3.1.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-3: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-4: ReportConfigNR

| Derivation Path: Table H.3.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -30 | To ensure reporting can always be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-5: CellGroupConfig-SCell (Table 6.5.3.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present | PCell always uses servCellIndex=0 |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 6.5.3.1.4.3-6 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 6.5.3.1.4.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-6: ServingCellConfig-SpCell (Table 6.5.3.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 1 |  |  |
| } |  |  |  |

Table 6.5.3.1.4.3-7: ServingCellConfig-SCell (Table 6.5.3.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS and No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 2 |  |  |
| } |  |  |  |

6.5.3.1.5	Test requirement

Table 6.5.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.5.3.1.5-1: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 |  | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 |  | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  |  | periodic |  |  | N/A |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  | N/A |  |  |
| CSI reporting periodicity | Config 1,2 |  | slot | 5 |  |  | N/A |  |  |
|  | Config 3 |  |  | 10 |  |  | N/A |  |  |
| CSI reporting offset | Config 1,2 |  | slot | 2 |  |  | N/A |  |  |
|  | Config 3 |  |  | 4 |  |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |  |  |  |  |  |
| NocNote2 |  | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  |  | Config 3 |  | -101 |  |  |  |  |  |
| EsIot |  |  | dB | 17 |  |  |  |  |  |
| EsNoc |  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 |  | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  |  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  |  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell in a slot n+1+THARQ+3msNR slot length, UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in TS 38.133 [6] section 8.3.

During T2 interruption of PCell/PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] section 8.3.

Figures 6.5.3.1.5-1 shows the derivation of the Test procedure requirement for DTX during T2, based on the core requirements for interruption.

Figure 6.5.3.1.5-1: Procedure derivation for Activation

1)	Activation command for SCell

2)	ACK for MAC-CE for SCell1 activation

3)	First CSI report timing (could be invalid CQI)

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

Figures 6.5.3.1.5-2 shows the derivation of the Test procedure requirement for NR PSCell DTX during T3, based on the core requirements for interruption.

Figure 6.5.3.1.5-2: Procedure derivation for Deactivation

1)	Deactivation command for SCell

2)	Latest slot stop sending CSI reports for SCell

The interruption on any activated serving cell shall not be more than the values specified for SA in TS 38.133 [6] clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in TS 38.133 [6] section 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

[Skip unchanged text]

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX

6.5.3.3.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements stated in TS 38.133 [6] clause 8.3, when the SCell in FR1 is unknown by the UE at the time of activation.

6.5.3.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and supporting 2DL CA.

6.5.3.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.3.3.

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Same initial conditions as described in section 6.5.3.1.4.1 with following exception:

-	The supported test configurations is replaced by Table 6.5.3.3.4.1-1.

-	The listed parameter values in Tables 6.5.3.3.4.1-2 will replace the values of corresponding parameters in Tables 6.5.3.1.4.1-3.

Table 6.5.3.3.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.5.3.3-1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 6.5.3.3-2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6.5.3.3-3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table 6.5.3.3.4.1-2: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

6.5.3.3.4.2	Test procedure

Same test procedure as described in section 6.5.3.1.4.2, except step3 and step 5 are replaced by following steps:

3.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1. The SCell (Cell 2) shall be powered OFF till T2 starts.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n, power ON the SCell (Cell2), T2 starts in slot n. If the SS receives ACK for MAC-CE sent by the UE, the test proceeds to step 6, otherwise go to step 9.

and,

-	step 3a is removed.

6.5.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.3.3.4.3-1: RRCReconfiguration in step 3: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 6.5.3.3.4.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.3.3.4.3-2: CellGroupConfig (Table 6.5.3.3.4.3-1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 6.5.3.3.4.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 6.5.3.3.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.3.3.4.3-3: ServingCellConfig-SpCell (Table 6.5.3.3.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| } |  |  |  |

Table 6.5.3.3.4.3-4: ServingCellConfig-SCell (Table 6.5.3.3.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| } |  |  |  |

6.5.3.3.5	Test requirement

Same test requirement as described in section 6.5.3.1.5, except Tactivation_time will be replaced with the valueTFirstSSB+5msTFirstSSB_MAX+TSMTC_MAX+2*Trs+5ms TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms

[End of change]