3GPP TSG-RAN5 Meeting #94-e	R5-220131

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3936 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of date for GMM NITZ TC 12.2.1.13 and 12.2.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to OS limitation, e.g. Andriod S, the current date “04/05/08” is out of date and is not allowed to be set in UE when checking the NITZ by MMI. Therefore it is suggested to update the date to a new fixed date for conformance requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the date to a new value in test steps and specific message contents. Editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformat UE will fail this case unfarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.13.4 and 12.2.1.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

12.2.1.13	NITZ / GMM/ Time zone, Time and DST Handling

12.2.1.13.1	Definition

To check that the UE correctly handles time zone, time and DST.

12.2.1.13.2	Conformance requirement

The feature Network Identities and Timezone shall make it possible for a serving PLMN to transfer its current identity, universal time, DST and LTZ to MSs, and for the MS to store and use this information. Each one of these elements is optional. The feature significantly enhances roaming as it enables the accurate indication of network identities that are either newer than the ME or have changed their name since the ME was manufactured or sold. Additionally time and timezone information can be utilised by MEs as desired.

When using the default character set (see 3GPP TS 23.038 [3]), the serving PLMN shall make both a "short" and a "long" name available to the MS. As an alternative or, in addition, to the default character set, the serving PLMN can make a name available in UCS2. The MS shall be free to choose one of these names depending upon its own characteristics and/or limitations, such as those of its display.

NOTE:	Guidance is sought, particularly from non-European operators, as to whether long and short name is required in UCS2 format.

The Network Operator may change the network identity at any time. However the change of network identity need not force immediate transfer of information to the MS.

As a network option, it shall be possible to send universal time (UT) by the network. Time information shall include: Year, Month, Day, Hour, Minute, Second, Timezone and DST. The expected accuracy of the time information is in the order of minutes.

NOTE:	Universal time indicates the time at which this information element (see 3GPP TS 24.008 [2]) may have been sent by the network. Thus it can be assumed that the accuracy of the time information when it arrives at the MS is usually within a couple minutes.

The serving PLMN shall make Local Time Zone (LTZ) available to the MS as an offset from Universal Time in units of 15 minutes.

When the LTZ is compensated for DST (summertime), the serving PLMN shall provide a DST parameter to indicate this. The adjustment for DST can be +1h or +2h.

The TP-Service-Centre-Time-Stamp field is given in semi-octet representation, and represents the local time in the following way:

|  | Year: | Month: | Day: | Hour: | Minute: | Second: | Time Zone |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Digits: (Semi-octets) | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

The Time Zone indicates the difference, expressed in quarters of an hour, between the local time and GMT. In the first of the two semi-octets, the first bit (bit 3 of the seventh octet of the TP-Service-Centre-Time-Stamp field) represents the algebraic sign of this difference (0: positive, 1: negative).

The Service-Centre-Time-Stamp, and any other times coded in this format that are defined in the present document, represent the time local to the sending entity.

If the MS has knowledge of the local time zone, then any time received (e.g. Service-Centre-Time-Stamp) at the MS may be displayed in the local time rather than the time local to the sending entity. Messages shall be stored as received without change to any time contained therein.

The Time Zone code enables the receiver to calculate the equivalent time in GMT from the other semi-octets in the Service-Centre-Time-Stamp, or indicate the time zone (GMT, GMT+1H etc.), or perform other similar calculations as required by the implementation. The value contained in the Time Zone field must take into account daylight saving time, such that when the sending entity changes from regular (winter) time to daylight saving (summer) time, there is a change to the value in the Time Zone field, for example in the UK the winter setting is 00000000 and the summer setting is 01000000.

If the MS receives a non-integer value in the SCTS, it shall assume that the digit is set to 0 but shall store the entire field exactly as received.

If the local time zone has been adjusted for Daylight Saving Time, the network shall indicate this by including the IE Network Daylight Saving Time.

References

3GPP TS 22.042 sub clause 4

3GPP TS 23.040 sub clause 9.2.3.11

3GPP TS 24.008 sub clause 9.4.19.4

12.2.1.13.3	Test purpose

To verify that a UE supporting any of the NITZ Time related feature (Local Time Zone, Universal Time and DST IE (and thus GMM Information) is able to handle them properly.

12.2.1.13.4	Method of test

Related ICS/IXIT statements

Support of FDD Yes/No.

Support of PS bearer service Yes/No.

-	Support of Switch on off button Yes/No.

-	Support of Automatic GPRS attach procedure at switch on or power on Yes/No.

-	Support of NITZ DST Yes/No.

-	Support of NITZ Universal Time for PLMN Yes/No.

-	Support of NITZ Local Time Zone for PLMN Yes/No.

Initial conditions

System Simulator:

Two cells operating in network operation mode I, cell A in MCC1/MNC1/LAC1/RAC1 (RAI-1), cell B in MCC1/MNC1/LAC1/RAC2 (RAI-4).

User Equipment:

-	The UE is "idle updated" on cell A.

-	The UE is switched off with a valid USIM inserted.

Test procedure

Following the GPRS attachment on Cell A, SS sends its local time and date (on GMT+1, Winter Time) using the GMM INFORMATION Message to the UE. The test operator verifies then the parameters and/or the time and date stored in the MS.

The UE is then moved to Cell B, and after a ROUTING AREA UPDATE procedure, the time is changed to “Summer Time” with the DST IE using a GMM INFORMATION Message. The test operator verifies then the parameters and/or the time and date stored in the UE and switches Off the UE

The UE is then moved to Cell A, and after a ROUTING AREA UPDATE procedure the Time Zone is then changed, no DST present (GMT+2, Winter Time), using GMM INFORMATION message. The operator verifies then the parameters and/or the time stored on the UE.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non Suitable neighbour cell". The UE is powered up or switched on and initiates an attach (see PICS). The following messages are sent and shall be received on cell A |
| 2 | -> |  | ATTACH REQUEST |  |
| 3 | SS |  |  | The SS starts integrity protection. |
| 4 | <- |  | ATTACH ACCEPT | Attach result = Attach type in step 2 Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 5 | -> |  | ATTACH COMPLETE |  |
| 6 | <- |  | GMM INFORMATION | Universal Time IE is included: “30/05/08 04:15:00” for Local Time “+ 1 hour” for Timezone No DST or Local Time Zone IE included See specific message content |
| 7 | SS |  |  | The SS releases the RRC connection |
| 8 | UE |  |  | Operator Action: The use of the supported Fields is checked: Universal Time: Year: 2030 Month: May Day: 8th Hour: 5 Hours Minute: 15 Minutes Timezone: GMT+1 Local Time Zone: Not sent DST: Daylight Saving Time not in use (i.e. winter time) (See Note) |
| 9 | SS |  |  | Set the cell type of cell B to the "Serving cell". Set the cell type of cell A to the "Suitable neighbour cell". The following messages are sent and shall be received on cell B |
| 10 | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-2 signature Routing area identity = RAI-1 |
| 11 | SS |  |  | The SS starts integrity protection. |
| 12 | <- |  | ROUTING AREA UPDATING ACCEPT | Update result = Update type in step 10 Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-4 |
| 13 | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 14 | <- |  | GMM INFORMATION | Local Time Zone IE and DST IE are included: “+ 2 hours” for Timezone (including DST) “+ 1 hour” for DST See specific message content |
| 15 | SS |  |  | The SS releases the RRC connection |
| 16 | UE |  |  | Operator Action: The use of the supported Fields is checked: Universal Time Year: 2030 Month: May Day: 8th Hour: 6 Hours Minutes: 15 Minutes Timezone: GMT+2 Local Time Zone Timezone: GMT+2 (DST included) DST: Daylight Saving Time in use (i.e. “summer time”) (See Note) |
| 17 | SS |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Suitable neighbour cell". The following messages are sent and shall be received on cell A |
| 18 | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-1 signature Routing area identity = RAI-4 |
| 19 | SS |  |  | The SS starts integrity protection. |
| 20 | <- |  | ROUTING AREA UPDATING ACCEPT | Update result = Update type in step 18 Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 21 | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 22 | <- |  | GMM INFORMATION | Only Local Time Zone IE is included: “+ 2 hours” for Timezone No DST IE included See specific message content |
| 23 | SS |  |  | The SS releases the RRC connection |
| 24 | UE |  |  | Operator Action: The use of the supported Fields is checked: Universal Time: Year: 2030 Month: MayDay: 8th Hour: 6 Hours Minutes: 15 Minutes Timezone: GMT+2 Local Time Zone: Timezone: GMT+2 DST: Daylight Saving Time not in use (i.e. winter time) (See Note) |
| Note: In step 6, 16 and 24 the “minute” is not so relevant and can be higher than “15” depending on operator’s action time. The check of Timezone and DST is done implicitly by checking the time only in case UE does not support the display of these two fields. Year is set to a fixed value. Steps 9-16 resp. 17-24 are only performed if the UE supports pc_LocalTimeZone. In Steps 8, 16, 24 Local Time Zone is checked. DST is to be checked only if the UE supports pc_DaylightSavingTime. |  |  |  |  |

Specific message contents:

GMM Information on step 6:

| Information element | Value/remark |
| --- | --- |
| Universal Time IE | 47 |
| Year | 03 - 2030 |
| Month | 50 - May |
| Day | 80 - 8th |
| Hour | 40 - 4 hours |
| Minute | 51 - 15 Minutes |
| Second | 00 - 0 second |
| Time Zone | 40 - GMT+1 (4*15 minutes+ 0*15 minutes DST) |

GMM Information on step 14:

| Information element | Value/remark |
| --- | --- |
| Local Time Zone IE | 46 |
| Time Zone | 80 - GMT+1+1(4*15 minutes+ 4*15 minutes DST) |
| Daylight Saving Time IE | 49 |
| Length of DST Content | 1 |
| Value | 1 - + 1 hour (summer time) |

GMM Information on step 22:

| Information element | Value/remark |
| --- | --- |
| Local Time Zone IE | 46 |
| Time Zone | 80 - GMT+2 (8*15 minutes+ 0*15 minutes DST) |

12.2.1.13.5	Test requirements

In step 6, 16 and 24 the UE populates the supported fields with the correct values.

[Skip unchanged text]

12.2.1.15	NITZ / GMM/ / MM and GMM Signalling

12.2.1.15.1	Definition

To check that the UE correctly stores NITZ parameters and displays the latest received NITZ parameters.

12.2.1.15.2	Conformance requirement

The feature Network Identities and Timezone shall make it possible for a serving PLMN to transfer its current identity, universal time, DST and LTZ to MSs, and for the MS to store and use this information. Each one of these elements is optional. The feature significantly enhances roaming as it enables the accurate indication of network identities that are either newer than the ME or have changed their name since the ME was manufactured or sold. Additionally time and timezone information can be utilised by MEs as desired.

When using the default character set (see TS 23.038 [4]), the serving PLMN shall make both a "short" and a "long" name available to the MS. As an alternative or, in addition, to the default character set, the serving PLMN can make a name available in UCS2. The MS shall be free to choose one of these names depending upon its own characteristics and/or limitations, such as those of its display.

It is expected that the MS will display the most up to date information transferred to it.

References

3GPP TS 22.042 sub clause 4 and 6.2

12.2.1.15.3	Test purpose

To verify that a UE supporting NITZ (any of the Fields) is able to handle both MM INFORMATION and GMM INFORMATION messages and those parameters set in the latest message override the previous ones.

12.2.1.15.4	Method of test

Related ICS/IXIT statements

Support of FDD Yes/No.

Support of PS bearer service Yes/No.

-	Support of Switch on off button Yes/No.

-	Support of Automatic GPRS attach procedure at switch on or power on Yes/No.

-	Support of NITZ DST Yes/No.

-	Support of NITZ Universal Time for PLMN Yes/No.

-	Support of NITZ Local Time Zone for PLMN Yes/No.

-	Support of NITZ Full name for PLMN Yes/No.

-	Support of NITZ Short name for PLMN Yes/No.

Initial conditions

System Simulator:

Two cells operating in network operation mode I, cell A in MCC1/MNC1/LAC1/RAC1 (RAI-1), cell B in MCC1/MNC1/LAC1/RAC2 (RAI-4).

User Equipment:

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | Value |
| --- | --- |
| EFUST | Services 19 and 51 are not supported |

The UE is "idle updated" on cell A.

The UE is switched off.

Test procedure

Following its GPRS Attachment procedure, the UE receives from SS, through the GMM INFORMATION a complete set of NITZ parameters (PLMN long name, short name, and Universal time with local time adjustment). The operator verifies then the names used and the time and date stored in the UE (according to specific PICS Statements).

The UE is then paged on its paging group with its IMSI. SS verifies that the UE sends a PAGING RESPONSE. SS sends back a MM INFORMATION message changing the NITZ PLMN names (short and full names) and the local time information.

The operator verifies then that the new NITZ names are used and the time information changed (if supported).

The serving cell is then lowered and the UE triggers a ROUTING AREA UPDATE procedure on the second cell. Following this procedure, SS sends some new NITZ PLMN names (full and short) using the GMM INFORMATION message.

The operator verifies then that the new NITZ names are used, if supported, and, if possible, that the time information is not changed.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non Suitable neighbour cell". The following messages are sent and shall be received on cell A The UE is powered up or switched on and initiates an attach (see PICS). The following messages are sent and shall be received on cell A |
| 2 | -> |  | ATTACH REQUEST |  |
| 3 | SS |  |  | The SS starts integrity protection. |
| 4 | <- |  | ATTACH ACCEPT | Attach result = Attach type in step 2 Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 5 | -> |  | ATTACH COMPLETE |  |
| 6 | <- |  | GMM INFORMATION | Full Name for Network, Short Name for Network and Universal Time IE are included: “NITZ GMM PLMN” for Full Name “GMM PLMN” for Short Name “30/05/08 04:15:00” for Local Time “+ 1 hour” for Timezone No DST or Local Time Zone IE included See specific message content |
| 7 | SS |  |  | The SS releases the RRC connection |
| 8 | UE |  |  | Operator Action: Verify that the names and time are stored and handled correctly according to specific PICS Statements: “NITZ GMM PLMN” for Full name, “GMM PLMN” for Short name The use of the supported Fields is checked: Universal Time: Year: 2030 Month: May Day: 8th Hour: 5 Hours Minute: 15 Minutes Timezone: GMT+1 Local Time Zone: Not Sent DST: Daylight Saving Time not in use (i.e. “winter time” See Note |
| 9 |  |  | Mobile terminated establishment of Radio Resource Connection | See TS 34.108 clause 7.1.2 "Initial UE identity" = TMSI1. Establishment Cause: Terminating Conversation Call. |
| 10 |  |  | PAGING RESPONSE | "Mobile identity" =TMSI1 |
| 11 |  |  | AUTHENTICATION REQUEST |  |
| 12 |  |  | AUTHENTICATION RESPONSE |  |
| 13 | SS |  |  | The SS starts CS domain integrity protection. |
| 14 | <- |  | MM INFORMATION | Full Name for Network, Short Name for Network, Local Time Zone and DST IE are included: “NITZ MM PLMN” for Full Name “MM PLMN” for Short Name “+ 2 hours” for Timezone (including DST) “+ 1 hour” for DST See specific message content |
| 15 | SS |  |  | The SS releases the RRC connection |
| 16 |  |  |  | Operator Action: Verify that the names and time are stored and handled correctly according to specific PICS Statements: “NITZ MM PLMN” for Full name, “MM PLMN” for Short name The use of the supported Fields is checked: Universal Time: Year: 2030 Month: May Day: 8th Hour: 6 Hours Minute: 15 Minutes Timezone: GMT+2 Local Time Zone: Timezone: GMT+2 (including DST) DST: Daylight Saving Time in use (i.e. “summer time”) See Note |
| 17 | SS |  |  | Set the cell type of cell B to the "Serving cell". Set the cell type of cell A to the "Suitable neighbour cell". The following messages are sent and shall be received on cell B |
| 18 | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-2 signature Routing area identity = RAI-1 |
| 19 | SS |  |  | The SS starts integrity protection. |
| 20 | <- |  | ROUTING AREA UPDATING ACCEPT | Update result = Update type in step 18 Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-4 |
| 21 | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 22 | <- |  | GMM INFORMATION | Full Name for Network and Short Name for Network IE are included: “NITZ GMM PLMN” for Full Name “GMM PLMN” for Short Name See specific message content |
| 23 | SS |  |  | The SS releases the RRC connection |
| 24 | UE |  |  | Operator Action: Verify that the names and time are stored and handled correctly according to specific PICS Statements: “NITZ GMM PLMN” for Full name, “GMM PLMN” for Short name if possible, verify that no change is done on the time information |
| Note: In step 8, 16 and 24 the “minute” is not so relevant and can be higher than “15” depending on operator’s action time. The check of Timezone and DST is done implicitly by checking the time only in case UE does not support the display of these two fields. Year is set to a fixed value. In steps 9-16 Local Time Zone Information is included only if the UE supports pc_LocalTimeZone. In Steps 8, 16 Local Time Zone is checked. DST is to be checked only if the UE supports pc_DaylightSavingTime. |  |  |  |  |

Specific message contents:

GMM Information, Step 6:

| Information element | Value/remark |
| --- | --- |
| Full Name IE | 43 |
| Length | 0D (13 octets) |
| Ext | 1 x x x x x x x (Spare Bit) |
| Coding scheme | x 0 0 0 x x x x (Cell Broadcast data coding scheme, GSM default alphabet, language unspecified) |
| Add CI | x x x x 0 x x x (Initials Country not included) |
| Spare Bit in final Octet | x x x x x 1 0 1 (Bit 4 to 8 of last Octet are set to ‘0’) |
| Text String | CE 24 55 0B 3A 36 9B 20 28 B3 E9 04 |
|  | (“NITZ GMM PLMN“) |
| Short Name IE | 45 |
| Length | 08 (8 octets) |
| Ext | 1 x x x x x x x (Spare Bit) |
| Coding scheme | x 0 0 0 x x x x (Cell Broadcast data coding scheme, GSM default alphabet, language unspecified) |
| Add CI | x x x x 0 x x x (Initials Country not included) |
| Spare Bit in final Octet | x x x x x 0 0 0 (“no information about the number of spare bits in last octet”) |
| Text String | C7 66 13 04 65 36 9D (“GMM PLMN“) |
| Universal Time IE | 47 |
| Year | 03 - 2030 |
| Month | 50 - May |
| Day | 80 - 8th |
| Hour | 40 - 4 hours |
| Minute | 51 - 15 Minutes |
| Second | 00 - 0 second |
| Time Zone | 40 - GMT+1 (4*15 minutes+ 0*15 minutes DST) |
| Note:The bit values of x |  |

MM Information, Step 14:

| Information element | Value/remark |
| --- | --- |
| Full Name IE | 43 |
| Length | 0C (12 octets) |
| Ext | 1 x x x x x x x (Spare Bit) |
| Coding scheme | x 0 0 0 x x x x (Cell Broadcast data coding scheme, GSM default alphabet, language unspecified) |
| Add CI | x x x x 0 x x x (Initials Country not included) |
| Spare Bit in final Octet | x x x x x 1 0 0 (Bit 5 to 8 of last Octet are set to ‘0’) |
| Text String | CE 24 55 0B 6A 36 41 50 66 D3 09 |
|  | (“NITZ MM PLMN“) |
| Short Name IE | 45 |
| Length | 08 (8 octets) |
| Ext | 1 x x x x x x x (Spare Bit) |
| Coding scheme | x 0 0 0 x x x x (Cell Broadcast data coding scheme, GSM default alphabet, language unspecified) |
| Add CI | x x x x 0 x x x (Initials Country not included) |
| Spare Bit in final Octet | x x x x x 1 1 1 (Bit 2 to 8 of last Octet are set to ‘0’) |
| Text String | CD 26 08 CA 6C 3A 01 (“MM PLMN“) |
| Local Time Zone IE | 46 |
| Time Zone | 80 - GMT+1+1(4*15 minutes+ 4*15 minutes DST) |
| Daylight Saving Time IE | 49 |
| Length of DST Content | 1 |
| Value | 1 - + 1 hour (summer time) |

GMM Information, Step 22:

| Information element | Value/remark |
| --- | --- |
| Full Name IE | 43 |
| Length | 0D (13 octets) |
| Ext | 1 x x x x x x x (Spare Bit) |
| Coding scheme | x 0 0 0 x x x x (Cell Broadcast data coding scheme, GSM default alphabet, language unspecified) |
| Add CI | x x x x 0 x x x (Initials Country not included) |
| Spare Bit in final Octet | x x x x x 1 0 1 (Bit 4 to 8 of last Octet are set to ‘0’) |
| Text String | CE 24 55 0B 3A 36 9B 20 28 B3 E9 04 |
|  | (“NITZ GMM PLMN“) |
| Short Name IE | 45 |
| Length | 08 (8 octets) |
| Ext | 1 x x x x x x x (Spare Bit) |
| Coding scheme | x 0 0 0 x x x x (Cell Broadcast data coding scheme, GSM default alphabet, language unspecified) |
| Add CI | x x x x 0 x x x (Initials Country not included) |
| Spare Bit in final Octet | x x x x x 0 0 0 (“no information about the number of spare bits in last octet”) |
| Text String | C7 66 13 04 65 36 9D (“GMM PLMN“) |

12.2.1.15.5	Test requirements

In step 8, 16 and 24 the UE populates the supported fields with the correct received values.

[End of change]