3GPP TSG-RAN5 Meeting #94-e	R5-220179

# Online, ,February 21 –March 04, 2022

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.918 | CR | n/a | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Test Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the test configuration for FQDN and IP 3 tuples based URSP rules |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated subclause 5.2.3 to add network slicing configurations - Updated subclause 5.2.5 to add FQDN and IP 3 tuples related parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.918 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

5.2.3	Network Slicing Configurations

Table 5.2.3-1: Network Slicing Configurations

| Application | OS App id | DNN | FQDN | IP 3 Tuples | S-NSSAI |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SST | SD |
| APP A | pc_OS_App_ID | pc_APN_ID_Specific | pc_Des_FQDN | pc_IP_Address, pc_Protocol_ID, pc_Sinlge_Remote_Port | ‘00000001'B | 0x000001 |
| APP B | FFS | FFS | FFS | FFS | ‘00000001'B | 0x000002 |

5.2.4	Message and Information Elements Contents

The common message and information elements contents are defined in TS 38.508-1 [6] clause 4. In addition, the following message and information elements contents apply:

–	MANAGE UE POLICY COMMAND

Table 5.2.4-1: MANAGE UE POLICY COMMAND

| Derivation Path: TS 24.501 Table D.5.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| MANAGE UE POLICY COMMAND message identity |  | ’0000 0001’B |  |  |
| UE policy section management list |  | See Table 5.2.4-2 |  |  |

–	UE policy section management list

Table 5.2.4-2: UE policy section management list

| Derivation Path: TS 24.501 Figure D.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management list IEI |  |  |  |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  |  |  |  |
| UE policy section management sublist (PLMN-1) |  |  |  |  |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management sublist contents |  |  |  |  |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UE policy section contents |  |  |  |  |
| UE policy part |  | See Table 5.2.4-3 |  |  |

–	UE policy part

Table 5.2.4-3: UE policy part with UE policy part type = {URSP}

| Derivation Path: TS 24.501 Figure D.6.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B |  | URSP |
| UE policy part contents |  |  |  |  |
| URSP rule 1 |  | See Table 4.8.5.2-1 in TS 38.508-1 |  |  |
| URSP rule 2 |  | See Table 4.8.5.2-1 in TS 38.508-1 |  |  |

5.2.5	UE Specific Items

The following parameters are recommended to be provided by UE or application supplier to indicate the information of application using network slicing during test.

| Parameter Name | Parameter Type | Description |
| --- | --- | --- |
| pc_APN_ID_Specific | charstring | APN/DNN value of the simulation application client defined in clause 5.3.1 The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [11], subclause 9.1 |
| pc_OS_App_ID | charstring | OS App Id value of the simulation application client defined in clause 5.3.1 The coding of OS App Id is up to UE implementation |
| pc_Des_FQDN | charstring | The destination FQDN value of Application Server Simulator(s) defined in clause 5.2.1 The destination FQDN as defined in subclause 28.3.2.1 in 3GPP TS 23.003 [11] |
| pc_Des_FQDN | charstring | The destination FQDN value of Application Server Simulator(s) defined in clause 5.2.1 The destination FQDN as defined in subclause 28.3.2.1 in 3GPP TS 23.003 [11] |
| pc_IP_Address | charstring | The IP address of Application Server Simulator(s) defined in clause 5.2.1 The coding of IP address is four octet IPv4 address and 4 octet IPv4 address mask for IPv4 or sixteen octet IPv6 address and one octet prefix length for IPv6 |
| pc_Protocol_ID | charstring | The IPv4 protocol identifier or IPv6 next header of Application Server Simulator(s) defined in clause 5.2.1 |
| pc_Sinlge_Remote_Port | charstring | The port number of Application Server Simulator(s) defined in clause 5.2.1 |

<<< END OF CHANGES >>>