3GPP TSG-RAN5 Meeting #94-e	R5-220192

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2702 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MDT TC 8.1.6.3.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Release information is missing in the case. FFS and Bluetooth beacon 1/2 should be clarified according to TS 38.508-1. The UE state in the preamble shall be 3N-A. Cell paramenter and unit shall be updated to NR Cell. Specific message contents of LoggedMeasurementConfiguration and RRCSetupComplete could be simplified based on the default message in TS 38.508-1. The message content of UEInformationResponse should align with TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Release information was added. Bluetooth beacon 1 & 2 were clarified as 40 and 41 and Editor`s Note was removed. UE state was changed to 3N-A in the preamble. Power level tables were corrected. Specific message contents were updated according to TS 38.508-1 and TS 38.331. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.6.3.2.1	Inter-System MDT / Logged MDT / Logging and reporting / Bluetooth measurement collection

8.1.6.3.2.1.1	Test Purpose (TP)

(1)

with { UE received RRCSetup message }

ensure that {

when { UE has Bluetooth logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the logMeasAvailableBT in the RRCSetupComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state and UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 including logMeasResultListBT }

}

(3)

with { UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a Bluetooth beacon is not included in BT-NameList }

ensure that {

when { UE in NR RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 not including the measurement result of the Bluetooth beacon not included in BT-NameList }

}

8.1.6.3.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.5a.1.3, 5.5a.3.2, 5.3.3.7. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-NameList, if included, in VarLogMeasConfig;

1>	store the received sensor-NameList, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]); or

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

…

2>	when performing the logging:

…

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

8.1.6.3.2.1.3	Test description

8.1.6.3.2.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- Cell 40 and Cell 41 are configured as per 38.508-1 [4] cl 4.4.1.3.

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.2.1.3.2	Test procedure sequence

Table 8.1.6.3.2.1.3.2-1/2 illustrate the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.2.1.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.1.6.3.2.1.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.1.6.3.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCRequest message on Cell 1. | --> | RRCRequest | - | - |
| 6 | SS transmit an RRCSetup message on Cell 1. | <-- | RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailableBT on Cell 1? | --> | RRCSetupComplete | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r16 with one entry (Bluetooth beacon 1) measurement result in the IE logMeasResultListBT? | --> | UEInformationResponse | 2, 3 | P |

8.1.6.3.2.1.3.3	Specific message contents

Table 8.1.6.3.2.1.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4] table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16-IEs SEQUENCE { |  |  |  |
| bt-NameList-r16 SEQUENCE { SIZE (1..maxBT-Name-r16)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 3GPP TS 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.2.1.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.3.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4] table 4.6.1-22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailable-r16 |  | true |  |  |  |  |  |
| logMeasAvailableBT-r16 |  | true |  |  |  |  |  |
| logMeasAvailableWLAN-r16 |  | Not Present |  |  |  |  |  |
| connEstFailInfoAvailable-r16 |  | Not Present |  |  |  |  |  |
| rlf-InfoAvailable-r16 |  | Not Present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.6.3.2.1.3.3-3: UEInformationRequest (step 12, Table 8.1.6.3.2.1.3.2-3)

Derivation Path: TS 38.508-1 [4] table 4.6.1-32A with Condition LOG

Table 8.1.6.3.2.1.3.3-4: UEInformationResponse (step 13, Table 8.1.6.3.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceRecordingSessionRef-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { |  | At least 3 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  |  | entry x |  |
| locationInfo-r16 SEQUENCE { |  |  |  |  |
| commonLocationInfo-r16 |  | Not checked |  |  |
| bt-LocationInfo-r16 SEQUENCE { |  |  |  |  |
| LogMeasResultListBT-r16 ::= SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 SEQUENCE { |  | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 |  | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 |  | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| relativeTimeStamp-r16 |  | SS record the value |  |  |
| servCellIdentity-r16 |  | Same as Cell 1 |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |  |
| resultsSSB-Cell-r16 |  | MeasQuantityResults of Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |  |
| best-ssb-Index |  | Not checked |  |  |
| best-ssb-Results |  | Not checked |  |  |
| numberOfGoodSSB |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells-r16 |  | Any allowed value |  |  |
| anyCellSelectionDetected-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[End of change]