3GPP TSG-RAN5 Meeting #94-e	R5-220211

# Online, , 21st Feb - 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1487 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TC 22.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RFC 4028 on Session Timer says in clause 7.3: If the response to a session refresh request is a 422 (Session Interval Too Small) response message, then the UAC MAY retry the request. The procedures for retrying are described in Section 7.4. This new request constitutes a new transaction and SHOULD have the same value as the Call-ID, To, and From of the previous request, but the CSeq should contain a new sequence number that is one higher than the previous. This implies that the UE can also use a new dialog rather than re-trying to create the same dialog |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed requirements on Call-ID, CSeq, To, and From headers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overly strict test spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 22.1	MO Call – UE is able to refresh the session

### 22.1.1	Definition

Test to verify that a UE supporting and using Session timer as described in RFC 4028 [146], and configured to be the refresher and triggered to perform an IMS mobile originated voice call when using IMS Multimedia Telephony with preconditions, correctly negotiates the Session-Expires header, processes received 422 Session Interval Too Small responses, and keeps the session alive as negotiated. This process is described in IR.92 [133] clause 2.2.8, RFC 4028 [146] sections 7.1-7.4 and 3GPP TS 24.229 [10], clause 5.1.2A.1.1.

### 22.1.2	Conformance requirement

[TS 24.229 clause 5.1.2A.1.1]

A UE supporting RFC 4028 [58], when it receives a 422 (Session Interval Too Small) to an INVITE request where the response contains a Min-SE header field, shall retry the request in accordance with RFC 4028 [58] subclause 7.4.

[TS 24.229 clause 5.2.7.2]

When the P-CSCF receives from the UE an INVITE request, the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1: Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it.

[TS 24.229 clause 5.2.7.3]

When the P-CSCF receives an INVITE request destined for the UE the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1: Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it in order to make it work.

[TS 24.229 clause 5.4.5.3]

If the S-CSCF requested the session to be refreshed periodically, and the S-CSCF got the indication that the session will be refreshed, when the session timer expires, the S-CSCF shall delete all the stored information related to the dialog.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.2A.1.1, 5.2.7.2, 5.2.7.3, and 5.4.5.3.

### 22.1.3	Test purpose

1)	To verify that, when setting up an MO call, the UE performs correct exchange of SIP protocol signalling messages for Session Timer extension; and

2)	To verify that within SIP signalling the UE is able to handle 422 Session Interval Too Small responses by retrying the initial INVITE request (as described by RFC 4028 [10], section 7.4); and

3)	To verify that the UE is able to refresh the session using UPDATE based on the session expiration value negotiated in SIP signalling during session set up; and

4)	To verify that the UE is able to keep the session active during session refreshes until released; and

5)	To verify that the UE does not change the role of refresher during session refreshes.

### 22.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2  up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS speech call | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 2 |  |  | INVITE | UE sends INVITE as described in C.21, step 2, with either the Session-Expires value set to 1800 or no Session-Expires header. |
| 3 |  |  |  |  |
| - |  |  | EXCEPTION: Steps 4a0 to 4a7 describe behaviour that depends on UE capability: the "lower case letter" identifies a step sequence that takes place if the UE included Session-Expires in step 2 |  |
| 4a0 |  |  | 100 Trying | SS sends 100 Trying response. |
| 4a1 |  |  | 422 Session Interval Too Small | SS sends 422 Session Interval Too Small response with Min-SE value of 1860. |
| 4a2 |  |  | ACK | UE sends ACK. |
| 4a3 |  |  | INVITE | UE sends INVITE with Min-SE value and Session-Expires value being 1860. |
| 4a4 |  |  | 100 Trying | SS sends 100 Trying response. |
| 4a5 |  |  | 422 Session Interval Too Small | SS sends 422 Session Interval Too Small response with Min-SE value of 1920. |
| 4a6 |  |  | ACK | UE sends ACK. |
| 4a7 |  |  | INVITE | UE sends INVITE with Min-SE value and Session-Expires value being 1920. |
| 5-13 |  |  | Steps 3-11 defined in annex C.21 |  |
| 14 |  |  | 200 OK | SS sends 200 OK for INVITE with negotiated Session-Expires value set to 1920 and refresher value set to uac. |
| 15 |  |  | ACK | UE sends ACK. |
| 16 |  |  | UPDATE | 960 seconds after step 15, UE sends an UPDATE request to refresh the session. |
| 17 |  |  | 200 OK | SS sends 200 OK for UPDATE. |
| 18 |  |  | UPDATE | 960 seconds after step 17, UE sends an UPDATE request to refresh the session. |
| 19 |  |  | 200 OK | SS sends 200 OK for UPDATE. |
| 20-23 |  |  | Steps defined in annex C.33 | SS releases the call. |

Specific Message Contents

INVITE (Step 2)

Use the default Message "INVITE" in Annex C.21 with conditions A1, A3, A4 and A26 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires | (if present) |
| delta-seconds | 1800 |
| refresher | uac (if present) |

422 Session Interval Too Small (Step 4a1)

Use the default Message "422 Session Interval Too Small" in Annex A.2.24 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Min-SE |  |
| delta-seconds | 1860 |

INVITE (Step 4a3)

Use the default Message "INVITE" in Annex C.21 with conditions A1, A3, A4 and A26 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires |  |
| delta-seconds | 1860 |
| Min-SE |  |
| delta-seconds | 1860 |

422 Session Interval Too Small (Step 4a5)

Use the default Message 422 Session Interval Too Small in Annex A.2.24 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Min-SE |  |
| delta-seconds | 1920 |

INVITE (Step 4a7)

Use the default Message "INVITE" in Annex A.2.1 with conditions A1, A3, A4 and A26 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires |  |
| delta-seconds | 1920 |
| Min-SE |  |
| delta-seconds | 1920 |

183 Session Progress (Step 6)

Use the default Message "183 Session Progress for INVITE" in Annex A.2.3 with condition A1 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |

200 OK (Step 14)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with conditions A1 and A10 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |
| Require | timer |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1920 |
| refresher | uac |
| Min-SE |  |
| delta-seconds | 1920 |

UPDATE (Steps 16 and 18)

Use the default Message "UPDATE" in Annex A.2.5 with condition A1 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1920 |
| refresher | uac |
| Min-SE |  |
| delta-seconds | 1920 |
| Content-Type | any value if present |

200 OK (Steps 17 and 19)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with conditions A1 and A10 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1920 |
| refresher | uac |
| Min-SE |  |
| delta-seconds | 1920 |

# *** End of Changes ***