3GPP TSG-RAN5 Meeting #94-e	R5-220213

# Online, , 21st Feb - 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0303 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for IMS5GS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Table A.12: PICS in A.12/62 was added for Data Off test cases but then not used in their applicability statements in TS 38.523-2. Analogously to applicability of TC 7.4, the applicability statement of other IMS5GS test cases should expanded where necessary. Title of TC 10.11 is wrong As R5-220238 proposes the change of some test case titles, this should also be reflected in the applicability table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table A.12: Voided pc_PS_data_off Expanded applicability statements, including new capabilities for NG.114. Fixed title entry for TC 10.11 Aligned test case titles with R5-200238 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unused capability for PS data off remains. Incomplete applicability statements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, A.4.5, A.4.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0198 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 4.2	Applicabilities for test cases specified in TS 34.229-5

Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title |  | Release | Applicability | Comments |
| --- | --- | --- | --- | --- | --- |
| Registration |  |  |  |  |  |
| 6.1 | Initial Registration / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR (see Note 1 below) |
| 6.2 | Initial Registration Failures / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.3 | Re-Registration Scenarios / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.4 | De-Registration Scenarios / 5GS |  | Rel-15 | C13 | UE supports IMS security and NR and IMS de-registration |
| 6.5 | Void |  |  |  |  |
| 6.6 | Re-Registration after capability update / 5GS |  | Rel-15 | C15 | UE supports IMS security and NR and Enabling/Disabling SMS over IMS |
| 6.7 | Authentication / MAC Parameter Invalid / Only two consecutive challenges / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.8 | Authentication / Security Server missing / SQN out of range / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.9 | Subscription / 503 Service Unavailable / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| Call Control |  |  |  |  |  |
| 7.1 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer |
| 7.2 | MTSI MO Voice Call / 504 Server Time-out / 5GS | Rel-15 |  | C03 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer |
| 7.3 | Void |  |  |  |  |
| 7.4 | MTSI MO Voice call with preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C14 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.4a | MTSI MO Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS |  | Rel-15 | C44 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG.114 v1.0 default configuration EVS/Br and NG.114 v1.0 default configuration EVS/Bw |
| 7.5 | MTSI MO Voice call without preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C27 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer |
| 7.6 | MTSI MT Voice Call with preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C04 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.6a | MTSI MT Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS |  | Rel-15 | C44 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.7 | MTSI MT Voice Call without preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C28 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions and NG114 v1.0 and NG114 answer to initial SDP offer |
| 7.8 | MTSI MT Voice Call without preconditions at originating UE and with preconditions at terminating UE / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 answer to initial SDP offer |
| 7.9 | MTSI MT Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 |  | C28 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions and NG114 v1.0 and NG114 answer to initial SDP offer |
| 7.10 | MTSI MT Voice call without preconditions and without SDP offer in MT INVITE / 5GS | Rel-15 |  | C28 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 initial SDP offer |
| 7.11 | MTSI MT Voice call without preconditions at terminating UE and originating UE requiring them / 5GS | Rel-15 |  | C05 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and (not preconditions or (preconditions and disabling preconditions)) |
| 7.12 | MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer |
| 7.13 | MTSI MT Voice Call with RTCP disabled / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.14 | MTSI MO Video Call with preconditions at both originating and terminating UE / 5GS | Rel-15 |  | C16 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.15 | MTSI MO Video call without preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C31 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer |
| 7.16 | MTSI MT Video call with preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C32 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.17 | MTSI MT Video call without preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C33 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer |
| 7.18 | MTSI MO Voice Call / EVS / AMR-WB / 5GS | Rel-15 |  | C14 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.19 | MTSI MT Voice Call / EVS / AMR-WB IO mode / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.20 | MTSI MO Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C16 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.21 | MTSI MO Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C31 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions and disabling preconditions and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.22 | MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C32 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.23 | MTSI MT Voice Call / add video and remove video / without preconditions at both originating UE and terminating UE / 5GS | Rel-15 |  | C33 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer |
| 7.24 | MTSI MT Voice Call / Forking / UE receives CANCEL request for a forked MT voice call / 5GS | Rel-15 |  | C14 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and preconditions and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1_0 forking |
| 7.24a | MTSI MO Voice Call / Forking / UE receives two preliminary responses and one early dialog termination / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer and NG114 v1_0 forking |
| 7.24b | MTSI MO Voice Call / UE receives two preliminary responses and one final response / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer and NG114 v1_0 forking |
| 7.25 | MTSI MT Voice call without SDP offer in INVITE / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 and NG114 v1.0 initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.26 | Mobile Originating CAT / Forking Model / MO Voice Call / 5GS | Rel-15 |  | C17 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and early media and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.27 | Session Timer / MO Voice Call / UE is able to refresh the session / 5GS | Rel-15 |  | C18 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.28 | Session Timer / MO Voice Call / Remote end is refresher / 5GS | Rel-15 |  | C18 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.29 | Session Timer / MO Voice Call / Remote end does not support Session Timer / 5GS | Rel-15 |  | C29 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and Session Timer for own benefit and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.30 | Session Timer / MO Voice Call / Remote end supports but does not use Session Timer / 5GS | Rel-15 |  | C18 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 7.31 | Session Timer / MT Voice Call / Remote end supports but does not send Session-Expires / 5GS | Rel-15 |  | C19 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.32 | Session Timer / MT Voice Call / Remote end sends Session-Expires but does not choose refresher / 5GS | Rel-15 |  | C19 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 7.33 | Session Timer / MT Voice Call / Remote end chooses UE as refresher / 5GS | Rel-15 |  | C30 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and re-INVITE for session refresh and NG114 v1.0 and NG114 answer to initial SDP offer |
| 7.34 | Session Timer / MT Voice Call / Remote end does not support Session Timer / 5GS | Rel-15 |  | C19 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| Supplementary Services |  |  |  |  |  |
| 8.1 | Originating Identification Presentation / Configuration / 5GS |  | Rel-15 | C06 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and MTSI Originating Identification Presentation |
| 8.2 | Originating Identification Restriction / Configuration / 5GS |  | Rel-15 | C45 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and MTSI Originating Identification Restriction - Configuration |
| 8.3 | Originating Identification Restriction / Signalling / 5GS |  | Rel-15 | C46 | UE supports IMS security and NR and IMS voice over NR and (GBA or HTTP Digest) and MTSI and MTSI Originating Identification Restriction and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.4 | Terminating Identification Presentation / Configuration / 5GS |  | Rel-15 | C47 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Presentation |
| 8.5 | Terminating Identification Restriction / Configuration / 5GS |  | Rel-15 | C48 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Restriction - Configuration |
| 8.6 | Terminating Identification Restriction / Signalling / 5GS |  | Rel-15 | C49 | UE supports IMS security and NR and IMS voice over NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Restriction and NG114 v1.0 and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 8.7 | Communication Forwarding Unconditional / Configuration / 5GS |  | Rel-15 | C50 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| 8.8 | Communication Forwarding Unconditional / Signalling / 5GS |  | Rel-15 | C53 | UE supports IMS security and NR and IMS voice over NR and (GBA or HTTP Digest) and MTSI and Communication Diversion and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.9 | Communication Forwarding on Not Logged-in / Configuration / 5GS |  | Rel-15 | C50 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| 8.11 | Communication Forwarding on Busy / Configuration / 5GS |  | Rel-15 | C50 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| 8.13 | Communication Forwarding on Subscriber Not Reachable / Configuration / 5GS |  | Rel-15 | C50 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| 8.15 | Communication Forwarding on No Reply / Configuration / 5GS |  | Rel-15 | C50 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| 8.17 | Barring of All Incoming Calls / Configuration / 5GS |  | Rel-15 | C51 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Incoming Communication Barring |
| 8.18 | Barring of All Incoming Calls / except for a specific user / Configuration / 5GS |  | Rel-15 | C07 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI andIncoming Communication Barring |
| 8.19 | Barring of All Incoming Calls from anonymous users / Configuration / 5GS |  | Rel-15 | C51 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Incoming Communication Barring |
| 8.21 | Barring of all Outgoing Calls / Configuration / 5GS |  | Rel-15 | C52 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Outgoing Communication Barring |
| 8.22 | Barring of Outgoing International Calls / Configuration / 5GS |  | Rel-15 | C52 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Outgoing Communication Barring |
| 8.23 | Barring of Outgoing International Calls / ex Home Country / Configuration / 5GS |  | Rel-15 | C52 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Outgoing Communication Barring |
| 8.24 | Barring of Outgoing International Calls / When Roaming / Configuration / 5GS |  | Rel-15 | C52 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Outgoing Communication Barring |
| 8.25 | Barring of Incoming Calls / When Roaming / Configuration / 5GS |  | Rel-15 | C51 | UE supports IMS security and NR and (GBA or HTTP Digest) and MTSI and Incoming Communication Barring |
| 8.26 | MO Voice Call Hold without announcement / 5GS |  | Rel-15 | C20 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Communication Hold and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.27 | MO Video Call Hold without announcement / 5GS |  | Rel-15 | C34 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and MTSI Communication Hold and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.28 | MT Voice Call Hold without announcement / 5GS |  | Rel-15 | C21 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI Communication Hold and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.29 | MT Video Call Hold without announcement / 5GS |  | Rel-15 | C34 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and MTSI Communication Hold and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.30 | Subscription to the MWI event package / 5GS |  | Rel-15 | C22 | UE supports IMS security and NR and MTSI Message Waiting Indication |
| 8.31 | Creating and leaving a conference / 5GS |  | Rel-15 | C23 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Conference and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.32 | Inviting user to conference by sending a REFER request to the conference focus / 5GS |  | Rel-15 | C23 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Conference and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.33 | Inviting user to conference by sending a REFER request to the conference focus / Video / 5GS |  | Rel-15 | C36 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI video and MTSI Conference and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.34 | Three way session creation / Voice / 5GS |  | Rel-15 | C24 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI Conference and MTSI three way session and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.35 | Three way session creation / Video / 5GS |  | Rel-15 | C35 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and MTSI Conference and MTSI three way session initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.36 | MO Voice Call Explicit Communication Transfer / Consultative Call Transfer / 5GS |  | Rel-15 | C25 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI Explicit Communication Transfer - consultative transfer and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer and NG114 v1.0 answer to initial SDP offer and NG114 v1.0 answer to second SDP offer |
| 8.37 | Communication Waiting and answering the voice call / 5Gs |  | Rel-15 | C26 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI Communication Waiting and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.38 | Communication Waiting and cancelling the voice call / 5GS |  | Rel-15 | C26 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI Communication Waiting and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| 8.40 | User initiated USSI / 5GS |  | Rel-15 | C37 | UE supports IMS security and NR and MTSI and USSI |
| 8.41 | Communication Forwarding on No Reply: MO Voice Call initiation with preconditions |  | Rel-15 | C38 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI Communication Diversion and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| SMS |  |  |  |  |  |
| 9.1 | Mobile Originating SMS / 5GS |  | Rel-15 | C08 | UE supports IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| 9.2 | Mobile Terminating SMS / 5GS |  | Rel-15 | C09 | UE supports IMS security and NR and SM-over-IP receiver and UE configured to use SMS over IP |
| 9.3 | Mobile Originating Concatenated SMS / 5GS |  | Rel-15 | C10 | UE supports IMS security and NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| 9.4 | Mobile Terminating Concatenated SMS / 5GS |  | Rel-15 | C11 | UE supports IMS security and NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| 9.5 | Mobile Originating SMS / RP-ERROR / 5GS |  | Rel-15 | C08 | UE supports IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| Emergency Calls |  |  |  |  |  |
| 10.1 | Emergency Call with emergency registration / Success / Location information available / 5GS |  | Rel-15 | C12 | UE supports IMS security and NR and IMS voice over NR and IMS emergency services and is capable of obtaining location information |
| 10.2 | Emergency Call with emergency registration / Success / Location information not available / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.3 | Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS |  | Rel-15 | C40 | UE supports IMS security and NR and IMS voice over NR MTSI and MTSI speech and preconditions and Communication Hold during emergency call |
| 10.4 | Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not / 5GS |  | Rel-15 | C41 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and preconditions |
| 10.6 | Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.9 | Emergency call without emergency registration / UE credentials are not accepted / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and IMS emergency services |
| 10.10 | Emergency call without emergency registration / Failure of registration / Rejected by 403 (Forbidden) / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.11 | New initial emergency registration after obtaining new IP address different than the IP address / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.12 | User-initiated emergency reregistration / UE has emergency related ongoing dialog / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.13 | User-initiated emergency reregistration / User initiates an emergency call / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.14 | In parallel emergency and non-emergency registrations / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| 10.15 | Deregistration upon emergency registration expiration / 5GS |  | Rel-15 | C39 | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| eCall over IMS |  |  |  |  |  |
| 11.1 | eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK / 5GS |  | Rel-16 | C42 | UE supports IMS security and MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Manual type of eCall |
| 11.2 | eCall over IMS / Automatic initiation / Normal registration / Emergency registration / Success / 200 OK with ACK / 5GS |  | Rel-16 | C43 | UE supports IMS security and MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Automatic type of eCall |
| 11.4 | eCall over IMS / Manual initiation / MSD transfer and 200 OK with ACK / SIP INFO for MSD Update / Success / 5GS |  | Rel-16 | C42 | UE supports IMS security and MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Manual type of eCall |

NOTE 1:	If any one of the other Registration test cases 6.2 until 6.9 is executed, this test case is optional.

| Conditions/Options |  |  |
| --- | --- | --- |
| C01 | IF A.6a/2 AND A.18/5 THEN R ELSE N/A | IMS security and NR |
| C02 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions |
| C03 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session |
| C04 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4 /16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and NG114 v1.0 |
| C05 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND ( NOT A.4/16 OR (A.4/16 AND A.12/59)) THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and (not preconditions or (preconditions and disabling preconditions)) |
| C06 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/1 AND A.6a/3 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Originating Identification Presentation and GBA for XCAP authentication |
| C07 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/7 AND A.6a/3 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Communication Barring and GBA for XCAP authentication |
| C08 | IF A.6a/2 AND A.18/5 AND A.3A/61 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| C09 | IF A.6a/2 AND A.18/5 AND A.3A/62 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and SM-over-IP receiver and UE configured to use SMS over IP |
| C10 | IF A.6a/2 AND A.18/5 AND A.3A/64 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| C11 | IF A.6a/2 AND A.18/5 AND A.3A/65 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| C12 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.12/26 AND A.12/27 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and is capable of obtaining location information |
| C13 | IF A.6a/2 AND A.18/5 AND A.12/39 THEN R ELSE N/A | IMS security and NR and IMS de-registration |
| C14 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.21/1 AND A.22/1 AND A.22/2 AND A.22/3 AND A.22/4 AND A.22/5 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG.114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| C15 | IF A.6a/2 AND A.18/5 AND A.12/58 THEN R ELSE N/A | IMS security and NR and Enabling/Disabling SMS over IMS |
| C16 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.15/3 AND A.15/9 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions |
| C17 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.12/45 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and early media and preconditions |
| C18 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.12/57 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer |
| C19 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 AND A.12/57 THEN R ELSE N/A | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer |
| C20 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.16/6 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Communication Hold and initiating a session and preconditions |
| C21 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.16/6 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Communication Hold and preconditions |
| C22 | IF A.6a/2 AND A.18/5 AND A.16/8 THEN R ELSE N/A | IMS security and NR and MTSI Message Waiting Indication |
| C23 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.16/9 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Conference and preconditions |
| C24 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.16/9 AND A.16/14 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI Conference and MTSI three way session and preconditions |
| C25 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 A.3A/50 AND A.15/1 AND A.16/11 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Explicit Communication Transfer - consultative transfer and preconditions |
| C26 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.16/13 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Communication Waiting and preconditions |
| C27 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.12/59 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and disabling preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| C28 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 AND A.12/59 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions |
| C29 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.12/57 AND A.12/60 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and Session Timer and Session Timer for own benefit |
| C30 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/16 AND A.12/57 AND A.12/61 THEN R ELSE N/A | UE supports IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and Session Timer and re-INVITE for session refresh |
| C31 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.15/3 AND A.15/11 AND A.15/12 AND A.15/13 AND A.4/2B AND A.4/16 AND A.12/61 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and initiating a session and preconditions and disabling preconditions |
| C32 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.15/3 AND A.15/11 AND A.15/12 AND A.15/13 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1and preconditions |
| C33 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/2 AND A.15/10 AND A.15/3 AND A.15/11 AND A.15/12 AND A.15/13 AND A.4/16 AND A.12/61 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and AMR-WB and EVS and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and preconditions and disabling preconditions |
| C34 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/3 AND A.16/6 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI video and MTSI Communication Hold and initiating a session and preconditions |
| C35 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/3 AND A.16/9 AND A.16/14 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI video and MTSI Conference and MTSI three way session and preconditions |
| C36 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32 AND A.3A/50 AND A.15/1 AND A.15/3 AND A.16/9 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI video and MTSI Conference and preconditions |
| C37 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/2B AND A.16/19 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and initiating a session and USSI |
| C38 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND A.3A/50 AND A.15/1 AND A.16/5 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and MTSI Communication Diversion and preconditions |
| C39 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND A.3A/50 AND A.15/1 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech |
| C40 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND A.3A/50 AND A.15/1 AND A.4/16 AND A.12/33 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and preconditions and Communication Hold during emergency call |
| C41 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND A.3A/50 AND A.15/1 AND A.4/16 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and MTSI and MTSI speech and preconditions |
| C42 | IF A.6a/2 AND A.3A/50 AND A.15/1 AND A.12/63 AND [3] A.10/16 THEN R ELSE N/A | UE supports IMS security and MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Manual type of eCall |
| C43 | IF A.6a/2 AND A.3A/50 AND A.15/1 AND A.12/64 AND [3] A.10/17 THEN R ELSE N/A | UE supports IMS security and MTSI and MTSI speech and IMS eCall type of emergency services over 5GS and Automatic type of eCall |
| C44 | IF A.18/5 AND [93] A.4.3.7-1/32AND A.3A/50 AND A.15/10 AND A.4/16 AND (A.21/1 OR A.21/2) THEN R ELSE N/A | NR and IMS voice over NR and MTSI and EVS and preconditions and (NG.114 v1.0 or NG.114 v2.0) |
| C45 | IF A.6a/2 AND A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/2A THEN R ELSE N/A | IMS security and NR and (GBA or HTTP Digest) and MTSI and Originating Identification Restriction - Configuration |
| C46 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/2 AND A.15/1 AND A.15/2 AND A.15/10 AND A.4/2B AND A.4/16 AND A.22/1 AND A.22/2 AND A.22/3 AND A.22/4 AND A.22/5 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and (GBA or HTTP Digest) and MTSI and MTSI Originating Identification Restriction and MTSI speech and AMR-WB and EVS and initiating a session and preconditions and NG114 v1.0 and NG114 v1.0 mmtel ICSI value and NG114 v1.0 audio feature tag and NG114 v1.0 video feature tag and NG114 v1.0 initial SDP offer and NG114 v1.0 final SDP offer |
| C47 | IF A.6a/2 AND A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/3 THEN R ELSE N/A | IMS security and NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Presentation |
| C48 | IF A.6a/2 AND A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/4A THEN R ELSE N/A | IMS security and NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Restriction - Configuration |
| C49 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/4 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and (GBA or HTTP Digest) and MTSI and Terminating Identification Restriction |
| C50 | IF A.6a/2 AND A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/5 THEN R ELSE N/A | IMS security and NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |
| C51 | IF A.6a/2 AND A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/7 THEN R ELSE N/A | IMS security and NR and (GBA or HTTP Digest) and MTSI and Incoming Communication Barring |
| C52 | IF A.6a/2 AND A.18/5 AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/15 THEN R ELSE N/A | IMS security and NR and (GBA or HTTP Digest) and MTSI and Outgoing Communication Barring |
| C53 | IF A.6a/2 AND A.18/5 AND [93] A.4.3.7-1/32AND (A.6a/3 OR A.6a/4) AND A.3A/50 AND A.16/5 THEN R ELSE N/A | IMS security and NR and IMS voice over NR and (GBA or HTTP Digest) and MTSI and Communication Diversion |

# *** Next Change ***

### A.4.5	Additional information

Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |  |
| 2 | Void |  |  |  |  |  |
| 3 | Void |  |  |  |  |  |
| 4 | UE capable of being configured to initiate Dedicated PDP Context | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiatesDedicatedPDPContext |  |
| 5 | UE capable of being configured to initiate P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaPCO |  |
| 6 | Void |  |  |  |  |  |
| 7 | UE capable of being configured to initiate P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv6 |  |
| 8 | UE supports P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaPCO |  |
| 9 | Void |  |  |  |  |  |
| 10 | UE supports P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv6 |  |
| 11 | Void |  |  |  |  |  |
| 12 | UE capable of initiating a bidirectional voice session over IMS | 24.229 [10], 5.1.6 | o | Rel-5 | pc_BidirecVoiceOverIMS |  |
| 13 | Void |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |
| 15 | Void |  |  |  |  |  |
| 16 | UE Supports " IPv6 address with embedded IPv4 address" in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 17 | UE Supports IPv4 address in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 18 | Void |  |  |  |  |  |
| 19 | UE supports UI capable of showing user notification for Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 |  |  |
| 20 | Void |  |  |  |  |  |
| 21 | Void |  |  |  |  |  |
| 22 | Void |  |  |  |  |  |
| 23 | Void |  |  |  |  |  |
| 24 | UE supports no reply timer setting | 24.604 [68], 4.9.1.4 | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.3.8 | m |  |  |  |
| 25 | UE supports sending DTMF events over RTP | 26.114 [56], Annex G | o | Rel-7 |  |  |
|  |  | IR.92 [83], 3.3 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 26 | Void |  |  |  |  |  |
| 27 | UE is capable of obtain ing location information | 24.229 [10] 4.7, 36.509 [79] 4.1, 5.5.2, 34.109 [78] 5.4.2 | o | Rel-9 | pc_IMS_Geolocation |  |
| 28 | Void |  |  |  |  |  |
| 29 | UE supports emergency speech call over 1xRTT | C.S0005-E [74] | o |  | pc_CS_Em_Call_in_1xRTT |  |
| 30 | Void |  |  |  |  |  |
| 31 | UE supports end-to-access-edge media security using SDES | 24.229 [10], 4.2.B.2 | o | Rel-9 |  |  |
| 32 | UE supports video media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | o | Rel-13 | pc_IMS_Video_FeatureTag |  |
|  |  | IR.94 [75], 2.2 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 33 | UE supports Communication Hold during emergency call | 23.167 [76], 7.1.1 | o | Rel-9 | pc_CommunicationHold_DuringEmergencyCall |  |
| 34 | UE indicates g.3gpp.srvcc-alerting media feature tag in INVITE request or 180 (Ringing) response | 24.237 [77], 12.2.3B.1 | o | Rel-10 | pc_IMS_SRVCCAlert |  |
| 35 | Void |  |  |  |  |  |
| 36 | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request | 24.237 [77] 6A.2.2.2 | o | Rel-12 | pc_BSRVCC |  |
| 37 | UE indicates OMA-TS-XDM_MO-V1_1-20080627-A.doc, section 5.2.8 "Node: /<X>/ AAUTHNAME" is configured | 24.623 [81], Annex B.2 | o | Rel-8 | pc_XCAP_UsernameIsConfiguredInUE |  |
| 38 | The UE use the default public user identity received in P-Associated -URI header in 200 OK for REGISTER as XCAP User Identity (XUI) | IR.92 [83] | m |  | pc_XCAP_XUIisDefaultPublicUserId |  |
|  |  | 24.623 [81] | o | Rel-8 |  |  |
| 39 | UE has the method that support IMS deregistration | 24.229 [10], 5.1.1.6 | o | Rel-5 | pc_IMS_Deregistration |  |
| 40 | UE supports Cs to PS SRVCC | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCC |  |
| 41 | UE supports Cs to PS SRVCC in alerting state | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCAlert |  |
| 42 | UE supports Cs to PS SRVCC and the MSC server assisted mid-call feature | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCMidCall |  |
| 43 | Void |  |  |  |  |  |
| 44 | Void |  |  |  |  |  |
| 45 | UE supports early media | 24.229 [10], 5.1.3 | o | Rel-8 | pc_EarlyMedia |  |
| 46 | UE indicates g.3gpp.accesstype media feature tag in REGISTER | 24.237 [77], 6.2.2 | c1 | Rel-11 |  |  |
| 47 | Void |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |
| 49 | The UE uses XCAP without PDN connection in WLAN | IR.51 [84], 4.6 | o | Rel-11 | pc_WLAN_XCAP_without_PDN | If PICS is true, Wi-Fi access without PDN connection applies. Otherwise, EPC-integrated WLAN applies |
| 50 | The UE uses XCAP in fixed broadband access |  | o | Rel-12 | pc_fixed_broadband_XCAP | In the context of the present specification, a UE supporting fixed broadband, A.18/3, shall set this PICS to true. |
| 51 | The UE uses same public user Identity for From/To as in initial REGISTER | 24.229 [10] | o | Rel-8 | pc_Same_IMPU |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 52 | UE reattempts IMS emergency service without registration after rejection of emergency registration. (For Rel-14, this is applicable only when the rejection indicates an anonymous emergency call support) | 23.167 [76], 4.1 | o | Rel-9 | pc_IMS_after_EmReg_rejection |  |
|  |  |  | m | Rel-14 |  |  |
| 53 | Void |  |  |  |  |  |
| 54 | UE supports IMS eCall type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Only |  |
| 56 | UE supports audio media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | m | Rel-13 | pc_IMS_Audio_FeatureTag |  |
|  |  | IR.92 [83], 2.2.1, 2.2.4 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 57 | UE supports Session Timer | 24.229 [10], 5.1.2A, 5.2.7.2, 5.2.7.3, 5.4.5.3 | o | Rel-6 | pc_IMS_Session_Timer |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 58 | UE supports enabling/disabling SMS over IMS | 27.007 [97], 8.72 | o | Rel-15 | pc_Enable_Disable_SMSoverIMS |  |
| 59 | UE supports disabling preconditions | 24.229 [10], 5.1.5A | o | Rel-15 | pc_IMS_Disable_Preconditions |  |
| 60 | UE uses Session Timer for its benefit alone | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_own_benefit |  |
|  |  | NG.114 [96] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 61 | UE supports usage of re-INVITE for session refresh | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_reINVITE |  |
|  |  | NG.114 [96] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 62 | Void |  |  |  |  |  |
| 63 | UE supports IMS eCall type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Capable |  |
| 64 | UE supports IMS eCall Only type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Only |  |
| 65 | UE supports IMS emergency call over WLAN | 24.229 [10], 5.1.6, R.2.2.6 | o | Rel-13 | pc_WLAN_IMS_EmergencyCall |  |
| Conditions/Options |  |  |  |  |  |  |
| c1: IF [73] A.4.4-1/32 THEN m ELSE o - - SC UE indicates accesstype. |  |  |  |  |  |  |

# *** Next Change ***

### A.4.13	NG.114 versions

Table A.21: NG.114 versions

| Item |  | Ref. | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Version 1.0 as of 07 August 2020 | NG.114 v1.0 [96] | o | n/a | pc_NG114_v1_0 |  |
| 2 | Version 2.0 as of 07 August 2021 | NG.114 v2.0 [98] | o | n/a | pc_NG114_v2_0 |  |

### A.4.14	NG.114 capabilities

Table A.22: NG.114 capabilities

| Item |  | Ref. | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | mmtel ICSI value | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_mmtel |  |
| 2 | audio feature tag | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_audio |  |
| 3 | video feature tag | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_video |  |
| 4 | Codecs and attributes in initial SDP offer | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_initial_SDP_offer |  |
| 5 | final codec selection | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_final_SDP_offer |  |
| 6 | Annex C.3 default configuration | NG.114 v1.0 [96] | m | n/a |  |  |
| 7 | Annex C.3 default configuration | NG.114 v2.0 [98] | m | n/a |  |  |
| 8 | Annex C.3 default configuration for voice exempt | NG.114 v2.0 [98] | m | n/a | pc_NG114_v1_0_default_config_voice_exempt |  |
| 9 | Annex C.3 default configuration for video exempt | NG.114 v2.0 [98] | m | n/a | pc_NG114_v2_0_default_config_video_exempt |  |
| 10 | Annex C.3 default configuration for EVS/Br | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_default_config_evs_br |  |
| 11 | Annex C.3 default configuration for EVS/Bw | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_default_config_evs_bw |  |
| 12 | Codecs and attributes in answer to initial SDP offer | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_first_SDP_answer |  |
| 13 | Codecs and attributes in answer to second SDP offer | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_second_SDP_answer |  |
| 14 | Forking behavior | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_forking |  |

# *** End of Changes ***