3GPP TSG-RAN5 Meeting # 94-e	R5-220283

# Electronic Meeting, 21st Feb- 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38.533 | CR |  | 1589 |  |  |  |  | rev |  |  | - |  |  | Current version: |  |  |  |  |  |  | 17.1.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  | UICC apps |  |  |  |  |  |  | ME |  |  | Radio Access Network |  |  |  |  |  |  | Core Network |  |  |
| Title: |  |  | Annexure update for test tolerance for FR1 CLI-RSSI measurement with non-DRX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  | Qualcomm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  | R5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  | NR_CLI-UEConTest |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 2022-1-22 |  |  |  |  |  |
| Category: |  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |  |  |  |
| Reason for change: |  |  |  |  | Test tolerance analysis for FR1 CLI-RSSI measurement was not yet updated for Annexure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  | Updated test tolerance analysis of CLI RSSI measurement for Annexure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  | TT analysis will remain missing in TS 38.533. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  |  | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### F.1.1.2	Measurement of RRM requirements

<Many lines are skipped here>

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC |  |  | Same as 4.3.2.2.1 |  |  |  | Same as 4.3.2.2.1 |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | ± 0.6dB (AWGN conditions) |  |  |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  |  |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB |  |  |  | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  |  |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  |  |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  |  | Same as 4.5.5.1 |  |  |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  |  | Same as 4.5.5.3 |  |  |  | Same as 4.5.5.3 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  | ±0.6 dB |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  |  |  | Same as 4.5.6.1.1 |  |  |  | Same as 4.5.6.1.1 |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  | ±0.6 dB |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  |  |  | Noc is the AWGN on cell 2 frequency Ês / Noc is the ratio of cell 2 signal / AWGN |  |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.1 |  |  |  | Same as 4.6.4.1 |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.3 |  |  |  | Same as 4.6.4.3 |  |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.4.1 |  |  |  | Same as 4.6.4.1 |  |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 (NR) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  | Same as 4.7.1.1.1 |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB |  |  |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  | Same as 4.7.1.1.1 |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  | Same as 4.7.1.1.1 |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |  |  |

<Many lines are skipped here>

## F.1.2	Interpretation of measurement results (normative)

See TS 38.521-1 [17] Annex F.2.

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

### F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR SA FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |
| 4.3.2.2.2 | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | SNRs as specified |  | 0.6dB |  | Formula: SNR + TT |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  | For Test Configurations 1,2,4,5 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS |  | -0.6dB 0dB -3dB -0.6dB 0dB -3dB |  | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -99 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  | 0dB 0dB |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX | During T1: Noc on CLI RSSI: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc on CLI RSSI: -108dBm/15kHz Io on CLI RSSI : -89.43 dBm/ 1.08 Mhz |  | During T1: 0dB 0dB During T2: 0dB 0.5dB |  | During T1: Noc: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc: -116 dBm/15kHz Io on CLI RSSI : -88.93 dBm/ 1.08 MHz |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  | 0dB 0.3dB 0dB 0dB |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  |