3GPP TSG-RAN5 Meeting #94-e	R5-220298

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2194 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS MO speech call establishment generic procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the UE is configured for preconditions, the SIP UPDATE from the UE can be received in parallel with PDU SESSION MODIFICATION COMPLETE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an exception in section 4.9.15.2.2-1 to handle steps 12 and 13a1 in any order |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.9.15	Test procedure for IMS MO speech call establishment in 5GC

4.9.15.1	Scope

The purpose of this procedure is to establish an IMS MO speech call.

4.9.15.2	Procedure description

4.9.15.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 1N-A and registered to the IMS.

4.9.15.2.2	Procedure sequence

Table 4.9.15.2.2-1: IMS MO speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Make the UE attempt an IMS speech call. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mo-VoiceCall'? | --> | NR RRC: RRCSetupRequest | - | P |
| 3 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Check: Does the UE transmit a SecurityModeComplete message and establish the initial security configuration? | --> | NR RRC: SecurityModeComplete | - | P |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| - | EXCEPTION: In parallel to step 8 below, the steps specified in Table 4.9.15.2.2-2 take place. | - | - | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 9a1 to 9b4 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 9a1-9a4 | IF the UE is configured to use preconditions THEN steps 2-5 of Annex A.4.1 of TS 34.229-5 [47] take place. | - | - | - | - |
| 9b1-9b4 | ELSE steps 2-5 of Annex A.4.2 of TS 34.229-5 [47] take place. | - | - | - | - |
| 10 | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND message. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: If the UE is configured for preconditions, steps 12 and 13a1 can happen in any order | - | - | - | - |
| 12 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and a PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |
| - | EXCEPTION: Steps 13a1 to 13b8 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 13a1-13a7 | IF the UE is configured to use preconditions THEN steps 6-12 of Annex A.4.1 of TS 34.229-5 [47] take place. | - | - | - | - |
| 13b1-13b3 | ELSE steps 6-8 of Annex A.4.2 of TS 34.229-5 [47] take place. | - | - | - | - |

Table 4.9.15.2.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 1a1 | IF the UE is configured to use preconditions THEN step 1 of Annex A.4.1 of TS 34.229-5 [47] takes place. | - | - | - | - |
| 1b1 | ELSE step 1 of Annex A.4.2 of TS 34.229-5 [47] takes place. | - | - | - | - |

4.9.15.3	Specific message contents

Table 4.9.15.3-1: RRCSetupRequest (step 2, Table 4.9.15.2.2-1)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | mo-VoiceCall |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.15.3-2: SERVICE REQUEST (step 4, Table 4.9.15.2.2-1)

| Derivation Path: Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  |  |  |  |
| Service type value |  | ‘0001’B | data |  |

Table 4.9.15.3-3: PDU SESSION MODIFICATION COMMAND (step 10, Table 4.9.15.2.2-1)

| Derivation Path: Table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.15.3-4: RRCReconfiguration (step 10, Table 4.9.15.2.2-1)

Derivation Path: Table 4.8.1-1C