3GPP TSG-RAN5 Meeting #94-e	R5-220352

# Electronic Meeting, 21st Feb - 4th Mar 2022

# Source:	Rohde & Schwarz

# Title:	Ongoing exchange with CT1

# Document for:	Endorsement

## 1.	Introduction

To report on ongoing exchange with 3GPP CT WG1.

## 2.	Discussion

The RAN5 IMS5GS rapporteur was tasked to conduct direct discussions with CT1 for relieving unclarities in CT1 specifications or RAN5 understanding of them.

Currently, we have two topics between RAN5 and CT1:

- Confirming resource reservation

- Re-using resources in scenarios with more than one remote UE

On A:

RAN5 had sent LS R5-211311 to CT 1. CT1 responded in LS C1-213557, acknowledging the issue, and updated TS 24.229 Rel-17. Yet, TS 24.229 now misses to state what applies when preconditions are not used. Excerpt from v17.4.0 cl 5.1.3.1:

Upon successful reservation of local resources the UE shall confirm the successful resource reservation (see subclause 6.1.2) within the next SIP request.
NOTE 3:  In case of the precondition mechanism being used on both sides, this confirmation will be sent in either a PRACK request or an UPDATE request.

On this remaining issue, we had an email exchange between RAN5 IMS5GS rapporteur and a group of CT1 IMS specialists from Qualcomm, MTK, Apple, Huawei, Nokia and Ericsson. The conclusion was that CT1 will fix this such that the specification will look similar to this (exact wording to be decided):

If precondition mechanism is used, the UE shall confirm successful resource reservation (see subclause 6.1.2) within the next SIP request.
NOTE 3:  This confirmation will be sent in either a PRACK request or an UPDATE request.

With that, issue A will be solved for good. There is no action necessary on RAN5 side as we already work based on this understanding.

Note that during this discussion, we also stumbled over a misleading phrase in GMSA IR.92 and NG.114:

If preconditions are used, and the originating UE receives the selected codec in the SDP of a SIP 18x response, then the UE must include only the same codec with its selected configuration parameters in the SDP of the SIP UPDATE request, used for precondition status update.

Above phrase suggests that PRACK cannot be used anymore for confirming resources. We agreed that this was not the intent of that phrase. CT1 delegates intend to take this to their colleagues looking after GSMA PRDs.

On B:

This is a more complex issue. When the UE interacts with more than one remote UE (for instance in 3way conf calls, forking, call transfer or call forwarding), it is not clear what that means regarding resource reservation: can the UE re-use the same resources or does it need at least partly new resources? This is currently being discussed with CT1, see C1-221029. RAN5 will be updated when we have results.

## 3.	Proposals

Allow late inputs to RAN5#94-e in case we gain insights into above issue B before meeting end.