3GPP TSG-RAN WG5 Meeting #94-e	R5-220462

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0232 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.4 - Generic test procedures for UE operation over E-UTRA/EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Dedicated EPS bearers for MCVideo and MCData have not been considered in clause 5.4.1A yet. - Generic procedures of clause 5.3 are using procedures of 5.4.3 and 5.4.4 even for cases of SIP signalling only; in addition there have been inconsistencies in 5.4.3 and 5.4.13. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Dedicated EPS bearers for MCVideo and MCData added to clause 5.4.1A; further clarifications regarding use of PDNs and QCIs - Procedures of 5.4.3 and 5.4.4 extended to allow them to be used for any kind of RRC connection establishment (i.e. with and without dedicated bearer, on-demand and pre-established session); clarifications added for the respective MC services. As consequence clauses 5.4.3A, 5.4.3B, 5.4.4A, 5.4.4B and 5.4.13 are removed. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.4	Generic test procedures for UE operation over E-UTRA/EPC

### 5.4.1	General

The purpose of the procedures specified in the following clauses is to facilitate test description by providing procedure sequences which can be referred from the relevant test cases specified e.g. in 3GPP TS 36.579-2 [2], 3GPP TS 36.579-3 [3], 3GPP TS 36.579-6 [84], 3GPP TS 36.579-7 [85].

The intention is, wherever possible, that E-UTRA/EPC signalling and initial conditions should not be provided in the test descriptions rather should be referred to the procedure steps described in the generic procedures below, whereas, the MCS SIP signalling and initial conditions when relevant for the test purposes shall be explicitly provided in the tests description itself.

Throughout the generic test procedures E-UTRA/EPC behaviour is denoted as "SS" for the System Simulator simulating the NWK side of the communication, and, "UE" for the Implementation Under Test (IUT), whereas the MCPTT/MCVideo/MCData relevant behaviour is denoted as "SS (MCPTT/MCVideo/MCData server)" and "UE (MCPTT/MCVideo/MCData client)"/"UE (MCPTT/MCVideo/MCData user)" respectively. ProSe related SS behaviour when the SS simulates an UE device is denoted e.g. as "SS-UE1".

Depending on the TS 36.579-5[5] test model being used, the E-UTRA/EPC signalling is:

-	MCX EUTRA test model: normative.

-	MCX IPCAN test model: informative, unless specifically specified otherwise elsewhere.

### 5.4.1A	UE APN/PDN support assumptions

According to TS 23.280 [110] clause 5.2.7.0 an MC service UE shall use APNs for the SIP-1, HTTP-1 and CSC-1 reference points, which may be different or all the same. To limit the test specification complexity it is assumed that only one APN is used and therefore there is a single MCX PDN. In addition there might be an IMS PDN and an internet PDN so that three PDNs need to be taken into account:

1.	MCX PDN with default EPS bearer using QCI=69

NOTE 1:	It should be noted that the core specs impose a requirement that the QCI value 8 or better shall be used for the EPS bearer that transports HTTP-1 reference point messaging. Using a single APN and having for the EPS bearer QCI=69 will satisfy this.

NOTE 2:	Void.

2.	Internet PDN with default EPS bearer using QCI=9

3.	IMS PDN with default EPS bearer using QCI=5

This results in the need to handle up to three PDNs during MCX conformance tests.

NOTE 3:	It should be noted that, handling IMS and MCX with one APN is theoretically possible but may have undesirable implications e.g. VoLTE signalling could delay MCX signalling therefore the assumption is that such implementations will be undesirable and unlikely.

Consequently, for IMS and MCX it should be assumed that the UE will do 2 different registrations, i.e. for each of them there will be a separate IP connection (different IP addresses at the UE and the SS).

Depending on UE configuration PDN connectivities for the up-to three PDNs may be established. There are two major scenarios:

1.	The MCX PDN connectivity gets established automatically after switch-on during the initial registration procedure. In addition the UE may establish PDN connectivities to the IMS PDN and/or the internet PDN. The connectivity to these PDNs may be requested in any order. There can be 1, 2 or 3 PDNs.

2.	The UE requests PDN connectivities for IMS and/or internet but not for MCX. If IMS and internet are requested, it may be in any order. Establishment of the MCX PDN connectivity is triggered after the initial registration in a separate procedure. There can be 2 or 3 PDNs in total.

To serve the above scenarios the following parameters are defined in TS 36.579-5 [5]:

-	px_MCX_InitialRegistration_TypeOfPDN1: 
First PDN registered during initial registration (either ‘ims’ or ‘internet’ or ‘mcx’)

-	px_MCX_InitialRegistration_TypeOfPDN2: 
Second PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no second PDN connectivity requested by the UE during initial registration.

-	px_MCX_InitialRegistration_TypeOfPDN3: 
Third PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no third PDN connectivity requested by the UE during initial registration.

The type of the parameters is a TTCN-3 enumerated type with values ‘ims’,‘internet’, ‘mcx’ and ‘none’.

In addition there is the parameter px_AccessPointName in TS 36.523-3 [74] which is used as default APN, i.e. for a PDN for which the UE does not provide an APN (NOTE: Any, but only one, of the three PDNs can be the one with default APN).

Regarding the default EPS bearers for the respective mission critical services the following applies for MCX conformance tests:

-	MCPTT: 
A single dedicated EPS bearer with QCI=65 is used with packet filters for the audio stream and media plane control signalling (see also TS 23.379 [126] clause 5.7.3)

-	MCVideo: 
A single dedicated EPS bearer with QCI=67 is used with packet filters for the audio and video streams and transmission control signalling (see also TS 23.281 [90] clause 5.5.3)

-	MCData:
A single dedicated EPS bearer with QCI=70 is used with packet filter for the TCP data stream (see also TS 23.282 [91] clause 5.8.3)

### 5.4.2	Generic Test Procedure for MCPTT UE registration

5.4.2.1	Initial conditions

System Simulator:

-	SS (MCPTT server)

-	E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [6] clause 4.4, unless otherwise specified in the test case. Requirements in regard to the PLMN which the simulated Cell(s) belongs to are specified in the test case using the present procedure.

IUT:

-	UE (MCPTT client)

-	The UE is MCPTT capable. The MCPTT preconditions required for initiation of MCPTT service authorization for the MCPTT client and the MCPTT service are specified in the test cases.

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	The UE shall be switched off.

5.4.2.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.4.2.3	Procedure

Table 5.4.2.3-1: E-UTRA/EPC signalling for UE registration

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 0 | Switch the UE on. | - | - |
| 1 | Void | - | - |
| 2 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 3 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message. The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. (NOTE 1) | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST |
| 5 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST |
| 6 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION RESPONSE |
| 7 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND |
| 8 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - |
| 9a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | RRC: DLInformationTransfer NAS: ESM INFORMATION REQUEST |
| 9a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | RRC: ULInformationTransfer NAS: ESM INFORMATION RESPONSE |
| 10 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 11 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 12 | The SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 13 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 14 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to TS 36.508 [6] clause 4.8.2.2.1.1. This message includes the ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. (NOTE 1) | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in steps 16 and 16A below, if initiated by the UE the generic procedure for IP address allocation in the U-plane as defined in TS 36.508 [6] clause 4.5A.1 takes place. | - | - |
| - | EXCEPTION: IF the UE is configured to register for MCX as first PDN during initial registration, THEN in parallel to the event described in steps 16 and 16Abelow the events described in table 5.4.2.3-2 take place. | - | - |
| - | EXCEPTION: IF the UE is configured to register for IMS as first PDN during initial registration, THEN in parallel to the event described in steps 16 and 16A below the generic procedure for IMS signalling in the U-plane specified in TS 36.508 clause 4.5A.3 takes place if requested by the UE | - | - |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| - | EXCEPTION: Depending on the UE capability step 16A may be performed 0, 1 or 2 times. (NOTE 1) | - | - |
| 16A | The E-UTRA/EPC signalling for establishment of an additional PDN connectivity according to table 5.4.2.3-1A takes place | - | - |
| 17 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease |
| - | EXCEPTION: IF the UE is not configured to register for MCX during initial registration, THEN steps 18 to 27 take place. | - | - |
| 18 | Make the UE user request MCPTT service authorisation/configuration. NOTE 2 | - | - |
| 19 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest |
| 20 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 21 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 22 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 23 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 24 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(N, 0) with N being the number of PDN connectivities established during initial registration (steps 0 – 17). The DRBs associated with the respective default EPS bearer context obtained during the attach procedure are established | <-- | RRC: RRCConnectionReconfiguration |
| 25 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 26 | The E-UTRA/EPC signalling for establishment of an additional PDN connectivity according to table 5.4.2.3-1A takes place | - | - |
| 27 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease |
| NOTE 1: The assumptions for the PDN support of a MCPTT capable UE, including the default EPS bearer context QCI requirements in regard to the different PDN are described in 5.4.1A. NOTE 2: This will start a 5 stage process. The first stage involves MCPTT User Authentication and includes Steps 3a1 through 10 of Table 5.3.2.3-1. The end result of the first stage is the MCPTT Client receives 3 tokens: access token, ID token, and refresh token. |  |  |  |

Table 5.4.2.3-1A: E-UTRA/EPC signalling for establishment of an additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN. | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST |
| 2 | The SS configures a new data radio bearer, associated with the additional default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of additional default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 4 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 clause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| - | EXCEPTION: IF ADD_IMS THEN in parallel to the event described in step 4 below the generic procedure for IMS signalling in the U-plane specified in TS 36.508 clause 4.5A.3 takes place if requested by the UE | - | - |
| - | EXCEPTION: IF ADD_MCX THEN in parallel to the event described in step 4 below the SIP registration for MCPTT as specified in table 5.4.2.3-2 takes place | - | - |
| 4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |

| Condition | Explanation |
| --- | --- |
| ADD_IMS | true if PDN CONNECTIVITY REQUEST is for IMS |
| ADD_MCX | true if PDN CONNECTIVITY REQUEST is for MCX |

Table 5.4.2.3-2: SIP registration for MCPTT

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: In parallel to the event described in steps 1 to 4 below the MCPTT user authentication as according to table 5.3.2.3-1 take place. |  |  |
| 1 | The UE sends initial registration for IMS services. | --> | SIP REGISTER |
| 2 | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. | <-- | SIP 401 Unauthorized |
| 3 | The UE completes the security negotiation procedures, sets up a temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. | --> | SIP REGISTER |
| 4 | The SS responds with 200 OK. | <-- | SIP 200 OK |
| 5-6 | Void |  |  |
| 6A | The generic procedure for MCPTT Service Authorization as specified in table 5.3.2.3-2 takes place |  |  |
| 7 | The SS (MCPTT server) sends SIP MESSAGE for configuring Location Info reporting. | <-- | SIP MESSAGE |
| 8 | The UE (MCPTT client) responds with SIP 200 (OK) | --> | SIP 200 (OK) |

5.4.2.4	Specific message contents

All specific E-UTRA/EPC signalling message contents shall be referred to TS 36.508 [6] clause 4.6 and 4.7.

The MCPTT relevant SIP message contents, Table 5.4.2.3-2, are specified in the present document clause 5.5.2, except for the following messages.

Table 5.4.2.4-1: SIP MESSAGE (step 7)

| Derivation Path: Table 5.5.2.7.2-1 SIP MESSAGE from the SS, condition LOCATION-CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | As described in Table 5.4.2.4-1A |  |  |  |

Table 5.4.2.4-1A: MCPTT Info in SIP MESSAGE (Table 5.4.2.4-1)

| Derivation Path: Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |

Table 5.4.2.4-2: SIP 200 (OK) (Step 8, Table 5.4.2.3-2)

Derivation Path: Table 5.5.2.17.1.1-1

Table 5.4.2.4-3: REGISTER (Step 1, Table 5.4.2.3-2)

Derivation Path: Table 5.5.2.13-1 with condition SIP_REGISTER_INITIAL

Table 5.4.2.4-4: SIP 401 (Unauthorized) (Step 2, Table 5.4.2.3-2)

Derivation Path: Table 5.5.2.19.7-1

Table 5.4.2.4-5: REGISTER (Step 3, Table 5.4.2.3-2)

Derivation Path: Table 5.5.2.13-1

Table 5.4.2.4-6: SIP 200 (OK) (Step 4, Table 5.4.2.3-2)

Derivation Path: Table 5.5.2.17.1.2-1

### 5.4.2A	Generic Test Procedure for MCVideo UE registration

The same as the procedure described in 5.4.2 with the following exception(s):

-	The term "MCPTT" is replaced with "MCVideo".

### 5.4.2B	Generic Test Procedure for MCData UE registration

The same as the procedure described in 5.4.2 with the following exception(s):

-	The term "MCPTT" is replaced with "MCData", and the term "call" with "communication".

### 5.4.3	Generic Test Procedure for MCX CO communication in E-UTRA

5.4.3.1	Initial conditions

System Simulator:

-	SS (MCX server)

-	SS E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [6] clause 4.4, unless otherwise specified in the test case. Requirements in regard to the PLMN which the simulated Cell(s) belongs to are specified in the test case using the present procedure.

IUT:

-	UE (MCX client)

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	The UE has performed the Generic Test Procedure for MCX UE registration as specified in clause 5.4.2, 5.4.2A or 5.4.2B and is in E-UTRA Registered, Idle Mode state with the MCX Client being active. During the attach a default EPS bearer context #3 (QCI 69) according to table 6.6.1-1, TS 36.508 [6] is established for MCX and SIP signalling.

NOTE 1:	The assumptions for the PDN support, including the default EPS bearer context QCI requirements in regard to the different PDN are described in 5.4.1A.

-	Detailed initial conditions for the UE (MCX client) shall be specified in the test case referring to the present procedure.

5.4.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.4.3.3	Procedure

Table 5.4.3.3-1: E-UTRA/EPC signalling for MCX CO communication

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Void | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'mo-Data'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) as specified in TS 36.508 [6] clause 4.8.2.2.1, with n=1..3 depending on the number of PDNs (see clause 5.4.1A) m=0..1 depending on the use case: IF the procedure is used for on-demand call or communication establishment, for establishment of a pre-established session or IF a pre-established session exists THEN m=1 ELSE m=0 | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described below, depending on the context in which the procedure is used, the MCX client may start with user plane signalling (NOTE 1). | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-15 | Void. | - | - |
| - | EXCEPTION: Steps 16a1-16a3 describe behaviour that depends on the context in which the procedure is used: The steps take place if the procedure is used for on-demand call or communication establishment or establishment of a pre-established session, | - | - |
| 16a1 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. The RRCConnectionReconfiguration message contains an ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message for a dedicated EPS bearer according to TS 36.508 [6] clause 6.6.2 with - MCPTT using dedicated EPS bearer context #5 (QCI 65) - MCVideo using dedicated EPS bearer context #10 (QCI 67) - MCData using dedicated EPS bearer context #9 (QCI 70) | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 16a2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the data radio bearer associated with the default EPS. | --> | RRC: RRCConnectionReconfigurationComplete |
| 16a3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| NOTE 1: User plane signalling can be SIP or HTTP signalling. |  |  |  |

Table 5.4.3.3-2: Void

5.4.3.4	Specific message contents

All specific E-UTRA/EPC signalling message contents shall be referred to TS 36.508 [6] clauses 4.6 and 4.7.

### 5.4.3A	Void

### 5.4.3B	Void

### 5.4.4	Generic Test Procedure for MCX CT communication in E-UTRA

5.4.4.1	Initial conditions

System Simulator:

-	SS (MCX server)

-	E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [6] clause 4.4, unless otherwise specified in the test case. Requirements in regard to the PLMN which the simulated Cell(s) belongs to are specified in the test case using the present procedure.

IUT:

-	UE (MCX client):

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	The UE has performed the Generic Test Procedure for MCX UE registration as specified in clause 5.4.2 and is in E-UTRA Registered, Idle Mode state with the MCX Client being active. During the attach a default EPS bearer context #3 (QCI 69) according to table 6.6.1-1, TS 36.508 [6] is established for MCX and SIP signalling.

NOTE 1:	The assumptions for the PDN support , including the default EPS bearer context QCI requirements in regard to the different PDN are described in 5.4.1A.

-	Detailed initial conditions for the UE (MCX client) shall be specified in the test case referring to the present procedure.

5.4.4.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.4.4.3	Procedure

Table 5.4.4.3-1: E-UTRA/EPC signalling for MCX CT communication

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'mt-Access'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) as specified in TS 36.508 [6] clause 4.8.2.2.1, with n=1..3 depending on the number of PDNs (see clause 5.4.1A) m=0..1 depending on the use case: IF the procedure is used for call or communication establishment or IF a pre-established session exists THEN m=1 ELSE m=0 | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-16 | Void. | - | - |
| - | EXCEPTION: Steps 17a1-17a3 describe behaviour that depends on the context in which the procedure is used: The steps take place if the procedure is used for on-demand call or communication establishment, | - | - |
| - | EXCEPTION: In parallel to the events described below there is SIP signalling for the on-demand call or communication establishment. | - | - |
| 17a1 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. The RRCConnectionReconfiguration message contains an ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message for a dedicated EPS bearer according to TS 36.508 [6] clause 6.6.2 with - MCPTT using dedicated EPS bearer context #5 (QCI 65) - MCVideo using dedicated EPS bearer context #10 (QCI 67) - MCData using dedicated EPS bearer context #9 (QCI 70) | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 17a2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the data radio bearer associated with the default EPS. | --> | RRC: RRCConnectionReconfigurationComplete |
| 17a3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 5.4.4.3-2: Void

5.4.4.4	Specific message contents

All specific E-UTRA/EPC signalling message contents shall be referred to TS 36.508 [6] clause 4.6 and 4.7.

### 5.4.4A	Void

### 5.4.4B	Void

### 5.4.5	Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment

5.4.5.1	Initial conditions

System Simulator:

-	SS-UE1 (MCPTT Client).

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [6], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator configured to simulate a location in the centre of Geographical area #1 and providing timing reference as defined in TS 36.508 [6] Table 4.11.2-2 scenario #1, for the assistance of E-UTRAN off-network testing.

NOTE:	For operation in off-network environment, it needs to be ensured that after the UE is powered up it considers the Geographical area #1 as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

IUT:

-	UE (MCPTT client):

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

UE state:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [6].

5.4.5.2	Definition of system information messages

N/a (out of E-UTRA coverage)

5.4.5.3	Procedure

Table 5.4.5.3-1: ProSe direct communication one-to-one out of E-UTRA coverage signalling for MCPTT CO communication-establishment

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Power up the UE. | - | - |
| 2 | Wait for 15 sec to allow the UE to establish that it is out of coverage and initiate scanning the frequency pre-set for ProSe communication for any activities. | - | - |
| 3 | Make the UE initiate one-to-one ProSe direct communication with the remote UE preconfigured (ProSe Layer-2 Group ID). | - | - |
| 4 | UE sends a DIRECT_COMMUNICATION_REQUEST message, IP Address Config IE set to "address allocation not supported". | --> | DIRECT_COMMUNICATION_REQUEST |
| 5 | SS-UE1 sends a DIRECT_SECURITY_MODE_COMMAND message. | <-- | DIRECT_SECURITY_MODE_COMMAND |
| 6 | UE sends a DIRECT_SECURITY_MODE_COMPLETE message ciphered and integrity protected with the new security context. | --> | DIRECT_SECURITY_MODE_COMPLETE |
| 7 | SS-UE1 sends a DIRECT_COMMUNICATION_ACCEPT message. | <-- | DIRECT_COMMUNICATION_ACCEPT |
| 8 | EXCEPTION: After the communication is established, an IP address configuration procedure is performed depending on what the UE has indicated in the IP Address Config IE (if it is not "address allocation not supported") in the DIRECT_COMMUNICATION_REQUEST message, and, the SS-UE1 itself indicating "address allocation not supported" in the DIRECT_COMMUNICATION_ACCEPT message. | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that depends on the UE implementation of keepalive procedure. | - | - |
| 9a1 | UE sends a DIRECT_COMMUNICATION_KEEPALIVE message. | --> | DIRECT_COMMUNICATION_KEEPALIVE |
| 9a2 | SS-UE1 sends a DIRECT_COMMUNICATION_KEEPALIVE_ACK message. | <-- | DIRECT_COMMUNICATION_KEEPALIVE_ACK |

5.4.5.4	Specific message contents

Table 5.4.5.4-1: DIRECT_COMMUNICATION_ACCEPT (step 7 Table 5.4.5.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IP Address Config | '0011'B | address allocation not supported |  |
| Link Local IPv6 Address | If the UE indicated 'address allocation not supported' in the IP Address Config IE in the DIRECT_COMMUNICATION_REQUEST message then a link-local IPv6 address formed locally | 128-bit IPv6 address |  |

Table 5.4.5.4-2: DIRECT_SECURITY_MODE_COMMAND (step 5, Table 5.4.5.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Security Capabilities | Set to the UE Security Capabilities received in the DIRECT_COMMUNICATION_REQUEST message |  |  |
| Chosen Algorithms | One of the non-null algorithms provided in UE Security Capabilities (i.e. different to EIA0 (null integrity protection algorithm)/EEA0 (null ciphering algorithm)) |  |  |
| MSB of KD ID | The MSB of KD ID of the new KD |  |  |
| KD Freshness | Not included |  |  |
| GPI | Not included |  |  |
| User Info { |  |  |  |
| Type of User Info | IMSI |  |  |
| Odd/even indication | Reflecting the number of digits in the IMSI |  |  |
| Identity digits | A value different to the IMSI of the UE |  |  |
| } |  |  |  |

Table 5.4.5.4-3: DIRECT_SECURITY_MODE_COMPLETE (step 6, Table 5.4.5.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-8. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LSB of KD ID | Not included |  |  |

Table 5.4.5.4-4: DIRECT_COMMUNICATION_KEEPALIVE (step 9a1, Table 5.4.5.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Keepalive Counter | 0 |  |  |
| Maximum Inactivity Period | Any allowed value |  |  |

### 5.4.6	Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment

5.4.6.1	Initial conditions

System Simulator:

-	SS-UE1 (MCPTT Client).

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [6], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator configured to simulate a location in the centre of Geographical area #1 and providing timing reference as defined in TS 36.508 [6] Table 4.11.2-2 scenario #1,for the assistance of E-UTRAN off-network testing.

NOTE:	For operation in off-network environment, it needs to be ensured that after the UE is powered up it considers the Geographical area #1 as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

UE state:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [6].

5.4.6.2	Definition of system information messages

N/a (out of E-UTRA coverage).

5.4.6.3	Procedure

Table 5.4.6.3-1: ProSe direct communication one-to-one out of E-UTRA coverage signalling for MCPTT CT communication-establishment

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Power up the UE. | - | - |
| 2 | Wait for 15 sec to allow the UE to establish that it is out of coverage and initiate scanning the frequency pre-set for ProSe communication for any activities. | - | - |
| 3 | SS-UE1 sends a DIRECT_COMMUNICATION_REQUEST message, IP Address Config IE set to "address allocation not supported". | <-- | DIRECT_COMMUNICATION_REQUEST |
| 4 | UE sends a DIRECT_SECURITY_MODE_COMMAND message uncyphered but integrity protected with the new security context. | --> | DIRECT_SECURITY_MODE_COMMAND |
| 5 | SS-UE1 sends a DIRECT_SECURITY_MODE_COMPLETE message ciphered and integrity protected with the new security context. | <-- | DIRECT_SECURITY_MODE_COMPLETE |
| 6 | UE sends a DIRECT_COMMUNICATION_ACCEPT message. | --> | DIRECT_COMMUNICATION_ACCEPT |
| 7 | EXCEPTION: After the communication is established, an IP address configuration procedure is performed depending on what the UE has indicated in the IP Address Config IE (if it is not "address allocation not supported") in the DIRECT_COMMUNICATION_REQUEST message, and, the SS-UE1 itself indicating "address allocation not supported" in the DIRECT_COMMUNICATION_ACCEPT message. | - | - |
| 8 | SS-UE1 sends a DIRECT_COMMUNICATION_KEEPALIVE message with a Keepalive Counter IE that contains the value of the keepalive counter for this link=0, and a Maximum Inactivity Period IE. | <-- | DIRECT_COMMUNICATION_KEEPALIVE |
| 9 | UE sends a DIRECT_COMMUNICATION_KEEPALIVE_ACK message including the Keepalive Counter IE set to the same value as that received in the DIRECT_COMMUNICATION_KEEPALIVE message. | --> | DIRECT_COMMUNICATION_KEEPALIVE_ACK |

5.4.6.4	Specific message contents

Table 5.4.6.4-1: DIRECT_COMMUNICATION_REQUEST (step 3, Table 5.4.6.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-5. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| User Info { |  |  |  |
| Type of User Info | IMSI |  |  |
| Odd/even indication | Reflecting the number of digits in the IMSI |  |  |
| Identity digits | A value different to the IMSI of the UE |  |  |
| } |  |  |  |
| IP Address Config | '0011'B | address allocation not supported |  |
| Maximum Inactivity Period | '10 0000 0000'B | 512 sec, randomly chosen to allow sufficient time for a TC which uses this procedure to be completed without need to repeat the keepalive procedure |  |
| Nonce_1 |  |  |  |
| UE Security Capabilities | 01111111 01111111 | All but null algorithms supported |  |
| MSB of KD-sess ID | the 8 most significant bits of the KD-sess ID |  |  |
| KD ID | Not present |  |  |
| Signature | the ECCSI signature calculated with the User Info and Nonce_1 as specified in 3GPP TS 33.303 [67] |  |  |
| Link Local IPv6 Address | a link-local IPv6 address formed locally |  |  |

Table 5.4.6.4-2: DIRECT_SECURITY_MODE_COMMAND (step 4 Table 5.4.6.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MSB of KD ID | Any allowed value |  |  |
| KD Freshness | Not included |  |  |
| GPI | Not included |  |  |
| Signature | The ECCSI signature calculated with the User Info and Nonce_1 as specified in 3GPP TS 33.303 [67] |  |  |
| Encrypted Payload | The SAKKE payload generated as specified in 3GPP TS 33.303 [67]. |  |  |

Table 5.4.6.4-3: DIRECT_SECURITY_MODE_COMPLETE (step 5, Table 5.4.6.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-8. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LSB of KD ID | 16 least significant bits of KD ID |  |  |

Table 5.4.6.4-4: DIRECT_COMMUNICATION_KEEPALIVE (step 8, Table 5.4.6.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Keepalive Counter | 0 |  |  |
| Maximum Inactivity Period | '10 0000 0000'B | 512 sec, randomly chosen to allow sufficient time for a TC which uses this procedure to be completed without need to repeat the keepalive procedure |  |

### 5.4.7	Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the SS

5.4.7.1	Initial conditions

System Simulator:

-	SS-UE1 (MCPTT Client).

-	Same as those defined in the 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.5, or, the 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.6.

IUT:

-	UE (MCPTT client)

ProSe related configuration

-	Same as those defined in the 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.5, or, the 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.6.

UE state

-	The UE has established ProSe direct communication one-to-one out of E-UTRA coverage using the 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.5, or, the 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.6.

5.4.7.2	Definition of system information messages

N/a (out of E-UTRA coverage).

5.4.7.3	Procedure

Table 5.4.7.3-1: ProSe direct communication one-to-one out of E-UTRA coverage signalling for MCPTT communication - release by the SS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS-UE1 sends a DIRECT_COMMUNICATION_RELEASE message with a Release Reason IE indicating 'Direct Communication to peer UE no longer needed'. | <-- | DIRECT_COMMUNICATION_RELEASE |
| 2 | UE sends a DIRECT_COMMUNICATION_RELEASE_ACCEPT message. | --> | DIRECT_COMMUNICATION_RELEASE_ACCEPT |

5.4.7.4	Specific message contents

Table 5.4.7.4-1: DIRECT_COMMUNICATION_RELEASE (step 1, Table 5.4.7.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-11. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Release Reason | '0001'B | Direct communication to the peer UE no longer needed |  |

### 5.4.8	Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the UE

5.4.8.1	Initial conditions

System Simulator:

-	SS-UE1 (MCPTT Client).

-	Same as those defined in the 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.5, or, the 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.6.

IUT:

-	UE (MCPTT client)

ProSe related configuration

-	Same as those defined in the 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.5, or, the 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.6.

UE state

-	The UE has established ProSe direct communication one-to-one out of E-UTRA coverage using the 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.5, or, the 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment', as described in clause 5.4.6.

5.4.8.2	Definition of system information messages

N/a (out of E-UTRA coverage).

5.4.8.3	Procedure

Table 5.4.8.3-1: ProSe direct communication one-to-one out of E-UTRA coverage signalling for MCPTT communication - release by the UE

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | UE sends a DIRECT_COMMUNICATION_RELEASE message with a Release Reason IE indicating 'Direct Communication to peer UE no longer needed'. | --> | DIRECT_COMMUNICATION_RELEASE |
| 2 | SS-UE1 sends a DIRECT_COMMUNICATION_RELEASE_ACCEPT message. | <-- | DIRECT_COMMUNICATION_RELEASE_ACCEPT |

5.4.8.4	Specific message contents

Table 5.4.8.4-1: DIRECT_COMMUNICATION_RELEASE (step 1, Table 5.4.8.3-1)

| Derivation path: 36.508 [6], Table 4.7F.3-11. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Release Reason | '0001'B | Direct communication to the peer UE no longer needed |  |

### 5.4.9	Generic Test Procedure for MCPTT communication in E-UTRA / Change of cells

5.4.9.1	Initial conditions

System Simulator:

-	SS (MCPTT server)

-	SS E-UTRA

-	Parameters are set to the default parameters for the basic E-UTRA single mode multi cell network scenarios, as defined in TS 36.508 [6] clause 4.4, unless otherwise specified in the test case.

-	3 cells (Cell 1, Cell 2 and Cell 4, all operating on the same frequency). Cells 1 and 2 are on the same PLMN1, whereas Cell 4 is on a different PLMN2.

NOTE:	The procedure only requires at maximum 2 cells to be active at any one instance.

IUT:

-	UE (MCPTT client)

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in clause 5.4.2 and is in E-UTRA Registered, Idle Mode state on Cell 1 with the MCPTT Client being active. During the attach a default EPS bearer context #3 (QCI 69) according to table 6.6.1-1, TS 36.508 [6] is established for MCPTT and SIP signalling. The UE is allowed to operate on both PLMN1 and PLMN2.

NOTE 1:	The assumptions for the PDN support of a MCPTT capable UE, including the default EPS bearer context QCI requirements in regard to the different PDN are described in 5.4.1A.

-	The UE has performed the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in clause 5.3.2 and thereby the MCPTT client is authorised for and able to use the MCPTT service including making group and private calls on- and off-network, and, the MCPTT user is registered for receiving MCPTT service through the MCPTT Client. The PLMN1 is set as HPLMN and PLMN2 is set as VPLMN in Table 5.5.8.1-1: MCPTT Initial UE Configuration Defaults.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

5.4.9.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.4.9.3	Procedure

Table 5.4.9.3-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" ... "Tn" are to be applied subsequently. The exact instants on which these values shall be applied are described elsewhere in the present clause.

Table 5.4.9.3-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -79 | "Off" | "Off" |
| T1 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -79 | "Off" |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | -79 |

Table 5.4.9.3-2: E-UTRA/EPC signalling for UE changing cells

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS configures: Cell 1 and Cell 2 parameters according to the row "T1" in table 5.4.9.3-1 in order to simulate needs for cell reselection to Cell2. | - | - |
| 2 | Wait for 5 sec to allow the UE to adjust to cell changes. NOTE 1. | - | - |
| 3 | The SS configures: Cell 2 and Cell 4 parameters according to the row "T2" in table 5.4.9.3-1 in order to simulate needs for cell reselection to Cell4. | - | - |
| 4 | The Generic test procedure for 'Tracking area updating procedure' defined in TS 36.508 [6] clause 4.5A.2 take place. NOTE 2. | - | - |
| NOTE 1: Depending on implementation the UE may start transmitting MCPTT protocol relevant data earlier. What may be transmitted is specified in the TCs. NOTE 2: The UE may start transmitting MCPTT protocol relevant data as soon as it receives TRACKING AREA UPDATE ACCEPT message. If this happens the SS shall not execute step 7 of the Generic test procedure for 'Tracking area updating procedure' and shall continue with the rest of the messages exchange defined in the test case. |  |  |  |

5.4.9.4	Specific message contents

None.

### 5.4.10	Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-many communication out of E-UTRA coverage / Announcing/Discoveree procedure for group member discovery

5.4.10.1	Initial conditions

System Simulator:

-	SS-UE1 (MCPTT Client).

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [6], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator configured to simulate a location in the centre of Geographical area #1 and providing timing reference as defined in TS 36.508 [6] Table 4.11.2-2 scenario #1, for the assistance of E-UTRAN off-network testing.

NOTE:	For operation in off-network environment, it needs to be ensured that after the UE is powered up it considers the Geographical area #1 as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

UE state:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [6].

5.4.10.2	Definition of system information messages

N/a (out of E-UTRA coverage)

5.4.10.3	Procedure

Table 5.4.10.3-1: ProSe Direct Discovery for public safety use / Announcing/Discoveree procedure for group member discovery for MCPTT off-network CT group calls

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Power up the UE. | - | - |
| 2 | Wait for 60 sec to allow the UE to determine that it is in the Geographical area #1 set in the USIM for operation when UE is "not served by E-UTRAN and acquire reference timing. | - | - |
| - | EXCEPTION: Steps 3a1-3b3b1 describe events which depend on the UE capabilities; the "lower case letter" identifies a step sequence that takes place if the UE is capable or not of Announcing for group member discovery. | - | - |
| 3a1 | IF pc_ProSeAnnForGroupMemberDiscovery (TS 36.523-2 [75]) THEN Force the UE upper layer application corresponding to ProSe Application ID px_ProSeAnnApplicationIdentity2 (TS 36.523-3 [74]) to initiate continuous announcing its availability in a discovery group. NOTE 1. | - | - |
| 3a2 | The UE transmits in the next transmission period a PC5_DISCOVERY message for Group Member Discovery Announcement applying DUIK, DUSK, and DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PC5_DISCOVERY message. | --> | PC5_DISCOVERY |
| 3b1 | ELSE SS sets WaitForMessageCounter=1 | - | - |
| - | EXCEPTION: Steps 3b2-3b3b1 are repeated until the event described in step 3b3a1 takes place OR WaitForMessageCounter=11. | - | - |
| 3b2 | SS-UE1 transmits in the next transmission period a PC5_DISCOVERY message for Group Member Discovery Solicitation applying DUIK, DUSK, and DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PC5_DISCOVERY message. WaitForMessageCounter=WaitForMessageCounter+1 | <-- | PC5_DISCOVERY |
| - | EXCEPTION: Steps 3b3a1-3b3b1 describe events which depend on the UE behaviour; the "lower case letter" identifies a step sequence that take place if the UE transmit or not in the next transmission period a PC5_DISCOVERY message. | - | - |
| 3b3a1 | The UE transmits in the next transmission period a PC5_DISCOVERY message for Group Member Discovery Response applying DUIK, DUSK, and DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PC5_DISCOVERY message and including the target Discovery Group ID of the discovery group to be discovered in step 3b2. | --> | PC5_DISCOVERY |
| 3b3b1 | The WaitForMessageCounter=11. | - | - |
| - | EXCEPTION: Steps 4 and 5 may be repeated multiple times depending on the MCPTT procedure taking place. | - | - |
| - | EXCEPTION: Step 4 is repeated until the MCPTT protocol data unit provided by the higher layers is transmitted in full. NOTE 2. | - | - |
| 4 | SS-UE1 sends sidelink communication over the PC5 interface in the next transmission period using the timing reference provided by the GNSS simulator (same to be used by the UE). NOTE 3. | <-- | STCH PDCP SDU packet |
| - | EXCEPTION: Step 5 is repeated until the MCPTT protocol data unit provided by the higher layers is transmitted in full. NOTE 4. | - | - |
| 5 | The UE sends sidelink communication over the PC5 interface in the next transmission period using the timing reference provided by the GNSS simulator (same to be used by the SS-UE1). NOTE 3. | --> | STCH PDCP SDU packet |
| NOTE 1: UEs which are capable of Announcing for group member discovery may start announcement automatically. NOTE 2: The SS-UE1 may need to send more than one MCPTT protocol data unit in sequence with no response expected between them from the UE. NOTE 3: What MCPTT protocol data units are included in the sidelink communication is defined in the test case using the present generic procedure. NOTE 4: The UE may need to send more than one MCPTT protocol data unit in sequence with no response expected between them from the SS-UE1. |  |  |  |

5.4.10.4	Specific message contents

Table 5.4.10.4-1: PC5_DISCOVERY (step 3a2 Table 5.4.10.3-1)

Derivation path: 36.508 [6], Table 4.7F.1-5A.

Table 5.4.10.4-2: PC5_DISCOVERY (step 3b2 Table 5.4.10.3-1)

Derivation path: 36.508 [6], Table 4.7F.1-5B.

Table 5.4.10.4-3: PC5_DISCOVERY (step 3b3a1 Table 5.4.10.3-1)

Derivation path: 36.508 [6], Table 4.7F.1-5C.

### 5.4.11	Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-many communication out of E-UTRA coverage / Monitoring/Discoverer procedure for group member discovery / One-to-many communication

5.4.11.1	Initial conditions

System Simulator:

-	SS-UE1 (MCPTT Client).

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [6], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator configured to simulate a location in the centre of Geographical area #1 and providing timing reference as defined in TS 36.508 [6] Table 4.11.2-2 scenario #1, for the assistance of E-UTRAN off-network testing.

NOTE:	For operation in off-network environment, it needs to be ensured that after the UE is powered up it considers the Geographical area #1 as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

UE state:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [6].

5.4.11.2	Definition of system information messages

N/a (out of E-UTRA coverage)

5.4.11.3	Procedure

Table 5.4.11.3-1: ProSe Direct Discovery for public safety use / Monitoring/Discoverer procedure for group member discovery for MCPTT off-network CO group calls

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Power up the UE. | - | - |
| 2 | Wait for 60 sec to allow the UE to determine that it is in the Geographical area #1 set in the USIM for operation when UE is "not served by E-UTRAN and acquire reference timing. | - | - |
| - | EXCEPTION: Steps 3a1-3b3 describe events which depend on the UE capabilities; the "lower case letter" identifies a step sequence that takes place if the UE is capable or not of Monitoring for group member discovery. | - | - |
| 3a1 | IF pc_ProSeMonForGtoupMemberDiscovery (TS 36.523-2 [75]) THEN the SS-UE1 starts continuously transmitting in the relevant transmission periods a PC5_DISCOVERY message for Group Member Discovery Announcement applying DUIK, DUSK, and DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PC5_DISCOVERY message. | <-- | PC5_DISCOVERY |
| 3b1 | ELSE Force the UE upper layer application corresponding to ProSe Application ID px_ProSeAnnApplicationIdentity2 (TS 36.523-3 [74]) to solicit proximity of other UEs in a discovery group. NOTE 1. | - | - |
| 3b2 | The UE transmits in the next transmission period a PC5_DISCOVERY message for Group Member Discovery Solicitation applying DUIK, DUSK, and DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PC5_DISCOVERY message. | --> | PC5_DISCOVERY |
| 3b3 | SS-UE1 transmits a PC5_DISCOVERY message for Group Member Discovery Response applying DUIK, DUSK, and DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PC5_DISCOVERY message and including the target Discovery Group ID of the discovery group to be discovered in step 2b2. | <-- | PC5_DISCOVERY |
| - | EXCEPTION: Steps 4 and 5 may be repeated multiple times depending on the MCPTT procedure taking place. | - | - |
| - | EXCEPTION: Step 4 is repeated until the MCPTT protocol data unit provided by the higher layers is transmitted in full. NOTE 2. | - | - |
| 4 | The UE sends sidelink communication over the PC5 interface in the next transmission period using the timing reference provided by the GNSS simulator (same to be used by the SS-UE1). NOTE 3. | --> | STCH PDCP SDU packet |
| - | EXCEPTION: Step 5 is repeated until the MCPTT protocol data unit provided by the higher layers is transmitted in full. NOTE 4. | - | - |
| 5 | SS-UE1 sends sidelink communication over the PC5 interface in the next transmission period using the timing reference provided by the GNSS simulator (same to be used by the UE). NOTE 3. | <-- | STCH PDCP SDU packet |
| NOTE 1: UEs which are not capable of Monitoring for group member discovery may start Discoverer procedure automatically. NOTE 2: The UE may need to send more than one MCPTT protocol data unit in sequence with no response expected between them from the SS-UE1. NOTE 3: Which MCPTT protocol data units are included in the sidelink communication is defined in the test case using the present generic procedure. NOTE 4: The SS-UE1 may need to send more than one MCPTT protocol data unit in sequence with no response expected between them from the UE. |  |  |  |

5.4.11.4	Specific message contents

Table 5.4.11.4-1: PC5_DISCOVERY (step 3a1 Table 5.4.11.3-1)

Derivation path: 36.508 [6], Table 4.7F.1-5A.

Table 5.4.11.4-2: PC5_DISCOVERY (step 3b2 Table 5.4.11.3-1)

Derivation path: 36.508 [6], Table 4.7F.1-5B.

Table 5.4.11.4-3: PC5_DISCOVERY (step 3b3 Table 5.4.11.3-1)

Derivation path: 36.508 [6], Table 4.7F.1-5C.

### 5.4.12	Generic Test Procedure for MCPTT communication over MBMS

5.4.12.1	Initial conditions

System Simulator:

-	SS (MCPTT server)

-	SS E-UTRA

-	E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [6] clause 4.4, unless otherwise specified in the test case.

-	MBSFNAreaConfiguration as defined in TS 36.508[6] table 4.6.1-4A is transmitted on MCCH

IUT:

-	UE (MCPTT client):

-	E-UTRAN UE supporting MBMS services. The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in clause 5.4.2 and is in E-UTRA Registered, Idle Mode state. The UE is made interested in receiving MBMS service in the PLMN of Cell 1 with MBMS Service ID 0.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

5.4.12.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used. System information combination 15 as defined in TS 36.508[6] clause 4.4.3.1 is used in the E-UTRA cell.

5.4.12.3	Procedure

Table 5.4.12.3-1: MCPTT communication over MBMS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS transmits MBSFNAreaConfiguration message | <-- | MBSFNAreaConfiguration |
| 2 | Wait for a period equal to the MCCH modification period for the UE to receive MBSFNAreaConfiguration message. | - | - |
| - | EXCEPTION: Step 3 is repeated continuously to carry the relevant MCPTT protocol data units provided by the higher layers. | - | - |
| 3 | The SS transmits 1 MBMS Packet on the MTCH in the next MCH Scheduling Period. NOTE: Which MCPTT protocol data units are sent and at which time is defined in the test case using the present generic procedure. | <-- | MBMS Packet |

5.4.12.4	Specific message contents

None.

### 5.4.13	Void