3GPP TSG-RAN WG5 Meeting #94-e	R5-220465

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0235 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.16 - SIP 1xx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Neither 24.379 nor 24.281 have any requirement for the UE to include the isfocus media tag in a provisional response for an INVITE and according to the IANA description for isfocus this would not make sense either (as the UE is no conference server). - There are no provisional responses for MCData - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - isfocus removed from Table 5.5.2.16.2.1-1 - Condition MCDATA removed from Table 5.5.2.16.2.1-1 and Table 5.5.2.16.2.2-1 - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.16	SIP 1xx

##### 5.5.2.16.1	SIP 100 (Trying)

This message is sent by the UE or the SS.

Table 5.5.2.16.1-1: SIP 100 (Trying)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "100" |  |  |  |
| Reason-Phrase | "Trying" |  |  |  |
| Via |  |  |  |  |
| via-parm | same value as received in INVITE message |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in INVITE message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| Content-Length | Optional in case of the message being sent by the UE |  |  |  |
| value | "0" | No message body included - end of SIP message |  |  |

##### 5.5.2.16.2	SIP 180 (Ringing)

5.5.2.16.2.1	SIP 180 (Ringing) from the UE

Table 5.5.2.16.2.1-1: SIP 180 (Ringing) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "180" |  |  |  |
| Reason-Phrase | "Ringing" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] |  |
| rec-route | same as received in INVITE message |  |  |  |
| Via | same as received in INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in INVITE message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in the INVITE message or any value if missing in the INVITE message. |  |  |  |
| Contact |  |  |  |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  | RFC 3262 [97] | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| Content-Length | if present |  |  |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Reponse sent reliable according to RFC 3262 [97] |

5.5.2.16.2.2	SIP 180 (Ringing) from the SS

Table 5.5.2.16.2.2-1: SIP 180 (Ringing) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "180" |  |  |  |
| Reason-Phrase | "Ringing" |  |  |  |
| Record-Route | same as spefied for the SIP 200 (OK) from the SS in table 5.5.2.17.1.2-1 with condition INVITE-RSP |  | RFC 3261 [22] |  |
| Via | same as received in the INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | Public user id of the callee (px_IMS_CalleeContactUri) | Callee contact Uri |  |  |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "isfocus" |  |  |  |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  | RFC 3262 [97] | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one; arbitrarily selected if there is no previous RSeq number |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| Content-Length |  |  |  |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Reponse sent reliable according to RFC 3262 [97] |

##### 5.5.2.16.3	SIP 183 (Session Progress)

5.5.2.16.3.1	SIP 183 (Session Progress) from the UE

Table 5.5.2.16.3.1-1: SIP 183 (Session Progress) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "183" |  |  |  |
| Reason-Phrase | "Session progress" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] |  |
| rec-route | same as received in INVITE message |  |  |  |
| Via | same as received in INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in INVITE message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in the INVITE message or any value if missing in the INVITE message. |  |  |  |
| Contact |  |  |  |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  |  | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| P-Answer-State | if present |  |  |  |
| value | "unconfirmed" |  |  |  |
| Content-Length | if present |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Reponse sent reliable according to RFC 3262 [97] |

5.5.2.16.3.2	SIP 183 (Session Progress) from the SS

Table 5.5.2.16.3.2-1: SIP 183 (Session Progress) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "183" |  |  |  |
| Reason-Phrase | "Session progress" |  |  |  |
| Record-Route | same as specified for the SIP 200 (OK) from the SS in table 5.5.2.17.1.2-1 with condition INVITE-RSP |  | RFC 3261 [22] |  |
| Via | same as received in the INVITE message |  | RFC 3261 [22] RFC 3581 [55] |  |
| Require |  |  |  | 100rel |
| option-tag | "100rel" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | Public user id of the callee (px_IMS_CalleeContactUri) | Callee contact Uri |  |  |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.icsi-ref= urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "isfocus" |  |  |  |
| Supported |  |  |  |  |
| option-tag | "norefersub" |  |  |  |
| Rseq |  |  |  | 100rel |
| response-num | previous RSeq number sent in the same direction incremented by one; arbitrarily selected if there is no previous RSeq number |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in INVITE message |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in INVITE message |  |  |  |
| P-Answer-State |  |  |  |  |
| value | "unconfirmed" |  |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
| port | not present |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included |  |  |

| Condition | Explanation |
| --- | --- |
| 100rel | Response sent reliable according to RFC 3262 [97] |