3GPP TSG-RAN WG5 Meeting #94-e	R5-220466

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0236 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.17 - SIP 2xx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 24.379 clause 6.3.2.1.5.2 (Response to an INVITE request, Final response) the participating function shall include in the Contact header field of the final 200 OK for an INVITE - the g.3gpp.mcptt media feature tag - the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" - the isfocus media feature tag Similar requirements are in 24.281 (clause 6.3.2.1.4.2) and in 24.282 (clause 9.2.3.3.3 for SDS, clause 9.2.4.3.3 for SDS session and clause 10.2.5.3.3 for FD). - Neither 24.379 nor 24.281 nor 24.282 have any requirement for the UE to include the isfocus media tag in the final 200 OK for an INVITE and according to the IANA description for isfocus this would not make sense either (as the UE is no conference server). - The Contact address sent to the UE in the final 200 OK for an INVITE of an MCData call is a session identity according to 24.282 clause 4.2.2. - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Missing media tags added for condition INVITE-RSP in Table 5.5.2.17.1.2-1. - isfocus removed from Table 5.5.2.17.1.1-1 - Contact address corrected for MCDATA in Table 5.5.2.17.1.2-1. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.17	SIP 2xx

##### 5.5.2.17.1	SIP 200 (OK)

5.5.2.17.1.1	SIP 200 (OK) from the UE

Table 5.5.2.17.1.1-1: SIP 200 (OK) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "200" |  |  |  |
| Reason-Phrase | "OK" |  |  |  |
| Via | same as received in the request |  | RFC 3261 [22] RFC 3581 [55] |  |
| Record-Route |  |  | RFC 3261 [22] | INVITE-RSP |
| rec-route | same as received in the request |  |  |  |
| From |  |  |  |  |
| addr-spec | Same value as received in the request |  |  |  |
| tag | same value as received in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in the request |  |  |  |
| tag | same value as received in the request or any value if missing in the request. |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref= urn:urn- 7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| feature-param | "text" |  |  | MCDATA |
| Call-ID |  |  |  |  |
| callid | same value as received in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in the request |  |  |  |
| Require |  |  |  | INVITE-RSP |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  |  | INVITE-RSP |
| delta-seconds | Same value as session expires header in SIP INVITE |  | RFC 4028 [30] TS 24.229 [16] cl.5.1.4.1 |  |
| refresher | "uas" |  |  |  |
| Content-Type |  |  | RFC 5621 [58] | INVITE-RSP |
| value | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| P-Answer-State | If present |  | RFC 4964 [118] TS 24.379 [9] clause 6.2.3.1.2 | INVITE-RSP AND GROUP-CALL |
| answer-type | “confirmed” |  |  |  |
| Message-body | not present |  | RFC 3261 [22] |  |
| Message-body |  |  | RFC 3261 [22] | INVITE-RSP |
| MIME body part |  | SDP message |  |  |
| MIME-part-header |  |  |  |  |
| MIME-Content-Type | "application/sdp" |  | RFC 4566 [27] |  |
| MIME-part-body | SDP message as described in Table 5.5.3.1.1-1 |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.1-2 |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.1-3 |  |  | MCDATA |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-header |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| INVITE-RSP | 200 OK is the response to the SIP INVITE |
| MCDATA_SDS | INVITE for SDS communication |
| MCDATA_FD | INVITE for FD communication |

5.5.2.17.1.2	SIP 200 (OK) from the SS

Table 5.5.2.17.1.2-1: SIP 200 (OK) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "200" |  |  |  |
| Reason-Phrase | "OK" |  |  |  |
| Via | same as received in the request |  | RFC 3261 [22] RFC 3581 [55] |  |
| Record-Route |  |  | RFC 3261 [22] | INVITE-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | pcscf.other.com |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | scscf.other.com |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[3] | SIP URI |  |  |  |
| user-info and host | orig@scscf.3gpp.org |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[4] | SIP URI |  |  |  |
| user-info and host | same address as sent by the UE in the first entry of the Route header of the INVITE | P-CSCF address |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] | SUBSCRIBE-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Expires |  |  | RFC 3261 [22] RFC 3903 [43] | SUBSCRIBE-RSP, PUBLISH-RSP |
| value | same value as in the request |  |  |  |
| Contact |  |  |  | REGISTER-RSP |
| addr-spec | same value as received in the REGISTER |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
| feature-param | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA |
| feature-param | "+g.3gpp.mcdata.fd" |  |  | MCDATA |
| expires | "600000" |  |  |  |
| Contact |  |  |  | SUBSCRIBE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
|  | "sip:" & tsc_MCX_CMS_Hostname |  |  | CONFIG |
|  | "sip:" & tsc_MCX_GMS_Hostname |  |  | GROUPCONFIG |
| port | not present |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_SessionId |  |  | MCDATA |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref= urn:urn- 7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | “audio” |  |  | MCPTT OR MCVIDEO |
| feature-param | “video” |  |  | MCVIDEO |
| feature-param | “text” |  |  | MCDATA |
| feature-param | "isfocus" |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in the request |  |  |  |
| Require |  |  |  | INVITE-RSP |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  |  | INVITE-RSP |
| generic-param | "3600" |  |  |  |
| refresher | "uac" |  |  |  |
| Supported |  |  |  | INVITE-RSP |
| option-tag | "tdialog" |  |  |  |
| option-tag | "norefersub" |  |  |  |
| option-tag | "explicitsub" |  |  |  |
| option-tag | "nosub" |  |  |  |
| Refer-Sub |  |  | RFC 4488 [36] | REFER-RSP |
| refer-sub-value | “false” |  |  |  |
| P-Associated-URI |  |  | RFC 7315 [52] | REGISTER-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| host | px_MCX_SIP_PublicUserId_A_1 |  |  |  |
| port | not present |  |  |  |
| Service-Route |  |  | RFC 3261 [22] | REGISTER-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| host | scscf.3gpp.org |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| SIP-ETag |  |  | RFC 3903 [43] | PUBLISH-RSP |
| entity-tag | unique value arbitrarily selected by the SS |  |  |  |
| Content-Type |  |  | RFC 4566 [27] | INVITE-RSP |
| media-type | "application/sdp" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] | INVITE-RSP |
| SDP message | SDP message as described in Table 5.5.3.1.2-1 |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.2-2 |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.2-3 | FFS |  | MCDATA |

| Condition | Explanation |
| --- | --- |
| REGISTER-RSP | 200 OK is the response to a SIP REGISTER |
| INVITE-RSP | 200 OK is the response to a SIP INVITE |
| SUBSCRIBE-RSP | 200 OK is the response to a SIP SUBSCRIBE |
| PUBLISH-RSP | 200 OK is the response to a SIP PUBLISH |
| REFER-RSP | 200 OK is the response to a SIP REFER |

##### 5.5.2.17.2	SIP 202 (Accepted)

Table 5.5.2.17.2-1: SIP 202 (Accepted)

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  | RFC 3261 [22] |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "202" |  |  |  |
| Reason-Phrase | "Accepted" |  |  |  |
| Via | same value as received in request |  | RFC 3261 [22] |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec | same value as received in request |  |  |  |
| tag | same value as received in request |  |  |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | same value as received in request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | same value as received in request |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | same value as received in request |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" |  |  |  |