3GPP TSG-RAN WG5 Meeting #94-e	R5-220472

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0242 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.10 - MCData Protected Payload Message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - MCData Protected Payload Message tables have not considered file distribution yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MCData Protected Payload Message tables extended to support file distribution |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.10	MCData Protected Payload Message

Table 5.5.3.10-1: MCData Protected Payload Message from the UE

| Derivation Path: TS 33.180 [94] clause 8.5.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type | Same message type as in the MCData message contained as Payload but with bit 7 set to '1'B |  |  | PROTECTED_MESSAGE |
|  | '01??????'B | NOTE: TS 33.180 [94] does not specify any message type |  | PROTECTED_FILE |
|  | '01111010B | '7A'O; IEI | TS 24.282 [87] clause 15.1.4 | PROTECTED_PAYLOAD |
| Date and Time | Any allowed value | Date and Time of creation of protected payload message |  |  |
| Payload ID | Any allowed value | The identifier for the payload. |  |  |
| Payload sequence number | Any allowed value | The sequence number of the protected payload. |  |  |
| Payload Algorithm | '01'O | DP_AES_128_GCM |  |  |
| Signalling algorithm | not present |  |  |  |
| IV | Any allowed value | Initialisation vector (or nonce) for message. Length depends on the algorithm and key used. 128 bits or 256 bits depending on the algorithm. |  |  |
| DPPK-ID | PCK-ID |  |  | PROTECTED_PAYLOAD, PCK |
|  | GMK-ID |  |  | GMK |
|  | CSK-ID |  |  | CSK |
| Payload |  | Protected Payload (Ciphertext) | TS 24.282 [87] clause 15.2.13 |  |
| Payload IEI | '78'O | Value as used in MCData messages in TS 24.282 [87] |  |  |
| Length of Payload contents | length of the content |  |  |  |
| Payload content type | '02'O | BINARY |  |  |
| Payload contents | Encrypted MCData message (NOTE 1) |  |  | PROTECTED_MESSAGE |
|  | Encrypted file or portion of file |  |  | PROTECTED_FILE |
|  | Encrypted Payload(s) of the unprotected DATA PAYLOAD message (NOTE 2) |  |  | PROTECTED_PAYLOAD |
| NOTE 1: The whole message is encrypted (including its message type) NOTE 2: The whole payload(s) are encrypted (including their IEI and length); in general there is only one payload |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PROTECTED_MESSAGE | The MCData Protected Payload message contains a whole encrypted MCData message |
| PROTECTED_FILE | The MCData Protected Payload message contains encrypted binary data representing a file or portion of a file |
| PROTECTED_PAYLOAD | The MCData Protected Payload message contains the Payload IE(S) of the MCData DATA PAYLOAD message |
| PCK | Encryption uses PCK |
| GMK | Encryption uses GMK |
| CSK | Encryption uses CSK |

Table 5.5.3.10-2: MCData Protected Payload Message from the SS

| Derivation Path: TS 33.180 [94] clause 8.5.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type | Same message type as in the MCData message contained as Payload but with bit 7 set to '1'B |  |  | PROTECTED_MESSAGE |
|  | '01000011'B | '43'O; same as for protected DATA PAYLOAD |  | PROTECTED_FILE |
|  | '01111010B | '7A'O; IEI | TS 24.282 [87] clause 15.1.4 | PROTECTED_PAYLOAD |
| Date and Time | The current date and time | Date and Time of creation of protected payload message |  |  |
| Payload ID | “1” | The identifier for the payload. |  |  |
| Payload sequence number | “1” | The sequence number of the protected payload. |  |  |
| Payload Algorithm | '01'O | DP_AES_128_GCM |  |  |
| Signalling algorithm | not present |  |  |  |
| IV | 'DCB9085150B3CF21E2F7DF5B542C25C2'O | Initialisation vector (or nonce) for message. Length depends on the algorithm and key used. 128 bits or 256 bits depending on the algorithm. |  |  |
| DPPK-ID | PCK-ID |  |  | PROTECTED_PAYLOAD, PCK |
|  | GMK-ID |  |  | GMK |
|  | CSK-ID |  |  | CSK |
| Payload |  | Protected Payload (Ciphertext) | TS 24.282 [87] clause 15.2.13 |  |
| Payload IEI | '78'O | Value as used in MCData messages in TS 24.282 [87] |  |  |
| Length of Payload contents | length of the content |  |  |  |
| Payload content type | '02'O | BINARY |  |  |
| Payload contents | Encrypted MCData message (NOTE 1) |  |  | PROTECTED_MESSAGE |
|  | Encrypted fiel or portion of file |  |  | PROTECTED_FILE |
|  | Encrypted Payload(s) of the unprotected DATA PAYLOAD message (NOTE 2) |  |  | PROTECTED_PAYLOAD |
| NOTE 1: The whole message is encrypted (including its message type) NOTE 2: The whole payload(s) are encrypted (including their IEI and length); in general there is only one payload |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PROTECTED_MESSAGE | The MCData Protected Payload message contains a whole encrypted MCData message |
| PROTECTED_FILE | The MCData Protected Payload message contains encrypted binary data representing a file or portion of a file |
| PROTECTED_PAYLOAD | The MCData Protected Payload message contains the Payload IE(S) of the MCData DATA PAYLOAD message |
| PCK | Encryption uses PCK |
| GMK | Encryption uses GMK |
| CSK | Encryption uses CSK |