3GPP TSG-RAN WG5 Meeting #94-e	R5-220474

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0244 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.8 - MCData Data signalling messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - MCDATA FD SIGNALLING PAYLOAD messages have been specified in a separate clause (5.5.3.13) even though they are also MCData Data signalling messages - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MCDATA FD SIGNALLING PAYLOAD messages corrected and moved to clause 5.5.3.8; FD NOTIFICATION messages added - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.8	MCData Data signalling messages

The MCData Data signalling messages specified in this clause are protected according to TS 33.180 clause 8.5.4, i.e. a MCData Data signalling message is contained in the protected payload of a MCData Protected Payload Message according to clause 5.5.3.10 with condition PROTECTED_MESSAGE and CSK.

The following conditions apply throughout clause 5.5.3.8:

Table 5.5.3.8-1: Conditions

| Condition | Explanation |
| --- | --- |
| DELIVERED | Disposition request/notification type DELIVERED |
| READ | Disposition request/notification type READ |
| DELIVERED_READ | Disposition request/notification type DELIVERED AND READ |
| FD_ACCEPTED | Disposition notification type FILE DOWNLOAD REQUEST ACCEPTED |
| FD_REJECTED | Disposition notification type FILE DOWNLOAD REQUEST REJECTED |
| FD_COMPLETED | Disposition notification type FILE DOWNLOAD COMPLETED |
| FD_DEFERRED | Disposition notification type FILE DOWNLOAD DEFERRED |
| FD_HTTP | FD Message for FD using using HTTP |
| FD_MSRP | FD Message for FD using media plane |

##### 5.5.3.8.1	SDS SIGNALLING PAYLOAD message from the UE

Table 5.5.3.8.1-1: SDS SIGNALLING PAYLOAD message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS signalling payload message identity | '00000001'B | SDS SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Any allowed value | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Any allowed value | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| SDS disposition request type | '0001'B |  | TS 24.282 [87] clause 15.2.3 | DELIVERED |
|  | '0010'B |  |  | READ |
|  | '0011'B |  |  | DELIVERED_READ |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| User location | Any allowed value if present |  | TS 24.282 [87] clause 15.2.25 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.2	SDS SIGNALLING PAYLOAD message from the SS

Table 5.5.3.8.2-1: SDS SIGNALLING PAYLOAD message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS signalling payload message identity | '00000001'B | SDS SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | '01010101010101010101010101010101'O | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | '01010101010101010101010101010101'O | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| SDS disposition request type | '0001'B |  | TS 24.282 [87] clause 15.2.3 | DELIVERED |
|  | '0010'B |  |  | READ |
|  | '0011'B |  |  | DELIVERED_READ |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| User location | Not present |  | TS 24.282 [87] clause 15.2.25 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.3	SDS NOTIFICATION message from the UE

Table 5.5.3.8.3-1: SDS NOTIFICATION message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS notification message identity | '00000101'B | SDS NOTIFICATION | TS 24.282 [87] clause 15.2.2 |  |
| SDS disposition notification type | '00000010'B |  | TS 24.282 [87] clause 15.2.5 | DELIVERED |
|  | '00000011'B |  |  | READ |
|  | '00000100'B |  |  | DELIVERED_READ |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD sent to the UE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD sent to the UE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.4	SDS NOTIFICATION message from the SS

Table 5.5.3.8.4-1: SDS NOTIFICATION message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS notification message identity | '00000101'B | SDS NOTIFICATION | TS 24.282 [87] clause 15.2.2 |  |
| SDS disposition notification type | '00000010'B |  | TS 24.282 [87] clause 15.2.5 | DELIVERED |
|  | '00000011'B |  |  | READ |
|  | '00000100'B |  |  | DELIVERED_READ |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD received from the UE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD received from the UE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.5	FD SIGNALLING PAYLOAD message from the UE

Table 5.5.3.8.5-1: FD SIGNALLING PAYLOAD message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD signalling payload message identity | '00000010'B | FD SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Any allowed value | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Any allowed value | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| FD disposition request type | “0001” | FILE DOWNLOAD COMPLETED UPDATE | TS 24.282 [87] clause 15.2.4 |  |
| Mandatory download | Not present | Not present indicates a Non-Mandatory download | TS 24.282 [87] clause 15.2.16 |  |
|  | '0001'B | MANDATORY DOWNLOAD |  | FD_MSRP |
| Payload |  |  | TS 24.282 [87] clause 15.2.13 | FD_HTTP |
| Length of Payload contents | Length of the payload contents |  |  |  |
| Payload content type | “00000100” | FILEURL |  |  |
| Payload contents | same URL as assigned by the SS in the HTTP 201 (Created) response to the HTTP POST request |  |  |  |
| Metadata | if present | Metadata is optional | TS 24.282 [87] clause 15.2.17 | FD_HTTP |
| file-selector | Any allowed value |  |  |  |
| file-date | Any allowed value |  |  |  |
| file-availability | Any allowed value |  |  |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |

##### 5.5.3.8.6	FD SIGNALLING PAYLOAD message from the SS

Table 5.5.3.8.6-1: FD SIGNALLING PAYLOAD message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD signalling payload message identity | '00000010'B | FD SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | '01010101010101010101010101010101'O | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | '01010101010101010101010101010101'O | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| FD disposition request type | '0001'B | FILE DOWNLOAD COMPLETED UPDATE | TS 24.282 [87] clause 15.2.4 |  |
| Mandatory download | Not present | Not present indicates a Non-Mandatory download | TS 24.282 [87] clause 15.2.16 |  |
|  | '0001'B | MANDATORY DOWNLOAD |  | FD_MSRP |
| Length of Payload contents | Length of the payload contents |  |  |  |
| Payload content type | “00000100” | FILEURL |  |  |
| Payload contents | tsc_MCData_MSF_URI & "/" & sub-path | URL identifying the location of the stored file; sub-path is arbitrarily selected by the SS and shall be different for every file upload of a test case |  |  |
| Metadata |  | NOTE 1 | TS 24.282 [87] clause 15.2.17 | FD_HTTP |
| file-selector |  |  | RFC 5547 [124] |  |
| filename | name of the file | e.g. "TestFile.txt" |  |  |
| filesize | size of the file |  |  |  |
| type | type of the file | e.g. "text/plain" |  |  |
| hash |  |  |  |  |
| algorithm | "sha-1" |  |  |  |
| value | hash value of the file |  |  |  |
| file-date |  |  | RFC 5547 [124] |  |
| date-param[1] |  |  |  |  |
| type | "creation" |  |  |  |
| date-time | date and time when the file has been created | e.g. "Mon, 20 Dec 2021 15:01:31 +0100" | RFC 5322 [109] |  |
| file-availability | Date and time until which the file is available | e.g. "Fri, 30 Dec 2050 23:59:59 +0100" | TS 24.282 [87] table 15.2.17-1 |  |
| file-description | "Test file" |  | TS 24.282 [87] table 15.2.17-1 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |
| NOTE 1: file-selector, file-date, file-availability and file-description are concatenated using CRLF (carriage-return/line-feed) as separator |  |  |  |  |

##### 5.5.3.8.7	FD NOTIFICATION message from the UE

Table 5.5.3.8.7-1: FD NOTIFICATION message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD notification message identity | '00000110'B | FD NOTIFICATION | TS 24.282 [87] clause 15.2.2 |  |
| FD disposition notification type | '00000001'B |  | TS 24.282 [87] clause 15.2.6 | FD_ACCEPTED |
|  | '00000010'B |  |  | FD_REJECTED |
|  | '00000011'B |  |  | FD_COMPLETED |
|  | '00000100'B |  |  | FD_DEFERRED |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Same value as in the corresponding FD SIGNALLING PAYLOAD sent to the UE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Same value as in the corresponding FD SIGNALLING PAYLOAD sent to the UE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.8	FD NOTIFICATION message from the SS

Table 5.5.3.8.8-1: FD NOTIFICATION message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD notification message identity | '00000110'B | FD NOTIFICATION | TS 24.282 [87] clause 15.2.2 |  |
| FD disposition notification type | '00000001'B |  | TS 24.282 [87] clause 15.2.6 | FD_ACCEPTED |
|  | '00000010'B |  |  | FD_REJECTED |
|  | '00000011'B |  |  | FD_COMPLETED |
|  | '00000100'B |  |  | FD_DEFERRED |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Same value as in the corresponding FD SIGNALLING PAYLOAD received from the UE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Same value as in the corresponding FD SIGNALLING PAYLOAD received from the UE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |