3GPP TSG-RAN WG5 Meeting #94-e	R5-220476

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0246 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.6 - Default MCPTT media plane control messages and other information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In all cases where condition ON-NETWORK is used in tables for floor control it is to express that in contrast to OFF-NETWORK a particular field is not present; but this can be considered as default case i.e. the fields are per default not present but present only for OFF-NETWORK ON-NETWORK is not needed to specified where the tables are referred to. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition ON-NETWORK removed from tables of floor control messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.6	Default MCPTT media plane control messages and other information elements

#### 5.5.6.1	General

The media plane control protocols messages specified in the present document are based on those specified in TS 24.380 [10] which in term are based on the RTCP Application Packets (RTCP: APP), as defined in IETF RFC 3550 [76].

Depending on the TC scenario, the same MCPTT media plane control message can be sent by the SS or by the UE. Throughout the default content specified in below a particular value has been chosen to satisfy one or the other scenario. It is expected that when a message is used in a TC in a particular context then the relevant for the usage in the TC values will be defined in the TC.

The following conditions apply throughout clause 5.5.6:

Table 5.5.6.1-1: Conditions

| Condition | Explanation |
| --- | --- |
| FA | IE for when an active Functional Alias is used |
| Multi-Talker | IE for when a Multi Talker call is active |
| ACK | Message requests a Floor Ack |
| NOTE: For further conditions see table 5.5.1-1 |  |

Considerations in regard to describing specific values:

-	SSRC

-	Synchronization SouRCe (SSRC) values are used in most of the messages specified in clause 5.5.6. The SSRC value is randomly chosen by the participant in, and globally unique within, an RTP session as specified in IETF RFC 3550 [76]. Because the value chosen by the UE (MCPTT client) cannot be controlled, specifying a "hard coded" value to be used by the SS (MCPTT server) or the SS-UE (MCPTT Client) is prone to triggering a collision by choosing a value which may be the same as the one chosen by the UE. How to resolve SSRC collisions is described in IETF RFC 3550 [76] however, resolving them as part of the MCPTT test case definitions e.g. in TS 36.579-2 [2] is not foreseen and is left to the test implementation.

-	For the purposes of default and specific messages definition throughout the present specification, as well as, throughout the rest of the MCPTT conformance test specifications e.g. the TS 36.579-2 [2] no explicit SSRC values are defined and instead the following notation is used to clarify the messages origin/destination:

-	When there is no danger for misunderstanding the notation 'The SSRC of the message sender' and the 'The SSRC of the intended recipient of the message' are used whereas the "sender" and the "recipient" are to be understood in the context of the test i.e. the test entities being involved to exchange messages.

-	When in doubt, the notations 'UE (MCPTT client) SSRC', SS (MCPTT server) SSRC', 'SS-UE1 (MCPTT Client) SSRC' or 'SS-UE2 (MCPTT Client) SSRC' are used.

#### 5.5.6.2	Floor Request

Table 5.5.6.2-1: Floor Request

| Derivation Path: 24.380 [10], Table 8.2.4-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Floor Request |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor participant sending the message. Notation in accordance with clause 5.5.6.1. |  |
| name | MCPT |  |  |
| Floor priority | Not present or Any allowed value | If present, a value between ‘0’ and ‘255’ where '0' is the lowest priority If the Floor Priority field is not included in the message the default priority (='0') is used as the Floor Priority value The max floor priority that can be requested in a Floor Request message is negotiated between the MCPTT client and the controlling MCPTT function using the "mc_priority" fmtp parameter e.g. at call setup |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant requesting the floor. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 10000x0000000000 | Normal call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise |  |
|  | 00010x0000000000 | Emergency call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | EMERGENCY-CALL |
|  | 00001x0000000000 | Imminent Peril call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | IMMPERIL-CALL |
| Functional Alias | Not present |  |  |
|  | px_MCPTT_ID_FA_A | Functional Alias = URI | FA |
| Location | optional |  |  |
| Location Type | Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present or Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location |  |  | REL-15 |
| Location Type | Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present or Any allowed value | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |

| Condition | Explanation |
| --- | --- |
| REL-15 | In effect when PICS "PICS FFS" is in effect |

#### 5.5.6.3	Floor Granted

Table 5.5.6.3-1: Floor Granted

| Derivation Path: 24.380 [10], Table 8.2.5-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Floor Granted with acknowledgment not required |  |
|  | 10001 | Floor Granted with acknowledgment required | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| Duration |  |  |  |
| Duration | "00000000 10000000" | 128 sec (an arbitrary value) |  |
| SSRC of granted floor participant | The SSRC of the intended recipient of the message | Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| Floor priority | Not present | If the Floor Priority field is not included in the message the default priority (='0') is used as the Floor Priority value |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant granted the floor. |  |
| Queue Size | Not present |  |  |
| Queue Size | "0" | the number of queued MCPTT clients in the MCPTT call | OFF-NETWORK |
| SSRC of queued floor participant | Not present |  |  |
| Queued User ID | Not present |  |  |
| Queue Info | Not present |  |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 0000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.4	Floor Deny

Table 5.5.6.4-1: Floor Deny

| Derivation Path: 24.380 [10], Table 8.2.6-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00011 | Floor Deny with acknowledgment not required |  |
|  | 10011 | Floor Deny with acknowledgment required | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| Reject Cause |  |  |  |
| Reject Cause | "1" | Cause #1 - Another MCPTT client has permission |  |
| Reject Phrase | "Another MCPTT client has permission" | An additional text string explaining the reason for rejecting the floor request. |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant being denied floor request. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.5	Floor Release

Table 5.5.6.5-1: Floor Release

| Derivation Path: 24.380 [10], Table 8.2.7-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00100 | Floor Release with acknowledgment not required |  |
|  | 10100 | Floor Release with acknowledgment required | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the floor participant sending the message. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76] |  |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT User ID of the floor participant releasing the floor. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 10000x0000000000 | Normal call x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise |  |
|  | 00010x0000000000 | Emergency call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | EMERGENCY-CALL |
|  | 00001x0000000000 | Imminent Peril call: x:=1 if pc_MCPTT_FloorRequestQueueing = “true”, x:=0 otherwise | IMMPERIL-CALL |

#### 5.5.6.6	Floor Idle

Table 5.5.6.6-1: Floor Idle

| Derivation Path: 24.380 [10], Table 8.2.8-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00101 | Floor Idle with acknowledgment not required |  |
|  | 10101 | Floor Idle with acknowledgment required | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| Message Sequence Number |  |  |  |
| Message Sequence Number | The value sent in the previous Floor Idle message, if any, increased with 1 | Any value between '0' and '65535' When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.7	Floor Taken

Table 5.5.6.7-1: Floor Taken

| Derivation Path: 24.380 [10], Table 8.2.9-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Floor Taken with acknowledgment not required |  |
|  | 10010 | Floor Taken with acknowledgment required | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT user ID of the floor participant sending the Floor Taken message |  |
| Granted Party's Identity |  |  |  |
| Granted Party's Identity | px_MCPTT_ID_User_B | The MCPTT User ID of the floor participant being granted the floor. |  |
| Granted Party's Identity | Not Present |  | Multi-Talker |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "1" | The receiver is permitted to request floor |  |
| Message Sequence Number |  |  |  |
| Message Sequence Number | The value sent in the previous Floor Taken message, if any, increased with 1 | Any value between '0' and '65535' When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |
| Floor Indicator | 1000010010000000 | Normal call, queueing supported, multi-talker | Multi-Talker |
|  | 0001010010000000 | Emergency call, queueing supported, multi-talker | EMERGENCY-CALL |
|  | 0001100010000000 | Imminent peril call, queueing supported, multi-talker | IMMPERIL-CALL |
| SSRC of granted floor participant | SS-UE1 (MCPTT Client) SSRC | The SSRC of the granted floor participant. |  |
| SSRC of granted floor participant | Not present |  | Multi-Talker |
| Functional Alias | Not present |  |  |
|  | px_MCPTT_ID_FA_B | Functional Alias = URI | FA AND NOT Multi-Talker |
| List of Granted Users | Not present |  |  |
| List of Granted Users |  |  | Multi-Talker |
| No of users | '10' |  |  |
| User ID | px_MCPTT_ID_User_A |  |  |
| User ID | px_MCPTT_ID_User_B |  |  |
| List of SSRCs of granted floor participants | Not present |  |  |
| List of SSRCs of granted floor participants |  |  | Multi-Talker |
| Number of SSRCs | '10' |  |  |
| SSRC | The SSRC of User A |  |  |
| SSRC | The SSRC of User B |  |  |
| List of Functional Aliases | Not present |  |  |
| List of Functional Aliases |  |  | FA AND Multi-Talker |
| No of FAs | '10' |  |  |
| Functional Alias | px_MCPTT_ID_FA_A |  |  |
| Functional Alias | px_MCPTT_ID_FA_B |  |  |
| Location |  |  | NOT Multi-Talker |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location | Not present |  | Multi-Talker |
| List of Locations | Not present |  | NOT Multi-Talker |
| List of Locations |  | The location information shall be maintained in the same order as the users in the List of Granted Users to allow location information to be matched to the correct user. | Multi-Talker |
| Number of Locations | '10' |  |  |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |
| Location Type | '00000000' | Not provided See TS 24.380 [10] Table 8.2.3.21-3 |  |
| Location Value | Not present | See TS 24.380 [10] Table 8.2.3.21-3. Not present if Location Type is set to "Not provided" |  |

#### 5.5.6.8	Floor Revoke

Table 5.5.6.8-1: Floor Revoke

| Derivation Path: 24.380 [10], Table 8.2.10.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00110 | Floor Revoke |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| Reject Cause |  |  |  |
| Reject Cause | "4" | Cause#4 - Media Burst pre-empted |  |
| Reject Phrase | "Media Burst pre-empted" | a text string encoded the text string in the SDES item CNAME as specified in IETF RFC 3550 [76], clause 6.5.1. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.9	Floor Queue Position Request

Table 5.5.6.9-1: Floor Queue Position Request

| Derivation Path: 24.380 [10], Table 8.2.11-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01000 | Floor Queue Position Request |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor participant sending the message. Notation in accordance with clause 5.5.6.1. Codedas specified in IETF RFC 3550 [76] |  |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_A | The MCPTT ID of the floor participant requesting the information. |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |

#### 5.5.6.10	Floor Queue Position Info

Table 5.5.6.10-1: Floor Queue Position Info

| Derivation Path: 24.380 [10], Table 8.2.12-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01001 | Floor Queue Position Info with acknowledgment not required |  |
|  | 11001 | Floor Queue Position Info with acknowledgment required | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPT |  |  |
| User ID | Not present |  |  |
| User ID |  |  | OFF-NETWORK |
| User ID | px_MCPTT_ID_User_B | the MCPTT ID of the floor participant sending the Floor Queue Position Info message |  |
| SSRC of queued floor participant | Not present |  |  |
|  | The SSRC of the message recepient | The SSRC field carries the SSRC of the queued floor participant | OFF-NETWORK |
| Queued User ID | Not present |  |  |
| Queued User ID |  |  | OFF-NETWORK |
| Queued User ID | px_MCPTT_ID_User_A | the MCPTT ID of the queued floor participant |  |
| Queue Info |  |  |  |
| Queue Position Info | "1" |  |  |
| Queue Priority Level | "0" |  |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010000000000 | Normal call, queueing supported |  |
|  | 0001010000000000 | Emergency call, queueing supported | EMERGENCY-CALL |
|  | 000110000000000 | Imminent peril call, queueing supported | IMMPERIL-CALL |

#### 5.5.6.11	Floor Ack

Table 5.5.6.11-1: Floor Ack

| Derivation Path: 24.380 [10], Table 8.2.13-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01010 | Floor Ack |  |
| SSRC | The SSRC of the SS | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. | DOWNLINK |
|  | The SSRC of the UE | The SSRC of the floor participant sending the message Notation in accordance with subclause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. | UPLINK |
| name | MCPT |  |  |
| Source |  |  |  |
| Source | "2" | The controlling MCPTT function is the sender of the message see TS 24.380[10] cl 4.2.1 and cl. 8.2.3.12 | DOWNLINK |
| Source | "0" | The Floor participant is the sender of the message see TS 24.380[10] cl 6.2 and cl. 8.2.3.12 | UPLINK |
| Message Type |  |  |  |
| Message Type | "0001xxxx" | Message Type of the Floormessage which requested the acknowledgment |  |
| Track Info | Not present | The MCPTT call does not involve a non-controlling MCPTT function |  |

| Condition | Explanation |
| --- | --- |
| UPLINK | The message is sent from the UE |
| DOWNLINK | The message is sent from the SS |
| For further conditions see table 5.5.6.1-1 |  |

#### 5.5.6.11A	Floor Release Multi Talker

Table 5.5.6.11A-1: Floor Release Multi Talker

| Derivation Path: 24.380 [10], Table 8.2.14-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 01111 | Floor Release Multi Talker |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor participant sending the message. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76] |  |
| name | MCPT |  |  |
| User ID |  |  |  |
| User ID | px_MCPTT_ID_User_B | The MCPTT User ID of the floor participant releasing the floor. |  |
| Floor Indicator |  |  |  |
| Floor Indicator | 1000010010000000 | Normal call, queueing, multi-talker |  |
|  | 0001010010000000 | Emergency call, queueing supported, multi-talker | EMERGENCY-CALL |
|  | 0001100010000000 | Imminent peril call, queueing supported, multi-talker | IMMPERIL-CALL |

#### 5.5.6.12	Connect

Table 5.5.6.12-1: Connect

| Derivation Path: 24.380 [10], Table 8.3.4-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Connect with acknowledgment required |  |
|  | 10000 | Connect with acknowledgment required | ACK |
| SSRC | The SSRC of the SS | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPC |  |  |
| MCPTT Session Identity field |  |  |  |
| Session Type | “00000000” | No session type |  |
|  | “00000001” | private | PRIVATE-CALL |
|  | "00000011" | prearranged | GROUP-CALL |
|  | “00000100” | chat | CHAT-GROUP-CALL |
| MCPTT Session Identity | tsc_MCX_SessionID_B | SIP URI, which identifies the MCPTT session between the MCPTT client and the controlling MCPTT function |  |
| MCPTT Group Identity field | Not Present |  | PRIVATE-CALL |
| MCPTT Group Identity field |  |  | GROUP-CALL |
| MCPTT Group Identity | px_MCPTT_Group_A_ID | a URI, which identifies the MCPTT group |  |
| Media Streams |  |  |  |
| Media Stream field | "1" | 8 bit parameter giving the number of the" m=audio" m-line negotiated in the pre-established session |  |
| Control Channel | "2" | 8 bit parameter giving the number of the "m=application" m-line negotiated in the pre-established session |  |
|  | "0" | no floor control | WITHOUT_FLOORCONTROL |
| Warning Text field | Not Present |  |  |
| Answer State field |  |  |  |
| Answer State | "1" | confirmed |  |
| Inviting MCPTT User Identity field |  |  |  |
| Inviting MCPTT User Identity | px_MCPTT_ID_User_B | URI, which identifies the inviting MCPTT user |  |
| PCK I_MESSAGE field | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| WITHOUT_FLOORCONTROL | There shall be no floor control during the call (e.g. in case of private or first-to-answer call) |
| For further conditions see table 5.5.1-1 |  |

#### 5.5.6.13	Disconnect

Table 5.5.6.13-1: Disconnect

| Derivation Path: 24.380 [10], Table 8.3.5-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Disconnect with acknowledgment not required |  |
|  | 10001 | Disconnect with acknowledgment required | ACK |
| SSRC | The SSRC of the SS | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPC |  |  |
| MCPTT Session Identity field | Same MCPTT Session Identity as used in the connect message at call establishment | TS 24.380 [10] clause 9.3.2.4.5 |  |

#### 5.5.6.14	Acknowledge

Table 5.5.6.14-1: Acknowledge

| Derivation Path: 24.380 [10], Table 8.3.6-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Acknowledge |  |
| SSRC | The SSRC of the UE | The SSRC of the floor participant sending the message Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCPC |  |  |
| Reason Code |  |  |  |
| Reason Code | "0" | Accepted |  |

#### 5.5.6.15	Map Group To Bearer

Table 5.5.6.15-1: Map Group To Bearer

| Derivation Path: 24.380 [10], Table 8.4.4-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Map Group To Bearer |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCMC |  |  |
| MCPTT Group ID | px_MCPTT_Group_A_ID | The group ID of the call |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| MBMS Subchannel |  |  |  |
| Audio m-line Number | "1" | The number of the "m=audio" m-line in the SIP MESSAGE request announcing the MBMS bearer |  |
| Floor m-line Number | "2" | The number of the "m=application" m-line in the SIP MESSAGE request announcing the MBMS bearer. The <Floor m-line Number> value is set to "0" when the same subchannel is used for media and for floor control. |  |
| IP version | "0" | '0' = IP version 4 '1' = IP version 6 All other values are reserved for future use |  |
| Floor control Port Number | "9" | The port to be used if the<Floor m-line Number> value is greater than '0'. If the <Floor m-line Number> value is equal to '0', the <Floor control Port Number> value is not included in the MBMS Subchannel field |  |
| Media Port Number | "9" |  |  |
| IP Address | "0.0.0.0" |  |  |

#### 5.5.6.16	Unmap Group To Bearer

Table 5.5.6.16-1: Unmap Group To Bearer

| Derivation Path: 24.380 [10], Table 8.4.5-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Unmap Group To Bearer |  |
| SSRC | The SSRC of the message sender | The SSRC of the floor control server for on-network and floor arbitrator for off-network. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCMC |  |  |
| MCPTT Group ID | px_MCPTT_Group_A_ID | The group ID of the call |  |

#### 5.5.6.17	Application Paging

Table 5.5.6.17-1: Application Paging

| Derivation Path: 24.380 [10], Table 8.4.6-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Application Paging |  |
| SSRC | The SSRC of the message sender | The SSRC of the participating MCPTT function. Notation in accordance with clause 5.5.6.1. Coded as specified in IETF RFC 3550 [76]. |  |
| name | MCMC |  |  |
| MCPTT Group ID | px_MCPTT_Group_A_ID | The group ID of the call |  |

#### 5.5.6.18	Bearer Announcement

Table 5.5.6.18-1: Bearer Announcement

| Derivation Path: 24.380 [10], Table 8.4.7-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00011 | Bearer Announcement |  |
| name | MCMC |  |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| Alternative TMGI | Not present |  |  |
| Monitoring State | '1' | The <Monitoring State> value is a binary value where the following values are defined: '0' Monitoring is inactive '1' Monitoring is active |  |