3GPP TSG-RAN WG5 Meeting #94-e	R5-220477

# Electronic Meeting, , 21th February  – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0247 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.7 - Default MCX group management messages and other information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Group creation and temporary group creation documents for MCVideo and MCData missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Group creation and temporary group creation documents for MCVideo and MCData defined within new clause with respective conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid, inconsistent or incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.7	Default MCX group management messages and other information elements

#### 5.5.7.1	MCPTT Group Configuration

The structure of a group configuration document is specified in TS 24.481 [11] clause 7, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 6.3.

The structure of the configuration document is based on several XML schemas. To distinguish the schemas the prefixes of their corresponding name spaces are used in the 'Information Element' column as according to table 7.2.2-2 of TS 24.481 [11].

Table 5.5.7.1-1: MCPTT Group Configuration Defaults

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| list-service[1] |  |  |  | Group 1 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_Group_A_ID |  | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 |  | TS 24.483 [13] clause 6.2.7 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 |  | TS 24.483 [13] clause 6.2.8 |  |  |  |
| list |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  | group member 1 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_A |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "3" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCX_User_A_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| entry[2] |  |  |  | group member 2 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_B |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "2" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCX_User_B_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| entry[3] |  |  |  | group member 3 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_C |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "1" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCX_User_C_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| cp:ruleset |  |  |  |  |  |  |  |  |  |
| cp:rule |  |  |  |  |  |  |  |  |  |
| cp:id attribute |  | "rule1" |  |  |  |  |  |  |  |
| cp:actions |  |  |  |  |  |  |  |  |  |
| cp:on-network-allow-getting-member-list |  | "true" |  | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures |  |  |  |  |  |
| cp:allow-initiate-conference |  | "true" |  |  |  |  |  |  |  |
| cp:join-handling |  | "true" |  |  |  |  |  |  |  |
| cp:allow-MCPTT-emergency-call |  | "true" |  | Indicates whether an MCPTT emergency group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 6.2.19 |  |  |  |
| cp:allow-imminent-peril-call |  | "true" |  | Indicates whether an MCPTT imminent peril group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 6.2.20 |  |  |  |
| cp:allow-MCPTT-emergency-alert |  | "true" |  | Indicates whether an MCPTT emergency alert is possible on the MCPTT group |  | TS 24.483 [13] clause 6.2.21 |  |  |  |
| cp:on-network-allow-getting-affiliation-list |  | "true" |  | Indicates that the identity is allowed to get the list of MCPTT users affiliated to the MCPTT group in on-network MCPTT procedures |  |  |  |  |  |
| cp:on-network-allow-conference-state |  | "true" |  | indicates that the identity is allowed to subscribe to the conference event package of an MCPTT group session of the MCPTT group in on-network MCPTT procedures |  |  |  |  |  |
| oxe:supported-services |  |  |  |  |  |  |  |  |  |
| oxe:service |  |  |  |  |  | TS 24.481 [11] |  |  |  |
| oxe:enabler |  | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |  |  |  |  |
| oxe:group-media |  |  |  |  |  |  |  |  |  |
| mcpttgi:mcptt-speech |  | Present |  |  |  |  |  |  |  |
| mcpttgi:owner |  | px_MCX_Group_A_Owner_Organization |  | Group’s owner (Mission Critical Organisation). |  | TS 24.483 [13] clause 6.2.15 |  |  |  |
| mcpttgi:preferred-voice-encodings |  |  |  |  |  |  |  |  |  |
| mcpttgi:encoding- |  |  |  |  |  |  |  |  |  |
| mcpttgi:name[1] |  | px_MCPTT_Group_A_preferred_VCodec |  | Preferred voice codec is a RTP payload. MCPTT clients shall support the AMR-WB codec. |  | RFC 4566 [27] TS 26.171 [66] TS 24.483 [13] clause 6.2.16 |  |  |  |
| mcpttgi:level-within-group-hierarchy |  | "0" |  | Indicates the level within a group hierarchy (only applicable for group-broadcast group). |  | TS 24.483 [13] clause 6.2.17 |  |  |  |
| mcpttgi:level-within-user-hierarchy |  | "0" |  | Indicates the level within user hierarchy (only applicable for user-broadcast group). |  | TS 24.483 [13] clause 6.2.18 |  |  |  |
| mcpttgi:protect-media |  | "true" |  | Indicates whether confidentiality and integrity of media is required on the MCPTT group |  | TS 24.483 [13] clause 6.2.22 |  |  |  |
| mcpttgi:protect-floor-control-signalling |  | "true" |  | Indicates whether confidentiality and integrity of floor control signalling is required on the MCPTT group |  | TS 24.483 [13] clause 6.2.23 |  |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id |  | tsc_MCX_Group_A_ProSeLayer2GroupID |  | Indicates the Prose layer-2 group ID |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |  |  |
| mcpttgi:off-network-IP-multicast-address |  | "0.0.0.0" |  | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |  |  |
| mcpttgi:off-network-ProSe-relay-service-code |  | "123456" |  | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |  |  |
| mcpttgi:off-network-in-progress-emergency-state-cancellation-timeout |  | "PT18H12M15S" |  | Indicates the timeout value for the cancellation of an in progress emergency for an MCPTT group call. "PT18H12M15S" corresponds to 65535 seconds what is maximum allowed value according to TS 24.483 [13] |  | TS 24.483 [13] clause 6.2.31 |  |  |  |
| mcpttgi:off-network-in-progress-imminent-peril-state-cancellation-timeout |  | "PT18H12M15S" |  | Indicates the timeout value for the cancellation of an in progress imminent peril for an MCPTT group call. "PT18H12M15S" corresponds to 65535 seconds what is maximum allowed value according to TS 24.483 [13] |  | TS 24.483 [13] clause 6.2.32 |  |  |  |
| mcpttgi:off-network-hang-timer |  | "PT5S" |  | Indicates the group call hang timer. "PT5S" corresponds to 5 seconds |  | TS 24.483 [13] clause 6.2.33 |  |  |  |
| mcpttgi:off-network-maximum-duration |  | "PT1M" |  | Indicates the max duration of group calls. "PT1M" corresponds to 1 minute |  | TS 24.483 [13] clause 6.2.34 |  |  |  |
| mcpttgi:off-network-queue-usage |  | "true" |  | Indicates if queuing is enabled or not |  | TS 24.483 [13] clause 6.2.34A |  |  |  |
| mcpttgi:off-network-ProSe-signalling-PPPP |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.36 |  |  |  |
| mcpttgi:off-network-ProSe-media-PPPP |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.37 |  |  |  |
| mcpttgi:off-network-ProSe-emergency-call-signalling-PPPP |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.38 |  |  |  |
| mcpttgi:off-network-ProSe-emergency-call-media-PPPP |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.39 |  |  |  |
| mcpttgi:off-network-ProSe-imminent-peril-call-signalling-PPPP |  | "7" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.40 |  |  |  |
| mcpttgi:off-network-ProSe-imminent-peril-call-media-PPPP |  | "7" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.41 |  |  |  |
| mcpttgi:multi-talker-control |  | "false" |  | "true" indicates that multi-talker control is enabled for the group "false" indicates that multi-talker control is disabled for the group |  |  |  |  |  |
| mcpttgi:max-number-simultaneous-talkers |  | "1" |  | Indicates the maximum number of parallel talkers in a MCPTT group session in on-network MCPTT procedures |  |  |  |  |  |
| mcpttgi:audio-mixing-entity |  | Not present |  | Absence of the <audio-mixing-entity> element indicates that audio mixing is performed in the network |  |  |  |  |  |

#### 5.5.7.2	MCVideo Group Configuration

The structure of a group configuration document is specified in TS 24.481 [11] clause 7, single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 6.

Table 5.5.7.2-1: MCVideo Group Configuration Defaults

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  | Group 1 |  |  |
| uri attribute | px_MCVideo_Group_A_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.7 |  |
| display-name | px_MCVideo_Group_A_Name | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.8 |  |
| list |  |  |  |  |
| entry[1] |  | group member 1 |  |  |
| uri attribute | px_MCVideo_ID_User_A | Indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "3" | Indicates the user priority of the MCVideo group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCX_User_A_ParticipantType | Participant type of the MCVideo group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcvideo-mcvideo-id |  |  |  |  |
| uri attribute | px_MCVideo_ID_User_A |  |  |  |
| entry[2] |  | Group member 2 |  |  |
| uri attribute | px_MCVideo_ID_User_B | Indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "2" | Indicates the user priority of the MCVideo group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCX_User_B_ParticipantType | Participant type of the MCVideo group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcvideo-mcvideo-id |  |  |  |  |
| uri attribute | px_MCVideo_ID_User_B |  |  |  |
| entry[3] |  | Group member 3 |  |  |
| uri attribute | px_MCVideo_ID_User_C | Indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "1" | Indicates the user priority of the MCVideo group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCX_User_C_ParticipantType | Participant type of the MCVideo group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcvideo-mcvideo-id |  |  |  |  |
| uri attribute | px_MCVideo_ID_User_C |  |  |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule1" |  |  |  |
| cp:actions |  |  |  |  |
| mcpttgi:on-network-allow-getting-member-list | "true" | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures. |  |  |
| mcpttgi:mcvideo-allow-emergency-call | "true" | Indicates that the identity is allowed to request an MCVideo-emergency call on the MCVideo group. |  |  |
| mcpttgi:mcvideo-allow-emergency-alert | "true" | Indicates that the identity is allowed to request an MCVideo-emergency alert on the MCVideo group. |  |  |
| mcpttgi:mcvideo-allow-imminent-peril-call | "true" | Indicates that the identity is allowed to request an MCVideo imminent peril call on the MCVideo group. |  |  |
| mcpttgi:mcvideo-on-network-allow-conference-state | "true" | Indicates that the identity is allowed to subscribe to the conference event package of an MCVideo group session of the MCVideo group in on-network MCVideo procedures. |  |  |
| mcpttgi:mcvideo-on-network-allow-getting-affiliation-list | "true" | Indicates that the identity is allowed to get the list of MCVideo users affiliated to the MCVideo group in on-network MCVideo procedures. |  |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  |  |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" | String defining an enabler |  |  |
| oxe:group-media |  |  |  |  |
| oxe:mcvideo-video-media |  |  |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id | tsc_MCX_Group_A_ProSeLayer2GroupID | Indicates the Prose layer-2 group ID | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |
| mcpttgi:off-network-IP-multicast-address | "0.0.0.0" | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |
| mcpttgi:off-network-ProSe-relay-service-code | "123456" | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |
| mcpttgi:owner | px_MCX_Group_A_Owner_Organization | Group’s owner (Mission Critical Organisation). | TS 24.483 [13] clause 6.2.15 |  |
| mcpttgi:level-within-group-hierarchy | "0" | Indicates the level within a group hierarchy (only applicable for group-broadcast group). | TS 24.483 [13] clause 6.2.17 |  |
| mcpttgi:level-within-user-hierarchy | "0" | Indicates the level within user hierarchy (only applicable for user-broadcast group). | TS 24.483 [13] clause 6.2.18 |  |
| mcpttgi:mcvideo-on-network-invite-members | "true" |  |  |  |
| mcpttgi:mcvideo-on-network-maximum-duration | "1800" | Indicates the max duration of MCVideo group calls. | TS 24.483 [13] clause 6.2.56 |  |
| mcpttgi:mcvideo-urgent-real-time-video-mode | "true" | Indicates that urgent real-time video mode is allowed for the MCVideo group. |  |  |
| mcpttgi:mcvideo-non-urgent-real-time-video-mode | "true" | indicates that non urgent real-time video mode is allowed for the MCVideo group. |  |  |
| mcpttgi:mcvideo-non-real-time-video-mode | "true" | indicates that non real-time video mode is allowed for the MCVideo group. |  |  |
| mcpttgi:mcvideo-active-real-time-video-mode | "non-urgent-real-time" | Indicates the the active real time video mode of the current group session |  |  |
| mcpttgi:mcvideo-maximum-simultaneous-mcvideo-transmitting-group-members | "1" | Indicates the allowed maximum number of simultaneous transmitting MCVideo Group Members. |  |  |
| mcpttgi:mcvideo-on-network-minimum-number-to-start | "1" | Indicates the minimum number of affiliated group members acknowledging before start of video transmission specified in 3GPP TS 23.281 [24] in on-network MCVideo procedures. |  |  |
| mcpttgi: mcvideo-on-network-group-priority | "1" | Indicates the priority level of the group in on-network MCVideo procedures. Higher value indicates higher priority. Absence of the <mcvideo-on-network-group-priority> element of the <list-service> element of the MCVideo group document indicates the lowest possible priority. |  |  |
| mcpttgi:mcvideo-off-network-arbitration-approach | "self" | This leaf node indicates the arbitration approach used for off-network video tranmissions on the group. | TS 24.483 [13] clause 6.2.47 |  |
| mcpttgi:mcvideo-off-network-maximum-simultaneous-transmissions | "1" | indicates maximum number of simultaneous transmissions for off-network MCVideo procedures. | TS 24.483 [13] clause 6.2.48 |  |
| mcpttgi:mcvideo-off-network-ProSe-signalling-PPPP | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 24.483 [13] clause 6.2.50 |  |
| mcpttgi:mcvideo-off-network-ProSe-emergency-call-signalling-PPPP | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo emerency group call signalling. | TS 24.483 [13] clause 6.2.52 |  |
| mcpttgi:mcvideo-off-network-ProSe-imminent-peril-call-signalling-PPPP | "7" | Indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo imminent peril group call signalling. | TS 24.483 [13] clause 6.2.54 |  |
| mcpttgi:mcvideo-off-network-ProSe-media-PPPP | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 24.483 [13] clause 6.2.51 |  |
| mcpttgi:mcvideo-off-network-ProSe-emergency-call-media-PPPP | "8" |  | TS 24.483 [13] clause 6.2.53 |  |
| mcpttgi:mcvideo-off-network-ProSe-imminent-peril-call-media-PPPP | "7" | Indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo imminent peril group call media. | TS 24.483 [13] clause 6.2.55 |  |
| mcpttgi:mcvideo-off-network-maximum-duration | "60 | Indicates the maximum duration of group calls |  |  |
| mcpttgi:mcvideo-off-network-in-progress-emergency-state-cancellation-timeout | "65535" | Indicates the timeout value for the cancellation of an in progress emergency in off-network MCVideo procedures |  |  |
| mcpttgi:mcvideo-off-network-in-progress-imminent-peril-state-cancellation-timeout | "65535" | Indicates the timeout value for the cancellation of an in progress imminent-peril group call in off-network MCVideo procedures |  |  |

#### 5.5.7.3	MCDATA Group Configuration

The structure of a group configuration document is specified in TS 24.481 [11] clause 7.

Single MCDATA group configuration parameters are defined in TS 24.483 [13] clause 6.3.

Table 5.5.7.3-1: MCDATA Group Configuration Defaults

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  | Group 1 |  |  |
| uri attribute | px_MCDATA_Group_A_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.7 |  |
| display-name | px_MCData _Group_A_Name | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 | TS 24.483 [13] clause 6.2.8 |  |
| list |  |  |  |  |
| entry[1] |  | group member 1 |  |  |
| uri attribute | px_MCData_ID_User_A | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "3" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCX _User_A_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_A |  |  |  |
| entry[2] |  | Group member 2 |  |  |
| uri attribute | px_MCData_ID_User_B | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "2" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCX _User_B_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_B |  | TS 24.483 [13] clause 6.2.11 |  |
| entry[3] |  | Group member 3 |  |  |
| uri attribute | px_MCData_ID_User_C | Indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user | TS 24.483 [13] clause 6.2.11 |  |
| display-name | Not present |  |  |  |
| mcpttgi:user-priority | "1" | Indicates the user priority of the MCData group member | TS 24.483 [13] clause 6.2.12 |  |
| mcpttgi:participant-type | px_MCX _User_C_ParticipantType | Participant type of the MCData group | TS 24.483 [13] clause 6.2.13 |  |
| rl:mcdata-mcdata-id |  |  |  |  |
| uri attribute | px_MCData_ID_User_C |  | TS 24.483 [13] clause 6.2.11 |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule1" |  |  |  |
| cp:actions |  |  |  |  |
| mcpttgi:on-network-allow-getting-member-list | "true" | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures. |  |  |
| mcpttgi:mcdata-on-network-allow-getting-affiliation-list | "true" | Indicates that the identity is allowed to get the list of MCData users affiliated to the MCData group in on-network MCData procedures |  |  |
| mcpttgi:mcdata-allow-transmit-data-in-this-group | "true" | Indicates that the identity is allowed to transmit data in this group |  |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  |  |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | String defining an enabler |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id | tsc_MCX_Group_A_ProSeLayer2GroupID | Indicates the Prose layer-2 group ID | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |
| mcpttgi:off-network-IP-multicast-address | "0.0.0.0" | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |
| mcpttgi:off-network-ProSe-relay-service-code | "123456" | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |
| mcpttgi:owner | px_MCX_Group_A_Owner_Organization | Group’s owner (Mission Critical Organisation). | TS 24.483 [13] clause 6.2.15 |  |
| mcpttgi:level-within-group-hierarchy | "0" | Indicates the level within a group hierarchy (only applicable for group-broadcast group). | TS 24.483 [13] clause 6.2.17 |  |
| mcpttgi:mcdata-enhanced-status-operational-values |  | A list of operational values used for the enhanced status service and two text strings used to display a meaningful message to the user. |  |  |
| mcpttgi:status |  |  |  |  |
| id | "0" |  |  |  |
| mcpttgi:shortText |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "going" |  |  |  |
| mcpttgi:description |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "going to the operation site" |  |  |  |
| mcpttgi:status |  |  |  |  |
| id | "1" |  |  |  |
| mcpttgi:shortText |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "arrived" |  |  |  |
| mcpttgi:description |  |  |  |  |
| langType | "English" |  |  |  |
| langText | "just arrived at the operation site" |  |  |  |
| mcpttgi:level-within-user-hierarchy | "0" | Indicates the level within user hierarchy (only applicable for user-broadcast group). | TS 24.483 [13] clause 6.2.18 |  |
| mcpttgi:mcdata-on-network-group-priority | "1" | Indicates the priority level of the group in on-network MCData procedures. Higher value indicates higher priority |  |  |
| mcpttgi:mcdata-on-network-max-data-size-for-SDS | "10000" | Indicates the maximum size of data (in bytes) that the originating MCData client is allowed to send to the MCData server for on-network SDS communications |  |  |
| mcpttgi:mcdata-on-network-max-data-size-for-FD | "10000" | Indicates the maximum size of data (in bytes) that the originating MCData client is allowed to send to the MCData server for on-network FD communications |  |  |
| mcpttgi:mcdata-on-network-max-data-size-auto-recv | "2000" | Indicates the maximum size of data (in bytes) which the MCData server always requests the terminating MCData client to automatically download for on-network FD communications using HTTP |  |  |
| mcpttgi:mcdata-off-network-ProSe-signalling-PPPP | "1" | Indicates the ProSe Per-Packet Priority value to be used when transmitting IP packets carrying signalling for a call on the MCData group in off-network MCData procedures |  |  |
| mcpttgi:mcdata-off-network-ProSe-media-PPPP | "1" | Indicates the ProSe Per-Packet Priority value to be used when transmitting IP packets carrying media for a call on the MCData group in off-network MCData procedures |  |  |

#### 5.5.7.4	MCX Group Creation Documents

Table 5.5.7.4-1: MCX Group Creation Document

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service [1] |  |  |  |  |
| uri-attribute | px_MCPTT_Group_B_ID | uri of the MCPTT group | TS 24.481 [11] | MCPTT |
|  | px_MCVideo_Group_B_ID |  |  | MCVIDEO |
|  | px_MCData_Group_B_ID |  |  | MCDATA |
| display-name | any value | group display name | TS 24.481 [11] |  |
| list |  |  |  |  |
| entry[1] |  | User-C |  |  |
| uri-attribute | px_MCPTT_ID_User_C | User ID allowed to participate in this group | TS 24.481 [11] | MCPTT |
|  | px_MCVideo_ID_User_C |  |  | MCVIDEO |
|  | px_MCData_ID_User_C |  |  | MCDATA |
| display-name | Not present | User display name | TS 24.481 [11] |  |
| entry[2] |  | User-D |  |  |
| uri-attribute | px_MCPTT_ID_User_D | User ID allowed to participate in this group | TS 24.481 [11] | MCPTT |
|  | px_MCVideo_ID_User_D |  |  | MCVIDEO |
|  | px_MCData_ID_User_D |  |  | MCDATA |
| display-name | Not present | User display name | TS 24.481 [11] |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  | TS 24.481 [11] |  |
| oxe:enabler | “urn:urn-7:3gpp-service.ims.icsi.mcptt” |  |  | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA |
| oxe:group-media |  |  |  |  |
| mcpttgi:mcptt-speech | Present |  |  | MCPTT |
| mcpttgi:mcvideo-video-media | Present |  |  | MCVIDEO |

Table 5.5.7.4-2: MCX Temporary Group Creation Document

| Derivation Path: TS 24.481 [11] clause 7.3.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| gmop:document |  |  |  |  |
| gmop:request |  |  |  |  |
| gmop:group-regroup-creation |  |  |  |  |
| group |  |  |  |  |
| list-service[1] |  |  |  |  |
| uri attribute | px_MCPTT_Group_T_ID | MCS temporary group identity |  | MCPTT |
|  | px_MCVideo_Group_T_ID |  |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  |  | MCDATA |
| display-name | Not present |  |  |  |
| list | Not present | Temporary group contains constituent groups but no group members |  |  |
| mcpttgi:on-network-temporary |  |  | TS 24.481 [11] |  |
| constituent-MCPTT-group-IDs |  |  |  |  |
| constituent-MCPTT-group-ID[1] | px_MCPTT_Group_A_ID | MCS group ID of a constituent MCS group of the temporary MCS group |  | MCPTT |
|  | px_MCVideo_Group_A_ID |  |  | MCVIDEO |
|  | px_MCData_Group_A_ID |  |  | MCDATA |
| constituent-MCPTT-group-ID[2] | px_MCPTT_Group_B_ID | MCS group ID of a constituent MCS group of the temporary MCS group |  | MCPTT |
|  | px_MCVideo_Group_B_ID |  |  | MCVIDEO |
|  | px_MCData_Group_B_ID |  |  | MCDATA |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  | TS 24.481 [11] |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA |
| oxe:group-media |  |  |  |  |
| mcpttgi:mcptt-speech | Present |  |  | MCPTT |
| mcpttgi:mcvideo-video-media | Present |  |  | MCVIDEO |