3GPP TSG-RAN WG5 Meeting #94-e	R5-220496

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0015 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 6.1 - Short Data Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test cases for MCData file distribution need to be cleaned up to use generic procedures and default message contents with the latest conditions. In addition there are the following technical issues: - Similar to MCPTT the SIP 200 OK sent to the UE does not contain any mcdata-info as neither a terminating client nor any server generates it - Handling of the display of the SDS message send to the UE in test cases 6.1.2, 6.1.9, 6.1.11 is not consistent in case of the disposition request for "READ" or "DELIVERY AND READ" - It depends on the parameter <max-payload-size-sds-cplane-bytes> in the Service Configuration document whether a client shall use the of control plane or media plane ( R5w220011 clause 2.6.2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of the test case sequences and the specific message contents to use generic procedures and default message contents according to the latest 36.579-1 specifications. In addition there are the following following changes: - mcdata-info removed from SIP 200 OK messages sent by the SS - Parallel behaviour introduced for test cases 6.1.9, 6.1.11 in the same way as for test case 6.1.2. - <max-payload-size-sds-cplane-bytes> set to not present (unlimited use of control plane) in the preamble of test cases using the control plane and set to 0 in test cases for standalone SDS using media plane. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.1	Short Data Service

### 6.1.1	On-network / Short Data Service (SDS) / Standalone SDS Using Signalling Control Plane / One-to-one Standalone SDS / Client Originated (CO)

6.1.1.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a standalone one-to-one SDS message with a disposition of only Delivery }

then { UE (MCDATA Client) sends a standalone one-to-one SDS message with a disposition of only Delivery via a SIP MESSAGE message }

}

(2)

with { UE (MCDATA Client) having sent a standalone one-to-one SDS message }

ensure that {

when { UE (MCDATA Client receives a disposition response via a SIP MESSAGE message from the SS (MCDATA Server) }

then { UE (MCDATA Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

(3)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a standalone one-to-one SDS message with a disposition of only Read }

then { UE (MCDATA Client) sends a standalone one-to-one SDS message with a disposition of only Read via a SIP MESSAGE message }

}

(4)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a standalone one-to-one SDS message with a disposition of both Read and Delivery }

then { UE (MCDATA Client) sends a standalone one-to-one SDS message with a disposition of both Read and Delivery via a SIP MESSAGE message }

}

6.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.2.2.1, 6.2.2.1, 6.2.4.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.2.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone SDS message is to be sent, shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9];

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-sds"; and

c)	if end-to-end security is required and the security context does not exist or if the existing security context has expired, an application/mikey MIME body with the MIKEY-SAKKE I_MESSAGE as specified in 3GPP TS 33.180 [26]. The MCData client:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26]; and

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter; and

viii)	shall include the MIKEY-SAKKE I_MESSAGE in an application/mikey MIME body as specified in 3GPP TS 33.180 [26];

…

4)	shall generate a standalone SDS message as specified in subclause 6.2.2.1; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.2.1]

In order to generate an SDS message, the MCData client:

1)	shall generate an SDS SIGNALLING PAYLOAD message as specified in subclause 15.1.2;

2)	shall generate a DATA PAYLOAD message as specified in subclause 15.1.4;

3)	shall include in the SIP request, the SDS SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

4)	shall include in the SIP request, the DATA PAYLOAD message in an application/vnd.3gpp.mcdata-payload MIME body as specified in subclause E.2.

When generating an SDS SIGNALLING PAYLOAD message as specified in subclause 15.1.2, the MCData client:

1)	shall set the Date and time IE to the current time as specified in subclause 15.2.8;

2)	if the SDS message starts a new conversation, shall set the Conversation ID IE to a newly generated Conversation ID value as specified in subclause 15.2.9;

3)	if the SDS message continues an existing unfinished conversation, shall set the Conversation ID IE to the Conversation ID value of the existing conversation as specified in subclause 15.2.9;

4)	shall set the Message ID IE to a newly generated Message ID value as specified in subclause 15.2.10;

5)	if the SDS message is in reply to a previously received SDS message, shall include the InReplyTo message ID IE with the Message ID value in the previously received SDS message;

6)	if the SDS message is for user consumption, shall not include an Application ID IE as specified in subclause 15.2.7;

7)	if the SDS message is intended for an application on the terminating MCData client, shall include an Application ID IE with an Application ID value representing the intended application as specified in subclause 15.2.7;

NOTE:	The value chosen for the Application ID value is decided by the mission critical organisation.

8)	if only a delivery disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY" as specified in subclause 15.2.3;

9)	if only a read disposition notification is required shall include a SDS disposition request type IE set to "READ" as specified in subclause 15.2.3; and

10)	if both a delivery and read disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY AND READ" as specified in subclause 15.2.3.

When generating an DATA PAYLOAD message for SDS as specified in subclause 15.1.4, the MCData client:

1)	shall set the Number of payloads IE to the number of Payload IEs that needs to be encoded, as specified in subclause 15.2.12;

2)	if end-to-end security is required for a one-to-one communication, shall include the Security parameters and Payload IE with security parameters as described in 3GPP TS 33.180 [26]. Otherwise, if end-to-end security is not required for a one-to-one communication, shall include the Payload IE as specified in subclause 15.1.4; and

3)	for each Payload IE included:

a)	if the payload is text, shall set the Payload content type as "TEXT" as specified in subclause 15.2.13;

b)	if the payload is binary data, shall set the Payload content type as "BINARY" as specified in subclause 15.2.13;

c)	if the payload is hyperlinks, shall set the Payload content type as "HYPERLINKS" as specified in subclause 15.2.13;

d)	if the payload is location, shall set the Payload content type as "LOCATION" as specified in subclause 15.2.13; and

e)	shall include the data to be sent in the Payload data.

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

…

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.1.1.3	Test description

6.1.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element shall not be present in the MCData Service Configuration document so that according to TS 24.484 [24] there is no size limit imposed for the use of C-plane procedures for the SDS message.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.1.3.2	Test procedure sequence

Table 6.1.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCDATA User request to send a standalone one-to-one SDS message with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2-2B | Check: Does the UE (MCData Client) perform steps 1a1-3 of the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a standalone one-to-one SDS message with disposition request "DELIVERY"? (NOTE 2) | - | - | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | Check: Does the UE (MCData Client) perform the generic procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the SDS message sent at step 2A? | - | - | 2 | P |
| 5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCDATA client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 2 | P |
| 7 | Make the MCDATA User request to send a standalone one-to-one SDS message with disposition request "READ". (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a standalone one-to-one SDS message with disposition request "READ"? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCData Client) perform the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the SDS message sent at step 8? | - | - | 2 | P |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCDATA client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 2 | P |
| 13 | Make the MCDATA User request to send a standalone one-to-one SDS message with disposition request "DELIVERY AND READ". (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a standalone one-to-one SDS message with disposition request "DELIVERY AND READ"? | - | - | 4 | P |
| 15 | Void | - | - | - | - |
| 16 | Check: Does the UE (MCData Client) perform the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the SDS message sent at step 8? | - | - | 2 | P |
| 17 | Void | - | - | - | - |
| 18 | Check: Does the UE (MCDATA client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The RRC connection is not released at the end of the procedure. |  |  |  |  |  |

6.1.1.3.3	Specific message contents

Table 6.1.1.3.3-1: SIP MESSAGE (step 2A, Table 6.1.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MIKEY, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.1.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.1.3.3-3 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.1.3.3-4 |  |  |  |

Table 6.1.1.3.3-2: MCDATA-Info (Table 6.1.1.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_1to1

Table 6.1.1.3.3-3: SDS SIGNALLING PAYLOAD (Table 6.1.1.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.1.3.3-4: DATA PAYLOAD (Table 6.1.1.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-1

Table 6.1.1.3.3-5: SIP MESSAGE (step 4, Table 6.1.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.1.3.3-7 |  |  |  |

Table 6.1.1.3.3-6: Void

Table 6.1.1.3.3-7: SDS NOTIFICATION (Table 6.1.1.3.3-5)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED

Table 6.1.1.3.3-8: Void

Table 6.1.1.3.3-9: SIP MESSAGE (step 8, Table 6.1.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MIKEY, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | As described in Table 6.1.1.3.3-3 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.1.3.3-10 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.1.3.3-4 |  |  |  |

Table 6.1.1.3.3-10: SDS SIGNALLING PAYLOAD (Table 6.1.1.3.3-9)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1 condition READ

Table 6.1.1.3.3-11: SIP MESSAGE (step 10, Table 6.1.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.1.3.3-12 |  |  |  |

Table 6.1.1.3.3-12: SDS NOTIFICATION (Table 6.1.1.3.3-11)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition READ

Table 6.1.1.3.3-13: SIP MESSAGE (step 14, Table 6.1.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MIKEY, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.1.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.1.3.3-14 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.1.3.3-4 |  |  |  |

Table 6.1.1.3.3-14: SDS SIGNALLING PAYLOAD (Table 6.1.1.3.3-13)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1 condition DELIVERED_READ

Table 6.1.1.3.3-15: SIP MESSAGE (step 16, Table 6.1.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION described in Table 6.1.1.3.3-16 |  |  |  |

Table 6.1.1.3.3-16: SDS NOTIFICATION (Table 6.1.1.3.3-15)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED_READ

### 6.1.2	On-network / Short Data Service (SDS) / Standalone SDS Using Signalling Control Plane / One-to-one Standalone SDS / Client Terminated (CT)

6.1.2.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a standalone one-to-one SDS message with a disposition request of "DELIVERY" }

then { UE (MCDATA Client) confirms the reception of the SDS message by sending a SIP 200 (OK) message, followed by a SIP MESSAGE with a disposition notification of "DELIVERED" and renders the contents of the Payload IE to the MCDATA User }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a standalone one-to-one SDS message with a disposition request of "READ" }

then { UE (MCDATA Client) confirms the reception of the SDS message by sending a SIP 200 (OK) message, and, renders the contents of the Payload IE to the MCDATA User, and, then sends a SIP MESSAGE message with a disposition notification of "READ" }

}

(3)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a standalone one-to-one SDS message with a disposition request of "DELIVERY AND READ" with pre-set timer TDU1 (delivery and read) }

then { UE (MCDATA Client) sends a delivered and read notification after the message is rendered to the user if the timer TDU1 (delivery and read) has not expired at this time, or, if the timer TDU1 (delivery and read) expires before the message is rendered to the user sends first a delivered notification and after the message is rendered to the user a read notification }

}

6.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.2.2.2, 9.2.1.2, 9.2.1.3, 12.2.1.1, 6.2.3.1, 6.2.4.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.2.2.2]

Upon receipt of a "SIP MESSAGE request for standalone SDS for terminating MCData client", the MCData client:

1)	may reject the SIP MESSAGE request if there are not enough resources to handle the SIP MESSAGE request;

2)	if the SIP MESSAGE request is rejected in step 1), shall respond toward participating MCData function with a SIP 480 (Temporarily unavailable) response and skip the rest of the steps of this subclause;

3)	if the SIP MESSAGE request contains an application/mikey MIME body containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP MESSAGE request with a SIP 606 (Not Acceptable) response, and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to exchange end-to-end secure message.

4)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

5)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5]; and

6)	shall handle the received message as specified in subclause 9.2.1.2.

[TS 24.282, clause 9.2.1.2]

When a MCData client has received a SIP request containing:

-	an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

-	an application/vnd.3gpp.mcdata-payload MIME body as specified in subclause E.2;

the MCData Client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body;

2)	shall decode the contents of the application/vnd.3gpp.mcdata-payload MIME body;

3)	if the SDS SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the SDS SIGNALLING PAYLOAD identifying the first message in the conversation thread;

4)	if the SDS SIGNALLING PAYLOAD message contains an existing Conversation ID and:

a)	if the SDS SIGNALLING PAYLOAD message does not contain an InReplyTo message ID, shall use the Message ID in the SDS SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

b)	if the SDS SIGNALLING PAYLOAD message contains an InReplyTo message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo message ID in the SDS SIGNALLING PAYLOAD, and use the Message ID in the SDS SIGNALLING PAYLOAD to identify the new message;

5)	shall identify the number of Payload IEs in the DATA PAYLOAD message from the Number of payloads IE in the DATA PAYLOAD message;

6)	if the SDS SIGNALLING PAYLOAD message does not contain an Application ID IE:

a)	shall determine that the payload contained in the DATA PAYLOAD message is for user consumption

b)	may notify the MCData user; and

c)	shall render the contents of the Payload IE(s) to the MCData user;

7)	if the SDS SIGNALLING PAYLOAD message contains an Application ID IE:

a)	shall determine that the payload contained in the DATA PAYLOAD message is not for user consumption,

b)	shall not notify the MCData user;

c)	if the Application ID value is unknown, shall discard the SDS message; and

d)	if the Application ID value is known, shall deliver the contents of the Payload IE(s) to the identified application;

NOTE 1:	If required, the MCData client decrypts the Payload IEs before rendering the SDS message to the user or delivering the SDS message to the application.

NOTE 2:	The actions taken when the payload contains application data not meant for user consumption or command instructions are based upon the contents of the payload. If the payload content is addressed to a non-MCData application that is not running, the MCData client starts the local non-MCData application and delivers the payload to that application.

NOTE 3:	User consent is not required before accepting the data.

8)	may store the message payload in local storage along with the Conversation ID, Message ID, InReplyTo message ID and Date and time; and

9)	if the received SDS SIGNALLING PAYLOAD message contains an SDS disposition request type IE shall follow the procedures in subclause 9.2.1.3.

[TS 24.282, clause 9.2.1.3]

To handle the disposition requests, the MCData client:

1)	If the SDS disposition request type IE is set to:

a)	"DELIVERY" then, shall send a delivered notification as described in subclause 12.2.1.1;

b)	"READ", shall send a read notification as described in subclause 12.2.1.1, when a display indication is received; or

c)	"DELIVERY AND READ" then, shall start timer TDU1 (delivery and read).

Upon receiving a display indication before timer TDU1 (delivery and read) expires, the MCData client:

1)	shall stop timer TDU1 (delivery and read); and

2)	shall send a delivered and read notification as described in subclause 12.2.1.1.

Upon expiry of timer TDU1 (delivery and read), the MCData client:

1)	shall send a delivered notification as described in subclause 12.2.1.1; and

2)	upon receiving a display indication, send a read notification as described in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the controlling MCData function that sent the SDS or FD message request. The MCData client determines the controlling MCData function from the contents of the <mcdata-controller-psi> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request;

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with an <mcdata-controller-psi> element containing the PSI of the controlling MCData function;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.3.1]

In order to generate an SDS notification, the MCData client:

1)	shall generate an SDS NOTIFICATION message as specified in subclause 15.1.5; and

2)	shall include in the SIP request, the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an SDS NOTIFICATION message as specified in subclause 15.1.5, the MCData client:

1)	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED" as specified in subclause 15.2.5;

2)	if sending a read notification, shall set the SDS disposition notification type IE as "READ" as specified in subclause 15.2.5;

3)	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ" as specified in subclause 15.2.5;

4)	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED" as specified in subclause 15.2.5;

5)	shall set the Date and time IE to the current time to as specified in subclause 15.2.8;

6)	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message as specified in subclause 15.2.9;

7)	shall set the Message ID to the value of the Message ID that was received in the SDS message as specified in subclause 15.2.10;

8)	if the SDS message was destined for the user, shall not include an Application ID IE as specified in subclause 15.2.7; and

9)	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message as specified in subclause 15.2.3.

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

…

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

6.1.2.3	Test description

6.1.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element shall not be present in the MCData Service Configuration document so that according to TS 24.484 [24] there is no size limit imposed for the use of C-plane procedures for the SDS message.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.2.3.2	Test procedure sequence

Table 6.1.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-1B | Check: Does the UE (MCData Client) perform steps 1a1-3 of the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive a standalone one-to-one SDS message with disposition request "DELIVERY"? (NOTE 3) | - | - | 1 | P |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "DELIVERED"? | - | - | 1 | P |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCDATA User? (NOTE 2) The exact expected content is specified in TS 36.579-1 [2], Table 5.5.3.10-2 | - | - | 1 | P |
| 6 | Make the MCDATA User close the message application to avoid unexpected behaviour when a subsequent message is sent in step 7. (NOTE 2) | - | - | - | - |
| 7-7B | Check: Does the UE (MCData Client) perform steps 1a1-3 of the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive a standalone one-to-one SDS message with disposition request "READ"? (NOTE 3) | - | - | 2 | P |
| 8 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 9 the events described in Table 6.1.2.3.2-2 take place. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCDATA User? (NOTE 2) The exact expected content is specified in TS 36.579-1 [2], Table 5.5.3.10-2. | - | - | 2 | P |
| 10 | Make the MCDATA User close the message application to avoid unexpected behaviour when a subsequent message is sent in step 11. (NOTE 2) | - | - | - | - |
| 11-11B | Check: Does the UE (MCData Client) perform steps 1a1-3 of the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive a standalone one-to-one SDS message with disposition request "DELIVERY AND READ"? (NOTE 3, 4) | - | - | 2 | P |
| 12 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 13 the events described in Table 6.1.2.3.2-3 take place. (NOTE 1) | - | - | - | - |
| 13 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCData User before the expiry of timer TDU1 (delivery and read)? (NOTE 2) The exact expected content is specified in TS 36.579-1 [2], Table 5.5.3.10-2. | - | - | 4 | P |
| 14 | Make the MCDATA User close the message application to avoid unexpected behaviour in case a subsequent message is sent (e.g. in a follow up test case). (NOTE 2) | - | - | - | - |
| NOTE 1: The behaviour is handled through parallel actions to allow for implementations which first indicate to the user that there is a message available, but render the message to the user only after the user takes an action to open the message. NOTE 2: This is expected to be done via a suitable implementation dependent MMI. NOTE 3: The RRC connection is not released at the end of the procedure. NOTE 4: Timer TDU1 (delivery and read) is started upon receipt of the SIP MESSAGE message that contains a "DELIVERY AND READ" disposition request. Timer TDU1 (delivery and read)=120ms according to the default value defined in TS 24.282 [31]. |  |  |  |  |  |

Table 6.1.2.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "READ"? | - | - | 2 | P |

Table 6.1.2.3.2-3: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1b1 describe behaviour that depends on the UE implementation in regard to how quick the UE (MCDATA Client) will render the contents of the Payload IE to the MCData User and the value of Timer TDU1. | - | - | - | - |
| 1a1-1a2 | Check: Does the UE (MCData Client) perform steps 2-3 of the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "DELIVERED"? | - | - | 3 | P |
| 1a3 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "READ"? | - | - | 3 | P |
| 1b1 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "DELIVERED AND READ"? | - | - | 3 | P |

6.1.2.3.3	Specific message contents

Table 6.1.2.3.3-1: SIP MESSAGE (step 1A, Table 6.1.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MIKEY, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.2.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.2.3.3-3 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.2.3.3-4 |  |  |  |

Table 6.1.2.3.3-2: MCDATA-Info (Table 6.1.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1

Table 6.1.2.3.3-3: SDS SIGNALLING PAYLOAD (Table 6.1.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.2-1, condition DELIVERED

Table 6.1.2.3.3-4: DATA PAYLOAD (Table 6.1.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-2

Table 6.1.2.3.3-5: Void

Table 6.1.2.3.3-6: SIP MESSAGE (step 3, Table 6.1.2.3.2-1 and step 1a1 , Table 6.1.2.3.2-3;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.2.3.3-7 |  |  |  |

Table 6.1.2.3.3-7: SDS NOTIFICATION (Table 6.1.2.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED

Table 6.1.2.3.3-8: SIP MESSAGE (step 7A, Table 6.1.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MIKEY, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.2.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.2.3.3-9 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.2.3.3-4 |  |  |  |

Table 6.1.2.3.3-9: SDS SIGNALLING PAYLOAD (Table 6.1.2.3.3-8)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.2-1 condition READ

Table 6.1.2.3.3-10: SIP MESSAGE (step 1, Table 6.1.2.3.2-2, step 1a3, Table 6.1.2.3.2-3;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.2.3.3-11 |  |  |  |

Table 6.1.2.3.3-11: SDS NOTIFICATION (Table 6.1.2.3.3-10)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition READ

Table 6.1.2.3.3-12: SIP MESSAGE (step 11A, Table 6.1.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MIKEY, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.2.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling messag |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.2.3.3-13 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.2.3.3-4 |  |  |  |

Table 6.1.2.3.3-13: SDS SIGNALLING PAYLOAD (Table 6.1.2.3.3-12)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.2-1 condition DELIVERED_READ

Table 6.1.2.3.3-14: SIP MESSAGE (step 1b1, Table 6.1.2.3.2-2;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.2.3.3-15 |  |  |  |

Table 6.1.2.3.3-15: SDS NOTIFICATION (Table 6.1.2.3.3-14)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED_READ

### 6.1.3	On-network / Short Data Service (SDS) / Standalone SDS Using Signalling Control Plane / Group Standalone SDS / Client Originated (CO)

6.1.3.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a group standalone SDS message with a disposition of only Delivery }

then { UE (MCDATA Client) sends a group standalone SDS message with a disposition of only Delivery via s SIP MESSAGE message }

}

(2)

with { UE (MCDATA Client) having sent a group standalone SDS message }

ensure that {

when { the UE (MCDATA Client) receives a disposition response via a SIP MESSAGE message from the SS (MCDATA Server) }

then { UE (MCData Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.1.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.2.2.1, 6.2.2.1, 6.2.4.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.2.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

...

3)	if a group standalone SDS message is to be sent:

a)	if the "/<x>/<x>/Common/MCData/AllowedSDS" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request to send SDS and not continue with the rest of the steps in this subclause; and

b)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with:

i)	the <request-type> element set to a value of "group-sds";

ii)	the <mcdata-request-uri> element set to the MCData group identity; and

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

4)	shall generate a standalone SDS message as specified in subclause 6.2.2.1; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.2.1]

In order to generate an SDS message, the MCData client:

1)	shall generate an SDS SIGNALLING PAYLOAD message as specified in subclause 15.1.2;

2)	shall generate a DATA PAYLOAD message as specified in subclause 15.1.4;

3)	shall include in the SIP request, the SDS SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

4)	shall include in the SIP request, the DATA PAYLOAD message in an application/vnd.3gpp.mcdata-payload MIME body as specified in subclause E.2.

When generating an SDS SIGNALLING PAYLOAD message as specified in subclause 15.1.2, the MCData client:

1)	shall set the Date and time IE to the current time as specified in subclause 15.2.8;

2)	if the SDS message starts a new conversation, shall set the Conversation ID IE to a newly generated Conversation ID value as specified in subclause 15.2.9;

3)	if the SDS message continues an existing unfinished conversation, shall set the Conversation ID IE to the Conversation ID value of the existing conversation as specified in subclause 15.2.9;

4)	shall set the Message ID IE to a newly generated Message ID value as specified in subclause 15.2.10;

5)	if the SDS message is in reply to a previously received SDS message, shall include the InReplyTo message ID IE with the Message ID value in the previously received SDS message;

6)	if the SDS message is for user consumption, shall not include an Application ID IE as specified in subclause 15.2.7;

7)	if the SDS message is intended for an application on the terminating MCData client, shall include an Application ID IE with an Application ID value representing the intended application as specified in subclause 15.2.7;

NOTE:	The value chosen for the Application ID value is decided by the mission critical organisation.

8)	if only a delivery disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY" as specified in subclause 15.2.3;

9)	if only a read disposition notification is required shall include a SDS disposition request type IE set to "READ" as specified in subclause 15.2.3; and

10)	if both a delivery and read disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY AND READ" as specified in subclause 15.2.3.

When generating an DATA PAYLOAD message for SDS as specified in subclause 15.1.4, the MCData client:

1)	shall set the Number of payloads IE to the number of Payload IEs that needs to be encoded, as specified in subclause 15.2.12;

2)	if end-to-end security is required for a one-to-one communication, shall include the Security parameters and Payload IE with security parameters as described in 3GPP TS 33.180 [26]. Otherwise, if end-to-end security is not required for a one-to-one communication, shall include the Payload IE as specified in subclause 15.1.4; and

3)	for each Payload IE included:

a)	if the payload is text, shall set the Payload content type as "TEXT" as specified in subclause 15.2.13;

b)	if the payload is binary data, shall set the Payload content type as "BINARY" as specified in subclause 15.2.13;

c)	if the payload is hyperlinks, shall set the Payload content type as "HYPERLINKS" as specified in subclause 15.2.13;

d)	if the payload is location, shall set the Payload content type as "LOCATION" as specified in subclause 15.2.13; and

e)	shall include the data to be sent in the Payload data.

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

…

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.1.3.3	Test description

6.1.3.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element shall not be present in the MCData Service Configuration document so that according to TS 24.484 [24] there is no size limit imposed for the use of C-plane procedures for the SDS message.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.3.3.2	Test procedure sequence

Table 6.1.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCDATA User request to send a group standalone SDS message with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2-2B | Check: Does the UE (MCData Client) perform steps 1a1-3 of the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a standalone group SDS message with disposition request "DELIVERY"? (NOTE 2) | - | - | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | Check: Does the UE (MCData Client) perform the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the SDS message sent at step 2A? | - | - | 2 | P |
| 5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCDATA Client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The RRC connection is not released at the end of the procedure. |  |  |  |  |  |

6.1.3.3.3	Specific message contents

Table 6.1.3.3.3-1: SIP MESSAGE (step 2A, Table 6.1.3.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.3.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.3.3.3-2A |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.3.3.3-3 |  |  |  |

Table 6.1.3.3.3-2: MCDATA-Info (Table 6.1.3.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp

Table 6.1.3.3.3-2A: SDS SIGNALLING PAYLOAD (Table 6.1.3.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.3.3.3-3: DATA PAYLOAD (Table 6.1.3.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-1

Table 6.1.3.3.3-4: SIP MESSAGE (step 4, Table 6.1.3.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | As described in Table 6.1.3.3.3-5 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.3.3.3-6 |  |  |  |

Table 6.1.3.3.3-5: MCDATA-Info (Table 6.1.3.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.2-3A |  |  |

Table 6.1.3.3.3-6: SDS NOTIFICATION (Table 6.1.3.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED

Table 6.1.3.3.3-7: Void

### 6.1.4	On-network / Short Data Service (SDS) / Standalone SDS Using Signalling Control Plane / Group Standalone SDS / Client Terminated (CT)

6.1.4.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a standalone group SDS message with a disposition request of "DELIVERY" }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message and sends a SIP MESSAGE message with a disposition notification of "DELIVERED" and renders the contents of the Payload IE to the MCDATA User }

}

6.1.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.2.2.2, 9.2.1.2, 9.2.1.3, 12.2.1.1, 6.2.3.1, 6.2.4.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.2.2.2]

Upon receipt of a "SIP MESSAGE request for standalone SDS for terminating MCData client", the MCData client:

…

3)	if the SIP MESSAGE request contains an application/mikey MIME body containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP MESSAGE request with a SIP 606 (Not Acceptable) response, and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to exchange end-to-end secure message.

4)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

5)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5]; and

6)	shall handle the received message as specified in subclause 9.2.1.2.

[TS 24.282, clause 9.2.1.2]

When a MCData client has received a SIP request containing:

-	an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

-	an application/vnd.3gpp.mcdata-payload MIME body as specified in subclause E.2;

the MCData Client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body;

2)	shall decode the contents of the application/vnd.3gpp.mcdata-payload MIME body;

…

5)	shall identify the number of Payload IEs in the DATA PAYLOAD message from the Number of payloads IE in the DATA PAYLOAD message;

6)	if the SDS SIGNALLING PAYLOAD message does not contain an Application ID IE:

a)	shall determine that the payload contained in the DATA PAYLOAD message is for user consumption

b)	may notify the MCData user; and

c)	shall render the contents of the Payload IE(s) to the MCData user;

7)	if the SDS SIGNALLING PAYLOAD message contains an Application ID IE:

a)	shall determine that the payload contained in the DATA PAYLOAD message is not for user consumption,

b)	shall not notify the MCData user;

c)	if the Application ID value is unknown, shall discard the SDS message; and

d)	if the Application ID value is known, shall deliver the contents of the Payload IE(s) to the identified application;

NOTE 1:	If required, the MCData client decrypts the Payload IEs before rendering the SDS message to the user or delivering the SDS message to the application.

…

NOTE 3:	User consent is not required before accepting the data.

8)	may store the message payload in local storage along with the Conversation ID, Message ID, InReplyTo message ID and Date and time; and

9)	if the received SDS SIGNALLING PAYLOAD message contains an SDS disposition request type IE shall follow the procedures in subclause 9.2.1.3.

[TS 24.282, clause 9.2.1.3]

To handle the disposition requests, the MCData client:

1)	If the SDS disposition request type IE is set to:

a)	"DELIVERY" then, shall send a delivered notification as described in subclause 12.2.1.1;

b)	"READ", shall send a read notification as described in subclause 12.2.1.1, when a display indication is received; or

c)	"DELIVERY AND READ" then, shall start timer TDU1 (delivery and read).

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the controlling MCData function that sent the SDS or FD message request. The MCData client determines the controlling MCData function from the contents of the <mcdata-controller-psi> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request;

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with an <mcdata-controller-psi> element containing the PSI of the controlling MCData function;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

…

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.3.1]

In order to generate an SDS notification, the MCData client:

1)	shall generate an SDS NOTIFICATION message as specified in subclause 15.1.5; and

2)	shall include in the SIP request, the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an SDS NOTIFICATION message as specified in subclause 15.1.5, the MCData client:

1)	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED" as specified in subclause 15.2.5;

…

5)	shall set the Date and time IE to the current time to as specified in subclause 15.2.8;

6)	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message as specified in subclause 15.2.9;

7)	shall set the Message ID to the value of the Message ID that was received in the SDS message as specified in subclause 15.2.10;

8)	if the SDS message was destined for the user, shall not include an Application ID IE as specified in subclause 15.2.7; and

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

…

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

6.1.4.3	Test description

6.1.4.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element shall not be present in the MCData Service Configuration document so that according to TS 24.484 [24] there is no size limit imposed for the use of C-plane procedures for the SDS message.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.4.3.2	Test procedure sequence

Table 6.1.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-1B | Check: Does the UE (MCData Client) perform steps 1a1-3 of the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive a standalone group SDS message with disposition request "DELIVERY"? (NOTE 2) | - | - | 1 | P |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "DELIVERED"? | - | - | 1 | P |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the UE (MCDATA client) render the contents of the Payload IE to the MCDATA User? (NOTE 1) | - | - | 1 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The RRC connection is not released at the end of the procedure. |  |  |  |  |  |

6.1.4.3.3	Specific message contents

Table 6.1.4.3.3-1: SIP MESSAGE (step 1A, Table 6.1.4.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING, MCDATA_PAYLOAD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.4.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in Table 6.1.4.3.3-3 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-body | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.4.3.3-3 |  |  |  |

Table 6.1.4.3.3-2: MCDATA-Info (Table 6.1.4.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_grp

Table 6.1.4.3.3-3: SDS SIGNALLING PAYLOAD (Table 6.1.4.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.2-1, condition DELIVERED

Table 6.1.4.3.3-4: DATA PAYLOAD (Table 6.1.4.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-2

Table 6.1.4.3.3-5: Void

Table 6.1.4.3.3-6: SIP MESSAGE (step 3, Table 6.1.4.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-headers | MCData-Info as described in Table 6.1.4.3.3-7 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.4.3.3-8 |  |  |  |

Table 6.1.4.3.3-7: MCDATA-Info (Table 6.1.4.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.1-3A |  |  |

Table 6.1.4.3.3-8: SDS NOTIFICATION (Table 6.1.4.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED

### 6.1.5	On-network / Short Data Service (SDS) / Standalone SDS Using Media Plane / One-to-one Standalone SDS / Client Originated (CO)

6.1.5.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a one-to-one standalone SDS message using the media plane}

then { UE (MCDATA Client) sends a request to establish an MSRP connection via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message }

}

(2)

with { UE (MCDATA Client) having requested the establishment of a MSRP connection }

ensure that {

when { UE (MCDATA Client) receives a SIP 200 (OK) message with the a=setup attribute set to "passive"  from the SS (MCDATA server) }

then { UE (MCDATA Client) sends a blank MSRP SEND message to bind the MSRP connection and then sends the one-to-one standalone SDS message via a MSRP SEND message with a disposition of "DELIVERY" }

}

(3)

with { UE (MCDATA Client) having sent a one-to-one standalone SDS message using the media plane }

ensure that {

when { UE (MCDATA Client receives a MSRP 200 (OK) message in response to the last MSRP SEND message indicating that the standalone SDS message has been successfully transferred }

then { UE (MCDATA Client) sends a SIP BYE message }

}

(4)

with { UE (MCDATA Client) having sent a one-to-one standalone SDS message using the media plane with a disposition of "DELIVERY" }

ensure that {

when { UE (MCDATA Client receives a disposition response via a SIP MESSAGE message from the SS (MCDATA Server }

then { UE (MCDATA Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.1.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.3.2.3, 9.2.3.2.1, 13.2.2.2.2.1, 12.2.1.2, TS 24.582 clauses 6.1.1.2.1, 6.1.1.2.2, 6.1.1.2.3, 6.1.1.2.4. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.3.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.sds media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	if a one-to-one standalone SDS message is to be sent:

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user, according to rules and procedures of IETF RFC 5366 [18];

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-sds"; and

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26];

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26];

…

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.3.2.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.1.2.

…

On receipt of an indication from the media plane indicating that the standalone SDS message has been successfully transferred, the MCData client shall:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "SIP";

b)	cause set to "200"; and

c)	text set to "transmission succeeded";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall interact with the media plane and indicate to terminate the session, as specified in 3GPP TS 24.582 [15].

[TS 24.282, clause 9.2.3.2.1]

When composing an SDP offer according to 3GPP TS 24.229 [5], IETF RFC 4975 [17], IETF RFC 6135 [19] and IETF RFC 6714 [20] the MCData client:

1)	shall include an "m=message" media-level section for the MCData media stream consisting of:

a)	the IP address and the port number;

b)	a protocol field value of "TCP/MSRP", or "TCP/TLS/MSRP" for TLS;

c)	a format list field set to '*';

d)	an "a=sendonly" attribute;

e)	an "a=path" attribute containing its own MSRP URI;

f)	set the content type as "a=accept-types:application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload"; and

g)	set the a=setup attribute as "actpass"; and

2)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [45].

[TS 24.282, clause 13.2.2.2.2.1]

When the MCData client wants to release a MCData communication established over the media plane, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5];

2)	shall set the Request-URI to the MCData session identity to be released; and

3)	shall send the SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall release all media plane resources corresponding to the MCData communication being released.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

[TS 24.582, clause 6.1.1.2.1]

Upon receiving an indication to establish MSRP connection for standalone SDS using media plane as the originating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11]; and

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12].

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

If MSRP chunking is not used then on receipt of a 200 (OK) response, the MCData client shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary;

2.	shall wait for a 200 (OK) response to each MSRP SEND request sent; and

3.	on receipt of the last 200 (OK) response shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks;

2.	shall indicate to MCData user that the SDS message could not be sent; and

3.	shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving an indication to terminate the session from the signalling plane, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks and may indicate to MCData user that the SDS message could not be sent.

[TS 24.582, clause 6.1.1.2.2]

In order to generate an SDS signalling payload, the MCData client:

1.	shall generate an SDS SIGNALLING PAYLOAD message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the SDS SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in 3GPP TS 24.282 [8]; and

When generating a an SDS SIGNALLING PAYLOAD message, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD message as defined in 3GPP TS 24.282 [8]. In the SDS SIGNALLING PAYLOAD message, the MCData client:

a.	may include and set the Disposition request type IE to:

i.	"DELIVERY", if only delivery disposition is requested;

ii.	"READ", if only read disposition is requested; or

iii.	"DELIVERY AND READ", if both delivery and read dispositions are requested;

b.	shall set Date and time IE to current UTC time;

c.	shall set Conversation ID IE to a universally unique message ID generated as per IETF RFC 4122 [10];

d.	shall set Message ID IE to a universally unique message ID generated as per IETF RFC 4122 [10];

e.	if indicated that the SDS message is in reply to another SDS message then, shall include the Reply ID IE set to the message identifier of the indicated SDS message; and

f.	if indicated that the target recipient of the SDS message is an application then, shall set Application Identifier IE to the application identifier.

[TS 24.582, clause 6.1.1.2.3]

In order to generate SDS data payload, the MCData client:

1.	shall generate a DATA PAYLOAD message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the DATA PAYLOAD message in an application/vnd.3gpp.mcdata-payload MIME body as specified in 3GPP TS 24.282 [8].

When generating a DATA PAYLOAD message, the MCData client:

1.	shall generate a SDS DATA PAYLOAD message as defined in 3GPP TS 24.282 [8]. In the SDS DATA PAYLOAD message, the MCData client:

a.	shall set Number of payloads IE to the total number of payloads being sent; and

b.	for each payload, shall include Payload IE. In the Payload IE:

i.	shall set Payload content type to "TEXT", or "BINARY", or "HYPERLINKS", or "LOCATION" according to the payload type; and

ii.	shall set Payload data IE to actual payload.

[TS 24.582, clause 6.1.1.2.4]

The MCData client shall take the procedures in subclause 6.4.1 into consideration when generating MSRP SEND messages.

The MCData client shall generate MSRP SEND for SDS message requests according to IETF RFC 4975 [11].

When generating an MSRP SEND for SDS message request containing an SDS SIGNALLING PAYLOAD message and an SDS DATA PAYLOAD message, the MCData client

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the first content type as Content-Type = "application/vnd.3gpp.mcdata-signalling";

3.	shall set the first body of the MSRP SEND request to the generated SDS SIGNALLING PAYLOAD message;

4.	shall set the second Content-Type as "application/vnd.3gpp.mcdata-payload"; and

5.	shall set the second body of the MSRP SEND request to the generated SDS DATA PAYLOAD message.

When generating an MSRP SEND for SDS message request containing only an SDS DATA PAYLOAD message, the MCData client:

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the Content-Type as "application/vnd.3gpp.mcdata-payload"; and

3.	shall set the body of the MSRP SEND request to the generated SDS DATA PAYLOAD message.

When generating an MSRP SEND for SDS message request containing only an SDS SIGNALLING PAYLOAD, the MCData client.

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the Content-Type as "application/vnd.3gpp.mcdata-signalling"; and

3.	shall set the body of the MSRP SEND request to the generated SDS SIGNALLING PAYLOAD message.

6.1.5.3	Test description

6.1.5.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element of the MCData Service Configuration document shall be set to 0 to force the MCData Client to send the data using the media plane.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.5.3.2	Test procedure sequence

Table 6.1.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCData User request to send a one-to-one standalone SDS message with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData Client) perform the procedure for CO MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.2.3-1? | - | - | 1,2 | P |
| 3-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CO MCData call release specified in TS 36.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCData Client) perform the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the SDS message sent at step 7? |  |  |  |  |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCDATA Client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.5.3.3	Specific message contents

Table 6.1.5.3.3-1: SIP INVITE (step 2, Table 6.1.5.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_SDS, MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | As described in Table 6.1.5.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.5.3.3-3 |  |  |  |

Table 6.1.5.3.3-2: SDP for SIP INVITE (Table 6.1.5.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_OFFER, MCD_1to1

Table 6.1.5.3.3-3: MCDATA-Info (Table 6.1.5.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_1to1

Table 6.1.5.3.3-4: SIP 200 (OK) (step 2, Table 6.1.5.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.5.3.3-5 |  |  |  |  |  |  |  |

Table 6.1.5.3.3-5: SDP for SIP 200 (OK) (Table 6.1.5.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_ANSWER |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.5.3.3-6: MSRP SEND (step 7, Table 6.1.5.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.5.3.3-6A |  |  |  |

Table 6.1.5.3.3-6A: MIME Message (step 7, Table 6.1.5.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.5.3.3-6B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | DATA PAYLOAD as described Table 6.1.5.3.3-7 |  |  |  |  |  |  |  |

Table 6.1.5.3.3-6B: SDS SIGNALLING PAYLOAD (Table 6.1.5.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.5.3.3-7: Data Payload (Table 6.1.5.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-1

Table 6.1.5.3.3-8..9: Void

Table 6.1.5.3.3-10: SIP BYE (step 8, Table 6.1.5.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.6.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1, condition MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  | RFC 3326 [125] |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.1.5.3.3-11: Void

Table 6.1.5.3.3-12: SIP MESSAGE (step 10, Table 6.1.5.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.5.3.3-14 |  |  |  |

Table 6.1.5.3.3-13: Void

Table 6.1.5.3.3-14: SDS NOTIFICATION (Table 6.1.5.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.5.3.3-15: Void

### 6.1.6	On-network / Short Data Service (SDS) / Standalone SDS Using Media Plane / One-to-one Standalone SDS / Client Terminated (CT)

6.1.6.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a standalone one-to-one SDS message using the media plane }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having responded to the SIP INVITE message that initiated a standalone one-to-one SDS message using the media plane }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message from the SS (MCDATA Server) }

then { UE (MCDATA Client) responds with an MSRP 200 (OK) message and if the MSRP SEND message is not blank, renders the contents of the Payload IE to the MCDATA User and sends a SIP MESSAGE message with a disposition notification of "DELIVERED" }

}

(3)

with { UE (MCDATA Client) having responded to the MSRP SEND message from the SS (MCDATA server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message to release communications }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message }

}

6.1.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.3.2.4, 9.2.3.2.2, 13.2.2.2.2.2, 9.2.1.3, 12.2.1.1, 6.2.4.1, 6.2.3.1, TS 24.582 clauses 6.1.1.3.1, 6.1.1.3.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.3.2.4]

Upon receipt of an initial SIP INVITE request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps after step 2;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in subclause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

3)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [45], and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to create an end-to-end secure session.

3)	may display to the MCData user the MCData ID of the inviting MCData user and the type of SDS request;

4)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

5)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

6)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

7)	shall include the g.3gpp.mcdata.sds media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP 200 (OK) response;

9)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.3.2.2; and

10)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client shall:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.1.3.

[TS 24.282, clause 9.2.3.2.2]

When the MCData client receives an initial SDP offer for an MCData standalone SDS, the MCData client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [5] and IETF RFC 4975 [17].

When composing an SDP answer, the MCData client:

1)	shall include an "m=message" media-level section for the accepted MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP", or "TCP/TLS/MSRP" for TLS according to the received SDP offer;

c)	a format list field set to '*';

d)	an "a=recvonly" attribute;

e)	an "a=path" attribute containing its own MSRP URI;

f)	set the content type as a=accept-types: application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload; and

g)	set the a=setup attribute according to IETF RFC 6135 [19].

[TS 24.282, clause 13.2.2.2.2.2]

Upon receiving a SIP BYE request, the MCData client:

1)	shall send SIP 200 (OK) response towards MCData server according to 3GPP TS 24.229 [5]; and

2)	shall release all media plane resources corresponding to the MCData communication being released.

NOTE:	Partially received data can be stored and processed.

[TS 24.282, clause 9.2.1.3]

To handle the disposition requests, the MCData client:

1)	If the SDS disposition request type IE is set to:

a)	"DELIVERY" then, shall send a delivered notification as described in subclause 12.2.1.1;

b)	"READ", shall send a read notification as described in subclause 12.2.1.1, when a display indication is received; or

c)	"DELIVERY AND READ" then, shall start timer TDU1 (delivery and read).

Upon receiving a display indication before timer TDU1 (delivery and read) expires, the MCData client:

1)	shall stop timer TDU1 (delivery and read); and

2)	shall send a delivered and read notification as described in subclause 12.2.1.1.

Upon expiry of timer TDU1 (delivery and read), the MCData client:

1)	shall send a delivered notification as described in subclause 12.2.1.1; and

2)	upon receiving a display indication, send a read notification as described in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the controlling MCData function that sent the SDS or FD message request. The MCData client determines the controlling MCData function from the contents of the <mcdata-controller-psi> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request;

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with an <mcdata-controller-psi> element containing the PSI of the controlling MCData function;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

2)	when sending FD messages, FD disposition notifications or FD media storage function discovery messages:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.fd media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

[TS 24.282, clause 6.2.3.1]

In order to generate an SDS notification, the MCData client:

1)	shall generate an SDS NOTIFICATION message as specified in subclause 15.1.5; and

2)	shall include in the SIP request, the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an SDS NOTIFICATION message as specified in subclause 15.1.5, the MCData client:

1)	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED" as specified in subclause 15.2.5;

2)	if sending a read notification, shall set the SDS disposition notification type IE as "READ" as specified in subclause 15.2.5;

3)	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ" as specified in subclause 15.2.5;

4)	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED" as specified in subclause 15.2.5;

5)	shall set the Date and time IE to the current time to as specified in subclause 15.2.8;

6)	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message as specified in subclause 15.2.9;

7)	shall set the Message ID to the value of the Message ID that was received in the SDS message as specified in subclause 15.2.10;

8)	if the SDS message was destined for the user, shall not include an Application ID IE as specified in subclause 15.2.7; and

9)	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message as specified in subclause 15.2.3.

[TS 24.582, clause 6.1.1.3.1]

Upon receiving an indication to establish MSRP connection for standalone SDS using media plane as the terminating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP connection is established, the MCData client:

1.	on receipt of an MSRP request in an MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData standalone message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.1.3.2.

[TS 24.582, clause 6.1.1.3.2]

The MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body;

2.	shall decode the contents of the application/vnd.3gpp.mcdata-payload MIME body;

3.	if the SDS SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the SDS SIGNALLING PAYLOAD identifying the first message in the conversation thread;

4.	if the SDS SIGNALLING PAYLOAD message contains an existing Conversation ID and:

a.	if the SDS SIGNALLING PAYLOAD message does not contain an InReplyTo Message ID, shall use the Message ID in the SDS SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

b.	if the SDS SIGNALLING PAYLOAD message contains an InReplyTo Message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo Message ID in the SDS SIGNALLING PAYLOAD and use the Message ID in the SDS SIGNALLING PAYLOAD to identify the new message;

5.	shall identify the number of Payload IEs in the DATA PAYLOAD message from the Number of Payloads IE in the DATA PAYLOAD message;

6.	if the SDS SIGNALLING PAYLOAD message does not contain an Application identifier IE:

a.	shall determine that the payload contained in the DATA PAYLOAD message is for user consumption;

b.	may notify the MCData user; and

c.	shall render the contents of the Payload IE(s) to the MCData user;

7.	if the SDS SIGNALLING PAYLOAD message contains an Application identifier IE:

a.	shall determine that the payload contained in the DATA PAYLOAD message is not for user consumption;

b.	shall not notify the MCData user;

c.	if the Application identifier value is unknown, shall discard the SDS message; and

d.	if the Application identifier value is known, shall deliver the contents of the Payload IE(s) to the identified application; and

8.	if SDS Disposition request type IE is present in the SDS SIGNALLING PAYLOAD message received in subclause 6.1.1.3.1 then, shall send a disposition notification as described in 3GPP TS 24.282 [8] subclause 9.2.1.3.

6.1.6.3	Test description

6.1.6.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element of the MCData Service Configuration document shall be set to 0 to force the MCData Client to send the data using the media plane.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.6.3.2	Test procedure sequence

Table 6.1.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData Client) perform the procedure for CT MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| 2-5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CT MCData call release specified in TS 36.579-1 [2] Table 5.3C.7.3-1? | - | - | 3 | P |
| 9-10 | Void | - | - | - | - |
| 11 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "DELIVERED"? | - | - | 2 | P |
| 12 | Check: Does the UE (MCData Client) render the contents of the Payload IE to the MCData User? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.6.3.3	Specific message contents

Table 6.1.6.3.3-1: SIP INVITE (step 1, Table 6.1.6.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.1.6.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.6.3.3-3 |  |  |  |

Table 6.1.6.3.3-2: SDP for SIP INVITE (Table 6.1.6.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_OFFER, MCD_1to1

Table 6.1.6.3.3-3: MCDATA-Info (Table 6.1.6.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.6.3.3-4: SIP 200 (OK) (step 2, Table 6.1.6.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.6.3.3-5 |  |  |  |  |  |  |  |

Table 6.1.6.3.3-5: SDP for SIP 200 (OK) (Table 6.1.6.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_ANSWER |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.6.3.3-6: MSRP SEND (step 6, Table 6.1.6.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message as specified in table 6.1.6.3.3-6A |  |  |  |

Table 6.1.6.3.3-6A: MIME Message (step 6, Table 6.1.6.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.6.3.3-6B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | DATA PAYLOAD as described in Table 6.1.6.3.3-7 |  |  |  |  |  |  |  |

Table 6.1.6.3.3-6B: SDS SIGNALLING PAYLOAD (Table 6.1.6.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.6.3.3-7: Data Payload (Table 6.1.6.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-2

Table 6.1.6.3.3-8: SIP BYE (step 8, Table 6.1.6.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1, condition MT_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  | RFC 3326 [125] |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.1.6.3.3-9: Void

Table 6.1.6.3.3-10: SIP MESSAGE (step 11, Table 6.1.6.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.6.3.3-11 |  |  |  |

Table 6.1.6.3.3-11: SDS NOTIFICATION (Table 6.1.6.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

### 6.1.7	On-network / Short Data Service (SDS) / Standalone SDS Using Media Plane / Group Standalone SDS / Client Originated (CO)

6.1.7.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a group standalone SDS message using the media plane}

then { UE (MCDATA Client) sends a request to establish an MSRP connection via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message }

}

(2)

with { UE (MCDATA Client) having requested the establishment of a MSRP connection }

ensure that {

when { UE (MCDATA Client) receives a SIP 200 (OK) message with the a=setup attribute set to "passive" }

then { UE (MCDATA Client) sends a blank MSRP SEND message to bind the MSRP connection and then sends the group standalone SDS message via a MSRP SEND message with a disposition of "DELIVERY" }

}

(3)

with { UE (MCDATA Client) having sent a group standalone SDS message using the media plane }

ensure that {

when { UE (MCDATA Client) receives a MSRP 200 (OK) message in response to the last MSRP SEND message indicating that the standalone SDS message has been successfully transferred }

then { UE (MCDATA Client) sends a SIP BYE message }

}

(4)

with { UE (MCDATA Client) having sent a group standalone SDS message using the media plane with a disposition of "DELIVERY" }

ensure that {

when { UE (MCDATA Client) receives a disposition response via a SIP MESSAGE message }

then { UE (MCDATA Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.1.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.3.2.3, 9.2.3.2.1, 13.2.2.2.2.1, 12.2.1.2, TS 24.582 clauses 6.1.1.2.1, 6.1.1.2.2, 6.1.1.2.3, 6.1.1.2.4. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.3.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.sds media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	if a one-to-one standalone SDS message is to be sent:

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user, according to rules and procedures of IETF RFC 5366 [18];

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-sds"; and

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26];

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26];

…

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.3.2.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.1.2.

…

On receipt of an indication from the media plane indicating that the standalone SDS message has been successfully transferred, the MCData client shall:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "SIP";

b)	cause set to "200"; and

c)	text set to "transmission succeeded";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall interact with the media plane and indicate to terminate the session, as specified in 3GPP TS 24.582 [15].

[TS 24.282, clause 9.2.3.2.1]

When composing an SDP offer according to 3GPP TS 24.229 [5], IETF RFC 4975 [17], IETF RFC 6135 [19] and IETF RFC 6714 [20] the MCData client:

1)	shall include an "m=message" media-level section for the MCData media stream consisting of:

a)	the IP address and the port number;

b)	a protocol field value of "TCP/MSRP", or "TCP/TLS/MSRP" for TLS;

c)	a format list field set to '*';

d)	an "a=sendonly" attribute;

e)	an "a=path" attribute containing its own MSRP URI;

f)	set the content type as "a=accept-types:application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload"; and

g)	set the a=setup attribute as "actpass"; and

2)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [45].

[TS 24.282, clause 13.2.2.2.2.1]

When the MCData client wants to release a MCData communication established over the media plane, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5];

2)	shall set the Request-URI to the MCData session identity to be released; and

3)	shall send the SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall release all media plane resources corresponding to the MCData communication being released.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

[TS 24.582, clause 6.1.1.2.1]

Upon receiving an indication to establish MSRP connection for standalone SDS using media plane as the originating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11]; and

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12].

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

If MSRP chunking is not used then on receipt of a 200 (OK) response, the MCData client shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary;

2.	shall wait for a 200 (OK) response to each MSRP SEND request sent; and

3.	on receipt of the last 200 (OK) response shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks;

2.	shall indicate to MCData user that the SDS message could not be sent; and

3.	shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving an indication to terminate the session from the signalling plane, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks and may indicate to MCData user that the SDS message could not be sent.

[TS 24.582, clause 6.1.1.2.2]

In order to generate an SDS signalling payload, the MCData client:

1.	shall generate an SDS SIGNALLING PAYLOAD message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the SDS SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in 3GPP TS 24.282 [8]; and

When generating a an SDS SIGNALLING PAYLOAD message, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD message as defined in 3GPP TS 24.282 [8]. In the SDS SIGNALLING PAYLOAD message, the MCData client:

a.	may include and set the Disposition request type IE to:

i.	"DELIVERY", if only delivery disposition is requested;

ii.	"READ", if only read disposition is requested; or

iii.	"DELIVERY AND READ", if both delivery and read dispositions are requested;

b.	shall set Date and time IE to current UTC time;

c.	shall set Conversation ID IE to a universally unique message ID generated as per IETF RFC 4122 [10];

d.	shall set Message ID IE to a universally unique message ID generated as per IETF RFC 4122 [10];

e.	if indicated that the SDS message is in reply to another SDS message then, shall include the Reply ID IE set to the message identifier of the indicated SDS message; and

f.	if indicated that the target recipient of the SDS message is an application then, shall set Application Identifier IE to the application identifier.

[TS 24.582, clause 6.1.1.2.3]

In order to generate SDS data payload, the MCData client:

1.	shall generate a DATA PAYLOAD message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the DATA PAYLOAD message in an application/vnd.3gpp.mcdata-payload MIME body as specified in 3GPP TS 24.282 [8].

When generating a DATA PAYLOAD message, the MCData client:

1.	shall generate a SDS DATA PAYLOAD message as defined in 3GPP TS 24.282 [8]. In the SDS DATA PAYLOAD message, the MCData client:

a.	shall set Number of payloads IE to the total number of payloads being sent; and

b.	for each payload, shall include Payload IE. In the Payload IE:

i.	shall set Payload content type to "TEXT", or "BINARY", or "HYPERLINKS", or "LOCATION" according to the payload type; and

ii.	shall set Payload data IE to actual payload.

[TS 24.582, clause 6.1.1.2.4]

The MCData client shall take the procedures in subclause 6.4.1 into consideration when generating MSRP SEND messages.

The MCData client shall generate MSRP SEND for SDS message requests according to IETF RFC 4975 [11].

When generating an MSRP SEND for SDS message request containing an SDS SIGNALLING PAYLOAD message and an SDS DATA PAYLOAD message, the MCData client

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the first content type as Content-Type = "application/vnd.3gpp.mcdata-signalling";

3.	shall set the first body of the MSRP SEND request to the generated SDS SIGNALLING PAYLOAD message;

4.	shall set the second Content-Type as "application/vnd.3gpp.mcdata-payload"; and

5.	shall set the second body of the MSRP SEND request to the generated SDS DATA PAYLOAD message.

When generating an MSRP SEND for SDS message request containing only an SDS DATA PAYLOAD message, the MCData client:

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the Content-Type as "application/vnd.3gpp.mcdata-payload"; and

3.	shall set the body of the MSRP SEND request to the generated SDS DATA PAYLOAD message.

When generating an MSRP SEND for SDS message request containing only an SDS SIGNALLING PAYLOAD, the MCData client.

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the Content-Type as "application/vnd.3gpp.mcdata-signalling"; and

3.	shall set the body of the MSRP SEND request to the generated SDS SIGNALLING PAYLOAD message.

6.1.7.3	Test description

6.1.7.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element of the MCData Service Configuration document shall be set to 0 to force the MCData Client to send the data using the media plane.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.7.3.2	Test procedure sequence

Table 6.1.7.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCData User request to send a group standalone SDS message with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData Client) perform the procedure for CO MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.2.3-1? | - | - | 1,2 | P |
| 3-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CO MCData call release specified in TS 36.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCData Client) perform the procedure for MCX SIP MESSAGE CT specified in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the SDS message sent at step 7? |  |  |  |  |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCDATA Client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.7.3.3	Specific message contents

Table 6.1.7.3.3-1: SIP INVITE (step 2, Table 6.1.7.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | As described in Table 6.1.7.3.3-1A |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.7.3.3-2 |  |  |  |

Table 6.1.7.3.3-1A: SDP for SIP INVITE (Table 6.1.7.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_OFFER

Table 6.1.7.3.3-2: MCDATA-Info (Table 6.1.7.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp

Table 6.1.7.3.3-3: SIP 200 (OK) (step 2, Table 6.1.7.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.7.3.3-4 |  |  |  |  |  |  |  |

Table 6.1.7.3.3-4: SDP for SIP 200 (OK) (Table 6.1.7.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_ANSWER |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.7.3.3-5: MSRP SEND (step 7, Table 6.1.7.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.7.3.3-5A |  |  |  |

Table 6.1.7.3.3-5A: MIME Message (step 7, Table 6.1.7.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.7.3.3-5B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.7.3.3-6 |  |  |  |  |  |  |  |

Table 6.1.7.3.3-5B: SDS SIGNALLING PAYLOAD (Table 6.1.7.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.7.3.3-6: Data Payload (Table 6.1.7.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-1

Table 6.1.7.3.3-7..8: Void

Table 6.1.7.3.3-9: SIP BYE (step 8, Table 6.1.7.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.6.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1, condition MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  | RFC 3326 [125] |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.1.7.3.3-10: Void

Table 6.1.7.3.3-11: SIP MESSAGE (step 10, Table 6.1.7.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_SDS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | As described in Table 6.1.7.3.3-12 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.7.3.3-13 |  |  |  |

Table 6.1.7.3.3-12: MCDATA-Info (Table 6.1.7.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.2-3A |  |  |

Table 6.1.7.3.3-13: SDS NOTIFICATION (Table 6.1.7.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.7.3.3-14: Void

### 6.1.8	On-network / Short Data Service (SDS) / Standalone SDS Using Media Plane / Group Standalone SDS / Client Terminated (CT)

6.1.8.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a standalone group SDS message using the media plane }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having responded to the SIP INVITE message that initiated a standalone group SDS message using the media plane }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message }

then { UE (MCDATA Client) responds with an MSRP 200 (OK) message and if the MSRP SEND message is not blank, renders the contents of the Payload IE to the MCDATA User and sends a SIP MESSAGE message with a disposition notification of "DELIVERED" }

}

(3)

with { UE (MCDATA Client) having responded to the MSRP SEND message from the SS (MCDATA server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message to release communications }

then { UE (MCDATA Client) sends a SIP 200 (OK) message }

}

6.1.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.3.2.4, 9.2.3.2.2, 13.2.2.2.2.2, 9.2.1.3, 12.2.1.1, 6.2.4.1, 6.2.3.1, TS 24.582 clauses 6.1.1.3.1, 6.1.1.3.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.3.2.4]

Upon receipt of an initial SIP INVITE request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps after step 2;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in subclause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

3)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [45], and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to create an end-to-end secure session.

3)	may display to the MCData user the MCData ID of the inviting MCData user and the type of SDS request;

4)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

5)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

6)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

7)	shall include the g.3gpp.mcdata.sds media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP 200 (OK) response;

9)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.3.2.2; and

10)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client shall:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.1.3.

[TS 24.282, clause 9.2.3.2.2]

When the MCData client receives an initial SDP offer for an MCData standalone SDS, the MCData client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [5] and IETF RFC 4975 [17].

When composing an SDP answer, the MCData client:

1)	shall include an "m=message" media-level section for the accepted MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP", or "TCP/TLS/MSRP" for TLS according to the received SDP offer;

c)	a format list field set to '*';

d)	an "a=recvonly" attribute;

e)	an "a=path" attribute containing its own MSRP URI;

f)	set the content type as a=accept-types: application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload; and

g)	set the a=setup attribute according to IETF RFC 6135 [19].

[TS 24.282, clause 13.2.2.2.2.2]

Upon receiving a SIP BYE request, the MCData client:

1)	shall send SIP 200 (OK) response towards MCData server according to 3GPP TS 24.229 [5]; and

2)	shall release all media plane resources corresponding to the MCData communication being released.

NOTE:	Partially received data can be stored and processed.

[TS 24.282, clause 9.2.1.3]

To handle the disposition requests, the MCData client:

1)	If the SDS disposition request type IE is set to:

a)	"DELIVERY" then, shall send a delivered notification as described in subclause 12.2.1.1;

b)	"READ", shall send a read notification as described in subclause 12.2.1.1, when a display indication is received; or

c)	"DELIVERY AND READ" then, shall start timer TDU1 (delivery and read).

Upon receiving a display indication before timer TDU1 (delivery and read) expires, the MCData client:

1)	shall stop timer TDU1 (delivery and read); and

2)	shall send a delivered and read notification as described in subclause 12.2.1.1.

Upon expiry of timer TDU1 (delivery and read), the MCData client:

1)	shall send a delivered notification as described in subclause 12.2.1.1; and

2)	upon receiving a display indication, send a read notification as described in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the controlling MCData function that sent the SDS or FD message request. The MCData client determines the controlling MCData function from the contents of the <mcdata-controller-psi> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request;

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with an <mcdata-controller-psi> element containing the PSI of the controlling MCData function;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.4.1]

This subclause is referenced from other procedures.

In a SIP MESSAGE request, the MCData client:

1)	when sending SDS messages or SDS disposition notifications:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

2)	when sending FD messages, FD disposition notifications or FD media storage function discovery messages:

a)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.fd media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

b)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8]; and

c)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5]; and

4)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the MCData user.

[TS 24.282, clause 6.2.3.1]

In order to generate an SDS notification, the MCData client:

1)	shall generate an SDS NOTIFICATION message as specified in subclause 15.1.5; and

2)	shall include in the SIP request, the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an SDS NOTIFICATION message as specified in subclause 15.1.5, the MCData client:

1)	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED" as specified in subclause 15.2.5;

2)	if sending a read notification, shall set the SDS disposition notification type IE as "READ" as specified in subclause 15.2.5;

3)	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ" as specified in subclause 15.2.5;

4)	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED" as specified in subclause 15.2.5;

5)	shall set the Date and time IE to the current time to as specified in subclause 15.2.8;

6)	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message as specified in subclause 15.2.9;

7)	shall set the Message ID to the value of the Message ID that was received in the SDS message as specified in subclause 15.2.10;

8)	if the SDS message was destined for the user, shall not include an Application ID IE as specified in subclause 15.2.7; and

9)	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message as specified in subclause 15.2.3.

[TS 24.582, clause 6.1.1.3.1]

Upon receiving an indication to establish MSRP connection for standalone SDS using media plane as the terminating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP connection is established, the MCData client:

1.	on receipt of an MSRP request in an MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData standalone message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.1.3.2.

[TS 24.582, clause 6.1.1.3.2]

The MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body;

2.	shall decode the contents of the application/vnd.3gpp.mcdata-payload MIME body;

3.	if the SDS SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the SDS SIGNALLING PAYLOAD identifying the first message in the conversation thread;

4.	if the SDS SIGNALLING PAYLOAD message contains an existing Conversation ID and:

a.	if the SDS SIGNALLING PAYLOAD message does not contain an InReplyTo Message ID, shall use the Message ID in the SDS SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

b.	if the SDS SIGNALLING PAYLOAD message contains an InReplyTo Message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo Message ID in the SDS SIGNALLING PAYLOAD and use the Message ID in the SDS SIGNALLING PAYLOAD to identify the new message;

5.	shall identify the number of Payload IEs in the DATA PAYLOAD message from the Number of Payloads IE in the DATA PAYLOAD message;

6.	if the SDS SIGNALLING PAYLOAD message does not contain an Application identifier IE:

a.	shall determine that the payload contained in the DATA PAYLOAD message is for user consumption;

b.	may notify the MCData user; and

c.	shall render the contents of the Payload IE(s) to the MCData user;

7.	if the SDS SIGNALLING PAYLOAD message contains an Application identifier IE:

a.	shall determine that the payload contained in the DATA PAYLOAD message is not for user consumption;

b.	shall not notify the MCData user;

c.	if the Application identifier value is unknown, shall discard the SDS message; and

d.	if the Application identifier value is known, shall deliver the contents of the Payload IE(s) to the identified application; and

8.	if SDS Disposition request type IE is present in the SDS SIGNALLING PAYLOAD message received in subclause 6.1.1.3.1 then, shall send a disposition notification as described in 3GPP TS 24.282 [8] subclause 9.2.1.3.

6.1.8.3	Test description

6.1.8.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The <max-payload-size-sds-cplane-bytes> element of the MCData Service Configuration document shall be set to 0 to force the MCData Client to send the data using the media plane.

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.8.3.2	Test procedure sequence

Table 6.1.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData Client) perform the procedure for CT MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| 2-5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CT MCData call release specified in TS 36.579-1 [2] Table 5.3C.7.3-1? | - | - | 3 | P |
| 9-10 | Void | - | - | - | - |
| 11 | Check: Does the UE (MCData Client) perform the procedure for CO SDS or FD message transfer using signalling plane specified in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification of "DELIVERED"? | - | - | 2 | P |
| 12 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCData User? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.8.3.3	Specific message contents

Table 6.1.8.3.3-1: SIP INVITE (step 1, Table 6.1.8.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.1.8.3.3-1A |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.8.3.3-2 |  |  |  |

Table 6.1.8.3.3-1A: SDP for SIP INVITE (Table 6.1.8.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_OFFER

Table 6.1.8.3.3-2: MCDATA-Info (Table 6.1.8.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_grp

Table 6.1.8.3.3-3: SIP 200 (OK) (step 2, Table 6.1.8.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.8.3.3-4 |  |  |  |  |  |  |  |

Table 6.1.8.3.3-4: SDP for SIP INVITE (Table 6.1.8.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_ANSWER |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.8.3.3-5: MSRP SEND (step 6, Table 6.1.8.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message as specified in table 6.1.8.3.3-5A |  |  |  |

Table 6.1.8.3.3-5A: MIME Message (step 6, Table 6.1.8.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.8.3.3-5B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.8.3.3-6 |  |  |  |  |  |  |  |

Table 6.1.8.3.3-5B: SDS SIGNALLING PAYLOAD (Table 6.1.8.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.8.3.3-6: Data Payload (Table 6.1.8.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-2

Table 6.1.8.3.3-7: SIP BYE (step 8, Table 6.1.8.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1, condition MT_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  | RFC 3326 [125] |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.1.8.3.3-8: Void

Table 6.1.8.3.3-9: SIP MESSAGE (step 11, Table 6.1.8.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_SDS, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.8.3.3-10 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS NOTIFICATION as described in Table 6.1.6.3.3-11 |  |  |  |

Table 6.1.8.3.3-10: MCDATA-Info (Table 6.1.8.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.1-3A |  |  |

Table 6.1.8.3.3-11: SDS NOTIFICATION (Table 6.1.8.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

### 6.1.9	On-network / Short Data Service (SDS) / SDS Session / One-to-one SDS Session / Client Originated (CO)

6.1.9.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to initiate a one-to-one SDS session using the media plane}

then { UE (MCDATA Client) sends a request to establish a one-to-one SDS session and a MSRP connection via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message }

}

(2)

with { UE (MCDATA Client) having received a SIP 200 (OK) message with the a=setup attribute set to "passive" in response to a SIP INVITE message}

ensure that {

when { UE (MCDATA Client) responds to the SIP 200 (OK) message with a SIP ACK message }

then { UE (MCData Client) sends a blank MSRP SEND message to bind the MSRP connection and then sends the one-to-one session SDS message via a MSRP SEND message with a disposition of "DELIVERY" }

}

(3)

with { UE (MCDATA Client) having sent a one-to-one session SDS message using the media plane with a disposition of "DELIVERY" }

ensure that {

when { UE (MCDATA Client) receives a disposition response via a MSRP SEND message }

then { UE (MCDATA Client) responds to the MSRP SEND message by sending a MSRP 200 (OK) message and delivers the notification to the MCDATA User }

}

(4)

with { UE (MCDATA Client) having established a one-to-one SDS session and a MSRP connection }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message with a disposition of "READ" }

then { UE (MCDATA Client) responds with a MSRP 200 (OK) message and then renders the contents of the Payload IE to the MCDATA User and then sends a MSRP SEND message with a disposition notification of "READ" }

}

(5)

with { UE (MCDATA Client) having established a one-to-one SDS session }

ensure that {

when { the MCDATA User requests to release the one-to-one SDS session }

then { UE (MCDATA Client) sends a SIP BYE message }

}

6.1.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.4.2.3, 9.2.4.2.1, 13.2.2.2.2.1, TS 24.582 clauses 6.1.2.2.1, 6.1.2.4, 6.1.2.5.1, 6.1.2.5.2, 6.1.2.5.3, 6.1.2.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.4.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.sds media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	if a one-to-one SDS session is requested:

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user, according to rules and procedures of IETF RFC 5366 [18];

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-sds-session"; and

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26];

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26];

…

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.4.2.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.2.2.

[TS 24.282, clause 9.2.4.2.1]

When composing an SDP offer according to 3GPP TS 24.229 [5], IETF RFC 4975 [17], IETF RFC 6135 [19] and IETF RFC 6714 [20] the MCData client:

1)	shall include an "m=message" media-level section for the MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP" or "TCP/TLS/MSRP" for TLS;

c)	an "a=sendrecv" attribute;

d)	an "a=path" attribute containing its own MSRP URI;

e)	set the content type as "a=accept-types:application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload"; and

f)	set the a=setup attribute as "actpass"; and

2)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [45].

[TS 24.282, clause 13.2.2.2.2.1]

When the MCData client wants to release a MCData communication established over the media plane, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5];

2)	shall set the Request-URI to the MCData session identity to be released; and

3)	shall send the SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall release all media plane resources corresponding to the MCData communication being released.

[TS 24.582, clause 6.1.2.2.1]

Upon receiving an indication to establish MSRP connection for SDS session as the originating MCData client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData SDS message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.2.6.

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client can generate and send an SDS message as specified in subclause 6.1.2.4, or can generate and send an SDS disposition notification for a received SDS message as specified in subclause 6.1.2.5, if requested.

Received content and disposition requests shall be handled as specified in subclause 6.1.2.6.

[TS 24.582, clause 6.1.2.4]

An MCData client is allowed to send an one-to-one SDS message only if

1.	the <allow-transmit-data> element of an <actions> element is present with a value "true" (see the MCData user profile document in 3GPP TS 24.484 [7]);

2.	the size of the SDS message is less than or equal to the value of the <max-data-size-sds-bytes> element in the MCData service configuration document as specified in 3GPP TS 24.484 [7]; and

3.	the size of the SDS message is less than or equal to the value of <MaxData1To1> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [7]).

An MCData client is allowed to send a group SDS message only if

1.	the <mcdata-allow-transmit-data-in-this-group> element of an <action> element is present with a value "true" as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4];

2.	the size of the SDS message is less than or equal to the value contained in the <mcdata-on-network-max-data-size-for-SDS> as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4]; and

3.	the size of the SDS message is less than or equal to the value contained in the <mcdata-max-data-in-single-request> element of the <entry> element of the MCData group document for this MCData group as specified in 3GPP TS 24.481 [11].

If the above mentioned conditions satisfy, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

[TS 24.582, clause 6.1.2.5.1]

To send an SDS disposition notification, the MCData client:

1.	shall generate a SDS NOTIFICATION as specified in subclause 6.1.2.5.2;

2.	shall include the SDS NOTIFICATION in an MSRP SEND request as specified in subclause 6.1.2.5.3, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

3.	shall send the MSRP SEND request on the established MSRP connection.

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary.

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks; and

2.	shall indicate to MCData user that the SDS message or the SDS disposition notification could not be sent.

[TS 24.582, clause 6.1.2.5.2]

In order to generate an SDS notification, the MCData client:

1.	shall generate an SDS NOTIFICATION message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in 3GPP TS 24.282 [8].

When generating an SDS NOTIFICATION message, the MCData client:

1.	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED";

2.	if sending a read notification, shall set the SDS disposition notification type IE as "READ";

3.	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ";

4.	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED";

5.	shall set the Date and time IE to the current time;

6.	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message;

7.	shall set the Message ID to the value of the Message ID that was received in the SDS message;

8.	if the SDS message was destined for the user, shall not include an Application ID IE; and

9.	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message.

[TS 24.582, clause 6.1.2.5.3]

The MCData client shall generate MSRP SEND requests for SDS disposition notification according to IETF RFC 4975 [11].

When generating an MSRP SEND request for SDS disposition notification containing an SDS NOTIFICATION message, the MCData client

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the content type as Content-Type = "application/vnd.3gpp.mcdata-signalling"; and

3.	shall set the body of the MSRP SEND request to the generated SDS NOTIFICATION message.

[TS 24.582, clause 6.1.2.6]

Upon receiving an SDS message, the MCData client:

1.	shall follow the procedure defined in subclause 6.1.1.3.2, with the following clarification:

a.	if SDS Disposition request type IE is present in the received SDS SIGNALLING PAYLOAD message then, shall send an SDS disposition notification as described in subclause 6.1.2.5.

Upon receiving an SDS disposition notification, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2.	shall deliver the notification to the user or application.

6.1.9.3	Test description

6.1.9.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.9.3.2	Test procedure sequence

Table 6.1.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCData User request to send a one-to-one session SDS message with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData Client) perform the procedure for CO MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.2.3-1? | - | - | 1,2 | P |
| 3-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive the disposition notification for the SDS message sent at step 7? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCDATA Client) deliver the notification to the MCData User? (NOTE 1) | - | - | 3 | P |
| 11 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an SDS message with disposition notification type "READ"? | - | - | 4 | P |
| 12 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 13 the events described in Table 6.1.9.3.2-2 take place. (NOTE 2) | - | - | - | - |
| 13 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCDATA User? (NOTE 1) | - | - | 4 | P |
| 14-15 | Void | - | - | - | - |
| 16 | Make the MCDATA User request to release the one-to-one session SDS message over the media plane. (NOTE 1) | - | - | - | - |
| 17 | Check: Does the UE (MCData Client) perform the procedure for CO MCData call release specified in TS 36.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 18 | The SS releases the RRC connection | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The behaviour is handled through parallel actions to allow for implementations which first indicate to the user that there is a message available, but render the message to the user only after the user takes an action to open the message. |  |  |  |  |  |

Table 6.1.9.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send a disposition notification of "READ"? | - | - | 2 | P |

6.1.9.3.3	Specific message contents

Table 6.1.9.3.3-1: SIP INVITE (step 2, Table 6.1.9.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_SDS, MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | As described in Table 6.1.9.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.9.3.3-3 |  |  |  |

Table 6.1.9.3.3-2: SDP for SIP INVITE (Table 6.1.9.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_OFFER, SDS_SESSION, MCD_1to1

Table 6.1.9.3.3-3: MCDATA-Info (Table 6.1.9.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-sds-session" |  |  |  |

Table 6.1.9.3.3-4: SIP 200 (OK) (step 2, Table 6.1.9.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.5.9.3-5 |  |  |  |  |  |  |  |

Table 6.1.9.3.3-5: SDP for SIP 200 (OK) (Table 6.1.9.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.9.3.3-6: MSRP SEND (step 7, Table 6.1.9.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.9.3.3-6A |  |  |  |

Table 6.1.9.3.3-6A: MIME Message (step 7, Table 6.1.9.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.9.3.3-6B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | DATA PAYLOAD as described in Table 6.1.9.3.3-7 |  |  |  |  |  |  |  |

Table 6.1.9.3.3-6B: SDS SIGNALLING PAYLOAD (Table 6.1.9.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.9.3.3-7: Data Payload (Table 6.1.9.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-1

Table 6.1.9.3.3-8..9: Void

Table 6.1.9.3.3-10: MSRP SEND (step 8, Table 6.1.9.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.9.3.3-11 |  |  |  |

Table 6.1.9.3.3-11: SDS NOTIFICATION (Table 6.1.9.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.9.3.3-12: MSRP SEND (step 11, Table 6.1.9.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message as specified in table 6.1.9.3.3-12A |  |  |  |

Table 6.1.9.3.3-12A: MIME Message (step 11, Table 6.1.9.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.9.3.3-13 |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | DATA PAYLOAD as described in Table 6.1.9.3.3-13A |  |  |  |  |  |  |  |

Table 6.1.9.3.3-13: SDS SIGNALLING PAYLOAD (Table 6.1.9.3.3-12A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.9.3.3-13A: Data Payload (Table 6.1.9.3.3-12A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-2

Table 6.1.9.3.3-14: MSRP SEND (step 1, Table 6.1.9.3.2-2;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.9.3.3-15 |  |  |  |

Table 6.1.9.3.3-15: SDS NOTIFICATION (Table 6.1.9.3.3-14)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.9.3.3-16: SIP BYE (step 17, Table 6.1.9.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.6.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1, condition MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.9.3.3-17: Void

### 6.1.10	On-network / Short Data Service (SDS) / SDS Session / One-to-one SDS Session / Client Terminated (CT)

6.1.10.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a one-to-one SDS session using the media plane }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having responded to the SIP INVITE message that initiated a one-to-one SDS session using the media plane }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message }

then { UE (MCDATA Client) responds with an MSRP 200 (OK) message and if the MSRP SEND message is not blank, renders the contents of the Payload IE to the MCDATA User and sends a SIP MESSAGE message with a disposition notification of "DELIVERED" }

}

(3)

with { UE (MCDATA Client) being in a one-to-one SDS session initiated by the SS (MCDATA server) }

ensure that {

when { the MCDATA User requests to send a one-to-one SDS Session message with a disposition of "READ" }

then { UE (MCDATA Client) sends a one-to-one session SDS message via a MSRP SEND message with a disposition of "DELIVERY" }

}

(4)

with { UE (MCDATA Client) having sent a one-to-one SDS session message using the media plane with a disposition of "READ" }

ensure that {

when { UE (MCDATA Client) receives a disposition response via a MSRP SEND message }

then { UE (MCDATA Client) responds to the MSRP SEND message by sending a MSRP 200 (OK) message and delivers the notification to the MCDATA User }

}

(5)

with { UE (MCDATA Client) being in a one-to-one SDS session initiated by the SS (MCDATA server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

6.1.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.4.2.4, 9.2.4.2.2, 13.2.2.2.2.2, TS 24.582 clauses 6.1.2.3.1, 6.1.2.3.1, 6.1.2.4, 6.1.2.5.1, 6.1.2.5.2, 6.1.2.5.3, 6.1.2.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.4.2.4]

Upon receipt of an initial SIP INVITE request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps after step 2;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in subclause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

3)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [45], and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to create an end-to-end secure session.

4)	may display to the MCData user the MCData ID of the inviting MCData user and the type of SDS request;

5)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

6)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

7)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

8)	shall include the g.3gpp.mcdata.sds media feature tag in the Contact header field of the SIP 200 (OK) response;

9)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP 200 (OK) response;

10)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.4.2.2; and

11)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client shall:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.2.3.

To send a disposition notification after the media plane is released, the MCData client:

1)	shall follow the procedures described in subclause 12.2.1.1.

[TS 24.282, clause 9.2.4.2.2]

When the MCData client receives an initial SDP offer for an MCData SDS session, the MCData client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [5] and IETF RFC 4975 [17].

When composing an SDP answer, the MCData client:

1)	shall include an "m=message" media-level section for the accepted MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP" or "TCP/TLS/MSRP" for TLS according to the received SDP offer;

c)	an "a=sendrecv" attribute;

d)	an "a=path" attribute containing its own MSRP URI;

e)	set the content type as a=accept-types: application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload; and

f)	set the a=setup attribute according to IETF RFC 6135 [19].

[TS 24.282, clause 13.2.2.2.2.2]

Upon receiving a SIP BYE request, the MCData client:

1)	shall send SIP 200 (OK) response towards MCData server according to 3GPP TS 24.229 [5]; and

2)	shall release all media plane resources corresponding to the MCData communication being released.

NOTE:	Partially received data can be stored and processed.

[TS 24.582, clause 6.1.2.3.1]

Upon receiving an indication to establish MSRP connection for SDS session as the terminating MCData client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData SDS message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.2.6.

On receiving MSRP 200 (OK) response to the first MSRP SEND request sent as "active" endpoint, or after sending MSRP 200 (OK) response to the first MSRP SEND request received as "passive" endpoint, the MCData client can generate and send an SDS message as specified in subclause 6.1.2.4, or can generate and send an SDS disposition notification for a received SDS message as specified in subclause 6.1.2.5, if requested.

Received content and disposition requests shall be handled as specified in subclause 6.1.2.6.

[TS 24.582, clause 6.1.2.4]

An MCData client is allowed to send an one-to-one SDS message only if

1.	the <allow-transmit-data> element of an <actions> element is present with a value "true" (see the MCData user profile document in 3GPP TS 24.484 [7]);

2.	the size of the SDS message is less than or equal to the value of the <max-data-size-sds-bytes> element in the MCData service configuration document as specified in 3GPP TS 24.484 [7]; and

3.	the size of the SDS message is less than or equal to the value of <MaxData1To1> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [7]).

An MCData client is allowed to send a group SDS message only if

1.	the <mcdata-allow-transmit-data-in-this-group> element of an <action> element is present with a value "true" as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4];

2.	the size of the SDS message is less than or equal to the value contained in the <mcdata-on-network-max-data-size-for-SDS> as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4]; and

3.	the size of the SDS message is less than or equal to the value contained in the <mcdata-max-data-in-single-request> element of the <entry> element of the MCData group document for this MCData group as specified in 3GPP TS 24.481 [11].

If the above mentioned conditions satisfy, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

[TS 24.582, clause 6.1.2.5.1]

To send an SDS disposition notification, the MCData client:

1.	shall generate a SDS NOTIFICATION as specified in subclause 6.1.2.5.2;

2.	shall include the SDS NOTIFICATION in an MSRP SEND request as specified in subclause 6.1.2.5.3, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

3.	shall send the MSRP SEND request on the established MSRP connection.

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary.

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks; and

2.	shall indicate to MCData user that the SDS message or the SDS disposition notification could not be sent.

[TS 24.582, clause 6.1.2.5.2]

In order to generate an SDS notification, the MCData client:

1.	shall generate an SDS NOTIFICATION message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in 3GPP TS 24.282 [8].

When generating an SDS NOTIFICATION message, the MCData client:

1.	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED";

2.	if sending a read notification, shall set the SDS disposition notification type IE as "READ";

3.	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ";

4.	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED";

5.	shall set the Date and time IE to the current time;

6.	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message;

7.	shall set the Message ID to the value of the Message ID that was received in the SDS message;

8.	if the SDS message was destined for the user, shall not include an Application ID IE; and

9.	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message.

[TS 24.582, clause 6.1.2.5.3]

The MCData client shall generate MSRP SEND requests for SDS disposition notification according to IETF RFC 4975 [11].

When generating an MSRP SEND request for SDS disposition notification containing an SDS NOTIFICATION message, the MCData client

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the content type as Content-Type = "application/vnd.3gpp.mcdata-signalling"; and

3.	shall set the body of the MSRP SEND request to the generated SDS NOTIFICATION message.

[TS 24.582, clause 6.1.2.6]

Upon receiving an SDS message, the MCData client:

1.	shall follow the procedure defined in subclause 6.1.1.3.2, with the following clarification:

a.	if SDS Disposition request type IE is present in the received SDS SIGNALLING PAYLOAD message then, shall send an SDS disposition notification as described in subclause 6.1.2.5.

Upon receiving an SDS disposition notification, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2.	shall deliver the notification to the user or application.

6.1.10.3	Test description

6.1.10.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.10.3.2	Test procedure sequence

Table 6.1.10.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData Client) perform the procedure for CT MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| 2-5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send a disposition notification of "DELIVERED"? | - | - | 2 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCData user? (NOTE 1) | - | - | 2 | P |
| 11 | Make the MCDATA User request to send a one-to-one session SDS message over the media plane with a disposition notification type of "READ". (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition notification type "READ"? | - | - | 3 | P |
| 13 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive the disposition notification for the SDS message sent at step 12? | - | - | 4 | P |
| 14 | Void | - | - | - | - |
| 15 | Check: Does the UE (MCDATA Client) deliver the notification to the MCDATA User? (NOTE 1) | - | - | 4 | P |
| 16 | Check: Does the UE (MCData Client) perform the procedure for CT MCData call release specified in TS 36.579-1 [2] Table 5.3C.7.3-1? | - | - | 5 | P |
| 17 | The SS releases the RRC connection | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.10.3.3	Specific message contents

Table 6.1.10.3.3-1: SIP INVITE (step 1, Table 6.1.10.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.1.10.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.10.3.3-3 |  |  |  |

Table 6.1.10.3.3-2: SDP for SIP INVITE (Table 6.1.10.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_OFFER, MCD_1to1, SDS_SESSION

Table 6.1.10.3.3-3: MCDATA-Info (Table 6.1.10.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-sds-session" |  |  |  |

Table 6.1.10.3.3-4: SIP 200 (OK) (step 1, Table 6.1.10.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.10.3.3-5 |  |  |  |  |  |  |  |

Table 6.1.10.3.3-5: SDP for SIP 200 (OK) (Table 6.1.10.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.10.3.3-6: MSRP SEND (step 6, Table 6.1.10.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message as specified in table 6.1.10.3.3-6A |  |  |  |

Table 6.1.10.3.3-6A: MIME Message (step 6, Table 6.1.10.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.10.3.3-6B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | DATA PAYLOAD as described in Table 6.1.10.3.3-7 |  |  |  |  |  |  |  |

Table 6.1.10.3.3-6B: SDS SIGNALLING PAYLOAD (Table 6.1.10.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.10.3.3-7: Data Payload (Table 6.1.10.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-2

Table 6.1.10.3.3-8: MSRP SEND (step 8, Table 6.1.10.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.10.3.3-9 |  |  |  |

Table 6.1.10.3.3-9: SDS NOTIFICATION (Table 6.1.10.3.3-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.10.3.3-10: MSRP SEND (step 12, Table 6.1.10.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.10.3.3-10A |  |  |  |

Table 6.1.10.3.3-10A: MIME Message (step 12, Table 6.1.10.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.10.3.3-11 |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | DATA PAYLOAD as described in Table 6.1.10.3.3-12 |  |  |  |  |  |  |  |

Table 6.1.10.3.3-11: SDS SIGNALLING PAYLOAD (Table 6.1.10.3.3-10A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.10.3.3-12: Data Payload (Table 6.1.10.3.3-10A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-1

Table 6.1.10.3.3-13..14: Void

Table 6.1.10.3.3-15: MSRP SEND (step 13, Table 6.1.10.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.10.3.3-16 |  |  |  |

Table 6.1.10.3.3-16: SDS NOTIFICATION (Table 6.1.10.3.3-15)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.10.3.3-17: SIP BYE (step 16, Table 6.1.10.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1, condition MT_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.10.3.3-18: Void

### 6.1.11	On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Client Originated (CO)

6.1.11.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to initiate a group SDS session using the media plane}

then { UE (MCDATA Client) sends a request to establish a group SDS session and a MSRP connection via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message }

}

(2)

with { UE (MCDATA Client) having received a SIP 200 (OK) message with the a=setup attribute set to "passive" }

ensure that {

when { UE (MCDATA Client) responds to the SIP 200 (OK) message with a SIP ACK message }

then { UE (MCDATA Client)  sends a blank MSRP SEND message to bind the MSRP connection and then sends the group session SDS message via a MSRP SEND message with a disposition of "DELIVERY" }

}

(3)

with { UE (MCDATA Client) having sent a group session SDS message using the media plane with a disposition of "DELIVERY" }

ensure that {

when { UE (MCDATA Client) receives a disposition response via a MSRP SEND message }

then { UE (MCDATA Client) responds to the MSRP SEND message by sending a MSRP 200 (OK) message and delivers the notification to the MCDATA User }

}

(4)

with { UE (MCDATA Client) having established a group SDS session and a MSRP connection }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message with a disposition of "DELIVERY AND READ" }

then { UE (MCDATA Client) responds with a MSRP 200 (OK) message }

}

(5)

with { UE (MCDATA Client) having responded with a MSRP 200(OK) message to a group SDS session message with a disposition of "DELIVERY AND READ" and timer TDU1 (delivery and read) not yet expired }

ensure that {

when { the UE (MCDATA Client) determines that the payload contained in the DATA PAYLOAD message is for user consumption and before timer TDU1 (delivery and read) expires }

then { the UE (MCDATA Client) renders the contents of the Payload IE to the MCDATA User and then sends a MSRP SEND message with a disposition notification of "DELIVERED AND READ" }

}

(6)

with { UE (MCDATA Client) having responded with a MSRP 200(OK) message to a group SDS session message with a disposition of "DELIVERY AND READ" and the UE (MCDATA Client) not yet rendering the contents of the Payload IE to the MCDATA User }

ensure that {

when { timer TDU1 (delivery and read) expires }

then { UE (MCDATA Client) sends a MSRP SEND message with a disposition notification of "DELIVERED" and then renders the contents of the Payload IE to the MCDATA User and then sends a MSRP SEND message with a disposition notification of "READ" }

}

(7)

with { UE (MCDATA Client) having established a group SDS session }

ensure that {

when { the MCDATA User requests to release the group SDS session }

then { UE (MCDATA Client) sends a SIP BYE message }

}

6.1.11.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.4.2.3, 9.2.4.2.1, 13.2.2.2.2.1, TS 24.582 clauses 6.1.2.2.1, 6.1.2.3.1, 6.1.2.4, 6.1.2.5.1, 6.1.2.5.2, 6.1.2.5.3, 6.1.2.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.4.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.sds media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.sds media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

…

8)	if a group SDS session is requested:

a)	if the "/<x>/<x>/Common/MCData/AllowedSDS" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request to send SDS and not continue with the rest of the steps in this subclause; and

b)	shall contain in an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-sds-session";

ii)	the <mcdata-request-uri> element set to the MCData group identity; and

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

NOTE 1:	The MCData client does not include the MCData ID of the originating MCData user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCData function.

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.4.2.1; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.2.2.

[TS 24.282, clause 9.2.4.2.1]

When composing an SDP offer according to 3GPP TS 24.229 [5], IETF RFC 4975 [17], IETF RFC 6135 [19] and IETF RFC 6714 [20] the MCData client:

1)	shall include an "m=message" media-level section for the MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP" or "TCP/TLS/MSRP" for TLS;

c)	an "a=sendrecv" attribute;

d)	an "a=path" attribute containing its own MSRP URI;

e)	set the content type as "a=accept-types:application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload"; and

f)	set the a=setup attribute as "actpass"; and

2)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [45].

[TS 24.282, clause 13.2.2.2.2.1]

When the MCData client wants to release a MCData communication established over the media plane, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5];

2)	shall set the Request-URI to the MCData session identity to be released; and

3)	shall send the SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCData client shall release all media plane resources corresponding to the MCData communication being released.

[TS 24.582, clause 6.1.2.2.1]

Upon receiving an indication to establish MSRP connection for SDS session as the originating MCData client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData SDS message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.2.6.

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client can generate and send an SDS message as specified in subclause 6.1.2.4, or can generate and send an SDS disposition notification for a received SDS message as specified in subclause 6.1.2.5, if requested.

Received content and disposition requests shall be handled as specified in subclause 6.1.2.6.

[TS 24.582, clause 6.1.2.3.1]

Upon receiving an indication to establish MSRP connection for SDS session as the terminating MCData client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData SDS message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.2.6.

On receiving MSRP 200 (OK) response to the first MSRP SEND request sent as "active" endpoint, or after sending MSRP 200 (OK) response to the first MSRP SEND request received as "passive" endpoint, the MCData client can generate and send an SDS message as specified in subclause 6.1.2.4, or can generate and send an SDS disposition notification for a received SDS message as specified in subclause 6.1.2.5, if requested.

Received content and disposition requests shall be handled as specified in subclause 6.1.2.6.

[TS 24.582, clause 6.1.2.4]

An MCData client is allowed to send an one-to-one SDS message only if

1.	the <allow-transmit-data> element of an <actions> element is present with a value "true" (see the MCData user profile document in 3GPP TS 24.484 [7]);

2.	the size of the SDS message is less than or equal to the value of the <max-data-size-sds-bytes> element in the MCData service configuration document as specified in 3GPP TS 24.484 [7]; and

3.	the size of the SDS message is less than or equal to the value of <MaxData1To1> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [7]).

An MCData client is allowed to send a group SDS message only if

1.	the <mcdata-allow-transmit-data-in-this-group> element of an <action> element is present with a value "true" as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4];

2.	the size of the SDS message is less than or equal to the value contained in the <mcdata-on-network-max-data-size-for-SDS> as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4]; and

3.	the size of the SDS message is less than or equal to the value contained in the <mcdata-max-data-in-single-request> element of the <entry> element of the MCData group document for this MCData group as specified in 3GPP TS 24.481 [11].

If the above mentioned conditions satisfy, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

[TS 24.582, clause 6.1.2.5.1]

To send an SDS disposition notification, the MCData client:

1.	shall generate a SDS NOTIFICATION as specified in subclause 6.1.2.5.2;

2.	shall include the SDS NOTIFICATION in an MSRP SEND request as specified in subclause 6.1.2.5.3, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

3.	shall send the MSRP SEND request on the established MSRP connection.

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary.

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks; and

2.	shall indicate to MCData user that the SDS message or the SDS disposition notification could not be sent.

[TS 24.582, clause 6.1.2.5.2]

In order to generate an SDS notification, the MCData client:

1.	shall generate an SDS NOTIFICATION message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in 3GPP TS 24.282 [8].

When generating an SDS NOTIFICATION message, the MCData client:

1.	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED";

2.	if sending a read notification, shall set the SDS disposition notification type IE as "READ";

3.	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ";

4.	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED";

5.	shall set the Date and time IE to the current time;

6.	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message;

7.	shall set the Message ID to the value of the Message ID that was received in the SDS message;

8.	if the SDS message was destined for the user, shall not include an Application ID IE; and

9.	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message.

[TS 24.582, clause 6.1.2.6]

Upon receiving an SDS message, the MCData client:

1.	shall follow the procedure defined in subclause 6.1.1.3.2, with the following clarification:

a.	if SDS Disposition request type IE is present in the received SDS SIGNALLING PAYLOAD message then, shall send an SDS disposition notification as described in subclause 6.1.2.5.

Upon receiving an SDS disposition notification, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2.	shall deliver the notification to the user or application.

6.1.11.3	Test description

6.1.11.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.11.3.2	Test procedure sequence

Table 6.1.11.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCData User request to send a group session SDS message with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData Client) perform the procedure for CO MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.2.3-1? | - | - | 1,2 | P |
| 3-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive the disposition notification for the SDS message sent at step 7? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCDATA Client) deliver the notification to the MCData User? (NOTE 1) | - | - | 3 | P |
| 11 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an SDS message with disposition notification type "DELIVERY AND READ"? (NOTE 3) | - | - | 4 | P |
| 12 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 13 the events described in Table 6.1.11.3.2-2 take place. (NOTE 2) | - | - | - | - |
| 13 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCData User? (NOTE 1) | - | - | 4 | P |
| 14 | Make the MCDATA User request to release the group session SDS message over the media plane. (NOTE 1) | - | - | - | - |
| 15 | Check: Does the UE (MCData Client) perform the procedure for CO MCData call release specified in TS 36.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 16 | The SS releases the RRC connection | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The behaviour is handled through parallel actions to allow for implementations which first indicate to the user that there is a message available, but render the message to the user only after the user takes an action to open the message. NOTE 3: Timer TDU1 (delivery and read) is started upon receipt of the SIP MESSAGE message that contains a "DELIVERY AND READ" disposition request. Timer TDU1 (delivery and read)=120ms according to the default value defined in TS 24.282 [31]. |  |  |  |  |  |

Table 6.1.11.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1b2 describe behaviour that depends on the timing of UE execution; the "lower case letter" identifies a step sequence that takes place if the UE receives a display indication before (step 1a1) or after (steps 1b1-1b2) timer TDU1 (delivery and read) expires. | - | - | - | - |
| 1a1 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send a disposition notification of "DELIVERED AND READ"? | - | - | 3 | P |
| 1b1 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send a disposition notification of "DELIVERED"? | - | - | 3 | P |
| 1b2 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send a disposition notification of "READ"? | - | - | 3 | P |

6.1.11.3.3	Specific message contents

Table 6.1.11.3.3-1: SIP INVITE (step 2, Table 6.1.11.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | As described in Table 6.1.11.3.3-1A |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.11.3.3-2 |  |  |  |

Table 6.1.11.3.3-1A: SDP for SIP INVITE (Table 6.1.11.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_OFFER, SDS_SESSION

Table 6.1.11.3.3-2: MCDATA-Info (Table 6.1.11.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-sds-session" |  |  |  |

Table 6.1.11.3.3-3: SIP 200 (OK) (step 2, Table 6.1.11.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.11.3.3-4 |  |  |  |  |  |  |  |

Table 6.1.11.3.3-4: SDP for SIP 200 (OK) (Table 6.1.11.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-5: MSRP SEND (step 7, Table 6.1.11.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.11.3.3-5A |  |  |  |

Table 6.1.11.3.3-5A: MIME Message (step 7, Table 6.1.11.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.11.3.3-5B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.11.3.3-6 |  |  |  |  |  |  |  |

Table 6.1.11.3.3-5B: SDS SIGNALLING PAYLOAD (Table 6.1.11.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.11.3.3-6: Data Payload (Table 6.1.11.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-1

Table 6.1.11.3.3-7..8: Void

Table 6.1.11.3.3-9: MSRP SEND (step 8, Table 6.1.11.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.11.3.3-10 |  |  |  |

Table 6.1.11.3.3-10: SDS NOTIFICATION (Table 6.1.11.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-11: MSRP SEND (step 11, Table 6.1.11.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message as specified in table 6.1.11.3.3-11A |  |  |  |

Table 6.1.11.3.3-11A: MIME Message (step 11, Table 6.1.11.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.11.3.3-12 |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.11.3.3-12A |  |  |  |  |  |  |  |

Table 6.1.11.3.3-12: SDS SIGNALLING PAYLOAD (Table 6.1.11.3.3-11A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED_READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-12A: Data Payload (Table 6.1.11.3.3-11A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-2

Table 6.1.11.3.3-13: MSRP SEND (step 1a1, Table 6.1.11.3.2-2;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.11.3.3-14 |  |  |  |

Table 6.1.11.3.3-14: SDS NOTIFICATION (Table 6.1.11.3.3-13)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED_READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-15: MSRP SEND (step 1b1, Table 6.1.11.3.2-2;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.11.3.3-16 |  |  |  |

Table 6.1.11.3.3-16: SDS NOTIFICATION (Table 6.1.11.3.3-15)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-17: MSRP SEND (step 1b2, Table 6.1.11.3.2-2;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.11.3.3-18 |  |  |  |

Table 6.1.11.3.3-18: SDS NOTIFICATION (Table 6.1.11.3.3-17)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-19: SIP BYE (step 15, Table 6.1.5.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.6.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1, condition MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.11.3.3-20: Void

### 6.1.12	On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Client Terminated (CT)

6.1.12.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a group SDS session using the media plane }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having responded to the SIP INVITE message that initiated a group SDS session using the media plane }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message }

then { UE (MCDATA Client) responds with a MSRP 200 (OK) message and if the MSRP SEND message is not blank, renders the contents of the Payload IE to the MCDATA User and sends a MSRP SEND message with a disposition notification of "DELIVERED" }

}

(3)

with { UE (MCDATA Client) being in a group SDS session initiated by the SS (MCDATA server) }

ensure that {

when { the MCDATA User requests to send a group SDS Session message with a disposition of "DELIVERY AND READ" }

then { UE (MCDATA Client) sends a group session SDS message via a MSRP SEND message with a disposition of "DELIVERY AND READ" }

}

(4)

with { UE (MCDATA Client) having sent a group SDS session message using the media plane with a disposition of "DELIVERY AND READ" }

ensure that {

when { UE (MCDATA Client) receives a disposition response via a MSRP SEND message }

then { UE (MCDATA Client) responds to the MSRP SEND message by sending a MSRP 200 (OK) message and delivers the notification to the MCDATA User }

}

(5)

with { UE (MCDATA Client) being in a group SDS session initiated by the SS (MCDATA Server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

6.1.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.4.2.4, 9.2.4.2.2, 13.2.2.2.2.2, TS 24.582 clauses 6.1.2.3.1, 6.1.2.3.1, 6.1.2.4, 6.1.2.5.1, 6.1.2.5.2, 6.1.2.5.3, 6.1.2.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 9.2.4.2.4]

Upon receipt of an initial SIP INVITE request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps after step 2;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in subclause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

3)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [45], and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to create an end-to-end secure session.

4)	may display to the MCData user the MCData ID of the inviting MCData user and the type of SDS request;

5)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

6)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

7)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

8)	shall include the g.3gpp.mcdata.sds media feature tag in the Contact header field of the SIP 200 (OK) response;

9)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" in the Contact header field of the SIP 200 (OK) response;

10)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in subclause 9.2.4.2.2; and

11)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client shall:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 6.1.2.3.

To send a disposition notification after the media plane is released, the MCData client:

1)	shall follow the procedures described in subclause 12.2.1.1.

[TS 24.282, clause 9.2.4.2.2]

When the MCData client receives an initial SDP offer for an MCData SDS session, the MCData client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [5] and IETF RFC 4975 [17].

When composing an SDP answer, the MCData client:

1)	shall include an "m=message" media-level section for the accepted MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP" or "TCP/TLS/MSRP" for TLS according to the received SDP offer;

c)	an "a=sendrecv" attribute;

d)	an "a=path" attribute containing its own MSRP URI;

e)	set the content type as a=accept-types: application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload; and

f)	set the a=setup attribute according to IETF RFC 6135 [19].

[TS 24.282, clause 13.2.2.2.2.2]

Upon receiving a SIP BYE request, the MCData client:

1)	shall send SIP 200 (OK) response towards MCData server according to 3GPP TS 24.229 [5]; and

2)	shall release all media plane resources corresponding to the MCData communication being released.

NOTE:	Partially received data can be stored and processed.

[TS 24.582, clause 6.1.2.3.1]

Upon receiving an indication to establish MSRP connection for SDS session as the terminating MCData client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData SDS message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.2.6.

On receiving MSRP 200 (OK) response to the first MSRP SEND request sent as "active" endpoint, or after sending MSRP 200 (OK) response to the first MSRP SEND request received as "passive" endpoint, the MCData client can generate and send an SDS message as specified in subclause 6.1.2.4, or can generate and send an SDS disposition notification for a received SDS message as specified in subclause 6.1.2.5, if requested.

Received content and disposition requests shall be handled as specified in subclause 6.1.2.6.

[TS 24.582, clause 6.1.2.4]

An MCData client is allowed to send an one-to-one SDS message only if

1.	the <allow-transmit-data> element of an <actions> element is present with a value "true" (see the MCData user profile document in 3GPP TS 24.484 [7]);

2.	the size of the SDS message is less than or equal to the value of the <max-data-size-sds-bytes> element in the MCData service configuration document as specified in 3GPP TS 24.484 [7]; and

3.	the size of the SDS message is less than or equal to the value of <MaxData1To1> element of the MCData user profile document (see the MCData user profile document in 3GPP TS 24.484 [7]).

An MCData client is allowed to send a group SDS message only if

1.	the <mcdata-allow-transmit-data-in-this-group> element of an <action> element is present with a value "true" as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4];

2.	the size of the SDS message is less than or equal to the value contained in the <mcdata-on-network-max-data-size-for-SDS> as defined in the MCData group document for this MCData group as specified in 3GPP TS 24.481 [4]; and

3.	the size of the SDS message is less than or equal to the value contained in the <mcdata-max-data-in-single-request> element of the <entry> element of the MCData group document for this MCData group as specified in 3GPP TS 24.481 [11].

If the above mentioned conditions satisfy, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

[TS 24.582, clause 6.1.2.5.1]

To send an SDS disposition notification, the MCData client:

1.	shall generate a SDS NOTIFICATION as specified in subclause 6.1.2.5.2;

2.	shall include the SDS NOTIFICATION in an MSRP SEND request as specified in subclause 6.1.2.5.3, with the following clarification;

a.	shall set To-Path header according to the MSRP URI in the received SDP; and

3.	shall send the MSRP SEND request on the established MSRP connection.

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary.

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks; and

2.	shall indicate to MCData user that the SDS message or the SDS disposition notification could not be sent.

[TS 24.582, clause 6.1.2.5.2]

In order to generate an SDS notification, the MCData client:

1.	shall generate an SDS NOTIFICATION message as specified in 3GPP TS 24.282 [8]; and

2.	shall include the SDS NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in 3GPP TS 24.282 [8].

When generating an SDS NOTIFICATION message, the MCData client:

1.	if sending a delivered notification, shall set the SDS disposition notification type IE as "DELIVERED";

2.	if sending a read notification, shall set the SDS disposition notification type IE as "READ";

3.	if sending a delivered and read notification, shall set the SDS disposition notification type IE as "DELIVERED AND READ";

4.	if the SDS message could not be delivered to the user or application (e.g. due to lack of storage), shall set the SDS disposition notification type IE as "UNDELIVERED";

5.	shall set the Date and time IE to the current time;

6.	shall set the Conversation ID to the value of the Conversation ID that was received in the SDS message;

7.	shall set the Message ID to the value of the Message ID that was received in the SDS message;

8.	if the SDS message was destined for the user, shall not include an Application ID IE; and

9.	if the SDS message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the SDS message.

[TS 24.582, clause 6.1.2.5.3]

The MCData client shall generate MSRP SEND requests for SDS disposition notification according to IETF RFC 4975 [11].

When generating an MSRP SEND request for SDS disposition notification containing an SDS NOTIFICATION message, the MCData client

1.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

2.	shall set the content type as Content-Type = "application/vnd.3gpp.mcdata-signalling"; and

3.	shall set the body of the MSRP SEND request to the generated SDS NOTIFICATION message.

[TS 24.582, clause 6.1.2.6]

Upon receiving an SDS message, the MCData client:

1.	shall follow the procedure defined in subclause 6.1.1.3.2, with the following clarification:

a.	if SDS Disposition request type IE is present in the received SDS SIGNALLING PAYLOAD message then, shall send an SDS disposition notification as described in subclause 6.1.2.5.

Upon receiving an SDS disposition notification, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2.	shall deliver the notification to the user or application.

6.1.12.3	Test description

6.1.12.3.1	Pre-test conditions

System Simulator:

-	SS (MCDATA Server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCDATA Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCDATA UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCDATA User performs the Generic Test Procedure for MCDATA Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2B.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCDATA Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.12.3.2	Test procedure sequence

Table 6.1.12.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData Client) perform the procedure for CT MCData Call Establishment specified in TS 36.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| 2-5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an SDS message with disposition notification type "DELIVERY"? | - | - | 2 | P |
| 7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send a disposition notification of "DELIVERY"? | - | - | 2 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCDATA Client) render the contents of the Payload IE to the MCDATA User? (NOTE 1) | - | - | 2 | P |
| 11 | Make the MCDATA User request to send a group session SDS message over the media plane with a disposition notification type of "DELIVERY AND READ". (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCData Client) perform the procedure for CO MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition notification type "DELIVERY AND READ"? | - | - | 3 | P |
| 13 | Check: Does the UE (MCData Client) perform the procedure for CT MSRP message transfer specified in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive the disposition notification for the SDS message sent at step 12? | - | - | 4 | P |
| 14 | Void | - | - | - | - |
| 15 | Check: Does the UE (MCDATA Client) deliver the notification to the MCData User? (NOTE 1) | - | - | 4 | P |
| 16 | Check: Does the UE (MCData Client) perform the procedure for CT MCData call release specified in TS 36.579-1 [2] Table 5.3C.7.3-1? | - | - | 5 | P |
| 17 | The SS releases the RRC connection | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.12.3.3	Specific message contents

Table 6.1.12.3.3-1: SIP INVITE (step 1, Table 6.1.12.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.1.12.3.3-1A |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.1.12.3.3-2 |  |  |  |

Table 6.1.12.3.3-1A: SDP for SIP INVITE (Table 6.1.12.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_OFFER, SDS_SESSION

Table 6.1.12.3.3-2: MCDATA-Info (Table 6.1.12.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-sds-session" |  |  |  |

Table 6.1.12.3.3-3: SIP 200 (OK) (step 1, Table 6.1.12.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message |  | As described in Table 6.1.12.3.3-4 |  |  |  |  |  |  |  |

Table 6.1.12.3.3-4: SDP for SIP 200 (OK) (Table 6.1.12.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.12.3.3-5: MSRP SEND (step 6, Table 6.1.12.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message as specified in table 6.1.12.3.3-5A |  |  |  |

Table 6.1.12.3.3-5A: MIME Message (step 6, Table 6.1.12.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.12.3.3-5B |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.12.3.3-6 |  |  |  |  |  |  |  |

Table 6.1.12.3.3-5B: SDS SIGNALLING PAYLOAD (Table 6.1.12.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.12.3.3-6: Data Payload (Table 6.1.12.3.3-5A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-2

Table 6.1.12.3.3-7: MSRP SEND (step 8, Table 6.1.12.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.12.3.3-8 |  |  |  |

Table 6.1.12.3.3-8: SDS NOTIFICATION (Table 6.1.12.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.3-1, condition DELIVERED |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.12.3.3-9: MSRP SEND (step 12, Table 6.1.12.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.12.3.3-9A |  |  |  |

Table 6.1.12.3.3-9A: MIME Message (step 12, Table 6.1.12.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-signalling" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.12.3.3-10 |  |  |  |  |  |  |  |
| MIME body part |  |  |  | MCData Data message |  |  |  |  |  |
| MIME-part-headers |  |  |  |  |  |  |  |  |  |
| Content-Type |  | "application/vnd.3gpp.mcdata-payload" |  |  |  |  |  |  |  |
| MIME-part-body |  | MCData Protected Payload Message containing DATA PAYLOAD as described in Table 6.1.12.3.3-11 |  |  |  |  |  |  |  |

Table 6.1.12.3.3-10: SDS SIGNALLING PAYLOAD (Table 6.1.12.3.3-9A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED_READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.12.3.3-11: Data Payload (Table 6.1.12.3.3-9A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.1-1

Table 6.1.12.3.3-12..13: Void

Table 6.1.12.3.3-14: MSRP SEND (step 13, Table 6.1.12.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.12.3.3-15 |  |  |  |

Table 6.1.12.3.3-15: SDS NOTIFICATION (Table 6.1.12.3.3-14)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED_READ |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.12.3.3-16: SIP BYE (step 16, Table 6.1.12.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1, condition MT_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 6.1.12.3.3-17: Void