3GPP TSG-RAN5 Meeting #94-e	R5-220508

# Electronic Meeting, February 21 - March 4, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2202 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of protocol testing applicability for 3CC inter-band NR DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR-DC configurations with 2CC NR CA are needed for protocol testing. Currently is only NR-DC 2CC configurations marked as applciable for protocol testing in Table 4.3.1.3.2.1-1. NR-DC configurations with 2CC NR CA need to be applicable for protocol testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed NR-DC configurations DC_n1A-n258G, DC_n78A-n257G, DC_n78A-n258G and DC_n79A-n257G to be indicated as applicable for protocol testing, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Protocol testing for NR-DC with 2CC NR CA not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.3.2.1	NR-DC configurations between FR1 and FR2 (two bands)

Table 4.3.1.3.2.1-1: NR-DC configurations between FR1 and FR2 (two bands)

| NR-DC configuration | Uplink NR-DC configuration | NR FR1 downlink configuration | NR FR2 downlink configuration | NR FR1 uplink configuration | NR FR2 uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_n1A-n258A | DC_n1A-n258A | n1A | n258A | n1A | n258A | Yes |
| DC_n1A-n258G | DC_n1A-n258A | n1A | CA_n258G | n1A | n258A | Yes (NR CA 2CC) |
| DC_n1A-n258H | DC_n1A-n258A | n1A | CA_n258H | n1A | n258A | No |
| DC_n1A-n258I | DC_n1A-n258A | n1A | CA_n258I | n1A | n258A | No |
| DC_n1A-n258J | DC_n1A-n258A | n1A | CA_n258J | n1A | n258A | No |
| DC_n1A-n258K | DC_n1A-n258A | n1A | CA_n258K | n1A | n258A | No |
| DC_n1A-n258L | DC_n1A-n258A | n1A | CA_n258L | n1A | n258A | No |
| DC_n1A-n258M | DC_n1A-n258A | n1A | CA_n258M | n1A | n258A | No |
| DC_n78A-n257A | DC_n78A-n257A | n78A | n257A | n78A | n257A | Yes |
| DC_n78A-n257G | DC_n78A-n257A | n78A | CA_n257G | n78A | n257A | Yes (NR CA 2CC) |
| DC_n78A-n257H | DC_n78A-n257A | n78A | CA_n257H | n78A | n257A | No |
| DC_n78A-n257I | DC_n78A-n257A | n78A | CA_n257I | n78A | n257A | No |
| DC_n78A-n258A | DC_n78A-n258A | n78A | n258A | n78A | n258A | Yes |
| DC_n78A-n258G | DC_n78A-n258A | n78A | CA_n258G | n78A | n258A | Yes (NR CA 2CC) |
| DC_n78A-n258H | DC_n78A-n258A | n78A | CA_n258H | n78A | n258A | No |
| DC_n78A-n258I | DC_n78A-n258A | n78A | CA_n258I | n78A | n258A | No |
| DC_n78A-n258J | DC_n78A-n258A | n78A | CA_n258J | n78A | n258A | No |
| DC_n78A-n258K | DC_n78A-n258A | n78A | CA_n258K | n78A | n258A | No |
| DC_n78A-n258L | DC_n78A-n258A | n78A | CA_n258L | n78A | n258A | No |
| DC_n78A-n258M | DC_n78A-n258A | n78A | CA_n258M | n78A | n258A | No |
| DC_n79A-n257A | DC_n79A-n257A | n79A | n257A | n79A | n257A | Yes |
| DC_n79A-n257G | DC_n79A-n257A | n79A | CA_n257G | n79A | n257A | Yes (NR CA 2CC) |
| DC_n79A-n257H | DC_n79A-n257A | n79A | CA_n257H | n79A | n257A | No |
| DC_n79A-n257I | DC_n79A-n257A | n79A | CA_n257I | n79A | n257A | No |
| Note 1: Protocol testing is limited to NR-DC configurations with 2CC and NR-DC with NR CA 2CC. |  |  |  |  |  |  |