3GPP TSG-RAN5 Meeting #94-e	R5-220535

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0359 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test applicability to allow for support of limited GNSS combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas, Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI6_Test |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN5#93e agreed CR R5-217676 is modified the applicabilities of TS37.571-3 to remove the limitation of allowing only the one superset combination of GNSS constellations to be tested. The relevant TS37.571-1 test applicability need to be corrected accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test applicability to remove the limitation of allowing only the one superset combination of GNSS constellations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-GNSS test cases applicability will not align in different test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 5.2.2.1, 5.3.1, 5.4.1, 5.5.1, 5.6.1, 6.2.1.1, 6.2.2.1, 6.3.1, 6.4.1, 6.5.1, 6.6.1, 7.1.1.1, 7.1.2.1, 7.2.1, 7.3.1, 7.4.1, 7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

## 5.2	Sensitivity

### 5.2.1	Sensitivity Coarse Time Assistance

#### 5.2.1.1	Definition and applicability

Sensitivity with coarse time assistance is the minimum level of GPS satellite signals required for the UE to make an A-GPS position estimate to a specific accuracy and within a specific response time when the network only provides coarse time assistance.

The requirements and this test apply to all types of UTRA for the FDD UE that supports A-GPS.

#### 5.2.1.2	Minimum requirements

<Unchanged Sections Skipped>

### 5.2.2	Sensitivity Fine Time Assistance

#### 5.2.2.1	Definition and applicability

Sensitivity with fine time assistance is the minimum level of GPS satellite signals required for the UE to make an A-GPS position estimate to a specific accuracy and within a specific response time when the network provides fine time assistance in addition to coarse time assistance.

The requirements and this test apply to all types of UTRA for the FDD UE that supports only A-GPS and that is capable of providing an enhanced performance when the network provides Fine Time Assistance.

#### 5.2.2.2	Minimum requirements

<Unchanged Sections Skipped>

## 5.3	Nominal Accuracy

### 5.3.1	Definition and applicability

Nominal accuracy is the accuracy of the UE's A-GPS position estimate under ideal GPS signal conditions.

The requirements and this test apply to all types of UTRA for the FDD UE that supports A-GPS.

### 5.3.2	Minimum requirements

<Unchanged Sections Skipped>

## 5.4	Dynamic Range

### 5.4.1	Definition and applicability

Dynamic Range is the maximum difference in level of the GPS signals from a number of satellites that allows the UE to make an A-GPS position estimate with a specific accuracy and a specific response time.

The requirements and this test apply to all types of UTRA for the FDD UE that supports A-GPS.

### 5.4.2	Minimum requirements

<Unchanged Sections Skipped>

## 5.5	Multi-path Performance

### 5.5.1	Definition and applicability

Multi-path performance measures the accuracy and response time of the UE's A-GPS position estimate in a specific GPS signal multi-path environment.

The requirements and this test apply to all types of UTRA for the FDD UE that supports A-GPS.

### 5.5.2	Minimum requirements

<Unchanged Sections Skipped>

## 5.6	Moving Scenario and Periodic Update Performance

### 5.6.1	Definition and applicability

Moving scenario and periodic update performance measures the accuracy of the UE's A-GPS position estimates and the periodic update capability of the UE in a moving scenario.

The requirements and this test apply to all types of UTRA for the FDD UE that supports A-GPS.

### 5.6.2	Minimum requirements

<Unchanged Sections Skipped>

## 6.2	Sensitivity

### 6.2.1	Sensitivity Coarse Time Assistance

#### 6.2.1.1	Definition and applicability

Sensitivity with coarse time assistance is the minimum level of GNSS satellite signals required for the UE to make an A-GNSS position estimate to a specific accuracy and within a specific response time when the network only provides coarse time assistance.

The requirements and this test apply to all types of UTRA for the UE that supports A-GNSS.

This test case includes sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 6.2.1.1.

Table 6.2.1.1: Sub-Test Case Number Definition

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS |
| 2 | UE supporting A-Galileo only |
| 3 | UE supporting A-GPS and Modernized GPS |
| 4 | UE supporting A-GPS and A-GLONASS |
| 8 | UE supporting A-GPS and A-Galileo |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS |

#### 6.2.1.2	Minimum requirements

<Unchanged Sections Skipped>

### 6.2.2	Sensitivity Fine Time Assistance

#### 6.2.2.1	Definition and applicability

Sensitivity with fine time assistance is the minimum level of GNSS satellite signals required for the UE to make an A-GNSS position estimate to a specific accuracy and within a specific response time when the network provides fine time assistance in addition to coarse time assistance.

The requirements and this test apply to all types of UTRA for the UE that supports A-GNSS and that is capable of providing an enhanced performance when the network provides Fine Time Assistance.

This test case includes sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 6.2.2.1.

Table 6.2.2.1: Sub-Test Case Number Definition

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS |
| 2 | UE supporting A-Galileo |
| 3 | UE supporting A-GPS and Modernized GPS |
| 4 | UE supporting A-GPS and A-GLONASS |
| 8 | UE supporting A-GPS and A-Galileo |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS |

#### 6.2.2.2	Minimum requirements

<Unchanged Sections Skipped>

## 6.3	Nominal Accuracy

### 6.3.1	Definition and applicability

Nominal accuracy is the accuracy of the UE's A-GNSS position estimate under ideal GNSS signal conditions.

The requirements and this test apply to all types of UTRA for the UE that supports A-GNSS.

This test case includes sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 6.3.1.

Table 6.3.1: Sub-Test Case Number Definition

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS |
| 2 | UE supporting A-Galileo |
| 3 | UE supporting A-GPS and Modernized GPS |
| 4 | UE supporting A-GPS and A-GLONASS |
| 8 | UE supporting A-GPS and A-Galileo |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS |

### 6.3.2	Minimum requirements

<Unchanged Sections Skipped>

## 6.4	Dynamic Range

### 6.4.1	Definition and applicability

Dynamic Range is the maximum difference in level of the GNSS signals from a number of satellites that allows the UE to make an A-GNSS position estimate with a specific accuracy and a specific response time.

The requirements and this test apply to all types of UTRA for the UE that supports A-GNSS.

This test case includes sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 6.4.1.

Table 6.4.1: Sub-Test Case Number Definition

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS |
| 2 | UE supporting A-Galileo |
| 3 | UE supporting A-GPS and Modernized GPS |
| 4 | UE supporting A-GPS and A-GLONASS |
| 8 | UE supporting A-GPS and A-Galileo |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS |

### 6.4.2	Minimum requirements

<Unchanged Sections Skipped>

## 6.5	Multi-path Performance

### 6.5.1	Definition and applicability

Multi-path performance measures the accuracy and response time of the UE's A-GNSS position estimate in a specific GNSS signal multi-path environment.

The requirements and this test apply to all types of UTRA for the UE that supports A-GNSS.

This test case includes sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 6.5.1.

Table 6.5.1: Sub-Test Case Number Definition

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS |
| 2 | UE supporting A-Galileo |
| 3 | UE supporting A-GPS and Modernized GPS |
| 4 | UE supporting A-GPS and A-GLONASS |
| 8 | UE supporting A-GPS and A-Galileo |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS |

### 6.5.2	Minimum requirements

<Unchanged Sections Skipped>

## 6.6	Moving Scenario and Periodic Update Performance

### 6.6.1	Definition and applicability

Moving scenario and periodic update performance measures the accuracy of the UE's A-GNSS position estimates and the periodic update capability of the UE in a moving scenario.

The requirements and this test apply to all types of UTRA for the UE that supports A-GNSS.

This test case includes sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 6.6.1.

Table 6.6.1: Sub-Test Case Number Definition

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS |
| 2 | UE supporting A-Galileo |
| 3 | UE supporting A-GPS and Modernized GPS |
| 4 | UE supporting A-GPS and A-GLONASS |
| 8 | UE supporting A-GPS and A-Galileo |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS |

### 6.6.2	Minimum requirements

<Unchanged Sections Skipped>

# 7	E-UTRA A-GNSS minimum performance requirements

## 7.0	General

This clause defines the minimum performance requirements for both UE based and UE assisted A-GNSS FDD and TDD E-UTRA UEs. If a UE supports both UE based and UE assisted modes then it shall be tested in both modes.

## 7.1	Sensitivity

### 7.1.1	Sensitivity Coarse time assistance

#### 7.1.1.1	Sub-tests

This test includes sub-tests dependent on the GNSS supported by the UE. Each sub-test is identified by a Sub-Test Number as defined in Table 7.1.1.1

Table 7.1.1.1: Sub-Test Number Definition

| Sub-Test Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A |
| 2 | UE supporting A-GLONASS |
| 3 | UE supporting A-Galileo |
| 4 | UE supporting A-GPS and Modernized GPS |
| 5 | UE supporting A-GPS and A-GLONASS (Note) |
| 8 | UE supporting A-GPS and A-Galileo (Note) |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

#### 7.1.1.2	Test purpose

To verify the performance of the first position estimate, when the UE is provided with only coarse time assistance.

#### 7.1.1.3	Test applicability

This test applies to all types of E-UTRA UE that supports A-GNSS except Category M1 and Category M2 devices that do not support VoLTE.

<Unchanged Sections Skipped>

### 7.1.2	Sensitivity Fine time assistance

#### 7.1.2.1	Sub-tests

This test includes sub-tests dependent on the GNSS supported by the UE. Each sub-test is identified by a Sub-Test Number as defined in Table 7.1.2.1

Table 7.1.2.1: Sub-Test Number Definition

| Sub-Test Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A |
| 2 | UE supporting A-GLONASS |
| 3 | UE supporting A-Galileo |
| 4 | UE supporting A-GPS and Modernized GPS |
| 5 | UE supporting A-GPS and A-GLONASS (Note) |
| 8 | UE supporting A-GPS and A-Galileo (Note) |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

#### 7.1.2.2	Test purpose

To verify the performance of the first position estimate, when the UE is additionally provided with fine time assistance.

#### 7.1.2.3	Test applicability

This test applies to all types of E-UTRA UE that supports A-GNSS and that is capable of providing an enhanced performance when the network provides Fine Time Assistance, except Category M1 and Category M2 devices that do not support VoLTE.

<Unchanged Sections Skipped>

## 7.2	Nominal Accuracy

#### 7.2.1	Sub-tests

This test includes sub-tests dependent on the GNSS supported by the UE. Each sub-test is identified by a Sub-Test Number as defined in Table 7.2.1

Table 7.2.1: Sub-Test Number Definition

| Sub-Test Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A |
| 2 | UE supporting A-GLONASS |
| 3 | UE supporting A-Galileo |
| 4 | UE supporting A-GPS and Modernized GPS |
| 5 | UE supporting A-GPS and A-GLONASS (Note) |
| 8 | UE supporting A-GPS and A-Galileo (Note) |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

#### 7.2.2	Test purpose

To verify the performance of the first position estimate, when the UE is provided with ideal GNSS signal conditions.

#### 7.2.3	Test applicability

This test applies to all types of E-UTRA UE that supports A-GNSS, except Category M1 and Category M2 devices that do not support VoLTE.

<Unchanged Sections Skipped>

## 7.3	Dynamic Range

#### 7.3.1	Sub-tests

This test includes sub-tests dependent on the GNSS supported by the UE. Each sub-test is identified by a Sub-Test Number as defined in Table 7.3.1

Table 7.3.1: Sub-Test Number Definition

| Sub-Test Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A |
| 2 | UE supporting A-GLONASS |
| 3 | UE supporting A-Galileo |
| 4 | UE supporting A-GPS and Modernized GPS |
| 5 | UE supporting A-GPS and A-GLONASS (Note) |
| 8 | UE supporting A-GPS and A-Galileo (Note) |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

#### 7.3.2	Test purpose

To verify the performance of the first position estimate, when the UE is provided with GNSS signals with large dynamic ranges.

#### 7.3.3	Test applicability

This test applies to all types of E-UTRA UE that supports A-GNSS, except Category M1 and Category M2 devices that do not support VoLTE.

<Unchanged Sections Skipped>

## 7.4	Multi-Path scenario

#### 7.4.1	Sub-tests

This test includes sub-tests dependent on the GNSS supported by the UE. Each sub-test is identified by a Sub-Test Number as defined in Table 7.4.1

Table 7.4.1: Sub-Test Number Definition

| Sub-Test Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A |
| 2 | UE supporting A-GLONASS |
| 3 | UE supporting A-Galileo |
| 4 | UE supporting A-GPS and Modernized GPS |
| 5 | UE supporting A-GPS and A-GLONASS (Note) |
| 8 | UE supporting A-GPS and A-Galileo (Note) |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

#### 7.4.2	Test purpose

To verify the performance of the first position estimate, when the UE is provided with GNSS signals with multi-path components.

#### 7.4.3	Test applicability

This test applies to all types of E-UTRA UE that supports A-GNSS, except Category M1 and Category M2 devices that do not support VoLTE.

<Unchanged Sections Skipped>

## 7.5	Moving scenario and periodic update (Rel-9 to Rel-13)

#### 7.5.1	Sub-tests

This test includes sub-tests dependent on the GNSS supported by the UE. Each sub-test is identified by a Sub-Test Number as defined in Table 7.5.1.

Table 7.5.1: Sub-Test Number Definition

| Sub-Test Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A |
| 2 | UE supporting A-GLONASS |
| 3 | UE supporting A-Galileo |
| 4 | UE supporting A-GPS and Modernized GPS |
| 5 | UE supporting A-GPS and A-GLONASS (Note) |
| 8 | UE supporting A-GPS and A-Galileo (Note) |
| 9 | UE supporting A-BDS |
| 10 | UE supporting A-GPS and A-BDS (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

#### 7.5.2	Test purpose

To verify the performance when the UE is requested to use periodical reporting with a reporting interval of 2 seconds.

#### 7.5.3	Test applicability

This test applies to all types of E-UTRA UE that supports A-GNSS with LPP Release 9 to 13, except Category M1 and Category M2 devices that do not support VoLTE.

## {End of changes}