3GPP TSG-RAN5 Meeting #94-e	R5-220574

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2213 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PC5 RRC message RRCReconfigurationSidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update default configuration of PC5 RRC message RRCReconfigurationSidelink - Table 4.6.1A-3 is specified from the DUT's side. But in some test case prose, these messages are sent by the NR-SS-UE so we need all fields to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 4.6.1A-3 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	RRCReconfigurationSidelink

Table 4.6.1A-3: RRCReconfigurationSidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier-r16 |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 |  | Not present |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { |  | 1 entry |  | SL_DRB |
| SLRB-Config-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| slrb-PC5-ConfigIndex-r16 |  | Not checked |  |  |  | TX |  |
|  |  | 1 |  |  |  | RX |  |
| sl-SDAP-ConfigPC5-r16 |  | Not checked |  | TX |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  | 1 entry |  |  |  |  |  |
| SL-PQFI-r16[1] |  | 1 |  | entry 1 |  |  |  |
| } |  |  |  |  |
| sl-MappedQoS-FlowsToReleaseList-r16 |  | Not present |  |  |
| sl-SDAP-Header-r16 |  | absent |  |  |
| } |  |  |  |  |
| sl-PDCP-ConfigPC5-r16 |  | Not checked |  | TX |
| sl-PDCP-ConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-PDCP-SN-Size-r16 |  | len12bits |  |  |
| sl-OutOfOrderDelivery-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RLC-ConfigPC5-r16 |  | Not checked |  | TX |
| sl-RLC-ConfigPC5-r16 CHOICE { |  |  |  | RX |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-SN-FieldLengthAM-r16 |  | size12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r16 |  | Not Checked |  | TX |
| sl-MAC-LogicalChannelConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-LogicalChannelIdentity-r16 |  | LogicalChannelIdentity with condition DRB1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| slrb-ConfigToReleaseList-r16 |  | Not present |  |  |
| sl-MeasConfig-r16 |  | Not present |  |  |
| sl-MeasConfig-r16 CHOICE { |  |  |  | SL_MEAS |
| setup |  | Not Checked |  | TX |
| setup SEQUENCE { |  |  |  | RX |
| sl-MeasObjectToRemoveList-r16 |  | Not present |  |  |
| sl-MeasObjectToAddModList-r16 |  | SL-MeasObjectList-r16 |  |  |
| sl-ReportConfigToRemoveList-r16 |  | Not present |  |  |
| sl-ReportConfigToAddModList-r16 |  | SL-ReportConfigList-r16 |  |  |
| sl-MeasIdToRemoveList-r16 |  | Not present |  |  |
| sl-MeasIdToAddModList-r16 |  | SL-MeasIdList-r16 |  |  |
| sl-QuantityConfig-r16 |  | SL-QuantityConfig-r16 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-CSI-RS-Config-r16 |  | Not present |  |  |
| sl-CSI-RS-Config-r16 CHOICE { |  |  |  | SL_CSI |
| setup |  | Not Checked |  | TX |
| setup SEQUENCE { |  |  |  | RX |
| sl-CSI-RS-FreqAllocation-r16 CHOICE { |  |  |  |  |
| sl-OneAntennaPort-r16 |  | 000000000001 |  |  |
| } |  |  |  |  |
| sl-CSI-RS-FirstSymbol-r16 |  | 6 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-ResetConfig-r16 |  | Not present |  |  |
| sl-LatencyBoundCSI-Report-r16 |  | Not present |  |  |
|  |  | 160 |  | SL_CSI |
| lateNonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| nonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_DRB | To provide peer UE SL DRB related configuration via PC5 RRC |
| SL_MEAS | To provide peer UE SL RSRP measurement and reporting related configuration via PC5 RRC |
| SL_CSI | To provide peer UE SL CSI reporting related configuration via PC5 RRC |
| TX | UE transmits and NR-SS-UE receives. |
| RX | UE receives and NR-SS-UE transmits. |

<End of modified section 1>