3GPP TSG-RAN5 Meeting #94-e	R5-220581

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2220 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SL-PreconfigurationNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To correct typos - sl-TDD-Configuration-r16 is used to determine sl-TDD-Config-r16 in MasterInformationBlockSidelink. Considering Pre-configuration is only used in PC5-only case. We suggest set sl-TDD-Configuration-r16 in Pre-configuration to sl-TDD-Configuration-r16 for simplify (i.e. all slots are applicable for SL transmission according to 38.213 cl.16.1). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 4.10.1-1 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.10	Reference configuration for V2X

4.10.1	Pre-configuration for V2X

–	SL-PreconfigurationNR

Table 4.10.1-1: SL-PreconfigurationNR

| Derivation Path: TS 38.331 [6], clause 9.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF { |  | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] |  | SL-FreqConfigCommon-r16 | entry 1 |  |
| } |  |  |  |  |
| sl-PreconfigNR-AnchorCarrierFreqList-r16 |  | Not present |  |  |
| sl-PreconfigEUTRA-AnchorCarrierFreqList-r16 |  | Not present |  |  |
| sl-RadioBearerPreConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF { |  | 1 entry |  |  |
| SL-RadioBearerConfig-r16[1] |  | SL-RadioBearerConfig-r16 | entry 1 |  |
| } |  |  |  |  |
| sl-RLC-BearerPreConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF { |  | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] |  | SL-RLC-BearerConfig-r16 | entry 1 |  |
| } |  |  |  |  |
| sl-MeasPreConfig-r16 |  | SL-MeasConfigCommon-r16 |  |  |
| sl-OffsetDFN-r16 |  | Not present |  |  |
| t400-r16 |  | ms1000 |  |  |
| sl-MaxNumConsecutiveDTX-r16 |  | n32 |  |  |
| sl-SSB-PriorityNR-r16 |  | 1 |  |  |
| sl-PreconfigGeneral-r16 SEQUENCE { |  |  |  |  |
| sl-TDD-Configuration-r16 |  | Not present |  |  |
| reservedBits-r16 |  | 00 |  |  |
| } |  |  |  |  |
| sl-UE-SelectedPreConfig-r16 |  | SL-UE-SelectedConfig-r16 |  |  |
| sl-CSI-Acquisition-r16 |  | Not present |  |  |
| sl-RoHC-Profiles-r16 |  | Not present |  |  |
| sl-MaxCID-r16 |  | Not present | default value 15 is used |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 1>