3GPP TSG-RAN5 Meeting #94-e	R5-220589

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2228 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to V2X message DIRECT LINK SECURITY MODE COMPLETE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, According to 24.587 6.1.2.7.3, if UE does not support IPv6 address allocation mechanism, it will set IP address configuration IE to “IPv6 address allocation not supported” and will include Link local IPv6 address, and it will mismatch with the Table 4.7.4-19. 2,According to 38.508-1 4.9.22 and 4.9.23 Authentication procedure is skipped, KNRP is not derived so LSBs of KNRP ID is not needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Set IP address configuration IE and Link local IPv6 address IE to “Not Checked” in Tx condition to be compatible with the UE not supporting IPv6 address allocation mechanism. 2,Set LSBs of KNRP ID to Not present in Rx condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail in the preamble procedure of V2X unicast TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	DIRECT LINK SECURITY MODE COMPLETE

Table 4.7.4-19: DIRECT LINK SECURITY MODE COMPLETE

| Derivation Path: TS 24.587 [54] Table 7.3.14.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK SECURITY MODE COMPLETE message identity |  | '0000 1111'B |  |  |
| Sequence number |  | Set in the test case, increase by 1 when a PC5 signalling message being sent or received | 0~255, uniquely identify a PC5 signalling message being sent or received |  |
| QoS flow descriptions |  |  |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0001'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| Number of parameters |  | 5 |  |  |
| E |  | 1 | parameters list is included |  |
| Associated V2X service identifiers |  |  |  |  |
| Length of V2X service identifier contents |  | '04'H |  |  |
| V2X service identifier 1 |  | '00 00 00 01'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 22 | Sensor sharing, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | 2 |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 4 |  |  |
| UE PC5 unicast user plane security policy |  | '0000 0000'B | Signalling integrity protection not needed, Signalling ciphering not needed. |  |
| IP address configuration |  | Not Checked |  | Tx |
| IP address configuration |  |  |  | Rx |
| IP address configuration IEI |  | '57'H |  |  |
| IP address configuration content |  | '0000 0001'B | IPv6 Router |  |
| Link local IPv6 address |  | Not Present |  | Rx |
|  |  | Not Checked |  | Tx |
| LSBs of KNRP ID |  | Not Checked |  | Tx |
|  |  | Not present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

<End of modified section 1>