3GPP TSG-RAN5 Meeting #94-e	R5-220596

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2773 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of V2X TC 13.2.6-Link keep alive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, NR V2X TC 13.2.6 is not completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Addition of NR V2X TC 13.2.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X TC 13.2.6 can’t be executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2... CR 0203 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

13.2.6	PC5 unicast / link keep alive

13.2.6.1	Test Purpose (TP)

(1)

with { UE transmitted a DIRECT LINK KEEPALIVE REQUEST message with Keep-alive counter value of n. }

ensure that {

when { UE didn't receive DIRECT LINK KEEPALIVE RESPONSE message when T5004 expires }

then { UE re-transmit a DIRECT LINK KEEPALIVE REQUEST message with same Keep-alive counter value of n }

}

(2)

with { UE received a first DIRECT LINK KEEPALIVE REQUEST message with Keep-alive counter value of n1, and UE responed a DIRECT LINK KEEPALIVE RESPONSE message.}

ensure that {

when { UE received a second DIRECT LINK KEEPALIVE REQUEST message with Keep-alive counter value of n2 < n1.}

then { UE does not transmit a DIRECT LINK KEEPALIVE RESPONSE message for the second DIRECT LINK KEEPALIVE REQUEST message.}

}

13.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.587 [FFS], subclause 6.1.2.8.5.1 and 6.1.2.8.5.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.587, subclause 6.1.2.8.5.1]

a)	Timer T5004 expires.

The initiating UE shall retransmit the DIRECT LINK KEEPALIVE REQUEST message with the last used value of the keep-alive counter and restart timer T5004. After reaching the maximum number of allowed retransmissions, the initiating UE shall abort the PC5 unicast link keep-alive procedure and locally release the PC5 unicast link.

NOTE:	The maximum number of allowed retransmissions is UE implementation specific.

b)	The need to use this PC5 unicast link no longer exists before the PC5 unicast link keep-alive procedure is completed.

The initiating UE shall abort the PC5 unicast link keep-alive procedure and initiate a PC5 unicast link release procedure.

c)	The initiating UE receives a DIRECT LINK KEEPALIVE RESPONSE message with a keep-alive counter value different from the value which the initiating UE had included in the last sent DIRECT LINK KEEPALIVE REQUEST message.

The initiating UE shall discard the DIRECT LINK KEEPALIVE RESPONSE message.

d)	The initiating UE receives a PC5 signalling message other than a DIRECT LINK KEEPALIVE RESPONSE message or PC5 user plane data from the target UE over this PC5 unicast link while timer T5004 is running.

The initiating UE shall stop timer T5004, abort the PC5 unicast link keep-alive procedure, start timer T5003 and increment the keep-alive counter for the PC5 unicast link.

e)	The initiating UE receives a DIRECT LINK KEEPALIVE RESPONSE message when T5004 is not running.

The initiating UE shall discard the DIRECT LINK KEEPALIVE RESPONSE message.

[TS 24.587, subclause 6.1.2.8.5.2]

a)	Timer T5005 expires.

The target UE shall:

1)	initiate a PC5 unicast link keep-alive procedure to check the link; or

2)	initiate the PC5 unicast link release procedure.

Whether the UE chooses 1) or 2) is left to UE implementation.

b)	The target UE receives a DIRECT LINK KEEPALIVE REQUEST message with a keep-alive counter value lower than the value which the target UE had included in the last sent DIRECT LINK KEEPALIVE RESPONSE message.

The target UE shall discard the DIRECT LINK KEEPALIVE REQUEST message.

c)	The target UE receives a DIRECT LINK KEEPALIVE REQUEST message if there is a pending PC5 signalling message or PC5 user plane data to be sent to the initiating UE over this PC5 unicast link.

The target UE:

1)	shall pass this PC5 signalling message to the lower layers for transmission along with the target UE's layer-2 ID for unicast communication and the initiating UE's layer-2 ID for unicast communication,  or perform the data transmission over PC5 unicast link as specified in clause 6.1.2.9; and

2)	shall consider transmission of this PC5 signalling message or PC5 user plane data to be an implicit DIRECT LINK KEEPALIVE RESPONSE message and skip generating a DIRECT LINK KEEPALIVE RESPONSE message. If a maximum inactivity period is included in the DIRECT LINK KEEPALIVE REQUEST message, the target UE shall stop T5005, if running, and start T5005 with its value set to the maximum inactivity period.

13.2.6.3	Test description

13.2.6.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

Preamble:

-	The UE is in state 4-A and Test Mode (On) , Test Loop Function(Off) with simulated NR SL UE configured as transmitting UE as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On).

13.2.6.3.2	Test procedure sequence

Table 13.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a DIRECT LINK KEEPALIVE REQUEST message with keep-alive counter = 0. | --> | PC5-S: DIRECT LINK KEEPALIVE REQUEST | - | - |
| 2 | The NR-SS-UE1 waits 5 seconds (T5004=5s). | - | - | - | - |
| 3 | Check: Does the UE transmit a DIRECT LINK KEEPALIVE REQUEST message with keep-alive counter = 0? | --> | PC5-S: DIRECT LINK KEEPALIVE REQUEST | 1 | P |
| 4 | The NR-SS-UE1 transmits a DIRECT LINK KEEPALIVE RESPONSE message with keep-alive counter = 0. | <-- | PC5-S: DIRECT LINK KEEPALIVE RESPONSE | - | - |
| 5 | The NR-SS-UE1 watis 4 seconds (less than T5003 (5s)) | - | - | - | - |
| 6 | The NR-SS-UE1 transmits a DIRECT LINK KEEPALIVE REQUEST message with keep-alive counter = 0. | <-- | PC5-S: DIRECT LINK KEEPALIVE REQUEST | - | - |
| 7 | The UE transmits a DIRECT LINK KEEPALIVE RESPONSE message with keep-alive counter = 0. | --> | PC5-S: DIRECT LINK KEEPALIVE RESPONSE | - | - |
| 8 | The NR-SS-UE1 watis 4 seconds (less than T5003 (5s)) | - | - | - | - |
| 9 | The NR-SS-UE1 transmits a DIRECT LINK KEEPALIVE REQUEST message with keep-alive counter = 1. | <-- | PC5-S: DIRECT LINK KEEPALIVE REQUEST | - | - |
| 10 | The UE transmits a DIRECT LINK KEEPALIVE RESPONSE message with keep-alive counter = 1. | --> | PC5-S: DIRECT LINK KEEPALIVE RESPONSE | - | - |
| 11 | The NR-SS-UE1 watis 4 seconds (less than T5003 (5s)) | - | - | - | - |
| 12 | The NR-SS-UE1 transmits a DIRECT LINK KEEPALIVE REQUEST message with keep-alive counter = 0. | <-- | PC5-S: DIRECT LINK KEEPALIVE REQUEST | - | - |
| 13 | Check: Does the UE transmit a DIRECT LINK KEEPALIVE RESPONSE message in next 30 seconds? | --> | PC5-S: DIRECT LINK KEEPALIVE RESPONSE | 2 | F |

13.2.6.3.3	Specific message contents

Table 13.2.6.3.3-1: Message DIRECT LINK KEEPALIVE RESPONSE (step 4, step7, Table 13.2.6.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.4-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Keep-alive counter | '00 00 00 00'H |  |  |

Table 13.2.6.3.3-2: Message DIRECT LINK KEEPALIVE REQUEST (step9, Table 13.2.6.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.4-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Keep-alive counter | '00 00 00 01'H |  |  |

<End of modified section 1>