3GPP TSG-RAN5 Meeting #94-e	R5-220611

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1367 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to applicability for LTE feMob |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To correct applicability for feMob TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct applicability for feMob TC 8.2.4.31.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not align with WP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4	Recommended Test Case Applicability

The applicability of each individual test is identified in Table 4-1. This is just a recommendation based on the purpose for which the test case was written.

The applicability of every test is formally expressed by the use of Boolean expression that are based on parameters (ICS) included in annex A of the present document.

Additional information related to the Test Case (TC), e.g. affecting its dynamic behaviour or its execution may be provided as well.

When a test case is to be executed against a category M1 UE and with IMS enabled, it is assumed that the UE is compliant to GSMA profile NG.108 [55].

The columns in Table 4-1 have the following meaning:

Clause

The clause column indicates the clause number in TS 36.523-1 [19] that contains the test body.

Title

The title column describes the name of the test and contains the clause title of the clause in TS 36.523-1 [19] that contains the test body.

Release

The release column indicates the earliest release from which the test case is applicable. In some specific cases it may indicate the release(s) for which the TC is only applicable.

Note:	Some exceptions to this interpretation may be indicated in Notes in column 'Release' e.g. see Note 3 Table 4-1.

Applicability - Condition

The following notations are used for the applicability column:

R	recommended - the test case is recommended

O	optional – the test case is optional

N/A	not applicable - in the given context, the test case is not recommended.

Ci	conditional - the test is recommended ("R") or not ("N/A") depending on the support of other items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

NOTE 1:	The conditions are defined in Table 4-1a.

Applicability - Comments

This column contains a verbal description of the condition.

Additional Information - Specific ICS

This column contains the mnemonics of ICS(s) affecting the dynamic behaviour of the TC.

NOTE 1A:	ICS items specified in 3GPP TS 34.123-2 [8] and 3GPP TS 34.229-2 [45] can be referred, to avoid redundant definitions.

NOTE 1B:	The ICS items pc_eFDD and pc_eFDD, as well as pc_NB_FDD and pc_NB_TDD, specified in the present document (Table A.4.1-1) are used to identify that a test case can be run in FDD or/and TDD branch. When none of them is provided it is assumed that the test case requires both FDD and TDD.

Additional Information - Specific IXIT

This column contains the mnemonics of IXIT(s) affecting the dynamic behaviour of the TC.

Additional Information - Number of TC Executions

This column contains, wherever applicable, the recommended for certification purposes number of TC executions. It may contain also other information e.g. exceptions to the release applicable to the test. Clarifying notes are listed in Table 4-1b.

Additional Information - Release other RAT

In regard to a particular test case, this column provides information on the release which is used by the simulated network in the other (i.e. non E-UTRA) RAT(s) where applicable. For each applicable RAT the release shall be indicated in the format 'Rel-X RAT'. When multiple RATs are applicable the entries per RAT shall be separated by a comma. When a value for a 3GPP RAT is not provided but the RAT is in the scope of the test case then for this RAT the release indicated in the Release column applies (per default), a Note extending the release applicability to an earlier version for E-UTRA in the ‘Release’ column is not applicable to the other RATs.

EXAMPLES:

Rel-9 UTRA FDD, Rel-8 GERAN or simply as Rel-9 UTRA FDD
(meaning that the UTRA FDD will simulate Rel-9 and the GERAN Rel-8 behaviours)

Rel-9 UTRA TDD
(meaning that the UTRA LCR TDD network will simulate Rel-9 behaviours)

NOTE 1C:	Some exceptions to this interpretation may be indicated in Notes in column 'Release other RAT' e.g. see Note 7A Table 4-1.

NOTE 2:	To meet the validation requirements from certification bodies then there is a need to uniquely reference the FDD and TDD branch of common FDD and TDD test cases. The FDD and TDD branches of common FDD and TDD test cases can be referenced by amending a "FDD" or "TDD" suffix to the test case clause number. For example for AM RLC test case 7.2.3.13 the FDD and TDD branches can be identified by "7.2.3.13 FDD" and "7.2.3.13 TDD".

Table 4-1: Applicability of tests and additional information for testing

| Clause |  | TC Title |  | Release |  | Applicability |  |  | Additional Information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment | Specific ICS |  |  | Specific IXIT |  | Number of TC Executions |  | Release other RAT |  |
| 6 |  | Idle mode operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1.1 |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode |  | Rel-8 |  | R |  | UEs supporting E-UTRA | pc_eFDD |  |  |  |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |  |  |
|  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | RRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.1.1 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.1.1a |  | RRC / Direct Indication Information / Notification of BCCH modification in idle mode |  | Rel-13 |  | C254 |  | UEs supporting E-UTRA and (CE Mode A or CE Mode B) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.1.2 |  | RRC / Paging for notification of BCCH modification in idle mode |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.1.2a |  | RRC / Paging for notification of BCCH modification in idle mode / eDRX cycle longer than the modification period / eDRX cycle with eDRX Allowed/Not Allowed |  | Rel-13 |  | C262 |  | UEs supporting E-UTRA and Extended DRX |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.1.3 |  | RRC / Paging for connection in idle mode / Multiple paging records |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.1.4 |  | RRC / Paging for connection in idle mode / Shared network environment |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.1.5 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.1.6 |  | RRC / BCCH modification in connected mode |  | Rel-8 |  | C224c |  | UEs supporting E-UTRA and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.1.7 |  | RRC / Paging / EAB active |  | Rel-11 |  | C194 |  | UEs supporting E-UTRA and EAB and LAP |  |  | pc_eFDD |  |  |  |  |  |  |
| 8.1.1.8 |  | RRC / Paging / DRX Operation / Enhanced Coverage / WUS |  | Rel-15 |  | C384 |  | UEs supporting E-UTRA FDD and (CE mode A or CE mode B) and WUS |  |  | pc_eFDD |  |  |  |  |  |  |
| 8.1.1.9 |  | RRC / Paging / eDRX Operation / Enhanced Coverage / WUS |  | Rel-15 |  | C385 |  | UEs supporting E-UTRA FDD and (CE mode A or CE mode B) and eDRX and WUS |  |  | pc_eFDD |  |  |  |  |  |  |
| 8.1.2.1 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.2.2 |  | RRC connection establishment / Reject with wait time |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.3 |  | RRC connection establishment / Return to idle state after T300 timeout |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.4 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.2.5 |  | RRC connection establishment / 0% access probability for MO calls, no restriction for MO signalling |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.6 |  | RRC connection establishment / Non-zero percent access probability for MO calls, no restriction for MO signalling |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.7 |  | RRC connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.8 |  | RRC connection establishment / Range of access baring time |  | Rel-8 |  | C97 |  | UEs supporting E-UTRA and Multiple PDN |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.9 |  | RRC Connection Establishment / 0% access probability for MO calls, non-zero percent access probability for MO signalling |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.10 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.2.11 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.2.12 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.2.13 |  | RRC connection establishment / 0% access probability for MO calls, 0% access probability for MO signalling |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.14 |  | RRC connection establishment / High speed flag |  | Rel-9 (Note 3) |  | C224c |  | UEs supporting E-UTRA and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.2.15 |  | RRC connection establishment / Extended and spare fields in SI |  | Rel-8 toRel-15 only |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.1 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.3.2 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.3.3 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.3.4 |  | RRC connection release / Redirection to another E-UTRAN frequency |  | Rel-8 |  | C388 |  | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.5 |  | RRC connection release / Success / With priority information |  | Rel-8 |  | C388 |  | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.5a |  | RRC connection release / Success / With extended priority information |  | Rel-12 |  | C388 |  | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.6 |  | RRC connection release / Redirection from E-UTRAN to UTRAN |  | Rel-8 |  | C01 |  | UEs supporting E-UTRA and UTRA and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  | Rel-9 UTRA TDD |  |
| 8.1.3.6a |  | RRC connection release / Redirection from E-UTRAN to UTRAN / Pre-redirection info |  | Rel-9 (Note 3) |  | C01 |  | UEs supporting E-UTRA and UTRA and NOT Category M1 |  |  | pc_eFDD |  |  |  |  | Rel-8 UTRA FDD |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  | Rel-9 UTRA TDD |  |
| 8.1.3.7 |  | RRC connection release / Redirection from UTRAN to E-UTRAN |  | Rel-8 |  | C01 |  | UEs supporting E-UTRA and UTRA and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  | Rel-9 UTRA TDD |  |
| 8.1.3.8 |  | RRC connection release / Redirection from E-UTRAN to GERAN |  | Rel-8 |  | C05 |  | UEs supporting E-UTRA and GERAN and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.9 |  | RRC connection release / Redirection from E-UTRAN to CDMA2000-HRPD |  | Rel-8 |  | C06 |  | UEs supporting E-UTRA and HRPD and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.10 |  | RRC connection release / Redirection from E-UTRAN to CDMA2000-1xRTT |  | Rel-8 |  | C07 |  | UEs supporting E-UTRA and 1xRTT and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.11 |  | RRC connection release / Redirection to another E-UTRAN band |  | Rel-9 (Note 3) |  | C184 a |  | UEs supporting E-UTRA and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.11a |  | RRC connection release / Redirection to another E-UTRAN band / Between FDD and TDD |  | Rel-9 (Note 3) |  | C389 |  | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |  |  |  |  |
| 8.1.3.12 |  | RRC connection release / Success / With priority information / Inter-band |  | Rel-9 (Note 3) |  | C184 a |  | UEs supporting E-UTRA and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  | Either TC 8.1.3.12 or TC 8.1.3.12b shall be executed. (Note 4) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.12a |  | RRC connection release / Success / With priority information / Inter-band / Between FDD and TDD |  | Rel-9 (Note 3) |  | C389 |  | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |  |  |  |  |
| 8.1.3.12b |  | RRC connection release / Success / With priority information / Inter-band (Single frequency operation in source band) |  | Rel-9 (Note 3) |  | C388 |  | UEs supporting E-UTRA and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  | Either TC 8.1.3.12 or TC 8.1.3.12b shall be executed. (Note 4) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.13 |  | LTE RRC connection release / Success / With idle mode measurement information from SIB5 |  | Rel-15 |  | C372 |  | UEs supporting E-UTRA and idle mode measurements |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.14 |  | LTE RRC connection release / Success / With idle mode measurement information from RRCConnectionRelease |  | Rel-15 |  | C372 |  | UEs supporting E-UTRA and idle mode measurements |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.1.3.15 |  | LTE RRC connection release / Success / With idle mode measurement information / No idle mode measurement capability provided |  | Rel-15 |  | C372 |  | UEs supporting E-UTRA and idle mode measurements |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.1.1 |  | RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC_CONNECTED / Success / Default bearer / Early bearer establishment |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.1.2 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.3 |  | RRC connection reconfiguration / Radio bearer establishment / Success / Dedicated bearer |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.1.4 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.5 |  | RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC CONNECTED / Success / Latency check |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.1.6 |  | RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC CONNECTED / Success / Latency check / SecurityModeCommand and RRCConnectionReconfiguration transmitted in the same TTI |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.1.7 |  | RRC connection reconfiguration / Radio bearer establishment / Success / SRB2 |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.1.8 |  | RRC connection reconfiguration / Radio bearer establishment / Success / Dedicated bearer / ROHC configured |  | Rel-9 (Note 3) |  | C120F |  | UEs supporting E-UTRA and Feature Group Indicator 7 and ROHC profile0x0001 and ROHC profile0x0002 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C120T |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.1 |  | RRC connection reconfiguration / Radio resource reconfiguration / Success |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.2 |  | RRC connection reconfiguration / SRB/DRB reconfiguration / Success |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.3.1 |  | CA / RRC connection reconfiguration / SCell addition/modification/release / Success / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.3.2 |  | CA / RRC connection reconfiguration / SCell addition/modification/release / Success / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.3.3 |  | CA / RRC connection reconfiguration / SCell addition/ modification/release / Success / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.4.1 |  | CA / RRC connection reconfiguration / SCell SI change / Success / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.4.2 |  | CA / RRC connection reconfiguration / SCell SI change / Success / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.4.3 |  | CA / RRC connection reconfiguration / SCell SI change / Success / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5.1 |  | CA / RRC connection reconfiguration / SCell addition without UL / Success / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  | a |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5.2 |  | CA / RRC connection reconfiguration / SCell addition without UL / Success / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5.3 |  | CA / RRC connection reconfiguration / SCell addition without UL / Success / Intra-band non-Contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5a.1 |  | CA / RRC connection reconfiguration / SCell addition without UL / SRS configuration / Periodic / multi-SRS switching |  | Rel-14 |  | C320 |  | UEs supporting E-UTRA FDD-TDD DL CA and SRS switching between a band pair. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C321 |  | UEs supporting E-UTRA TDD-TDD DL CA and SRS switching between a band pair. |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5a.2 |  | CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Aperiodic |  | Rel-14 |  | C320 |  | UEs supporting E-UTRA FDD-TDD DL CA and SRS switching between a band pair. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C321 |  | UEs supporting E-UTRA TDD-TDD DL CA and SRS switching between a band pair. |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5a.3 |  | CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Collision handling / Priority |  | Rel-14 |  | C320 |  | UEs supporting E-UTRA FDD-TDD DL CA and SRS switching between a band pair. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C321 |  | UEs supporting E-UTRA TDD-TDD DL CA and SRS switching between a band pair. |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.5a.4 |  | CA / RRC connection reconfiguration / TDD SCell addition without UL / SRS configuration / Collision handling / flexible SRS transmitting |  | Rel-14 |  | C320 |  | UEs supporting E-UTRA FDD-TDD DL CA and SRS switching between a band pair. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | C321 |  | UEs supporting E-UTRA TDD-TDD DL CA and SRS switching between a band pair. |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.6.1 |  | RRC connection reconfiguration/ UE Assistance Information/power preference indication setup and release |  | Rel-11 |  | C187 |  | UEs supporting E-UTRA and Power Preference Indication |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.6.2 |  | RRC connection reconfiguration/ UE Assistance Information/power preference indication release on connection re-establishment |  | Rel-11 |  | C 187 |  | UEs supporting E-UTRA and Power Preference Indication |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.6.3 |  | RRC connection reconfiguration/ UE Assistance Information/T340 running |  | Rel-11 |  | C187 |  | UEs supporting E-UTRA and Power Preference Indication |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.6.4 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.6.5 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.6.6 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.7.1 |  | CA / RRC connection reconfiguration / sTAG addition/modification/release / Success / Intra-band contiguous CA |  | Rel-11 |  | C190 |  | UEs supporting E-UTRA and Intra-band contiguous Uplink Carrier Aggregation and multiple timing advances |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.7.2 |  | CA / RRC connection reconfiguration / sTAG addition/modification/release / Success / Inter-band CA |  | Rel-11 |  | C191 |  | UEs supporting E-UTRA and Inter-band Uplink Carrier Aggregation and multiple timing advances and UL (Pcell) supported in each band of Inter-band CA combination under test |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.7.3 |  | CA / RRC connection reconfiguration / sTAG addition/modification/release / Success / Intra-band non-contiguous CA |  | Rel-11 |  | C192 |  | UEs supporting E-UTRA and Intra-band non-contiguous Uplink Carrier Aggregation and multiple timing advances |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.8 |  | RRC connection reconfiguration / SIB1 information / Success |  | Rel-11 |  | C268 |  | UEs supporting E-UTRA and Support of CRS interference handling and Synchronisation signal and common channel interference handling |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.9.1 |  | RRC connection reconfiguration / PSCell addition and SCG release / SCG / DRB |  | Rel-12 |  | C245 |  | UEs supporting E-UTRA and DC SCG DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.9.2 |  | RRC connection reconfiguration / PSCell addition and SCG release / Split DRB |  | Rel-12 |  | C244 |  | UEs supporting E-UTRA and DC Split DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.9.3 |  | RRC connection reconfiguration / SCG change without handover / SCG DRB to MCG DRB and SCG DRB modification |  | Rel-12 |  | C245 |  | UEs supporting E-UTRA and DC SCG DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.9.4 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.9.5 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.10 |  | eIMTA / RRC connection reconfiguration / Radio resource reconfiguration / Success |  | Rel-12 |  | C256 |  | UEs supporting E-UTRA and eIMTA and NOT Category M1 |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.11 |  | Short Processing Time / SRS configuration / Aperiodic |  | Rel-15 |  | C378 |  | UE supporting E-UTRA and short processing time |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.12 |  | Short TTI / SRS configuration / TDD / Aperiodic |  | Rel-15 |  | C382 |  | UEs supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs and SRS trigerring via DCI format 7 |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.13.1 |  | CA / RRC connection reconfiguration / SCell addition in dormant mode / Success / Intra-band Contiguous CA |  | Rel-15 |  | C374 |  | UEs supporting E-UTRA and Intra-band Carrier Aggregation and addition of SCell in dormant state |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.2.14.1 |  | CA / RRC connection reconfiguration / SCell addition in activated mode / Success / Intra-band Contiguous CA |  | Rel-15 |  | C375 |  | UEs supporting E-UTRA and Intra-band Carrier Aggregation and addition of SCell in activated state |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.3.1 |  | RRC connection reconfiguration / Radio bearer release / Success |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.1 |  | RRC connection reconfiguration / Handover / Success / Dedicated preamble |  | Rel-8 |  | C12 |  | (UEs supporting E-UTRA and NOT Category M1) or (UEs supporting E-UTRA and CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.2 |  | RRC connection reconfiguration / Handover / Success / Common preamble |  | Rel-8 |  | C12 |  | (UEs supporting E-UTRA and NOT C ategory M1) or (UEs supporting E-UTRA and CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.3 |  | RRC connection reconfiguration / Handover / Success / Intra-cell / Security reconfiguration |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.4 |  | RRC connection reconfiguration / Handover / Failure / Intra-cell / Security reconfiguration |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.5 |  | RRC connection reconfiguration / Handover / All parameters included |  | Rel-8 |  | C12 |  | (UEs supporting E-UTRA and NOT Category M1) or (UEs supporting E-UTRA and CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A””) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.6 |  | RRC connection reconfiguration / Handover / Success / Inter-frequency |  | Rel-8 |  | C21aF |  | UEs supporting E-UTRA and Feature Group Indicator 13 and Feature Group Indicator 25 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C21aT |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.7 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment successful |  | Rel-8 |  | C12 |  | (UEs supporting E-UTRA and NOT Category M1) or (UEs supporting E-UTRA and CE Mode A and eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A and intra-frequency handover to target cell in normal coverage and CE Mode A) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.8 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment failure |  | Rel-8 |  | C12 |  | (UEs supporting E-UTRA and NOT Category M1) or (UEs supporting E-UTRA and CE Mode A and “eventA3 for intra-frequency neighbouring cells in normal coverage CE Mode A” and “intra-frequency handover to target cell in normal coverage and CE Mode A”) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.9 |  | RRC connection reconfiguration / Handover / Inter-band blind handover / Success |  | Rel-8 |  | C185F |  | UEs supporting E-UTRA and Feature Group Indicator 13 and Feature Group Indicator 25 and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C185T |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.10 |  | RRC connection reconfiguration / Handover (between FDD and TDD) |  | Rel-8 |  | C63 |  | UEs supporting E-UTRA FDD and E-UTRA TDD and FDD Feature Group Indicator 25 and FDD Feature Group Indicator 30 and TDD Feature Group Indicator 25 and TDD Feature Group Indicator 30 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |  |  |  |  |
| 8.2.4.11 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.4.12 |  | RRC connection reconfiguration / Handover / Setup and release of MIMO |  | Rel-8 |  | C56 |  | UEs supporting E-UTRA and (UE Category 2 to UE Category 5) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.13 |  | RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band |  | Rel-9 (Note 3) |  | C185F |  | UEs supporting E-UTRA and Feature Group Indicator 13 and Feature Group Indicator 25 and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C185T |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.13a |  | RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band / Between FDD and TDD |  | Rel-9 (Note 3) |  | C63 |  | UEs supporting E-UTRA FDD and E-UTRA TDD and FDD Feature Group Indicator 25 and FDD Feature Group Indicator 30 and TDD Feature Group Indicator 25 and TDD Feature Group Indicator 30 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |  |  |  |  |
| 8.2.4.14 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band |  | Rel-9 (Note 3) |  | C185F |  | UEs supporting E-UTRA and Feature Group Indicator 13 and Feature Group Indicator 25 and more than 1 FDD or TDD E-UTRA band and NOT Category M1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C185T |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.14a |  | RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band / Between FDD and TDD |  | Rel-9 (Note 3) |  | C63 |  | UEs supporting E-UTRA FDD and E-UTRA TDD and FDD Feature Group Indicator 25 and FDD Feature Group Indicator 30 and TDD Feature Group Indicator 25 and TDD Feature Group Indicator 30 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |  |  |  |  |
| 8.2.4.15 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band |  | Rel-9 (Note 3) |  | C185F |  | UEs supporting E-UTRA and Feature Group Indicator 13 and Feature Group Indicator 25 and more than 1 FDD or TDD E-UTRA band and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C185T |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.15a |  | RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band / Between FDD and TDD |  | Rel-9 (Note 3) |  | C63 |  | UEs supporting E-UTRA FDD and E-UTRA TDD and FDD Feature Group Indicator 25 and FDD Feature Group Indicator 30 and TDD Feature Group Indicator 25 and TDD Feature Group Indicator 30 and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |  |  |  |  |  |
| 8.2.4.16.1 |  | CA / RRC connection reconfiguration / Setup and Change of MIMO / Intra-band Contiguous CA |  | Rel-10 |  | C176 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation and does not support Category 1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.16.2 |  | CA / RRC connection reconfiguration / Setup and Change of MIMO / Inter-band CA |  | Rel-10 |  | C177 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation and does not support Category 1 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.16.3 |  | CA / RRC connection reconfiguration / Setup and Change of MIMO / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.17.1 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.17.2 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Inter-band CA |  | Rel-10 |  | C242 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation and UL (Pcell) supported in each band of Inter-band CA combination under test |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.17.3 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.18.1 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.18.2 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.18.3 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.19.1 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.19.2 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.19.3 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change / SCell no Change / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
| 8.2.4.20.1 |  | CA / RRC connection reconfiguration / Handover / Success / SCell Change / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.20.2 |  | CA / RRC connection reconfiguration / Handover / Success / SCell Change / Inter-band CA |  | Rel-10 |  | C242 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation and UL (Pcell) supported in each band of Inter-band CA combination under test |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.20.3 |  | CA / RRC connection reconfiguration / Handover / Success / SCell Change Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.21.1 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.21.2 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.21.3 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band non-contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.22 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.4.23.1 |  | CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Intra-band Contiguous CA |  | Rel-10 |  | C132 |  | UEs supporting E-UTRA and Intra-band contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.23.2 |  | CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band CA |  | Rel-10 |  | C151 |  | UEs supporting E-UTRA and Inter-band Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.23.3 |  | CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Intra-band non-Contiguous CA |  | Rel-11 |  | C132a |  | UEs supporting E-UTRA and Downlink Intra-band non-Contiguous Carrier Aggregation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.24.1 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.4.25.1 |  | RRC connection reconfiguration / Intra-MeNB Handover / MCG DRB to MCG DRB and MCG DRB to/from SCG DRB |  | Rel-12 |  | C245 |  | UEs supporting E-UTRA and DC SCG DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.25.2 |  | RRC connection reconfiguration / Intra-MeNB Handover / MCG DRBs to/from Split DRB |  | Rel-12 |  | C246 |  | UEs supporting E-UTRA and DC Split DRB and DC SCG DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.25.3 |  | RRC connection reconfiguration / Intra-MeNB Handover / Split DRB to Split DRB |  | Rel-12 |  | C244 |  | UEs supporting E-UTRA and DC Split DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.25.4 |  | RRC connection reconfiguration / Handover with SCG release / MCG/SCG DRBs to MCG DRB |  | Rel-12 |  | C245 |  | UEs supporting E-UTRA and DC SCG DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.25.5 |  | RRC connection reconfiguration / Handover with SCG release / Split DRB to MCG DRB |  | Rel-12 |  | C244 |  | UEs supporting E-UTRA and DC Split DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.25.6 |  | RRC connection reconfiguration / Handover with SCG reconfiguration / SCG DRB to SCG DRB |  | Rel-12 |  | C245 |  | UEs supporting E-UTRA and DC SCG DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.25.7 |  | RRC connection reconfiguration / Handover with SCG reconfiguration / Split DRB to Split DRB |  | Rel-12 |  | C244 |  | UEs supporting E-UTRA and DC Split DRB |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.26 |  | eIMTA / RRC connection reconfiguration / Handover / Success |  | Rel-12 |  | C256 |  | UEs supporting E-UTRA and eIMTA and NOT Category M1 |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.27 |  | RRC connection reconfiguration / Handover / Success / Intra-frequency in Enhanced Coverage |  | Rel-13 |  | C254c |  | UEs supporting E-UTRA and CE mode A and eventA3 for intra-frequency neighbouring cells in normal coverage and intra-frequency handover to target cell in normal coverage |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.28 |  | eCall Only mode / RRC connection reconfiguration / Inter-frequency Handover / Success |  | Rel-14 (Note 7) |  | C314a |  | UEs supporting E-UTRA and IMS eCall Only type of emergency services over EPS and Automatic type of eCall initiation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.29 |  | UDC/ RRC connection reconfiguration / Handover / Success |  | Rel-15 |  | C352 |  | UEs supporting E-UTRA and the uplink data compression operation |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.30.1 |  | RRC connection reconfiguration / Handover / DAPS Handover / Success / Intra-Frequency |  | Rel-16 |  | C398 |  | UEs supporting E-UTRA and intra-frequency DAPS handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.30.2 |  | DAPS handover / Success / Radio Link Failure in source / Intra-Frequency |  | Rel-16 |  | C398 |  | UEs supporting E-UTRA and intra-frequency DAPS handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.30.3 |  | DAPS handover / Failure / source link available / Radio Link Failure in source / Intra-Frequency |  | Rel-16 |  | C398 |  | UEs supporting E-UTRA and intra-frequency DAPS handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.30.5 |  | DAPS handover / Success / Radio Link Failure in source / Inter-Frequency |  | Rel-16 |  | Caa01 |  | UEs supporting E-UTRA and inter-frequency DAPS handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.30.6 |  | DAPS handover / Failure / source link available / Radio Link Failure in source / Inter-Frequency |  | Rel-16 |  | Caa01 |  | UEs supporting E-UTRA and inter-frequency DAPS handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.30.4 |  | RRC connection reconfiguration / Handover / DAPS Handover / Success / Inter-Frequency |  | Rel-16 |  | C404 |  | UEs supporting E-UTRA and inter-frequency DAPS handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.31.1 |  | RRC connection reconfiguration / Handover / Conditional Handover/ Success / A3 / A5 / A3+A5 |  | Rel-16 |  | C399 |  | UEs supporting E-UTRA conditional handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.31.2 |  | Conditional handover / modify conditional handover configuration |  | Rel-16 |  | C399 |  | UEs supporting E-UTRA conditional handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.31.3 |  | Conditional handover / Failure |  | Rel-16 |  | C399 |  | UEs supporting E-UTRA conditional handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.4.31.4 |  | Conditional handover / Handover / Handover Failure |  | Rel-16 |  | C399 |  | UEs supporting E-UTRA conditional handover |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 8.2.5.1 |  | LWA / WLAN Release / WLAN Association / EUTRA RRC_Connected to WLAN (Event W2) |  | Rel-13 |  | C267 |  | UEs supporting E-UTRA and LWA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>