3GPP TSG-RAN5 Meeting #94-e	R5-220662

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2795 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcases 8.2.4.1.1.1, 8.2.4.1.1.2 and 8.2.4.1.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The derivation Table for IE BWP-DownlinkDedicated is wrongly set to 4.6.3-10, and should be changed to 4.6.3-11 of the latest 38.508-1. This will also address the issue of the IE RadioLinkMonitoringConfig being absent from IE BWP-DownlinkDedicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the derivation table to 4.6.3-11 in the Table 8.2.4.1.1.1.3.3-9: BWP-DownlinkDedicated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.1.1.1.ENDC, 8.2.4.1.1.2.ENDC and 8.2.4.1.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.4.1.1	NR CA / NR SCell addition / modification / release / Success / EN-DC

###### 8.2.4.1.1.1	NR CA / NR SCell addition / modification / release / Success / EN-DC / Intra-band Contiguous CA

8.2.4.1.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and SCG with initial AS security activated }

ensure that {

when { UE receives an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message containing sCellToAddModList with an sCellIndex set to the configured Scell }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in deactivated state and sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and SCG with SCell(s) configured }

ensure that {

when { UE receives an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message containing sCellToAddModList with an sCellIndex matching one of the current UE SCell configuration }

then { UE modifies the affected SCell dedicated configurations and sends an RRCConnectionReconfigurationComplete message }

}

(3)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and SCG with SCell(s) configured }

ensure that {

when { UE receives an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message containing sCellToReleaseList with an sCellIndex matching one of the current UE SCell configuration }

then { UE releases the SCell and sends an RRCConnectionReconfigurationComplete message }

}

8.2.4.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331: 5.3.5.3; TS 38.331: 5.3.5.3, 5.3.5.5.8, 5.3.5.5.9. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2>	re-establish PDCP for SRB2 configured with E-UTRA PDCP entity and for all DRBs that are established and configured with E-UTRA PDCP, if any;

2>	re-establish RLC for SRB2 and for all DRBs that are established and configured with E-UTRA RLC, if any;

2>	if the RRCConnectionReconfiguration message includes the fullConfig:

3>	perform the radio configuration procedure as specified in 5.3.5.8;

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 1:	Void

NOTE 2:	Void

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 3:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs configured with pdcp-Config and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

1>	if the received RRCConnectionReconfiguration includes the nr-Config and it is set to release: or

1>	if the received RRCConnectionReconfiguration includes endc-ReleaseAndAdd and it is set to TRUE:

2>	perform EN-DC release as specified in TS 38.331 [82], clause 5.3.5.10;

1>	if the received RRCConnectionReconfiguration includes the sk-Counter:

2>	perform key update procedure as specified in TS 38.331 [82], clause 5.3.5.7;

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2>	resume SRB2 and all DRBs that are suspended, if any, including RBs configured with NR PDCP;

NOTE 4:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 5:	The UE may discard SRB2 messages and data that it receives prior to completing the reconfiguration used to resume these bearers.

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2>	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1>	if the RRCConnectionReconfiguration message includes the dedicatedInfoNASList:

2>	forward each element of the dedicatedInfoNASList to upper layers in the same order as listed;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	if the RRCConnectionReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.10.9;

1>	upon RRC connection establishment, if UE does not need UL gaps during continuous uplink transmission:

2>	configure lower layers to stop using UL gaps during continuous uplink transmission in FDD for RRCConnectionReconfigurationComplete message and subsequent uplink transmission in RRC_CONNECTED except for UL transmissions as specified in TS36.211 [21];

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest:

3>	include perCC-GapIndicationList and numFreqEffective;

2>	if the frequencies are configured for reduced measurement performance:

3>	include numFreqEffectiveReduced;

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82, 5.3.5.3];

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

1>	if the RRCReconfiguration includes the fullConfig:

2>	perform the radio configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1> if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration message contains the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

1>	set the content of RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent, or;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2> if RRCReconfiguration was received via SRB1:

3> submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

3> if reconfigurationWithSync was included in spCellConfig of an SCG:

4> initiate the random access procedure on the SpCell, as specified in TS 38.321 [3];

3> else:

4> the procedure ends;

NOTE:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2> else (RRCReconfiguration was received via SRB3):

3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE:	For EN-DC, in the case RRCReconfiguration is received via SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1 > else:

2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2 and DRBs that are suspended;

1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a random access procedure triggered above;

2> stop timer T304 for that cell group;

2> apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2> apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2> if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3> if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4> acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2> the procedure ends.

NOTE:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

[TS 38.331, clause 5.3.5.5.8]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell.

[TS 38.331, clause 5.3.5.5.9]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

Editor’s Note: FFS Check automatic measurement handling for SCells.

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

1>	modify the SCell configuration in accordance with the sCellConfigDedicated.

8.2.4.1.1.1.3	Test description

8.2.4.1.1.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell. NR Cell 1 is the PSCell and NR Cell 3 is the SCell.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.

-	Else, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC), Bearers (MCG(s) and SCG) established and Test Loop Function (On) with UE test loop mode B according to TS 38.508-1 [4], clause 4.5.4.

8.2.4.1.1.1.3.2	Test procedure sequence

Table 8.2.4.1.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | The SS transmits a RRCConnectionReconfiguration message including NR RRCReconfiguration message to configure the NR SCell. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |  |
| 2 | Check: Does the UE transmit a RRCConnectionReconfigurationComplete message containing RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 1 | P |  |
| 2A | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? | - | - | - | - |
| 3 | The SS transmits a RRCConnectionReconfiguration message including NR RRCReconfiguration message to modify the startingBitOfFormat2-3 of the NR SCell. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |  |
| 4 | Check: Does the UE transmit a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 2 | P |  |
| 4A | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? | - | - | - | - |
| 5 | The SS transmits a RRCConnectionReconfiguration message including NR RRCReconfiguration message to release the NR SCell. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |  |
| 6 | Check: Does the UE release SCell and sends a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 3 | P |  |

8.2.4.1.1.1.3.3	Specific message contents

Table 8.2.4.1.1.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition EN-DC_NR_CA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-2: RRCReconfiguration (Table 8.2.4.1.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-3: CellGroupConfig (Table 8.2.4.1.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 With condition SCell_Add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { |  | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  |  | entry 1 |  |
| sCellIndex |  | 1 |  |  |
| sCellConfigCommon |  | ServingCellConfigCommon |  |  |
| sCellConfigDedicated |  | ServingCellConfig | TS 38.508-1 [4] table 4.6.3-167 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.4.1.1.1.3.3-4: ServingCellConfigCommon (Table 8.2.4.1.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| PhysCellId | Physical Cell Identity of NR Cell 3 |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-5: RRCConnectionReconfiguration (step 3, Table 8.2.4.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition EN-DC_NR_CA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-6: RRCReconfiguration (Table 8.2.4.1.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-7: CellGroupConfig (Table 8.2.4.1.1.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition SCell_Add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE(1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-8: ServingCellConfig (Table 8.2.4.1.1.1.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
| firstActiveDownlinkBWP-Id | Not present |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| firstActiveUplinkBWP-Id | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-9: BWP-DownlinkDedicated (Table 8.2.4.1.1.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-10: PDCCH-Config (Table 8.2.4.1.1.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| tpc-SRS SetupRelease { |  |  |  |
| setup | SRS-TPC-CommandConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-11: SRS-TPC-CommandConfig (Table 8.2.4.1.1.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-183 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-TPC-CommandConfig ::= SEQUENCE { |  |  |  |
| startingBitOfFormat2-3 | 1 |  |  |
| fieldTypeFormat2-3 | 1 |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-12: RRCConnectionReconfiguration (step 5, Table 8.2.4.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition EN-DC_NR_CA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-13: RRCReconfiguration (Table 8.2.4.1.1.1.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.1.1.3.3-14: CellGroupConfig (Table 8.2.4.1.1.1.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToReleaseList SEQUENCE (SIZE(1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |
| SCellIndex[1] | 2 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

###### 8.2.4.1.1.2	NR CA / NR SCell addition / modification / release / Success / EN-DC / Intra-band non-Contiguous CA

8.2.4.1.1.2.1	Test Purpose (TP)

Same as TC 8.2.4.1.1.1 but applied to Intra-band non-Contiguous CA.

8.2.4.1.1.2.2	Conformance requirements

Same as TC 8.2.4.1.1.1 but applied to Intra-band non-Contiguous CA.

8.2.4.1.1.2.3	Test description

8.2.4.1.1.2.3.1	Pre-test conditions

Same as TC 8.2.4.1.1.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

8.2.4.1.1.2.3.2	Test procedure sequence

Same as TC 8.2.4.1.1.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

###### 8.2.4.1.1.3	NR CA / NR SCell addition / modification / release / Success / EN-DC / Inter-band CA

8.2.4.1.1.3.1	Test Purpose (TP)

Same as TC 8.2.4.1.1.1 but applied to Inter-band CA

8.2.4.1.1.3.2	Conformance requirements

Same as TC 8.2.4.1.1.1 but applied to Inter-band CA

8.2.4.1.1.3.3	Test description

8.2.4.1.1.3.3.1	Pre-test conditions

Same as TC 8.2.4.1.1.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

8.2.4.1.1.3.3.2	Test procedure sequence

Same as TC 8.2.4.1.1.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3