3GPP TSG-RAN5 Meeting #94-e	R5-220675

# Online, , 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5399 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case in Annex F.1.5 and Annex F.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test Cases uncertainty parameters and test tolerances must be available in Annex F.1.5 and Annex F.3.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Uncertainty parameters and test tolerances have been defined of the following test cases: 1) TC 9.6.1.1_A.5 2) TC 9.6.1.1_A.6 3) TC 9.6.1.2_A.5 4) TC 9.6.1.2_A.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.1.5 and Annex F.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

F.1.5	Measurement of Channel State Information reporting

Table F.1.5-1: Maximum Test System Uncertainty for Channel State Information reporting

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 9.2.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 | ± 0.3 dB | Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |
| 9.2.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 | ± 0.3 dB | Same as 9.2.1.1 |
| <<Too many rows skipped>> |  |  |
| 9.6.1.1_A.2 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.1_A.4 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.1_A.5 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.1_A.6 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 2DL CA | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.3 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (4DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.4 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.6 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.3.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (2DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.3.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.3.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (4DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.3.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.4.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (2DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.4.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.4.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (4DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.4.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.7.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.7.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 | ± 0.6 dB | Same as 9.3.1.1.1 |
| 9.7.2.2 TDD CQI reporting definition under fading conditions for UE category 0 | ± 0.6 dB | Same as 9.3.1.1.1 |
| 9.7_1.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | ± 0.3 dB | Same as 9.2.1.1 |
| 9.7_1.1.2 TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | ± 0.3 dB | Same as 9.2.1.1 |
| 9.7_1.1.3 FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | ± 0.6 dB | Same as 9.3.1.1.1 |
| 9.7_1.1.4 TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | ± 0.6 dB | Same as 9.3.1.1.1 |
| 9.8.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.8.1.2 TDD CQI reporting definition under AWGN conditions for UE category M1 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.8.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category M1 | ± 0.6 dB | Same as 9.3.1.1.1 |
| 9.8.2.2 TDD CQI reporting definition under fading conditions for UE category M1 | ± 0.6 dB | Same as 9.3.1.1.1 |
| 9.9.1.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 4x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 8x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.3.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.3.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.4.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 4x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.1.4.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 8x4 | ± 0.3 dB | Same as 9.2.1.1 |
| 9.9.2.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Ês1/Noc ± 0.6 dB Ês2/Noc ± 0.3 dB | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB for single Tx Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |
| 9.9.2.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Ês1/Noc ± 0.6 dB Ês2/Noc ± 0.3 dB | Same as 9.9.2.1.1 |
| 9.9.2.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 | Ês1/Noc ± 0.8 dB Ês2/Noc ± 0.3 dB | Overall system uncertainty for fading conditions on Cell 1 comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.7 dB for two Tx antennas. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |
| 9.9.2.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 | Ês1/Noc ± 0.8 dB Ês2/Noc ± 0.3 dB | Same as 9.9.2.2.1 |
| 9.9.3.1.1 TDD PMI Reporting – PUSCH 3-1 (Single PMI) 8x4 | ± 0.6 dB | Same as 9.4.1.1.1 |
| 9.9.4.1.1 FDD RI Reporting– PUCCH 1-1 4x4 | ± 0.8 dB | Same as 9.5.1.1 |
| 9.9.4.1.2 TDD RI Reporting– PUSCH 3-1 4x4 | ± 0.8 dB | Same as 9.5.1.1 |
| 9.9.4.2.1 FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 | ± 0.8 dB | Same as 9.5.1.1 |
| 9.9.4.2.2 TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 | ± 0.8 dB | Same as 9.5.1.1 |
| 9.10.1.1 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |
| 9.10.1.2 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | ± 0.6 dB | Same as 9.10.1.1 |
| 9.12.1.1 FDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty ±0.5 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect AWGN absolute power uncertainty ±3.0 dB not expected to have any significant effect |
| 9.12.1.1 TDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) | ± 0.6 dB | Same as 9.12.1.1 |
| 9.12.2.1 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) | ± 0.6 dB | Same as 9.12.1.1 |
| 9.12.2.2 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) | ± 0.6 dB | Same as 9.12.1.1 |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |
| AWGN Bandwidth |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  | ±3 dB |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  | ±0.5 dB |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  | ±3 dB |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Fading profile power uncertainty |  | Test-specific |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |
| Downlink channel matrix uncertainties: |  |  |
| Tx phase error, as shown in Figure F.1.5-1 |  | is defined for each Tx antenna up to NTx ≤10° is constant for the duration of the test |
| Rx phase error, as shown in Figure F.1.5-1 |  | is defined for each Rx antenna up to NRx There is no constraint on the value of except that it is constant for the duration of the test |
| Note 1: Only the overall stimulus error is considered. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN parameters apply to all test cases except 9.3.3.1.1 and 9.3.3.1.2. The fading parameters apply to test cases using fading Note 3: Downlink channel matrix uncertainties apply to eDL-MIMO CSI test cases Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |

<< Skip unchanged sections >>

F.3.5	Measurement of Channel State Information reporting

Table F.3.5-1: Derivation of Test Requirements (Channel State Information reporting tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 9.2.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| <<Too many rows skipped>> |  |  |  |
| 9.6.1.1_A.2 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Same as 9.6.1.1_A.1 |
| 9.6.1.1_A.4 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Same as 9.6.1.1_A.1 |
| 9.6.1.1_A.5 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Same as 9.6.1.1_A.1 |
| 9.6.1.1_A.6 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 2DL CA | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.2_A.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) | Same as 9.6.1.1_A.2 | Same as 9.6.1.1_A.2 | Same as 9.6.1.1_A.2 |
| 9.6.1.2_A.3 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (4DL CA) | Same as 9.6.1.1_A.3 | Same as 9.6.1.1_A.3 | Same as 9.6.1.1_A.3 |
| 9.6.1.2_A.4 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) | Same as 9.6.1.1_A.4 | Same as 9.6.1.1_A.4 | Same as 9.6.1.1_A.4 |
| 9.6.1.2_A.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) | Same as 9.6.1.1_A.5 | Same as 9.6.1.1_A.5 | Same as 9.6.1.1_A.5 |
| 9.6.1.2_A.6 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) | Same as 9.6.1.1_A.6 | Same as 9.6.1.1_A.6 | Same as 9.6.1.1_A.6 |
| 9.6.1.3.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (2DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 The analysis is recorded in 3GPP TR 36.904 [17] |
| 9.6.1.3.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) | Same as 9.6.1.1_A.2 | Same as 9.6.1.1_A.2 | Same as 9.6.1.1_A.2 The analysis is recorded in 3GPP TR 36.904 [17] |
| 9.6.1.3.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (4DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.3.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.4.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (2DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 The analysis is recorded in 3GPP TR 36.904 [17] |
| 9.6.1.4.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) | Same as 9.6.1.1_A.2 | Same as 9.6.1.1_A.2 | Same as 9.6.1.1_A.2 The analysis is recorded in 3GPP TR 36.904 [17] |
| 9.6.1.4.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (4DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.6.1.4.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 | Same as 9.6.1.1_A.1 |
| 9.7.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.7.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 |
| 9.7.2.2 TDD CQI reporting definition under fading conditions for UE category 0 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 |
| 9.7_1.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.7_1.1.2 TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | Same as 9.2.1.2 | Same as 9.2.1.2 | Same as 9.2.1.2 |
| 9.7_1.1.3 FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 |
| 9.7_1.1.4 TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 |
| 9.8.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.8.1.2 TDD CQI reporting definition under AWGN conditions for UE category M1 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.8.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category M1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 |
| 9.8.2.2 TDD CQI reporting definition under fading conditions for UE category M1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 | Same as 9.3.1.1.1 |
| 9.9.1.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 4x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 8x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.3.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.3.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.4.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 4x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.1.4.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 8x4 | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR and limits unchanged |
| 9.9.2.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -4.00dB 1.8 BLER 0.02 | 0dB +0.03dB +0.03dB 0.01 | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -3.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 9.9.2.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Same as 9.9.2.1.1 | Same as 9.9.2.1.1 | Same as 9.9.2.1.1 |
| 9.9.2.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -4.00dB 1.8 BLER 0.02 | 0dB +0.03dB +0.03dB 0.01 | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -3.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 9.9.2.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 | Same as 9.9.2.2.1 | Same as 9.9.2.2.1 | Same as 9.9.2.2.1 |
| 9.9.3.1.1 TDD PMI Reporting – PUSCH 3-1 (Single PMI) 8x4 | Noc as specified in test procedure 2.50 | 0.01 | Noc unchanged Ior/ Noc=SNRrnd is a result of test procedure step 2 and is used as setting in test procedure step 3. 2.49 as per Table G.5.4-1 |
| 9.9.4.1.1 FDD RI Reporting– PUCCH 1-1 4x4 | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 | SNR 0dB 20.01 10.01 10.01 20.01 | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |
| 9.9.4.1.2 TDD RI Reporting– PUSCH 3-1 4x4 | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 | SNR 0dB 20.01 10.01 10.01 20.01 | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |
| 9.9.4.2.1 FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 | SNR 0dB 20.01 10.01 10.01 20.01 | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |
| 9.9.4.2.2 TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 | SNR 0dB 20.01 10.01 10.01 20.01 | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |
| 9.10.1.1 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | SNRs as specified Test 1-1: 1.2 Test 1-2: 1.2 Test 1-3: 1.3 Test 1-4:  1.35 | SNR 0dB 0.01 | SNR unchanged  1.19 as per Table G.5.4-1  1.19 as per Table G.5.4-1  1.29 as per Table G.5.4-1  1.34 as per Table G.5.4-1 |
| 9.10.1.2 TDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | SNRs as specified Test 1-1: 1.2 Test 1-2: 1.2 Test 1-3: 1.3 Test 1-4:  1.35 | SNR 0dB 0.01 | SNR unchanged  1.19 as per Table G.5.4-1  1.19 as per Table G.5.4-1  1.29 as per Table G.5.4-1  1.34 as per Table G.5.4-1 |
| 9.12.1.1 FDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) | SNRs as specified 20% 1.05 BLER 0.02 | SNR 0dB 0% 0.01 BLER 0 | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |
| 9.12.1.1 TDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) | SNRs as specified 20% 1.05 BLER 0.02 | SNR 0dB 0% 0.01 BLER 0 | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |
| 9.12.2.1 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) | SNRs as specified 20% 1.05 BLER [0.02] | SNR 0dB 0% 0.01 BLER 0 | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |
| 9.12.2.2 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) | SNRs as specified 20% 1.05 BLER [0.02] | SNR 0dB 0% 0.01 BLER 0 | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |

# << End of changes >>