3GPP TSG-RAN5 Meeting #94-e	R5-220725

# Electronic Meeting, 21th Feb– 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2237 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default configuration for NR SL RRM test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, To add default configuration for NR SL RRM tests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Default configuration for NR SL RRM tests are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.3.2	Sidelink information elements for RRM

As defined in clause 4.6.6 with the following exceptions:

–	SL-BWP-ConfigCommon

Table 7.3.2-1: SL-BWP-ConfigCommon

| Derivation Path: Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon-r16 | Table 7.3.2-2 |  |
| } |  |  |  |  |

–	SL-BWP-PoolConfigCommon

Table 7.3.2-2: SL-BWP-PoolConfigCommon

| Derivation Path: Table 4.6.6-4 with condition RXPOOL and SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  |  |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool-r16 | entry 1 Table 7.3.2-3 |  |
| } |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool-r16 | Table 7.3.2-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

–	SL-ResourcePool

Table 7.3.2-3: SL-ResourcePool

| Derivation Path: Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-NumSubchannel-r16 |  | 1 |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |  |
| sl-ThresPSSCH-RSRP-List-r16 |  | Set according to the configuration in specific test cases |  |  |
| sl-SelectionWindowList-r16 SEQUENCE (SIZE (8)) OF SL-SelectionWindowConfig-r16 { |  | 8 entries |  |  |
| SL-SelectionWindowConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-SelectionWindow-r16 |  | n20 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PreemptionEnable-r16 |  | enabled |  |  |
| sl-MinMaxMCS-List-r16 SEQUENCE (SIZE (1..3)) OF SL-MinMaxMCS-Config-r16 { |  | 1 entry |  |  |
| SL-MinMaxMCS-Config-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MCS-Table-r16 |  | qam64 |  |  |
| sl-MinMCS-PSSCH-r16 |  | 0 |  |  |
| sl-MaxMCS-PSSCH-r16 |  | 28 |  |  |
| } |  |  |  |  |
| sl-TimeResource-r16 |  | 11111111111111111111 |  |  |
| } |  |  |  |  |

–	SL-PSSCH-TxConfigList

Table 7.3.2-4: SL-PSSCH-TxConfigList

| Derivation Path: Table 4.6.6-19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 { |  | 1 entry |  |  |  |
| sl-ThresUE-Speed-r16 |  |  | kmph200 |  |  |  |
| sl-ParametersAboveThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 1 |  |  |  |
| } |  |  |  |  |  |  |
| sl-ParametersBelowThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  |  | 4 |  |  |  |
| sl-MaxMCS-PSSCH-r16 |  |  | 25 |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 1 |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |

–	SL-UE-SelectedConfig

Table 7.3.2-5: SL-UE-SelectedConfig

| Derivation Path: Table 4.6.6-35 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-UE-SelectedConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSSCH-TxConfigList-r16 |  | SL-PSSCH-TxConfigList-r16 | Table 7.3.2-4 |  |
| sl-ProbResourceKeep-r16 |  | v0dot8 |  |  |
| sl-ReselectAfter-r16 |  | n1 |  |  |
| } |  |  |  |  |

7.4	FFS

Void.

<End of modified section 1>