3GPP TSG-RAN5 Meeting #94-e	R5-220771

# Electronic Meeting, February 21 - March 4, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2242 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for CA_n66(2A) BCS1 and BCS2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | BCS1 and BCS2 for CA_n66(2A) have been introduced in Rel-17 and the test frequencies for BCS1 and BCS2 CBW combinations need to be added to clause 4.3.1.1.5.66. The introduced BCS1 and BCS2 CBW combinations in TS 38.101-1 are: The valid new CBW combinations for BCS1 and BCS2 need to take the TS 38.101-1 specified CBW vs SCS (Table 5.3.5-1) and the assymmetric UL and DL channel bandwidth conmbinations (Table 5.3.6-1) into account: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test frequencies for new CBW combinations for BCS1 and BCS2 in Table 4.3.1.1.5.66-1 (SCS=15kHz) and Table 4.3.1.1.5.66-2 (SCS=30 kHz). Added BCS info for existing and new CBW combinations in CBW combination column indicating for which BCS a CBW combination is part of. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test frequencies specified for CA_n66(2A) BCS1 and BCS2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.5.66 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 2245 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.5.66	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); A (5MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 5+5 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 5+10 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 5+15 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 5+20 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 5+25 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 5+30 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 5+40 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (10MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 10+5 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (15MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 15+5 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (20MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 20+5 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (25MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 25+5 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (30MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 30+5 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (40MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 40+5 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | able 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.66-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); A (10MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (15MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/20 |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (20MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (25MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (30MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (40MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (0,1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/40 or 20/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |