3GPP TSG-RAN5 Meeting #94-e	R5-220843

# Electronic Meeting, 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2808 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MDT test case 8.1.6.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies, Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Currently in Table 8.1.6.1.3.4.3.3-8, the fields ra-InformationCommon-r16 and rlf-Cause-r16 are set as “Not Present” which is incorrect as the UE is allowed to include these fields, as per the TS 38.331 RLF-Report field descriptions below: ra-InformationCommon This field is optionally included when connectionFailureType is set to 'hof' or when connectionFailureType is set to 'rlf' and the rlf-Cause equals to 'randomAccessProblem' or 'beamRecoveryFailure'; otherwise this field is absent. rlf-Cause This field is used to indicate the cause of the last radio link failure that was detected. In case of handover failure information reporting (i.e., the connectionFailureType is set to 'hof'), the UE is allowed to set this field to any value Since this GCF ensures that reporting of NR handover failure is checked using the Field connectionFailureType set to “hof”, the fields ra-InformationCommon-r16 and rlf-Cause-r16 are not mandatory to be checked. - As per TS 38.331 Cl 5.3.10.5, for connectionFailureType to hof; failedPCellId is set to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover. - As per TS38.331 Cl. 5.3.7.4, if RRCReestablishmentRequest was initiated due to handover failure, then reestablishmentCellId is to be set in the rlf-report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In Specific message contents Table 8.1.6.1.3.4.3.3-8, the fields ra-InformationCommon-r16 and rlf-Cause-r16 are set as “Not checked”. - In Specific message contents Table 8.1.6.1.3.4.3.3-8, reestablishmentCellId-r16 is set to the global cell identity of NRCell 1. - In Specific message contents Table 8.1.6.1.3.4.3.3-8, nrFailedPCellId-r16 is set to “Any Allowed value”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 8.1.6.1.3.4	Radio Link Failure / Reporting at NR handover

8.1.6.1.3.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with handover failure information available }

ensure that {

when { UE receives RRCReconfiguration message }

then { UE sends the RRCReconfigurationComplete message with rlf-InfoAvailable }

}

(2)

with { UE in RRC_CONNECTED state with the handover failure information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report included }

}

8.1.6.1.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.10.5, 5.3.5.8.3, 5.3.5.3 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	set the c-RNTI to the C-RNTI used in the source PCell(in case HO failure) or PCell (in case RLF);

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.4.3	Test description

8.1.6.1.3.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.4.3.2	Test procedure sequence

Table 8.1.6.1.3.4.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.3.4.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Only Cell 1 is available. |

Table 8.1.6.1.3.4.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | Only Cell 1 is available. |

Table 8.1.6.1.3.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 2 the steps specified in Table 8.1.6.1.3.4.3.2-4 should take place. | - | - | - | - |
| 2 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 3 | The UE transmits an RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 4 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 5 | The UE transmits an RRCReestablishmentComplete message using the security key derived from the nextHopChainingCount on NR Cell 1. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 6 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | Check: Does the UE transmit an RRCReconfigurationtComplete message with rlf-InfoAvailable included? | --> | NR RRC: RRCReconfigurationtComplete | 1 | P |
| 8 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 9 | Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? | --> | NR RRC: UEInformationResponse | 2 | P |

Table 8.1.6.1.3.4.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.1.6.1.3.4.3.3	Specific message contents

Table 8.1.6.1.3.4.3.3-1: RRCReconfiguration-HO (step 1, Table 8.1.6.1.3.4.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_KeyChange

Table 8.1.6.1.3.4.3.3-2: RRCReestablishmentRequest (step 3, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-3: RRCReestablishment (step 4, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-4: RRCReestablishmentComplete (step 5, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-5: RRCReconfiguration (step 6, Table 8.1.6.1.3.4.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.4.3.3-6: RRCReconfigurationComplete (step 7, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| rlf-InfoAvailable-r16 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.3.4.3.3-7: UEInformationRequest (step 8, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| rlf-ReportReq-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-8: UEInformationResponse (step 9, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 CHOICE { |  |  |  |
| nrPreviousCell-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| eutraFailedPCellId-r16 | Not present |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | hof |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |