3GPP TSG-RAN5 Meeting #94-e	R5-220873

# Online, , 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1550 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating reference sensitvity test requirement for CA combination of n24 and n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The reference sensitivity test requirement for CA_n24-n41 is absent in clause 7.3A.1.5 needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Table 7.3A.1.5-1 with CA_n24-n41 configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference sensitivity test requirement for CA_n24-n41 will not be specified for UE Conformance Testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.3A.1.5	Test requirement

For 2DL carrier aggregation, test parameters are specified in table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. For the CA configurations listed in table 7.3A.1.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1 for non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous 2 DL CA, the test requirement for shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1 for the SCC. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.1.5-1: Reference sensitivity requirement for 2DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
|  | CA_n40B | - |
|  | CA_n41C | - |
| Intra-band contiguous 2DL CA | CA_n66B | - |
|  | CA_n78B | - |
|  | CA_n78C | - |
|  | CA_n66(2A) | - |
| Intra-band non-contiguous 2DL CA | CA_n77(2A) | - |
|  | CA_n78(2A) | - |
|  | CA_n1A-n3A |  |
|  | CA_n1A-n77A | - |
|  | CA_n1A-n78A | - |
|  | CA_n3A-n77A | - |
| Inter-band 2DL CA | CA_n3A-n78A | - |
|  | CA_n8A-n78A | - |
|  | CA_n24A-n41A | - |
|  | CA_n28A-n41A | - |
|  | CA_n28A-n79A | - |
|  | CA_n41A-n79A | - |
|  | CA_n66A-n70A | - |
|  | CA_n66A-n71A | - |
|  | CA_n70A-n71A | - |
| SDL configuration | CA_n29A-n66A | - |
|  | CA_n29A-n70A | - |

Table 7.3A.1.5-2: Reference sensitivity for SDL bands

| NR band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n29 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, Table 7.3.2.4.1-3, Table 7.3.2.5-1, Table 7.3.2.5-2 and Table 7.3A.1.4-1   with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.0.2.2-1 for the SCC(s). For aggregation of two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.0.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

For band combinations including operating bands without uplink band (as noted in Table 5.2-1), the requirements are specified in Table 7.3A.1.5-1 and for any band with uplink the uplink configuration specified in Table 7.3.2.4.1-2. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels, as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one-sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal, as described in Annex A.5.1.1/A.5.2.1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------