3GPP TSG-RAN5 Meeting #94-e	R5-220879

# Online, 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1656 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 (6.1.1.7) is imcomlete due to undefined TT analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TT is specified. Editor’s Note are removed and make TCs completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.7, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

<<Start of change>>

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

6.1.1.7.1	Test purpose

The purpose of this test is to verify that when the current and target cell operates on the same carrier frequency the UE configured with highSpeedMeasFlag-r16 is able to search and measure cells to meet the intra-frequency NR cell re-selection requirements.

6.1.1.7.2	Test applicability

This test applies to all types of NR UE from release 16 onwards that supports measurement enhancements in high speed scenario

6.1.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.1.7.

6.1.1.7.4	Test description

6.1.1.7.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.1.1.7.4.1-1.

Table 6.1.1.7.4.1-1: Supported test configurations for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 6.1.1.7-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.1.1.7-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.1.1.7-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.1.1.7.4.1-2.

Table 6.1.1.7.4.1-2: Initial conditions for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.1.1.7.4.1-3.

2. Message contents are defined in clause 6.1.1.7.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 6.1.1.7.4.1-3: General test parameters for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial | Active cell |  | 1, 2, 3 | Cell1 |  |
| condition | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.32 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.1.7.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. highSpeedMeasFlag-r16 is broadcasted to UE.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1. Set Cell 2 physical cell identity = initial Cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 6.1.1.7.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.1.1.7.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

7a	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 6.1.1.7.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, continues with step 11a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

11a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5 on Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

6.1.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.1.7.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks on condition of HighSpeedMeas with exceptions | Table H.2.1-1 with Condition SMTC.2 and Asynchronous cells for configuration 6.1.1.7-1 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.7-2 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.7-3 Table H.2.1-2 Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions |  |

6.1.1.7.5	Test requirement

Tables 6.1.1.7.4.1-3 and 6.1.1.7.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16.

Table 6.1.1.7.5-1: Cell specific test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |  |
|  | dB | 1 | 16 | -3.55 | 3.24 | -infinity |  | 3.24 | -3.55 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16.45 | -infinity |  | 16.45 | 13 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -81.55 | -infinity |  | -81.55 | -85 |
|  |  | 2 | -82 | -85 | -81.55 | -infinity |  | -81.55 | -85 |
|  |  | 3 | -79 | -82 | -78.54 | -infinity |  | -78.54 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -51.91 | -51.91 | specified in Cell 1 columns- |  |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -51.91 | -51.91 |  |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -45.81 | -45.81 |  |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  | AWGN 1944Hz Note4 |  |  |
| Propagation Condition |  | 3 | AWGN |  |  |  | AWGN 3334Hz Note5 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell test requirement in this case is expressed as:

Cell re-selection delay to a newly detectable cell = Tdetect,NR_Intra + TSI-NR

Tdetect,NR_Intra = 2.56 s; as specified in Table 4.2.2.3-2 in clause 4.2.2.3 TS 38.133 [6].

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a newly detectable cell shall be less than a total of 3.84 seconds in this test case (note: this gives a total of 3.84 seconds but the test allows 4 seconds).

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 0.96 s; as specified in Table 4.2.2.3-2 in clause 4.2.2.3 TS 38.133 [6].

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to an already detected cell shall be less than a total of 2.24 seconds in this test case (note: this gives a total of 2.24 seconds but the test allows 3 seconds).

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

<<Unchaged sections skipped>>

F.1	Measurement uncertainties and test tolerances for FR1 and FR2

F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

F.1.1.2	Measurement of RRM requirements

<<Unchaged sections skipped>>

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.2.1 | Same as 6.1.2.1 |  |
| 6.3.1.1 NR SA FR1 handover with known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.3.2.1.2 NR SA FR1 – FR1 RRC re-establishment | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.2.1 | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.2.2 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.3.1 NR SA FR1 conditional handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.3.2 NR SA FR1-FR1 conditional handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB |  | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for Pcell configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for Pcell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for Pcell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for Pcell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.5.3.1 NR SA FR1 Scell activation and deactivation of known Scell in non-DRX for 160ms Scell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.3.2 NR SA FR1 Scell activation and deactivation of known Scell in non-DRX for 320ms Scell measurement cycle | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 NR SA FR1 Scell activation and deactivation of unknown Scell in non-DRX | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX | Same as 6.5.6.1.1 |  | Same as 6.5.6.1.1 |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Fading ±0.5 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB |  | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |

<<Unchaged sections skipped>>

F.1.2	Interpretation of measurement results (normative)

See TS 38.521-1 [17] Annex F.2.

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR SA FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

<<Unchaged sections skipped>>

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In Signalling SsearchDeltaP = 3dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB In Signalling 3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB In Signalling SsearchDeltaP = 6dB |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SsearchDeltaP of Cell 1 = 3dB SsearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SsearchDeltaP of Cell 1 = 3dB SsearchDeltaP of Cell 2 = 15dB |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SsearchThresholdP of Cell 1 = 50dB SsearchThresholdP of Cell 2 = 50dB | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SsearchThresholdP of Cell 1 = 50dB SsearchThresholdP of Cell 2 = 34dB |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 6.3.1.1 NR SA FR1 handover with known target cell |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
|  |  | TEST CONFIGURATION 3 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  | A3-Offset: 0dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB | -1dB During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.5dB | A3-Offset: -1dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +9.5dB |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell |  | During T1: Noc: -100dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: -1.54dB 0dB 0dB 0dB 0dB 0dB During T2: -1.54dB 0dB 0dB 0dB 0dB 0dB During T3: -1.54dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -101.54dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.2.1 |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 6.3.2.2.2 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.3.1 NR SA FR1 conditional handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.3.2 NR SA FR1-FR1 conditional handover |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.1 | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.3 | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  | Same as 64.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.5.3.1 |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB | During T1: 0dB 0dB +1.65dB 0dB During T2: 0dB 0dB -1.65dB 1.65dB | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +19.65dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -3.65dB Ês2 / Noc: +20.65dB |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3:-110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±9dB (±2dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±3.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_48 to RSRP_70 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |

<<End of change>>