3GPP TSG-RAN5 Meeting #94-e	R5-220917

# Online,  , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0492 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequencies for performance test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies specified in the Initial conditions section (“Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1”) correspond to a non-zero offset between PointA and the lowest usable subcarrier (offsetToCarrier) setting, which conflicts with the setting specified in the Common Test Parameters tables, which are aligned with settings in TS 38.101-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note in the initial conditions clause to consider the offsetToCarrier=0 setting as defined in the Common Test Parameters tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conflicting frequency setting specifications will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.1_1, 5.2.2.1.2_1, 5.2.2.1.3_1, 5.2.2.1.4_1, 5.2.2.1.5_1, 5.2.2.1.6_1, 5.2.2.1.7_1, 5.2.2.1.8_1, 5.2.2.1.9_1, 5.2.2.1.10_1, 5.2.2.1.11_1, 5.2.2.1.12_1, 5.2.2.1.13_1, 5.2.2.1.14_1, 5.2.2.2.1_1, 5.2.2.2.2_1, 5.2.2.2.3_1, 5.2.2.2.4_1, 5.2.2.2.5_1, 5.2.2.2.6_1, 5.2.2.2.7_1, 5.2.2.2.8_1, 5.2.2.2.9_1, 5.2.2.2.10_1, 5.2.2.2.11_1, 5.2.3.1.1_1, 5.2.3.1.2_1, 5.2.3.1.3_1, 5.2.3.1.5_1, 5.2.3.1.6_1, 5.2.3.1.7_1, 5.2.3.1.9_1, 5.2.3.1.10_1, 5.2.3.1.11_1, 5.2.3.1.12_1, 5.2.3.1.13_1, 5.2.3.1.14_1, 5.2.3.2.1_1, 5.2.3.2.2_1, 5.2.3.2.3_1, 5.2.3.2.5_1, 5.2.3.2.6_1, 5.2.3.2.7_1, 5.2.3.2.9_1, 5.2.3.2.11_1, 5.2A.2.2.1, 5.2A.2.2.1, 5.3.2.1.1, 5.3.2.1.2, 5.3.2.1.3, 5.3.2.2.1, 5.3.2.2.2, 5.3.2.2.3, 5.3.3.1.1, 5.3.3.1.2, 5.3.3.1.3, 5.3.3.2.1, 5.3.3.2.2, 5.3.3.2.3, 5.5.1, 6.2.2.1.1.1, 6.2.2.1.1.2, 6.2.2.1.2.1, 6.2.2.1.2.2, 6.2.2.2.1.1, 6.2.2.2.1.2, 6.2.3.1.1.1, 6.2.3.1.1.2, 6.2.3.2.1.1, 6.2.3.2.1.2, 6.3.2.1.1, 6.3.2.1.2, 6.3.2.1.3, 6.3.2.1.4, 6.3.2.1.5, 6.3.2.1.6, 6.3.2.2.1, 6.3.2.2.2, 6.3.2.2.3, 6.3.2.2.4, 6.3.2.2.5, 6.3.2.2.6, 6.3.3.1.1, 6.3.3.1.2, 6.3.3.1.3, 6.3.3.1.4, 6.3.3.1.5, 6.3.3.1.6, 6.3.3.2.1, 6.3.3.2.2, 6.3.3.2.3, 6.3.3.2.5, 6.3.3.2.6, 6.4.2.1_1, 6.4.2.2_1, 6.4.3.2_1, 7.2.2.2.1_1, 7.2A.2.1, 7.3.2.2.1, 7.3.2.2.2, 7.3.2.2.3, 8.2.2.2.1.1, 8.2.2.2.2.1, 8.3.2.2.1, 8.4.2.2.1, 9.4B.1.1, 9.4B.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

5.2.2.1.1_1	2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's Note: This test cases is incomplete in following aspects:

-	SNR in test requirements table is within square brackets for test point 1-6, 1-7.

5.2.2.1.1_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 1 and Rank 2 scenarios.

5.2.2.1.1_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.1.1_1.3	Test description

5.2.2.1.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.1.2_1	2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.2_1.1	Test purpose

Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH

5.2.2.1.2_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.1.2_1.3	Test description

5.2.2.1.2_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.3_1	2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.3_1.1	Test purpose

To verify PDSCH mapping Type B performance under 2 receive antenna conditions.

5.2.2.1.3_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting PDSCH mapping type B.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and PDSCH mapping type B.

5.2.2.1.3_1.3	Test description

5.2.2.1.3_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.1.4_1	2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.4_1.1	Test purpose

To verify the Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions with CRS rate matching configured.

5.2.2.1.4_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 15 and forward supporting capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test1-1 also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test 1-2 applies to all types of NR UE release 15 and forward supporting capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

Test 1-2 also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

5.2.2.1.4_1.3	Test description

5.2.2.1.4_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.5_1	2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU factor due to finite test time needs further study

5.2.2.1.5_1.1	Test purpose

To verify the PDSCH 0.001% BLER performance under 2 receive antenna conditions.

5.2.2.1.5_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE dl-64QAM-MCS-TableAlt and capability IE cqi-TableAlt.

5.2.2.1.5_1.3	Test description

5.2.2.1.5_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.6_1	2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Statistical significance in Annex G is FFS

-	MU factor due to finite test time needs further study

-	Test Procedure is incomplete

5.2.2.1.6_1.1	Test purpose

To Verify the PDSCH repetitions over multiple slots performance under 2 receive antenna conditions.

5.2.2.1.6_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-RepetitionMultiSlots-r16.

5.2.2.1.6_1.3	Test description

5.2.2.1.6_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.7_1	2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum requirements are within brackets.

5.2.2.1.7_1.1	Test purpose

To verify PDSCH mapping Type B performance and UE processing capability 2 under two receive antenna conditions.

5.2.2.1.7_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-ProcessingType2.

5.2.2.1.7_1.3	Test description

5.2.2.1.7_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.8_1	2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Statistical significance in Annex G is FFS

-	MU factor due to finite test time needs further study

-	Test Procedure is incomplete

5.2.2.1.8_1.1	Test purpose

To Verify the PDSCH pre-emption performance under 2 receive antenna conditions.

5.2.2.1.8_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pre-EmptIndication-DL-r16.

5.2.2.1.6_1.3	Test description

5.2.2.1.6_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

<<Unchaged sections skipped>>

5.2.2.1.9_1	2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.9_1.1	Test purpose

To verify the PDSCH performance under 2 receive antennas conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.1.9_1.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

5.2.2.1.9_1.3	Test description

5.2.2.1.9_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.1.10_1	2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.1.10_1.2	Test applicability

This test applies to all types of NR UE release 16 and forward.

This test also applies to all types of EUTRA UE release 16.

5.2.2.1.10_1.3	Test description

5.2.2.1.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.1.11_1	2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum requirements are within brackets

-	MU and TT are FFS

5.2.2.1.11_1.1	Test purpose

To verify the PDSCH performance with Single-DCI based SDM scheme under 2 receive antenna conditions.

5.2.2.1.11_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE singleDCI-SDM-scheme-r16.

5.2.2.1.11_1.3	Test description

5.2.2.1.11_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.12_1	2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA

5.2.2.1.12_1.1	Test purpose

To verify the PDSCH performance when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and when UE receives multiple PDCCHs scheduling PDSCHs.

5.2.2.1.12_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE multiDCI-MultiTRP-r16.

5.2.2.1.12_1.3	Test description

5.2.2.1.12_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.13_1	2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA

5.2.2.1.13_1.1	Test purpose

To verify the PDSCH performance under 2 receive antenna conditions when UE is configured with “FDMSchemeA” in “RepetitionScheme-r16”.

5.2.2.1.13_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE supportFDM-SchemeA-r16.

5.2.2.1.13_1.3	Test description

5.2.2.1.13_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.1.14_1	2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA

5.2.2.1.14_1.1	Test purpose

To verify the PDSCH performance under 2 receive antenna conditions when UE is configured with repetitionNumber-r16 with multiple slot level PDSCH transmission occasions of the same TB with two TCI states.

5.2.2.1.14_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE supportTDM-SchemeA-r16.

5.2.2.1.14_1.3	Test description

5.2.2.1.14_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.2.1_1	2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.1_1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers

5.2.2.2.1_1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.2.1_1.3	Test Description

5.2.2.2.1_1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of TS 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.2.2_1	2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.2_1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH

5.2.2.2.2_1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.2.2_1.3	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.2_1.3-3, with the addition of test parameters in table 5.2.2.2.2_1.3-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.2.2.2_1.3-1.

Table 5.2.2.2.2_1.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH | 1-1 |

Table 5.2.2.2.2_1.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| NZP CSI-RS for CSI acquisition | OFDM symbols in the PRB used for CSI-RS |  | l0 = 13 |
|  | CSI-RS periodicity | Slots | 5 |
| ZP CSI-RS for CSI acquisition | Subcarrier index in the PRB used for CSI-RS |  | (k0, k1, k2, k3)=(2, 4, 6, 8) |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CSI-RS periodicity | Slots | 5 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.2_1.3-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-7.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 | 14.8 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.2.2.1.2

5.2.2.2.2_1.4	Test Description

5.2.2.2.2_1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 5.1.2.1 and 5.1.2.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.2.2.3_1	2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.3_1.1	Test purpose

To verify the PDSCH mapping Type B normal performance under 2 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput with baseline receiver configuration.

5.2.2.2.3_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting PDSCH mapping type B.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and PDSCH mapping type B.

5.2.2.2.3_1.3	Test description

5.2.2.2.3_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.2.2.4_1	2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.4_1.1	Test purpose

To verify the PDSCH mapping Type A coexistence performance under 2 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput with baseline receiver configuration.

5.2.2.2.4_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 15 and forward supporting capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test 1-1 also applies to all types of E-UTRA UE release 15 and forward supporting EN-DC and capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test 1-2 applies to all types of NR UE release 15 and forward supporting capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

Test 1-2 also applies to all types of E-UTRA UE release 15 and forward supporting EN-DC and capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

5.2.2.2.4_1.3	Test description

5.2.2.2.4_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.2.2.5_1	2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU factor due to finite test time needs further study

5.2.2.2.5_1.1	Test purpose

To verify the PDSCH 0.001% BLER performance under 2 receive antenna conditions.

5.2.2.2.5_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE dl-64QAM-MCS-TableAlt and capability IE cqi-TableAlt.

5.2.2.2.5_1.3	Test description

5.2.2.2.5_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.2.6_1	2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Statistical significance in Annex G is FFS

-	MU factor due to finite test time needs further study

-	Test Procedure is incomplete

5.2.2.2.6_1.1	Test purpose

To Verify the PDSCH repetitions over multiple slots performance under 2 receive antenna conditions.

5.2.2.2.6_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-RepetitionMultiSlots-r16.

5.2.2.2.6_1.3	Test description

5.2.2.2.6_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.2.7_1	2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.7_1.1	Test purpose

To verify PDSCH mapping Type B performance and UE processing capability 2 under two receive antenna conditions.

5.2.2.2.7_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-ProcessingType2.

5.2.2.2.7_1.3	Test description

5.2.2.2.7_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.2.8_1	2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Statistical significance in Annex G is FFS

-	MU factor due to finite test time needs further study

-	Test Procedure is incomplete

5.2.2.2.8_1.1	Test purpose

To Verify the PDSCH pre-emption performance under 2 receive antenna conditions.

5.2.2.2.8_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pre-EmptIndication-DL-r16.

5.2.2.2.6_1.3	Test description

5.2.2.2.6_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.2.2.9_1	2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum test time in annex G .1.5 is missing

5.2.2.2.9_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.2.9_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

5.2.2.2.9_1.3	Test description

5.2.2.2.9_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.2.10_1	2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum test time in Annex G.1.5 is missing

5.2.2.2.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.2.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.2.10_1.3	Test description

5.2.2.2.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.2.2.11_1	2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum requirements are within brackets

-	MU and TT are FFS

5.2.2.2.11_1.1	Test purpose

To verify the PDSCH performance with Single-DCI based SDM scheme under 2 receive antenna conditions.

5.2.2.2.11_1.2	Test applicability

Test applies to all types of NR UE release 16 and forward supporting capability IE singleDCI-SDM-scheme-r16.

5.2.2.2.11_1.3	Test description

5.2.2.2.11_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.1_1	4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor’s note: Minimum test time is FFS for test point 1-5, 1-6, 1-7. SNR in test requirements table is within square brackets for test point 1-6, 1-7.

5.2.3.1.1_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 1 and Rank 2 scenarios.

5.2.3.1.1_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.2.3.1.1_1.3	Test description

5.2.3.1.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.3.1.2_1	4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA

5.2.3.1.2_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration for CSI-RS overlapped with PDSCH scenario.

5.2.3.1.2_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.2.3.1.2_1.3	Test description

5.2.3.1.2_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.3.1.3_1	4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.1.3_1.1	Test purpose

To verify the PDSCH mapping Type B normal performance under 4 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput with baseline receiver configuration.

5.2.3.1.3_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports and PDSCH mapping type B.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports and PDSCH mapping type B.

5.2.3.1.3_1.3	Test description

5.2.3.1.3_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.3.1.5_1	4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU factor due to finite test time needs further study

5.2.3.1.5_1.1	Test purpose

To verify the PDSCH 0.001% BLER performance under 4 receive antenna conditions.

5.2.3.1.5_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE dl-64QAM-MCS-TableAlt and capability IE cqi-TableAlt.

5.2.3.1.5_1.3	Test description

5.2.3.1.5_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.6_1	4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Statistical significance in Annex G is FFS

-	MU factor due to finite test time needs further study

-	Test Procedure is incomplete

5.2.3.1.6_1.1	Test purpose

To Verify the PDSCH repetitions over multiple slots performance under 4 receive antenna conditions.

5.2.3.1.6_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-RepetitionMultiSlots-r16.

5.2.3.1.6_1.3	Test description

5.2.3.1.6_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.7_1	4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum requirements are within brackets.

5.2.3.1.7_1.1	Test purpose

To verify PDSCH mapping Type B performance and UE processing capability 2 under four receive antenna conditions.

5.2.3.1.7_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-ProcessingType2.

5.2.3.1.7_1.3	Test description

5.2.3.1.7_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.11_1	4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum requirements are within brackets

-	MU and TT are FFS

5.2.3.1.11_1.1	Test purpose

To verify the PDSCH performance with Single-DCI based SDM scheme under 4 receive antenna conditions.

5.2.3.1.11_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE singleDCI-SDM-scheme-r16.

5.2.3.1.11_1.3	Test description

5.2.3.1.11_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.12_1	4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA

5.2.3.1.12_1.1	Test purpose

To verify the PDSCH performance when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and when UE receives multiple PDCCHs scheduling PDSCHs.

5.2.3.1.12_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE multiDCI-MultiTRP-r16.

5.2.3.1.12_1.3	Test description

5.2.3.1.12_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.13_1	4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA

5.2.3.1.13_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions when UE is configured with “FDMSchemeA” in “RepetitionScheme-r16”.

5.2.3.1.13_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE supportFDM-SchemeA-r16.

5.2.3.1.13_1.3	Test description

5.2.3.1.13_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.14_1	4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA

5.2.3.1.14_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions when UE is configured with repetitionNumber-r16 with multiple slot level PDSCH transmission occasions of the same TB with two TCI states.

5.2.3.1.14_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE supportTDM-SchemeA-r16.

5.2.3.1.14_1.3	Test description

5.2.3.1.14_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.1.9_1	4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU and TT are FFS

5.2.3.1.9_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.1.9_1.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

5.2.3.1.9_1.3	Test description

5.2.3.1.9_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.3.1.10_1	4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU and TT are FFS

5.2.3.1.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.1.10_1.2	Test applicability

This test applies to all types of NR UE release 16 and forward.

This test also applies to all types of EUTRA UE release 16.

5.2.3.1.10_1.3	Test description

5.2.3.1.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.3.2.1_1	4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.2.1_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 1 and Rank 2 scenarios.

5.2.3.2.1_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward, supporting 4Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4Rx antenna ports.

5.2.3.2.1_1.3	Test description

5.2.3.2.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.3.2.2_1	4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.2.2_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration for CSI-RS overlapped with PDSCH scenario.

5.2.3.2.2_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.2.3.2.2_1.3	Test description

5.2.3.2.2_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.3.2.3_1	4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.2.3_1.1	Test purpose

To verify the PDSCH mapping Type B normal performance under 4 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput with baseline receiver configuration.

5.2.3.2.3_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports and PDSCH mapping type B.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports and PDSCH mapping type B.

5.2.3.2.3_1.3	Test description

5.2.3.2.3_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.2.3.2.5_1	4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU factor due to finite test time needs further study

5.2.3.2.5_1.1	Test purpose

To verify the PDSCH 0.001% BLER performance under 4 receive antenna conditions.

5.2.3.2.5_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE dl-64QAM-MCS-TableAlt and capability IE cqi-TableAlt.

5.2.3.2.5_1.3	Test description

5.2.3.2.5_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.2.6_1	4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Statistical significance in Annex G is FFS

-	MU factor due to finite test time needs further study

-	Test Procedure is incomplete

5.2.3.2.6_1.1	Test purpose

To Verify the PDSCH repetitions over multiple slots performance under 4 receive antenna conditions.

5.2.3.2.6_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-RepetitionMultiSlots-r16.

5.2.3.2.6_1.3	Test description

5.2.3.2.6_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.2.7_1	4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.2.7_1.1	Test purpose

To verify PDSCH mapping Type B performance and UE processing capability 2 under four receive antenna conditions.

5.2.3.2.7_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE pdsch-ProcessingType2.

5.2.3.2.7_1.3	Test description

5.2.3.2.7_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2.3.2.9_1	4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Test tolerance is TBD

-	Minimum test time in Annex G.1.5 is missing

5.2.3.2.9_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.2.9_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

5.2.3.2.9_1.3	Test description

5.2.3.2.9_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.2.3.2.11_1	2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Minimum requirements are within brackets

-	MU and TT are FFS

5.2.3.2.11_1.1	Test purpose

To verify the PDSCH performance with Single-DCI based SDM scheme under 4 receive antenna conditions.

5.2.3.2.11_1.2	Test applicability

Test applies to all types of NR UE release 16 and forward supporting capability IE singleDCI-SDM-scheme-r16.

5.2.3.2.11_1.3	Test description

5.2.3.2.11_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.2A.2.1.1	2Rx Normal PDSCH Demodulation Performance for CA (2DL CA)

Editor’s note: Following aspects in this clause are incomplete

- 	Test point selection criteria is TBD

5.2A.2.1.1.1	Test Purpose

To verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions for multiple CA configurations and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput per CC.

5.2A.2.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA.

5.2A.2.1.1.3	Test description

5.2A.2.1.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

<<Unchaged sections skipped>>

5.2A.2.2.1	2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA)

Editor’s note: Following aspects in this clause are incomplete

-	Connection Diagram is in []

- 	Test point selection criteria is TBD

- 	Message contents is TBD

-	Annex F update is TBD

5.2A.2.2.1.1	Test Purpose

To verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only.

5.2A.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA.

5.2A.2.2.1.3	Test description

5.2A.2.2.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.2-1.

<<Unchaged sections skipped>>

5.3.2.1.1	2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.2.1.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.1.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.1.3-1: Minimum performance for 1 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 2 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 8.2 |
| 3 | 10 MHz | 48 | 2 | 4 | R.PDCCH.1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
| 4 | 10 MHz | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 4.4 |
| 5 | 10 MHz | 48 | 2 | 16 | R.PDCCH.1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.

5.3.2.1.1.4	Test description

5.3.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

5.3.2.1.2	2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.2.1.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.1.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.2.3-1: Minimum performance for 2 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low | 1 | 2.0 |
| 2 | 10 MHz | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low | 1 | -1.3 |
| 3 | 10 MHz | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | -0.2 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.

5.3.2.1.2.4	Test description

5.3.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.3.2.1.3	2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving

Editor's Note: This Test case in incomplete in following aspects:

-	Minimum requirements are still in brackets

5.3.2.1.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting DRX adaptation

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC that supporting DRX adaptation.

5.3.2.1.3.3	Minimum conformance requirements

The parameters specified in Table 5.3.2.1.3.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.2.1.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2I?/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.2.1.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.3.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | [5.5] |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.2.1.3.

5.3.2.1.3.4	Test description

5.3.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.3.2.2.1	2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.2.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.2.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.2.1.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLC300- 100 | 1x2 Low | 1 | -3.8 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.1.4	Test description

5.3.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.2.2.2	2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.2.2.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.2.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.2.2.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH.2-1.3 TDD | TDLC300-100 | 2x2 Low | 1 | -1.2 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.2.4	Test description

5.3.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.2.2.3	2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.2.2.3.1	Test Purpose

This test verifies the demodulation performance of DCI format 2_6 PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Long DRX Cycle and DRX adaptation.

5.3.2.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.2.2.3.3-1 are valid for all TDD tests for power saving unless otherwise stated.

Table 5.3.2.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2I?/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability.. |

For the parameters specified in Table 5.3.2.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.2.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.3.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.3.4	Test description

5.3.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.2.2.3.4.3.1and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.3.1.1	4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.3.1.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.1.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.1.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 2.2 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 2.7 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | -0.4 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -3.2 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.1.

5.3.3.1.1.4	Test description

5.3.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.3.1.2	4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.3.1.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.1.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.2.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -1.9 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -4.5 |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | -1.0 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.1.

5.3.3.1.2.4	Test description

5.3.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.3.1.3	4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.3.1.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting 4 Rx antenna ports and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward that supporting EN-DC, 4 Rx antenna ports and DRX adaptation.

5.3.3.1.3.3	Minimum conformance requirements

The parameters specified in Table 5.3.3.1.3.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.3.1.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2I?/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.3.1.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.3.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.3.1.3.

5.3.3.1.3.4	Test description

5.3.3.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

5.3.3.2.1	4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.3.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.2.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.2.1.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x4 Low | 1 | 2.1 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300-100 | 1x4 Low | 1 | -0.9 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLA30-10 | 1x4 Medium A | 1 | -3.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.1.4	Test description

5.3.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.3.2.2	4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.3.2.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.2.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.2.2.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH.2-1.3 | TDLC300-100 | 2x4 Low | 1 | -4.3 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.2.4	Test description

5.3.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.3.3.2.3	4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.3.2.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

5.3.3.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

For the parameters specified in Table 5.3.3.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.3.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2I?/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

Table 5.3.3.2.3.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | -0.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.3.4	Test description

5.3.3.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.3.2.3.4.3.1 and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

5.5.1	FR1 Sustained downlink data rate performance for single carrier

5.5.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement

5.5.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

5.5.1.3	Minimum conformance requirements

The requirements in this clause are applicable to the FR1 single carrier case.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the channel bandwidth with the downlink physical channel setup according to Annex C.3.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5.1.3-1. The parameters specified in Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in Table 5.5.1.3-3 are applicable for tests on TDD bands.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5.1.3-1: Common test parameters for FDD and TDD bands

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5.1.3-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 30 kHz / 10 MHz and 30 kHz / 20 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For 2Tx: Single Panel Type I, Random precoder chosen from precoder index 0 and 2, selection updated per slot For 4Tx: Single Panel Type I, Random precoder chosen from precoders with i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2}, selection updated per slot |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | WB |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

Table 5.5.1.3-2: Additional test parameters for FDD band

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 4 |
| K1 value |  |  | 2 |

Table 5.5.1.3-3: Additional test parameters for TDD band

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to each UL-DL pattern |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots |  |  |  |

Table 5.5.1.3-4: Number of PRBs in CORESET

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 48 | 78 | 102 | 132 | 156 | 216 | 270 | N/A | N/A | N/A |
| 30 | 6 | 24 | 36 | 48 | 60 | 78 | 102 | 132 | 162 | 216 | 270 |

Table 5.5.1.3-5: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

5.5.1.3.1	Procedure for test parameter selection

Below test parameter selection procedure is from 38.101-4 [5] by replacing CA configuration with operating band, and bandwidth instead of bandwidth combination.

The test parameters are determined by the following procedure:

-	Select one operating band among all supported operating bands and set of per band UE capabilities among all supported UE capabilities that provides the largest data rate [TS 38.306 [14, Section 4.1.2]].

-	Set of per band UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor [TS 38.306 [14, Section 4.1.2]].

-	When there are multiple sets of bandwidths and UE capabilities (channel bandwidth, subcarrier spacing, number of MIMO layer, modulation format, scaling factor) with same largest data rate, select one among sets with the smallest channel bandwidth.

-	For each operating band, use Table 5.5.1.3-5 to determine MCS based on test parameters and indicated UE capabilities

Pasting relevant portion of max data rate equation from TS 38.306 [14] section 4.1

For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink.

is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink.

is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4.

is the numerology (as defined in TS 38.211 [6])

is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

is the maximum RB allocation in bandwidth  with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where  is the UE supported maximum bandwidth in the given band or band combination.

is the overhead and takes the following values

0.14, for frequency range FR1 for DL

0.18, for frequency range FR2 for DL

0.08, for frequency range FR1 for UL

0.10, for frequency range FR2 for UL

NOTE:	Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL.

The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations.

The normative reference for this requirement is TS 38.101-4 [5], clause 5.5.1.

5.5.1.4	Test description

5.5.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of TS 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 5.5.1.3-1.

<<Unchaged sections skipped>>

6.2.2.1.1.1	2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA

6.2.2.1.1.1.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10% falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.1.1.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.2.2.1.1.1.3	Minimum requirement for periodic CQI reporting

For the parameters specified in Table 6.2.2.1.1.1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.1.1.1.3-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| CSI-reportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.1.1.

6.2.2.1.1.1.4	Test Description

6.2.2.1.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

6.2.2.1.1.2	2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.2.1.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.1.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.2.1.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.2.1.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.2.1.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |
| Duplex Mode |  |  |  | FDD |  |
| Subcarrier spacing |  |  | kHz | 15 |  |
| SNR |  |  | dB | 1 | 2 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-1 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.1.2.

6.2.2.1.1.2.4	Test Description

6.2.2.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

6.2.2.1.2.1	2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA

6.2.2.1.2.1.1	Test purpose

To verify the variance of the wideband CQI reports is within the limits defined, that the ratio of the throughput is within the limits defined and that the average PDSCH BLER is greater than or equal to 2% for the indicated transport format.

6.2.2.1.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of E-UTRA UE release 15 and forward supporting EN-DC.

6.2.2.1.2.1.3	Minimum conformance requirements

For the parameters specified in Table 6.2.2.1.2.1.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.2.1.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.2.1.2.1.3-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.1.2.1.3-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| SNR |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  | ULA high |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 |  |  |  |

Table 6.2.2.1.2.1.3-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.2.1.

6.2.2.1.2.1.4	Test description

6.2.2.1.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.2.2.1.2.2	2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA

6.2.2.1.2.2.1	Test purpose

To verify the variance of the wideband CQI reports is within the limits defined, that the ratio of the throughput is within the limits defined and that the average PDSCH BLER is greater than or equal to 2% for the indicated transport format.

6.2.2.1.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of E-UTRA UE release 15 and forward supporting EN-DC.

6.2.2.1.2.2.3	Minimum conformance requirements

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.2.1.2.2.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.2.1.2.2.3-2.

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.2.1.2.2.3-2.

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD.

Table 6.2.2.1.2.2.3-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.45μs |  |  |  |
| Antenna configuration |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report interval and offset |  |  | slot | Not configured |  |  |  |
| Aperiodic Report Slot Offset |  |  |  | 5 |  |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-5 |  |  |  |

Table 6.2.2.1.2.2.3-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | 2 | 2 |
| β [%] | 55 | 55 |
|  | 1.05 | 1.05 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.2.2.

6.2.2.1.2.2.4	Test description

6.2.2.1.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.2.2.2.1.1	2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA

6.2.2.2.1.1.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10% falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.2.1.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.2.2.2.1.1.3	Minimum requirement for periodic CQI reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

For the parameters specified in Table 6.2.2.2.1.1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.2.1.1.3-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| CSI-reportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-4 |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.2.1.1.

6.2.2.2.1.1.4	Test Description

6.2.2.2.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.2.2.2.1.2	2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.2.2.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.2.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.2.2.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.2.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.2.2.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |
| SNR |  |  | dB | 1 | 2 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-2 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.2.1.2.

6.2.2.2.1.2.4	Test Description

6.2.2.2.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

6.2.3.1.1.1	4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA

6.2.3.1.1.1.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10% falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.1.1.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.2.3.1.1.1.3	Minimum requirement for periodic CQI reporting

For the parameters specified in Table 6.2.3.1.1.1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.3.1.1.1.3-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| SNR |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.1.1.1.

6.2.3.1.1.1.4	Test Description

6.2.3.1.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

6.2.3.1.1.2	4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.3.1.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.1.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.3.1.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.3.1.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.3.1.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |
| Subcarrier spacing |  |  | kHz | 15 |  |
| Duplex Mode |  |  |  | FDD |  |
| SNR |  |  | dB | -2 | -1 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-1 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.1.1.2.

6.2.3.1.1.2.4	Test Description

6.2.3.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

6.2.3.2.1.1	4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA

6.2.3.2.1.1.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10% falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.2.1.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.2.3.2.1.1.3	Minimum requirement for periodic CQI reporting

For the parameters specified in Table 6.2.3.2.1.1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.3.2.1.1.3-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-4 |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.2.1.1.

6.2.3.2.1.1.4	Test Description

6.2.3.2.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.2.3.2.1.2	4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.3.2.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.2.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.3.2.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.3.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.3.2.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |
| SNR |  |  | dB | -2 | -1 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-2 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.2.1.2.

6.2.3.2.1.2.4	Test Description

6.2.3.2.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

<<Unchaged sections skipped>>

6.3.2.1.1	2Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA

6.3.2.1.1.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.1.1.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.1-2.

Table 6.3.2.1.1.3-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset |  | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.2.1.1.4	Test description

6.3.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of  PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.1.2	2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA

6.3.2.1.2.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.1.2.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.2.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.2.3-2.

Table 6.3.2.1.2.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the eNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.2.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.3.2.1.2.4	Test description

6.3.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of  PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.1.3	2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.1.3.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.1.3.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.3.3-2.

Table 6.3.2.1.3.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLC300-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.3.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 2.5 |

6.3.2.1.3.4	Test description

6.3.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.1.4	2Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.1.4.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.1.4.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.4.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.4.3-2.

Table 6.3.2.1.4.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 32 x 2 (N1,N2) = (4,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, 12) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.4.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 5.0 |

6.3.2.1.4.4	Test description

6.3.2.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.1.5	2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.1.5.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.1.5.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.5.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.5.3-2.

Table 6.3.2.1.5.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII |
|  | L (numberOfBeams) |  | 2 |
|  | NPSK (phaseAlphabetSize) |  | 8 |
|  | subbandAmplitude |  | True |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction) |  | 10 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.2.1.5.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.9 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.2.1.5.

6.3.2.1.5.4	Test description

6.3.2.1.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.1.6	2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.1.6.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.6.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

6.3.2.1.6.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.6.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.6.3-2.

Table 6.3.2.1.6.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.2.1.6.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 2.2 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.2.1.6.

6.3.2.1.6.4	Test description

6.3.2.1.6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.2.1	2Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA

6.3.2.2.1.1	Test Purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.2.1.3	Minimum Conformance Requirements

For the parameters specified in Table 6.3.2.2.1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.1.3-2.

Table 6.3.2.2.1.3-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 106 |
|  | Subcarrier spacing | kHz | 30 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM, lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified inAnnex B.2.3.2.3. |  |  |  |

Table 6.3.2.2.1.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.2.2.1.4	Test Description

6.3.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of TS 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.2.2	2Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA

6.3.2.2.2.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.2.2.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.2.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.2.3-2.

Table 6.3.2.2.2.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 106 |
|  | Subcarrier spacing | kHz | 30 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-6)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+6)]. Note 3: Randomization of the principle beam direction shall be used as specified inAnnex B.2.3.2.3. |  |  |  |

Table 6.3.2.2.2.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.3.2.2.2.4	Test description

6.3.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.2.3	2Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.2.3.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC.

6.3.2.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.2.3.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.3.3-2.

Table 6.3.2.2.3.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLC300-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.2.3.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 2.5 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.2.2.3.

6.3.2.2.3.4	Test description

6.3.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.2.4	2Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 32 Tx is missing

6.3.2.2.4.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC.

6.3.2.2.4.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.2.4.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.4.3-2.

Table 6.3.2.2.4.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 32 x 2 (N1,N2) = (4,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, 12) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.2.4.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 5.0 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.2.2.4.

6.3.2.2.4.4	Test description

6.3.2.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.2.5	2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.2.5.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.2.5.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.2.5.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.5.3-2.

Table 6.3.2.2.5.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII |
|  | L (numberOfBeams) |  | 2 |
|  | NPSK (phaseAlphabetSize) |  | 8 |
|  | subbandAmplitude |  | True |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction) |  | 10 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.2.2.5.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.9 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.2.2.5.

6.3.2.2.5.4	Test description

6.3.2.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.2.2.6	2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.2.2.6.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.6.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

6.3.2.2.6.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.2.6.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.6.3-2.

Table 6.3.2.2.6.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.2.2.6.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 2.2 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.2.2.6.

6.3.2.2.6.4	Test description

6.3.2.2.6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.1.1	4Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA

6.3.3.1.1.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.1.1.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.1-2.

Table 6.3.3.1.1.3-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.1.1.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.3.1.1.

6.3.3.1.1.4	Test description

6.3.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.1.2	4Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA

6.3.3.1.2.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.1.2.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.2.3-2.

Table 6.3.3.1.2.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3 |  |  |  |

Table 6.3.3.1.2.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.3.1.2.

6.3.3.1.2.4	Test description

6.3.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.1.3	4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.1.3.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.1.3.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.1.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.3.3-2.

Table 6.3.3.1.3.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLC300-5 |
| Antenna configuration |  |  | High XP 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.1.3.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 3.0 |

6.3.3.1.3.4	Test description

6.3.3.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.1.4	4Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.1.4.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.1.4.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.1.4.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.4.3-2.

Table 6.3.3.1.4.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 32 x 4 (N1,N2) = (4,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, 12) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.1.4.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 7.0 |

6.3.3.1.4.4	Test description

6.3.3.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.1.5	4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.1.5.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.1.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.1.5.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.1.5.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.5.3-2.

Table 6.3.3.1.5.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII |
|  | L (numberOfBeams) |  | 2 |
|  | NPSK (phaseAlphabetSize) |  | 8 |
|  | subbandAmplitude |  | True |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction) |  | 10 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.3.1.5.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.9 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.3.1.5.

6.3.3.1.5.4	Test description

6.3.3.1.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.1.6	4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.1.6.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.1.6.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

6.3.3.1.6.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.1.6.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.6.3-2.

Table 6.3.3.1.6.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.3.1.6.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 2.2 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.3.1.6.

6.3.3.1.6.4	Test description

6.3.3.1.6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.2.1	4Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA

6.3.3.2.1.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.2.1.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.2.1.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.1.3-2.

Table 6.3.3.2.1.3-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset |  | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3 |  |  |  |

Table 6.3.3.2.1.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.3.2.1.4	Test description

6.3.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.2.2	4Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA

6.3.3.2.2.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.2.2.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.2.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.2.3-2.

Table 6.3.3.2.2.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-6)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+6)]. Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3 |  |  |  |

Table 6.3.3.2.2.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.3.3.2.2.4	Test description

6.3.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.2.3	4Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.2.3.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and 4 Rx antenna ports.

6.3.3.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.2.3.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.3.3-2.

Table 6.3.3.2.3.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLC300-5 |
| Antenna configuration |  |  | High XP 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.2.3.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 3.0 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.3.2.3.

6.3.3.2.3.4	Test description

6.3.3.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.2.5	4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.2.5.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.2.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.3.2.5.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.2.5.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.5.3-2.

Table 6.3.3.2.5.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII |
|  | L (numberOfBeams) |  | 2 |
|  | NPSK (phaseAlphabetSize) |  | 8 |
|  | subbandAmplitude |  | True |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction) |  | 10 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.3.2.5.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.8 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.3.2.5.

6.3.3.2.5.4	Test description

6.3.3.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.3.3.2.6	4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Connection figure for 16 Tx is missing

6.3.3.2.6.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.3.2.6.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Enhanced Type II codebook with at least 16 ports per CSI-RS resource.

6.3.3.2.6.3	Minimum conformance requirements

For the parameters specified in Table 6.3.3.2.6.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.6.3-2.

Table 6.3.3.2.6.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  | 0010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Table 6.3.3.2.6.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 2.2 |

The normative reference for this requirement is TS 38.101-4 [5], clause 6.3.3.2.6.

6.3.3.2.6.4	Test description

6.3.3.2.6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.4.2.1_1	2Rx FDD FR1 RI reporting for both SA and NSA

6.4.2.1_1.1	Test Purpose

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

6.4.2.1_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.4.2.1_1.3	Minimum Conformance Requirements

The minimum performance requirement in Table 6.4.2.1_1.3-2 is defined as:

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.2.1_1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.2.1_1.3-2.

Table 6.4.2.1_1.3-1: RI Test (FDD)

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 10 | 10 |
| Subcarrier spacing |  |  | kHz | 15 | 15 | 15 |
| Duplex Mode |  |  |  | FDD | FDD | FDD |
| SNR |  |  | dB | 0 | 20 | 20 |
| Propagation channel |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  | ULA Low 2x2 | ULA Low 2x2 | ULA High 2x2 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 5/1 | 5/1 |
| ReportConfigType |  |  |  | Periodic | Periodic | Periodic |
| CQI-table |  |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 5/0 | 5/0 |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8 | 8 | 8 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |

Table 6.4.2.1_1.3-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1.0 | N/A | N/A |

The normative reference for this requirement is TS 38.101-4 [2] clause 6.4.2.1.

6.4.2.1_1.4	Test Description

6.4.2.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex D.

Test Environment: Normal as defined in TS 38.508 [6] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 38.508 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.4.2.2_1	2Rx TDD FR1 RI reporting for both SA and NSA

6.4.2.2_1.1	Test Purpose

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

6.4.2.2_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.4.2.2_1.3	Minimum Conformance Requirements

The minimum performance requirement in Table 6.4.2.2_1.3-2 is defined as:

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.2.2_1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.2.2_1.3-2.

Table 6.4.2.2_1.3-1: RI Test (TDD)

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 | 40 | 40 |
| Subcarrier spacing |  |  | kHz | 30 | 30 | 30 |
| Duplex Mode |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| SNR |  |  | dB | 0 | 20 | 20 |
| Propagation channel |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  | ULA Low 2x2 | ULA Low 2x2 | ULA High 2x2 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  | slot | 10/1 | 10/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 | 10/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 | 10/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic | Periodic | Periodic |
| CQI-table |  |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  | RB | 16 | 16 | 16 |
| csi-ReportingBand |  |  |  | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 10/9 | 10/9 | 10/9 |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 9.5 | 9.5 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |

Table 6.4.2.2_1.3-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1.0 | N/A | N/A |

The normative reference for this requirement is TS 38.101-4 [2] clause 6.4.2.2.

6.4.2.2_1.4	Test Description

6.4.2.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex D.

Test Environment: Normal as defined in TS 38.508 [6] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 38.508 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.4.3.1_1	4Rx FDD FR1 RI reporting for both SA and NSA

6.4.3.1_1.1	Test Purpose

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

6.4.3.1_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

6.4.3.1_1.3	Minimum Conformance Requirements

The minimum performance requirement in Table 6.4.3.1_1.3-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.3.1_1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.3.1_1.3-2.

Table 6.4.3.1_1.3-1: RI Test (FDD)

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 10 | 10 | 10 |
| Subcarrier spacing |  |  | kHz | 15 | 15 | 15 | 15 |
| Duplex Mode |  |  |  | FDD | FDD | FDD | FDD |
| SNR |  |  | dB | -2 | 16 | 16 | 22 |
| Propagation channel |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  | ULA Low 2x4 | ULA Low 2x4 | ULA High 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (9,-) | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| ReportConfigType |  |  |  | Periodic | Periodic | Periodic | Periodic |
| CQI-table |  |  |  | Table 2 | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  | 1111111 | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 5/0 | 5/0 | 5/0 |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 11111111 |
|  |  | RI Restriction |  | N/A | N/A | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8 | 8 | 8 | 8 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 | 1 |
| RI Configuration |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 2 and follow RI |

Table 6.4.3.1_1.3-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 0.9 | N/A | N/A | 0.9 |

The normative reference for this requirement is TS 38.101-4 [2] clause 6.4.3.1.

6.4.3.1_1.4	Test Description

6.4.3.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal as defined in TS 38.508 [6] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 38.508 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

6.4.3.2_1	4Rx TDD FR1 RI reporting for both SA and NSA

6.4.3.2_1.1	Test Purpose

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

6.4.3.2_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

6.4.3.2_1.3	Minimum Conformance Requirements

The minimum performance requirement in Table 6.4.3.2_1.3-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.3.2_1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.3.2_1.3-2.

Table 6.4.3.2_1.3-1: RI Test (TDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 | 40 | 40 | 40 |
| Duplex Mode |  |  |  |  | TDD | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  |  | FR1.30-1 | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| DL BWP configuration #1 |  |  | First PRB |  | 0 | 0 | 0 | 0 |
|  |  |  | Number of contiguous PRB |  | 106 | 106 | 106 | 106 |
|  |  |  | Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| SNR |  |  |  | dB | -2 | 16 | 16 | 22 |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x4 | ULA Low 2x4 | ULA High 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | 16 | 16 | 16 | 16 |
| csi-ReportingBand |  |  |  |  | 1111111 | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  |  | slot | 10/9 | 10/9 | 10/9 | 10/9 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 11111111 |
|  |  | RI Restriction |  |  | N/A | N/A | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 | 9.5 | 9.5 | 9.5 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 2 and follow RI |

Table 6.4.3.2_1.3-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 0.9 | N/A | N/A | 0.9 |

The normative reference for this requirement is TS 38.101-4 [2] clause 6.4.3.2.

6.4.3.2_1.4	Test Description

6.4.3.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1[7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 6.1.2-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

<<Unchaged sections skipped>>

7.2.2.2.1_1	2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA

7.2.2.2.1_1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance with different channel models, MCSs and number of MIMO layers.

7.2.2.2.1_1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

7.2.2.2.1_1.3	Test Description

7.2.2.2.1_1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 7.2-1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

<<Unchaged sections skipped>>

7.2A.2.1	2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA)

Editor's Note: Following aspects in this clause are incomplete.

-	TT analysis pending

7.2A.2.1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance with different channel models, MCSs and number of MIMO layers.

7.2A.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support NR 2DL CA.

7.2A.2.1.3	Test Description

7.2A.2.1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-2.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 7.2-1.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

<<Unchaged sections skipped>>

7.3.2.2.1	2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA

7.3.2.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH for a single-antenna port with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 7.3.2.2.1.3-1.

7.3.2.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

7.3.2.2.1.3	Minimum conformance requirements

For the parameters specified in Table 7.3.2.2.1.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.1.3-2. The downlink physical setup is in accordance with Annex C.2.2.

Table 7.3.2.2.1.3-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Table 7.3.2.2.1.3-2: Minimum performance requirements with 120 kHz SCS for 1Tx antenna

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | 100 MHz | 60 | 1 | 2 CCE | R.PDCCH.5-1.1 TDD | TDLA30-75 | 1x2 Low | 1 | 6.0 |
| 1-2 | 100 MHz | 60 | 1 | 4 CCE | R.PDCCH.5-1.2 TDD | TDLA30-300 | 1x2 Low | 1 | 2.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.3.2.2.1.

7.3.2.2.1.4	Test Description

7.3.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 7.3-1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

<<Unchaged sections skipped>>

7.3.2.2.2	2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA

7.3.2.2.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH for two-antenna port with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 7.3.2.2.2.3-1.

7.3.2.2.2.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

7.3.2.2.2.3	Minimum conformance requirements

For the parameters specified in Table 7.3.2.2.2.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.2.3-2. The downlink physical setup is in accordance with Annex C.2.2.

Table 7.3.2.2.2.3-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Table 7.3.2.2.2.3-2: Minimum performance requirements with 120 kHz SCS for 2Tx Antenna

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 2-1 | 100 MHz | 60 | 1 | 8 CCE | R.PDCCH.5-1.3 TDD | TDLA30-75 | 2x2 Low | 1 | 1.4 |
| 2-2 | 100 MHz | 60 | 2 | 16 CCE | R.PDCCH.5-2.1 TDD | TDLA30-75 | 2x2 Low | 1 | -1.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.3.2.2.2.

7.3.2.2.2.4	Test Description

7.3.2.2.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 7.3-1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

<<Unchaged sections skipped>>

7.3.2.2.3	2Rx TDD FR2 PDCCH 1 Tx antenna performance for power saving

7.3.2.2.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 7.3.2.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

7.3.2.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Long DRX Cycle and DRX adaptation.

7.3.2.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 7.3.2.2.3.3-1 are valid for normal PDCCH in DRX on period and PDCCH in DRX off period.

Table 7.3.2.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR2.120-1 |
| CCE to REG mapping type |  |  | Interleaved |
| REG bundle size |  |  | 6 |
| Interleaver size |  |  | 2 |
| Shift index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2I?/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 7.3.2.2.3.3-2, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 7.3.2.2.3.3-2. The downlink physical setup is in accordance with Annex C.5.1.

Table 7.3.2.2.3.3-2: Minimum performance requirements with 120 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 3-1 | 100 | 60 | 1 | 4 | R.PDCCH. 5-1.2 TDD | TDLA30-300 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | 8 | R.PDCCH. 5-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.3.2.2.3.

7.3.2.2.3.4	Test description

7.3.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of DRX, DCP, PDCCH before measurement are specified in 7.3.2.2.3.4.3.1 and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 7.3-1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

<<Unchaged sections skipped>>

8.2.2.2.1.1	2Rx TDD FR2 periodic wideband CQI reporting under AWGN performance for both SA and NSA

8.2.2.2.1.1.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10% falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

8.2.2.2.1.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

8.2.2.2.1.1.3	Minimum requirement for periodic CQI reporting

The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 8.2.2.2.1.1.3-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified by the following:

a)	the reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time;

b)	if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 8.2.2.2.1.1.3-1 Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-2 |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 8.2.2.2.1.1.

8.2.2.2.1.1.4	Test Description

8.2.2.2.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 8.1.2-1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

<<Unchaged sections skipped>>

8.2.2.2.2.1	2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA

8.2.2.2.2.1.1	Test Purpose

To verify the variance of the wideband CQI reports is within the limits defined, that the ratio of the throughput is within the limits defined and that the average PDSCH BLER is greater than or equal to 1% for the indicated transport format.

8.2.2.2.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

8.2.2.2.2.1.3	Minimum requirement for aperiodic CQI reporting

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI. To account for sensitivity of the input SNR the CQI reporting under frequency non-selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 8.2.2.2.2.1.3-1 and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified by the following:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α % of the time, where α% is specified in Table 8.2.2.2.2.1.3-2;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 8.2.2.2.2.1.3-2;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.01.

Table 8.2.2.2.2.1.3-1: Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |
| SNRBB |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  | TDLA30-35 |  |  |  |
| Antenna configuration |  |  |  | 2×2 ULA High |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS interval and offset |  | slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig interval and offset |  | slot | Not configured |  |  |  |
|  | aperiodicTriggeringOffset |  |  | 0 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured |  |  |  |
| ReportConfigType |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | Not configured |  |  |  |
| Aperiodic Report Slot Offset |  |  |  | 6 |  |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |  |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-1 |  |  |  |

Table 8.2.2.2.2.1.3-2 Minimum requirements

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 2 | 2 |
|  | 1.05 | 1.05 |

The normative reference for this requirement is TS 38.101-4 [5] clause 8.2.2.2.2.1.

8.2.2.2.2.1.4	Test Description

8.2.2.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 8.1.2-1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

<<Unchaged sections skipped>>

8.3.2.2.1	2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA

8.3.2.2.1.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

8.3.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

8.3.2.2.1.3	Minimum conformance requirements

For the parameters specified in Table 8.3.2.2.1.3-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.3.2.2.1.3-2.

Table 8.3.2.2.1.3-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 | 100 |
| Subcarrier spacing |  | kHz | 120 | 120 |
| TDD DL-UL configuration |  |  | FR2.120-2 as specified in Annex A.1.3 | FR2.120-1 as specified in Annex A.1.3 |
| Propagation channel |  |  | TDLA30-35 | TDLA30-35 |
| Antenna configuration |  |  | 2 x 2 ULA Low | 2 x 2 ULA Low |
| Beamforming Model |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | 8/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3, (6,-) | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| csi-ReportingBand |  |  | 111111111 | 111111111 |
| CSI-Report interval and offset |  | slot | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 6 | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A |
|  | CodebookSubsetRestriction |  | 001111 | 001111 |
|  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 1.375 | 1.75 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Measurement channel |  |  | R.PDSCH.5-8.1 TDD | R.PDSCH.5-7.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4)], this reported PMI cannot be applied at the gNB downlink before slot#(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 8.3.2.2.1.3-2: Minimum requirement

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.05 | 1.05 |

8.3.2.2.1.4	Test description

8.3.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 8.1.2-1.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

<<Unchaged sections skipped>>

8.4.2.2.1	2Rx TDD FR2 RI reporting for both SA and NSA

-8.4.2.2.1.1	Test Purpose

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

8.4.2.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

8.4.2.2.1.3	Minimum requirement

The minimum performance requirement in Table 8.4.2.2.1.3-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 8.4.2.2.1.3-1, and using the downlink physical channels specified in Annex C.2.2, the minimum requirements are specified in Table 8.4.2.2.1.3-2.

Table 8.4.2.2.1.3-1: RI Test (TDD)

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 | 100 | 100 |
| Subcarrier spacing |  |  | kHz | 120 | 120 | 120 |
| Duplex Mode |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 | FR2.120-2 | FR2.120-2 |
| SNR |  |  | dB | 0 | 16 | 16 |
| Propagation channel |  |  |  | TDLA30-35 | TDLA30-35 | TDLA30-35 |
| Antenna configuration |  |  |  | ULA Low 2x2 | ULA Low 2x2 | XP High 2x2 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 4, (8,-) | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | CSI-RS interval and offset |  | slot | 8/1 | 8/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig interval and offset |  | slot | Not configured | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  |  | 0 | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 1 | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8,13) | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured | Not configured | Not configured |
| ReportConfigType |  |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  |  | Table 1 | Table 1 | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  | 111111111 | 111111111] | 111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 6 | 6 | 6 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 1.375 | 1.375 | 1.375 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |
| Note 1: Measurements channels are specified in Table A.4-1. TBS.1-1 is used for Rank 1 case. TBS.1-2 is used for Rank 2 case. |  |  |  |  |  |  |

Table 8.4.2.2.1.3-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 1.05 |
| 2 | 1.0 | N/A | N/A |

The normative reference for this requirement is TS 38.101-4 [5] clause 8.4.2.2.

8.4.2.2.1.4	Test Description

8.4.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2, adjusted such that offsetToCarrier is equal to 0, as defined in Table 8.1.2-1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

<<Unchaged sections skipped>>

9.4B.1.1	Sustained downlink data rate performance for EN-DC within FR1

9.4B.1.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement

9.4B.1.1.2	Test Applicability

This test applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

9.4B.1.1.3	Minimum conformance requirements

During the test, the PDSCH performance on both the NR cell(s) and LTE cell(s) shall be verified.

The TB success rate shall be higher than 85% when NR PDSCH is scheduled with MCS defined for the selected EN-DC bandwidth combination and with the downlink physical channel setup according to Annex C.3.1 and when E-UTRA PDSCH is scheduled with FRC defined for the selected EN-DC bandwidth combination and with the downlink physical channel setup according to Annex C.3.2 from TS 36.101 [X].

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters for NR cell are specified in Table 9.4B.1.1.3-1. The parameters specified in Table 9.4B.1.1.3-2 are applicable for tests on FDD NR cell and parameters specified in Table 9.4B.1.1.3-3 are applicable for tests on TDD NR cell.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz for NR cell.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz for NR cell.

Table 9.4B.1.1.3-1: Common test parameters for FDD or TDD NR band

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 9.4B.1.1.3-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 30 kHz / 10 MHz and 30 kHz / 20 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For 2Tx: Single Panel Type I, Random precoder chosen from precoder index 0 and 2, selection updated per slot For 4Tx: Single Panel Type I, Random precoder chosen from precoders with i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2}, selection updated per slot |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | WB |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

Table 9.4B.1.1.3-2: Additional test parameters for NR FDD band

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 4 |
| K1 value |  |  | 2 |

Table 9.4B.1.1.3-3: Additional test parameters for NR TDD band

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to each UL-DL pattern |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots |  |  |  |

Table 9.4B.1.1.3-4: Number of PRBs in CORESET for NR cell

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 48 | 78 | 102 | 132 | 156 | 216 | 270 | N/A | N/A | N/A |
| 30 | 6 | 24 | 36 | 48 | 60 | 78 | 102 | 132 | 162 | 216 | 270 |

Table 9.4B.1.1.3-5: MCS indexes for indicated UE capabilities for NR cell

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |

Table 9.4B.1.1.3-6: Additional test setup for E-UTRA CC

| Parameter | Unit | Value |
| --- | --- | --- |
| Inter-TTI Distance |  | 1 |
| Number of OFDM symbols for PDCCH per component carrier | OFDM symbols | 1 |
| Cross carrier scheduling |  | Not configured |
| Propagation condition |  | Static propagation condition No external noise sources are applied |
| at antenna port | dBm/15kHz | -85 |
| Antenna configuration | 2 layer CC | 2x2 or 2x4 |
|  | 4 layer CC | 4x4 |
| Codebook subset restriction | 2 layer CC | 10 |
|  | 4 layer CC | 1000 |
| Downlink power allocation | 2 layer CC | = -3dB, = -3dB, = 0dB |
|  | 4 layer CC | = -6dB, = -6dB, = 3dB |

Table 9.4B.1.1.3-7: E-UTRA FRC for SDR test (FDD)

| MIMO layer | Bandwidth | Reference channel |  |  |
| --- | --- | --- | --- | --- |
|  |  | 64QAM | 256QAM | 1024QAM |
| 2 layer | 5 | R.PDSCH.4-1.1 FDD | R.PDSCH.4-3.1 FDD | R.PDSCH.4-5.1 FDD |
|  | 10 | R.PDSCH.4-1.2 FDD | R.PDSCH.4-3.2 FDD | R.PDSCH.4-5.2 FDD |
|  | 15 | R.PDSCH.4-1.3 FDD | R.PDSCH.4-3.3 FDD | R.PDSCH.4-5.3 FDD |
|  | 20 | R.PDSCH.4-1.4 FDD | R.PDSCH.4-3.4 FDD | R.PDSCH.4-5.4 FDD |
| 4 layer | 5 | R.PDSCH.4-2.1 FDD | R.PDSCH.4-4.1 FDD | R.PDSCH.4-6.1 FDD |
|  | 10 | R.PDSCH.4-2.2 FDD | R.PDSCH.4-4.2 FDD | R.PDSCH.4-6.2 FDD |
|  | 15 | R.PDSCH.4-2.3 FDD | R.PDSCH.4-4.3 FDD | R.PDSCH.4-6.3 FDD |
|  | 20 | R.PDSCH.4-2.4 FDD | R.PDSCH.4-4.4 FDD | R.PDSCH.4-6.4 FDD |

Table 9.4B.1.1.3-8: E-UTRA FRC for SDR test (TDD)

| MIMO layer | Bandwidth | Reference channel |  |  |
| --- | --- | --- | --- | --- |
|  |  | 64QAM | 256QAM | 1024QAM |
| 2 layer | 10 | R.PDSCH.6-1.1 TDD | R.PDSCH.6-3.1 TDD | R.PDSCH.6-5.1 TDD |
|  | 15 | R.PDSCH.6-1.2 TDD | R.PDSCH.6-3.2 TDD | R.PDSCH.6-5.2 TDD |
|  | 20 | R.PDSCH.6-1.3 TDD | R.PDSCH.6-3.3 TDD | R.PDSCH.6-5.3 TDD |
| 4 layer | 10 | R.PDSCH.6-2.1 TDD | R.PDSCH.6-4.1 TDD | R.PDSCH.6-6.1 TDD |
|  | 15 | R.PDSCH.6-2.2 TDD | R.PDSCH.6-4.2 TDD | R.PDSCH.6-6.2 TDD |
|  | 20 | R.PDSCH.6-2.3 TDD | R.PDSCH.6-4.3 TDD | R.PDSCH.6-6.3 TDD |

9.4B.1.1.3.1	Procedure for test parameter selection

The test parameters are determined by the following procedure:

-	Select one EN-DC bandwidth combination among all supported EN-DC configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate [TS 38.306 [14, Section 4.1.2]].

-	Set of per NR CC UE capabilities include channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor TS 38.306 [14] Section 4.1.2]].

-	Set of per E-UTRA CC UE capabilities includes channel bandwidth, number of PDSCH MIMO layers and modulation format [TS 38.306 [14] Section 4.1.2]].

-	When there are multiple sets of EN-DC bandwidth combinations and UE capabilities with same largest data rate, select one among sets with the smallest aggregated channel bandwidth.

-	For each NR FR1 CC in EN-DC bandwidth combination, use Table 9.4B.1.1.3-5 to determine MCS based on test parameters and indicated UE capabilities.

-	For each E-UTRA CC in EN-DC bandwidth combination, use Table 9.4B.1.1.3-7 and Table 9.4B.1.1.3-8 to determine FRC based on test parameters and indicated UE capabilities.

Pasting relevant portion of max data rate equation from TS 38.306 [14] section 4.1

For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink.

is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink.

is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4.

is the numerology (as defined in TS 38.211 [6])

is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

is the maximum RB allocation in bandwidth  with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where  is the UE supported maximum bandwidth in the given band or band combination.

is the overhead and takes the following values

0.14, for frequency range FR1 for DL

0.18, for frequency range FR2 for DL

0.08, for frequency range FR1 for UL

0.10, for frequency range FR2 for UL

NOTE:	Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL.

For EUTRA in case of MR-DC, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

Data rate (in Mbps) =

wherein

J is the number of aggregated EUTRA component carriers in MR-DC band combination

is the total maximum number of DL-SCH transport block bits received within a 1ms TTI for j-th CC, as derived from TS36.213 [22] based on the UE supported maximum MIMO layers for the j-th carrier, and based on the modulation order and number of PRBs based on the bandwidth of the j-th carrier.

The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations.

For MR-DC, the approximate maximum data rate is computed as the sum of the approximate maximum data rates from NR and EUTRA

The normative reference for this requirement is TS 38.101-4 [5], clause 9.4B.1.1.

9.4B.1.1.4	Test description

9.4B.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR and E-UTRA operating bands specified in Table 5.3.5-1 of TS 38.521-1.

Configurations of NR PDSCH and NR PDCCH before measurement are specified in Annex C.

E-UTRA configurations before measurement are specified in at Table 9.4B.1.1.3-6.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 9.4B.1.1.3-1.

<<Unchaged sections skipped>>

9.4B.1.2	Sustained downlink data rate performance for EN-DC including FR2 NR carrier

Editor's Note: MU analysis is complete for up to 100 MHz ChBW.

9.4B.1.2.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement.

9.4B.1.2.2	Test Applicability

This test applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

9.4B.1.2.3	Minimum conformance requirements

The test setup for E-UTRA Pcell is specified in Clause 9.1.2 and Table 9.4B.1.1.1-1. During the test, only the PDSCH performance on the NR cell(s) on FR2 carriers is verified.

The TB success rate shall be higher than 85% when NR PDSCH is scheduled with MCS defined for the selected EN-DC bandwidth combination and with the downlink physical channel setup according to Annex C.2.2.

The TB success rate of delivered PDCP SDU(s) by Layer2 is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and DL_correct_rx is the number of correctly received DL transport blocks. All the above numbers of transmitted, retransmitted or correctly received DL transport blocks are calculated as the sum of the numbers of DL transport blocks per CG used for DC.

The test parameters are specified in Tables 9.4B.1.2.3-1, 9.4B.1.2.3-2.

Unless otherwise stated, no user data is scheduled on slot #0, 40 and 41 within 20 ms for SCS 60 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 80 and 81 within 20 ms for SCS 120 kHz.

Table 9.4B.1.2.3-1: Test parameters for FR2 TDD

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| PTRS epre-Ratio |  |  |  | 0 |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 60 or 120 |
| DL BWP configuration #1 | RB Offset |  |  | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [3] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 60 or 120 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 7.5A.1-2 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1-1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH &PDCCH DMRS Precoding configuration |  |  | Single Panel Type I, Random per slot with equal probability of precoder index 0 and 2, and with REG bundling granularity for number of Tx larger than 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | WB |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | 13 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 |
| PTRS configuration | Frequency density (KPT-RS) |  |  | 2 |
|  | Time density (LPT-RS) |  |  | 1 |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 60 kHz SCS: 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 13 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| TCI state #0 | Tyoe 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Tyoe 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Tyoe 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Tyoe 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Number of HARQ Processes |  |  |  | 10 for FR2.60-1 and 8 for FR2.120-1 |
| K1 value |  |  |  | Specific to each UL-DL pattern |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| TDD UL-DL pattern |  |  |  | 60 kHz SCS: FR2.60-1 120 kHz SCS: FR2.120-1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity for Rank 2 |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: PDSCH is scheduled only on full DL slots not containing SSB or TRS. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 3: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [3] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

Table 9.4B.1.2.3-2: Number of PRBs in CORESET

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 30 | 66 | 132 | 264 |

Table 9.4B.1.2.3-3: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |

Table 9.4B.1.2.3-4: SNR required to achieve 85% of peak throughput under AWGN conditions

| MCS Index (Note 1) | SNRBB(dB) for maximum number of PDSCH MIMO Layers = 1 | SNRBB(dB) for maximum number of PDSCH MIMO Layers = 2 |
| --- | --- | --- |
| 13 | 6.2 | 9.0 |
| 14 | 7.2 | 9.9 |
| 15 | 8.2 | 10.9 |
| 16 | 8.7 | 11.6 |
| 17 | 10.1 | 13.2 |
| 18 | 10.7 | 13.7 |
| 19 | 11.7 | 14.7 |
| 20 | 12.7 | 15.6 |
| 21 | 13.6 | 16.5 |
| 22 | 14.8 | 17.6 |
| 23 | 15.6 | 18.6 |
| 24 | 16.9 | 19.7 |
| 25 | 18.3 | 21.2 |
| 26 | 19.3 | 22.3 |
| 27 | 20.5 | 23.3 |
| Note 1: MCS Index is based on MCS Table defined in clause 5.1.3 of TS 38.214 [12] when 256QAM is not enabled. |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 9.4B.1.2.

9.4B.1.2.3.1	Procedure for test parameter selection

The test parameters are determined by the following procedure:

-	Step 1: Calculate the NR FR2 data rate for EN-DC bandwidth combinations, using a procedure from Clause 7.5A, for all supported EN-DC configurations and set of per NR component carrier (CC) UE capabilities among all supported UE capabilities:

-	Set of per NR CC UE capabilities includes a channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor as defined in clause 4.1.2 of TS 38.306 [14].

-	Step 2: Calculate the E-UTRA data rate for EN-DC bandwidth combinations, using a procedure from clause 4.1.2 of TS 38.306 [14], for all supported EN-DC configurations and set of per E-UTRA component carrier (CC) UE capabilities among all supported UE capabilities:

-	Set of per E-UTRA CC UE capabilities includes a channel bandwidth, number of PDSCH MIMO layers and modulation format as defined in clause 4.1.2 of TS 38.306 [14].

-	Step 3: Select the EN-DC bandwidth combination among all supported EN-DC configurations that achieves maximum total data rate in steps 1 and 2 among all UE capabilities:

-	When there are multiple sets of EN-DC bandwidth combinations and UE capabilities with the same largest data rate, select a single set with the smallest aggregated channel bandwidth.

-	Step 4: For each NR FR2 CC in the selected EN-DC bandwidth combination, use MCS determined in step 1 for that EN-DC bandwidth combination based on test parameters and indicated UE capabilities.

Pasting relevant portion of max data rate equation from TS 38.306 [14] section 4.1

For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink.

is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink.

is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4.

is the numerology (as defined in TS 38.211 [6])

is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

is the maximum RB allocation in bandwidth  with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where  is the UE supported maximum bandwidth in the given band or band combination.

is the overhead and takes the following values

0.14, for frequency range FR1 for DL

0.18, for frequency range FR2 for DL

0.08, for frequency range FR1 for UL

0.10, for frequency range FR2 for UL

NOTE:	Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL.

For EUTRA in case of MR-DC, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

Data rate (in Mbps) =

wherein

J is the number of aggregated EUTRA component carriers in MR-DC band combination

is the total maximum number of DL-SCH transport block bits received within a 1ms TTI for j-th CC, as derived from TS36.213 [22] based on the UE supported maximum MIMO layers for the j-th carrier, and based on the modulation order and number of PRBs based on the bandwidth of the j-th carrier.

The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations.

For MR-DC, the approximate maximum data rate is computed as the sum of the approximate maximum data rates from NR and EUTRA

9.4B.1.2.4	Test description

9.4B.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1, adjusted such that offsetToCarrier is equal to 0, as defined in Table 9.4B.1.1.3-1.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

<<End of change>>