3GPP TSG-RAN5 Meeting #94-e	R5-220927

# Online,  , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1668 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Core Spec Alignment with TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update cl. 6.3.2.3.1.4.2 Test procedure to include description that Cell 1 and Cell 2 belong to different tracking areas. Update info for 6.3.2.3.1 in Table E.4-1: Cell configuration mapping for SA FR1 RRM testing: - 38.533 NR Cell2: NR Cell 3 NR Cell 23 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be correctly performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.1, E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependent CR: R4-2203572 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.3.2.3.1	NR SA FR1 RRC connection release with redirection

6.3.2.3.1.1	Test purpose

This test is to verify RRC connection release with redirection from NR to NR.

6.3.2.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] A.6.3.2.3.1.

6.3.2.3.1.4	Test description

6.3.2.3.1.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.3.1.4.1-1.

Table 6.3.2.3.1.4.1-1: Redirection from NR to NR test configurations

| Configuration | Description |
| --- | --- |
| 6.3.2.3.1-1 | Source cell: NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.3.2.3.1-2 | Source cell: NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.3.2.3.1-3 | Source cell: NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.3.1.4.1-2.

Table 6.3.2.3.1.4.1-2: Initial conditions for Redirection from NR to NR test case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameters for PCell and neighbour cell are given in Table 6.3.2.3.1.4.1-3 below.

2. Message contents are defined in clause 6.3.2.3.1.4.3.

3. There are two carriers and two NR cells specified in the test. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 6.3.2.3.1.4.1-3: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

6.3.2.3.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The “RRCRelease” message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2. Cell 1 and Cell 2 belong to different tracking areas.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.2.3.1.5-1. T1 starts.

3.	SS shall transmit an RRCRelease during period T1.

4.	The SS shall switch the power setting from T1 to T2 as specified in Table 6.3.2.3.1.5-1. When the last TTI containing the RRCRelease message is sent to UE, T2 starts.

5.	If the UE transmits the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	After T2 expires, the UE shall be switched off. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and Cell 2 shall be powered OFF.

7.	The SS shall set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

8.	Repeat step 2-7 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

6.3.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.3.2.3.1.4.3-1: RRCRelease for NR RRC redirection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |  |
| nr SEQUENCE { |  |  |  |  |
| carrierFreq |  | ARFCN-ValueNR | Frequency of Cell 2 |  |
| ssbSubcarrierSpacing |  | kHz15 |  | Config 1,2 |
|  |  | kHz30 |  | Config 3 |
| smtc SEQUENCE { |  |  |  |  |
| duration |  | sf1 |  | SMTC.1 |
|  |  | sf5 |  | SMTC.2 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.3.2.3.1.4.3-2: SIB2

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |

6.3.2.3.1.5	Test requirement

Table 6.3.2.3.1.5-1 defines the primary level settings for Redirection from NR to NR test case.

Table 6.3.2.3.1.5-1: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR 2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR 2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -64.59 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 ms and is specified in clause 12 in TS 38.331 [13].

Tidentify-NR = 680 ms in the test.

TSI-NR = 1280 ms is assumed, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 170 ms in the test.

This gives a total of 2240 ms.

<<Unchanged sections skipped>>

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for SA FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 23 |  |  |  |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 23 |  |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 23 |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.2 | SA event triggered reporting tests without gap under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 |  |  |  |  |  |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.3.2 | NR SA FR1-FR2 SS-RSRP relative measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |

<<End of change>>