3GPP TSG-RAN WG5 Meeting #94-e	R5-220983

# Online, , 21st February 2022 – 4th March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0304 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to applicability of test case 6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | On the similar lines of agreed discussion paper R5-206260, equivalent NR test case 6.4 is also impacted when UE is made to de-register from IMS services after step-23. When test case is executed on a voice centric device and IMS services are deregistered, UE may disable its “N1 mode capability for 3GPP access” due to “IMS Voice being unavailable over 3GPP access” as specified in TS 24.501. Table 4.3.4.1: Change of IMS voice availability for a UE in single-registration mode Change of IMS voice available condition; Procedure to execute. "IMS voice not available" over 3GPP access only and the UE's usage setting is "voice centric"; Disable the N1 mode capability for 3GPP access, if the UE is only registered over 3GPP access (see subclause 4.9.2). (NOTE 2). As N1 mode is disabled, subsequent steps (i.e., after step-29) can not be performed. |
| Summary of change: |  | Test case 6.4 is made applicable for data centric devices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case when run in voice centric mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of modified section>

## 4.2	Applicabilities for test cases specified in TS 34.229-5

Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title | Release | Applicability | Comments |
| --- | --- | --- | --- | --- |
| Registration |  |  |  |  |
| 6.1 | Initial Registration / 5GS | Rel-15 | C01 | UE supports IMS security and NR (see Note 1 below) |
| 6.2 | Initial Registration Failures / 5GS | Rel-15 | C01 | UE supports IMS security and NR |
| 6.3 | Re-Registration Scenarios / 5GS | Rel-15 | C01 | UE supports IMS security and NR |
| 6.4 | De-Registration Scenarios / 5GS | Rel-15 | C13 | UE supports IMS security and NR and IMS de-registration and UE's usage setting as data centric |

## <End of modifed section>

## <Start of modified section>

| Conditions/Options |  |  |
| --- | --- | --- |
| C01 | IF A.6a/2 AND A.18/5 THEN R ELSE N/A | IMS security and NR |
| … |  |  |
| C13 | IF A.6a/2 AND A.18/5 AND A.12/39 AND [93] A.4.4-2/10 THEN R ELSE N/A | IMS security and NR and IMS de-registration and UE's usage setting as data centric |

## <End of modifed section>