3GPP TSG-RAN WG5 Meeting #94-e	R5-221007

# Online, , 21st February 2022 - 4th March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0330 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new IMS over 5GS TC 11.7 eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fulfilling the work plan by introducing new test case 11.7 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 11.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR EIEI Workplan not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0305 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-4: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 4: Enabler for IP multimedia applications testing".

[6]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[7]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[9]	ETSI ES 201 873-2: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 2: TTCN-3 Tabular Presentation Format (TFT)".

[10]	ETSI TR 201 873-3: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 3: TTCN-3 Graphical Presentation Format (GFT)".

[11]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[12]	ISO/IEC 9646-1: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[13]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[14]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[15]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[16]	3GPP TS 33.203: "3G security;Access security for IP-based services".

[17]	IETF RFC 3329: "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[18]	IETF RFC 3680: "A Session Initiation Protocol (SIP) Event Package for Registrations".

[19]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[21]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[22]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[23]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[24]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[25]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[26]	3GPP TS 24.237: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity".

[27]	3GPP TS 23.003: "Numbering, addressing and identification".

[28]	IETF RFC 6665: “SIP-Specific Event Notification”.

[29]	IETF RFC 3312: “Integration of Resource Management and SIP”.

[30]	IETF RFC 3262: “Reliability of Provisional Responses in the Session Initiation Protocol (SIP)”.

[31]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[32]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem".

[33]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[34]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem".

[35]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem".

[36]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem".

[37]	IETF RFC 4028: "Session Timers in the Session Initiation Protocol (SIP)".

[38]	IETF RFC 4566: "SDP: Session Description Protocol".

[39]	IETF RFC 7462: "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP) ".

[40]	IETF RFC 3891: "The Session Initiation Protocol (SIP) "Replaces" Header".

[41]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[42]	IETF RFC 3326: "The Reason Header Field for the Session Initiation Protocol (SIP)".

[43]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[44]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[45]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[46]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[47]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[48]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[49]	IETF RFC 6228: "Session Initiation Protocol (SIP) Response Code for Indication of Terminated Dialog".

[50]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[51]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[52]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Common Test Environments for User Equipment (UE) Conformance Testing".

<End of modifed section>

<Start of modified section>

11.7	eCall only mode / Manual initiation / Emergency Registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS

11.7.1	Test Purpose (TP)

(1)

with { UE is in state 5GMM-DEREGISTERED.eCALL-INACTIVE }

ensure that {

when { UE is being made to initiate a manual eCall over IMS }

then { UE performs IMS and emergency registration and sends correctly composed INVITE with MSD }

}

(2)

with { UE having sent an  INVITE }

ensure that {

when { UE receives 486 Busy Here }

then { UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates ecall in CS domain }

}

11.7.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229 clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

11.7.3	Test description

11.7.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell 1 connected to 5GC, default parameters.

-	if px_NR_RATComb_Tested = NR_UTRA, UTRA cell 5 as defined in TS 36.508 [52].

-	if px_NR_RATComb_Tested = NR_GERAN, GERAN cell 24 as defined in TS 36.508 [52].

-	NR Cell 1 power level is set as “serving cell”

-	Cell 24/Cell 5 power level is set as “suitable cell” as defined in TS 36.508 [52].

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [21] Table 6.4.1-24.

Preamble:

-	UE is in state 5GMM-DEREGISTERED.eCALL-INACTIVE.

11.7.3.2	Test procedure sequence

Table 11.7.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start a manual eCall | - | - | - | - |
| 2-19 | Steps 2 to 19 of generic procedure specified in Table 4.5.2.2-2 of 38.508-1 [21] takes place. | - | - | - | - |
| 20-22 | Steps 8-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 23 and 24, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 23 | Step 1 of Annex A.23 happens Check: Does the UE include MSD in the message body? | --> | INVITE | 1 | P |
| 24 | TE sends 600 Busy Everywhere response | <-- | 486 Busy Here | - | - |
| - | EXCEPTION: Steps 25a1-25b5 takes place depending on the value of px_NR_RATComb_Tested | - | - | - | - |
| 25a1 | IF px_NR_RATComb_Tested = NR_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration. | - | - | 2 | P |
| 25a2 | SS configures NR Cell 1 as a “non-suitable cell” | - | - | - | - |
| 25a3 | eCall is maintained for 5 seconds | - | - | - | - |
| - | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex A.12 can take place | - | - | - | - |
| - | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex A.11 can take place | - | - | - | - |
| 25a4 | eCall is cleared by SS | - | - | - | - |
| 25b1 | IF px_NR_RATComb_Tested = NR_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain | - | - | 2 | P |
| 25b2 | SS configures NR Cell 1 as a “non-suitable cell” | - | - | - | - |
| 25b3 | eCall is maintained for 5 seconds | - | - | - | - |
| 25b2 | eCall is cleared by SS | - | - | - | - |
| 25b5 | UE performs MM/GMM registration | - | - | - | - |

11.7.3.3	Specific message contents

Table 11.7.3.3-1: INVITE (step 23, table 11.7.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A20

Table 11.7.3.3-2: 486 Busy Here (step 24, table 11.7.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.21 with Condition A1

Table 11.7.3.3-4: EMERGENCY SETUP (step 25a1 or 25b1, table 11.7.3.2-1)

| Information Element | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Service Category IE |  | bit 6 = 1 and all other bits are set to 0. |  |  |

Table 11.7.3.3-3: ROUTING AREA UPDATE ACCEPT (step 25b5, table 11.7.3.2-1)

| Derivation path: TS 36.508, Table 4.7B.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Cond | Value/remark | Rel | Reference |  |
| PDP Context Status |  | 0 NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |  |  |

<End of modifed section>