3GPP TSG-RAN WG5 Meeting # 94-e	R5-221049

# Electronic meeting, February 21 - March 4, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2267 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 4.3.1.1.5.48 for test frequencies of CA_n48(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test frequencies for NR intra-band non-congiguous CA configuration CA_n48(2A) are not corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the test frequencies for CA_n48(2A) in Table 4.3.1.1.5.48-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequencies for CA_n48(2A) are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.5.48 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

4.3.1.1.5.48	CA_n48(2A)

Editor’s note:	Test frequencies for CA_n48(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.48-1: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination |  | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-50MHz) |  |  |  |  |  |  |  |
| 10+10 |  | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  |  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 10+15 |  | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  |  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 10+20 |  | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  |  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 10+40 |  | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  |  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 10+50 |  | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  |  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-50MHz) |  |  |  |  |  |  |  |
| 15+10 |  | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  |  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 15+15 |  | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  |  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 15+20 |  | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  |  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 15+40 |  | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  |  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 15+50 |  | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  |  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-50MHz) |  |  |  |  |  |  |  |
| 20+10 |  | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  |  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 20+15 |  | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  |  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 20+20 |  | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  |  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 20+40 |  | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  |  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 20+50 |  | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  |  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-50MHz) |  |  |  |  |  |  |  |
| 40+10 |  | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  |  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 40+15 |  | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  |  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 40+20 |  | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  |  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 40+40 |  | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  |  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 40+50 |  | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  |  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-50MHz) |  |  |  |  |  |  |  |
| 50+10 |  | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  |  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 50+15 |  | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  |  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 50+20 |  | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  |  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 50+40 |  | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  |  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 50+50 |  | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  |  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| Note 1: CA_n48(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |  |

Table 4.3.1.1.5.48-2: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination |  | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-100MHz) |  |  |  |  |  |  |  |
| 10+10 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 10+15 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 10+20 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 10+40 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 10+50 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 10+60 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 10+80 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 10+90 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 10+100 |  | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  |  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-100MHz) |  |  |  |  |  |  |  |
| 15+10 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 15+15 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 15+20 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 15+40 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 15+50 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 15+60 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 15+80 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 15+90 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 15+100 |  | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  |  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-100MHz) |  |  |  |  |  |  |  |
| 20+10 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 20+15 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 20+20 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 20+40 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 20+50 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 20+60 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 20+80 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 20+90 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 20+100 |  | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  |  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-100MHz) |  |  |  |  |  |  |  |
| 40+10 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 40+15 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 40+20 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 40+40 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 40+50 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 40+60 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 40+80 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 40+90 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 40+100 |  | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  |  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-90MHz) |  |  |  |  |  |  |  |
| 50+10 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 50+15 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 50+20 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 50+40 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 50+50 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 50+60 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 50+80 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 50+90 |  | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  |  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| CA_n48(2A); n48A (60MHz) + n48A (10-80MHz) |  |  |  |  |  |  |  |
| 60+10 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 60+15 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 60+20 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 60+40 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 60+50 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 60+60 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 60+80 |  | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  |  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| CA_n48(2A); n48A (80MHz) + n48A (10-60MHz) |  |  |  |  |  |  |  |
| 80+10 |  | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 80+15 |  | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 80+20 |  | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 80+40 |  | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 80+50 |  | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 80+60 |  | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  |  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| CA_n48(2A); n48A (90MHz) + n48A (10-50MHz) |  |  |  |  |  |  |  |
| 90+10 |  | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 90+15 |  | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 90+20 |  | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 90+40 |  | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 90+50 |  | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  |  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| CA_n48(2A); n48A (100MHz) + n48A (10-40MHz) |  |  |  |  |  |  |  |
| 100+10 |  | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  |  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 100+15 |  | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  |  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 100+20 |  | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  |  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 100+40 |  | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  |  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| Note 1: CA_n48(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>