3GPP TSG-RAN5 Meeting #94-e	R5-221114

# Electronic Meeting, 21st Feb – 04th Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1578 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating A-MPR for CA testing for CA_n41A-n79A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 6.2A.3.1 is incomplete for CA_n41A-n79A and it need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add test configuration table for NS_47 in Initial conditions. 2. Updated tables in test procedure. 3. Added NS_47 in message contents. 4. Added NS_47 for CA_n41A-n79A in test requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some test procedure cannot be correctly executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0562 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-221115 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2A.3.1.4.1-1: Test Configuration Table (network signalling value NS_47/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range for PCC and SCC PCC specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  | Lowest,Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 | N/A for A-MPR testing. | Default | 30 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 |  | Default | 30 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | Default | 30 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.4.1-2: Test Configuration Table (network signalling value NS_100/NS_01)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest NRB_agg for PCC and SCC Highest NRB_agg for PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  | Lowest,Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |
|  |  | Modulation |  |  | RB allocation |  |
|  |  | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Inner Full |
| 2 |  | CP-OFDM 256QAM | CP-OFDM 256QAM |  | Outer Full | Outer Full |
| 3 |  | DFT-s-OFDM QPSK | CP-OFDM 256QAM |  | Inner Full | Outer Full |

Table 6.2A.3.1.4.1-3: Test Configuration Table (network signalling value NS_43/NS_01, NS_43U/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 910 | 10MHz | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.3.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.3.1.4.3.

6.2A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.3.1.4.1-1, Table 6.2A.3.1.4.1-2, Table 6.2A.3.1.4.1-3 as appropriate on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level corresponding to Power Class 3.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2A.3.1.4.1-1, Table 6.2A.3.1.4.1-2, Table 6.2A.3.1.4.1 send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

6.2A.3.1.4.3.1	Message contents exceptions (network signalling value " NS_100" on PCC)

Table 6.2A.3.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2A.3.1.4.3.2	Message contents exceptions (network signalling value " NS_43" on PCC)

Table 6.2A.3.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_43) |  |  |

6.2A.3.1.4.3.3	Message contents exceptions (network signalling value " NS_43U" on PCC)

Table 6.2A.3.1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (NS_43U) |  |  |

6.2A.3.1.4.3.4	Message contents exceptions (network signalling value "NS_47" on PCC)

Table 6.2A.3.1.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance (Additional Maximum Power Reduction (A-MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78 and CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20+TT |
| 2 | 23 | 6.5 | 6.5 | 7 | 0 | 0 | 0 | 3 | 9.8 | 7 | 25+TT | 2.8+TT |
| 3 | 23 | 6.5 | 0 | 7 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14+TT |

For the UE which supports inter-band NR CA configuration, ΔTIB,c in Table 6.2A.4.0.2.3-1 applies. Unless otherwise stated, ΔTIB,c is set to zero.

<End of Changes>